3GPP TSG-RAN WG4 Meeting # 109															R4-2321795

Chicago, USA, November 13 – November 17, 2023

Agenda item:			8.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR38.846 of Guidelines on Co-existence analysis for triple beat

Document for:	Discussion and Approval

# Introduction

This is a TP to TR 38.846 to add the guidelines for analysis of triple beat, that may cause self-interference. It is based on the discussion paper [1] where also the proposal to add this analysis was presented.

TP for TR 38.846

**************** Start of TP ***************

## 6.5	Guidelines on Co-Existence analysis

### 6.5.2	Uplink Intra-Band CA with two UL CC transmissions

*************************** Unchanged CLAUSES Omitted ***************************

### 6.5.3	Uplink triple beat

When adding a band combination including three uplink transmission - one with UL intra-band carrier aggregation, which makes two tones and a third in the form of a single uplink component carrier this study is needed regardless if the intra-band CA is non-contiguous or contiguous intra-band uplink CA.

Table 6.5.2-1: Band nX and Band nY triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | - | - | - | - |  | . |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | - | - | - | - |  | - |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | - | - | - | - |  | - |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | - | - | - | - |  |  |

If any issues are identified via the calculations presented in Table 6.5.2-1 additional REFSENS requirements may be needed.

In the Table 6.5.2-1 the following abbreviations is used:

fU1L = minimum frequency of TX aggressor band of ULCC1 lower band range (i.e. Minimum frequency edge of the band)

fU2L = minimum frequency of TX aggressor band of ULCC2 lower band range (i.e. Minimum frequency edge of the band)

fU3L = maximum frequency of TX aggressor band of ULCC2 lower band range (i.e. Minimum frequency edge of the band + 			Maximum Instantaneous UL BW)

fU1H = maximum frequency of TX aggressor band of ULCC1 higher band range (i.e. Maximum frequency edge of the band)

fU2H = minimum frequency of TX aggressor band of ULCC2 higher band range

fU3H = maximum frequency of TX aggressor band of ULCC2 higher band range (i.e. Minimum frequency edge of the band - 			Maximum Instantaneous UL BW)

fSCCL = minimum frequency in single CC band

fSCCH = maximum frequency in single CC band

Channel BW = Channel bandwidth of the component carrier
Minimum channel separation = Minimum frequency separation between the two component carriers or the inter CC GB
Maximum channel separation = Maximum frequency separation between the two CCs or aggregated uplink BW

Figure 6.5.2-1: Co-existence studies triple beat

For two-band inter-band CA or DC combinations, the precondition is that:

- The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 6.5.2-2.

For three-band inter-band CA or DC combinations and triple beat in third band, the precondition is that:

- The 3rd DL band belongs to the same band group or belongs to a band group which is adjacent to either one of the UL bands, where band groups are defined in Table 6.5.2-2.

For the case when the victim band may be affected by a 1st order triple-beat product, proponents should systematically check if the downlink band may be affected by dual uplink IMD3 interference. If the test point is missing, a dual UL IMD3 MSD test point should be specified.

If the triple beat frequency is composed of the frequency sum of the 2 discrete RBs in the contiguous UL CA, there is no need to specify the TB test configuration as the requirement can already be verified by the fallback 2UL IMD3. The generic guidelines can be found in clause 7.4 for type 3 UL configurations.

  - Table 6.5.2-2: Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

**************** End of TP ***************

- References

[1] R4-2304941 Discussion of triple beat rules for MSD analysis, WG4 Meeting #106-bis-e