3GPP TSG-RAN WG4 Meeting#109      	R4-2321796

Chicago Meeting, November 13th – November 17th, 2023

Agenda item:			8.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.846 Addition of Guidelines on Harmonic mixing MSD requirements

Document for:	Approval

- Introduction

As discussed in [1] RAN4 is in need of generic rules across any and all architectures that support the radio system. This TP adds a common ruleset for band analysis for band combinations which needs this type of analysis to the TR for band combination simplification.

- TP

***************************** Start of changes ************************************

## 6.5	Guidelines on Co-Existence analysis

### 6.5.1	Uplink Intra-Band CA with one UL CC transmissions

For harmonic mixing analysis where the UL of either band can directly or in the form of uplink harmonic interfere the DL receive band of the other band in the combination the following rules applies for the study of the combinations besides the frequency range checks. The rules are split into those of PC3 in Table 6.5.1-1 and those for PC2 or PC1.5 in Table 6.5.1-2. The cells in grey do not require harmonic mixing analysis. For certain UL/DL combinations an additional check on the DL frequency of the impacted receive band must be made.

Table 6.5.1-1: PC3 and PC5 harmonic mixing rules of analysis applicability.

| PC3 and PC5 of UL band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | UL1 | UL2 | UL3 | UL4 |  |
| DL2 | TBD | N/A | DL > 3GHz | N/A |  |
| DL3 | All | All | N/A | TBD |  |
| DL4 | TBD | N/A | DL > [3 or 5]GHz | N/A |
| DL5 | All | TBD | N/A | N/A |

Table 6.5.1-2: PC2 and PC1.5 harmonic mixing rules of analysis applicability.

| PC2 and PC1.5 of UL band |  |  |  |
| --- | --- | --- | --- |
|  | UL1 | UL2 | UL3 |  |  | UL4 |  |
| DL2 | TBD | N/A | All |  |  | N/A |  |
| DL3 | All | All | N/A |  |  | TBD |  |
| DL4 | All | N/A | DL > [3]GHz |  | N/A |  |  |
| DL5 | All | TBD | N/A |  | N/A |  |

The TBD’s in tables 6.5.1-1 and 6.5.1-2 shall be further evaluated based on companies technical analysis during Rel-18.

### 6.5.2	Uplink Intra-Band CA with two UL CC transmissions

When adding a band combination including two uplink transmission in one UL Intra-Band Carrier Aggregation this study is needed for both non-contiguous and contiguous intra-band uplink CA.

There are 5 cases to consider where IMDs of the two CCs of an intra-band UL CA may cause MSD issues:

–	FDD band with DL and UL contiguous intra-band CA (CA_nXXB/ C UL and DL) => IMDs of intra band CA can cause MSD up to 7th order, only odd orders should be an issue.

–	FDD band with DL and UL non-contiguous intra-band CA CA_nXX(2A) UL and DL) => IMDs of intra band CA can cause MSD up to 7th order, only odd orders should be an issue.

–	Two band simultaneous Rx/Tx combinations with 1 band UL with contiguous intra-band CA => IMDs of intra band CA can cause MSD up to 9th order, only odd order should be an issue as 2nd and 4th order are limited to 400MHz and higher even orders are typically low.

–	Two band simultaneous Rx/Tx combinations with 1 band UL with non-contiguous intra-band CA => IMDs of intra band CA can cause MSD up to 7th order, 2nd order can be up to 600MHz and should not be an issue while 4th order can be an issue for low bands as it can reach up to 1200Mhz and higher even orders are typically low.

–	Two band simultaneous Rx/Tx combinations with 2 band UL with contiguous intra-band CA in one of the bands => IMDs of intra band CA can cause MSD up to 7th order, 2nd order can be up to 600MHz and should not be an issue while 4th order can be an issue for low bands as it can reach up to 1200Mhz and higher even orders are typically low.

–	The assumption that out of even order IMDs, only 4th order IMD is an issue, is based on the fact that in R18:

○    Maximum UL aggregated BW for contiguous intra-band is 200MHz.

○    Maximum bandwidth separation class is 600MHz.

–	Two band simultaneous Rx/Tx combinations with 2 band UL with non-contiguous intra-band CA in one of the bands is not considered because it would require 3 non-contiguous UL clusters which is not allowed in R18.

Based on the above, an IMD calculation table can be made generic for all above cases as shown in Table 6.5.2-1.

Table 6.5.2-1: Co-existence studies for uplink configurations including intra-band UL CA

| Configuration | Channel BW | Minimum Channel separation | Maximum Instantaneous UL BW | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data |  |  |  |  |  | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency |  |  |  |  |  |  |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H | - | - |
| Interference ranges |  |  |  |  | - | - |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges |  |  |  |  | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges |  |  |  |  |  |  |
| 5th | I 3*fU1L-2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges |  |  |  |  |  |  |
| 6th | I 3*fU1L-3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fUH1-fU3H |
| Interference ranges |  |  |  |  |  |  |
| 7th | I 4*fU1L-3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges |  |  |  |  |  |  |
| 9th | I 5*fU1L-4*fU3L I | I 5*fU1H-4*fU3H I | 6*fU1L-3*fU3L | 6*fU1H-3*fU3H | 7*fU1L-2*fU3L | 7*fU1H-2*fU3H |
| Interference ranges |  |  |  |  |  |  |

If any issues are identified via the calculations presented in Table 6.5.2-1 additional REFSENS requirements may be needed.

In the Table 6.5.2-1 the following abbreviations is used:

fU1L = 	minimum frequency of TX aggressor band of ULCC1 lower band range (i.e. Minimum frequency edge of the band)

fU2L = 	minimum frequency of TX aggressor band of ULCC2 lower band range (i.e. Minimum frequency edge of the band)

fU3L = 	maximum frequency of TX aggressor band of ULCC2 lower band range (i.e. Minimum frequency edge of the band + 			Maximum Instantaneous UL BW)

fU1H = 	maximum frequency of TX aggressor band of ULCC1 higher band range (i.e. Maximum frequency edge of the band)

fU2H = 	minimum frequency of TX aggressor band of ULCC2 higher band range

fU3H = 	maximum frequency of TX aggressor band of ULCC2 higher band range (i.e. Minimum frequency edge of the band - 			Maximum Instantaneous UL BW)

fD1L = 	minimum frequency of RX victim band of DLCC placed on the lower frequency side of the TX aggressor band

fD1H = 	maximum frequency of RX victim band of DLCC placed on the lower frequency side of the TX aggressor band

fD2L = 	minimum frequency of RX victim band of DLCC placed on the higher frequency side of the TX aggressor band

fD2H = 	maximum frequency of RX victim band of DLCC placed on the higher frequency side of the TX aggressor band

Channel BW = 	Channel bandwidth of the component carrier.
						- Equal to minimum UL CBW for non-contiguous UL CA
						- Equal to the maximum UL CBW combination that fits the minimum between the maximum aggregated 						  CBW for the BCS and the band bandwidth for contiguous UL CA

Minimum channel separation =	Minimum frequency separation between the two component carriers or the inter CC GB
										- Equal to 0 for contiguous intra-band UL CA
										- Equal to minimum CBW for non-contiguous ULCA

Maximum channel separation = 	Maximum frequency separation between the two component carriers or aggregated uplink 									BW
										- For contiguous intra-band UL CA it is equal to either the aggregated bandwidth or total 										   bandwidth of the band, whatever is the smallest.
										- For non-contiguous intra-band UL CA it is equal to either the bandwidth separation 											   class bandwidth or total bandwidth of the band, whatever is the smallest.

Figure 6.5.2-1: Co-existence studies for Uplink Intra-Band Non-Contiguous CA

***************************** End of changes ************************************

## References

[1] R4-2320025, Discussion on rules of Harmonic mixing MSD requirements, Nokia, Nokia Shanghai Bell