3GPP TSG-RAN WG4 Meeting # 109	R4-2321813

Chicago, USA, November 13 – November 17, 2023

Title: 	WF on IoT_NTN_extLband

Agenda Item:	9.3.6

Source: 	Moderator (Inmarsat)

Document for:	Approval

# Topic #1: UE RF Requirements

Issue 1-2: REFSENS

Agreement:

  - For the extended L-band, the reference sensitivity level for eMTC NTN can be defined as Table 2.2-1 and Table 2.2-2 and the reference sensitivity level for NB-IoT can be defined as Table 2.2-3.

    - The REFSENS equation will be captured in the TR

- Table 2.2-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -102.7 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

- Table 2.2-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -103.5 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

- Table 2.2-3: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B in TS 36.102 | - 108.2 |

Issue 1-2-1: In-Band Blocking Requirements for Cat M1 + Issue 1-2-2: In-Band Blocking Requirements for Cat NB1 NB2

Agreement:

- Put the action on capturing the additional blocking requirements from EN 301 681 and ECC Report 263 on hold until further information from ETSI, considering the UE implementation difficulty and the ongoing discussion in ETSI.

  - Send LS to ETSI to show the discrepancy and applicability to device types

  - Add the note in the TS and TR that the requirements would be defined after the feedback of ETSI is received.

Issue 1-2-2: Out-of-Band Blocking Requirements for Cat M1 + Issue 1-2-2: In-Band Blocking Requirements for Cat NB1 NB2

Agreement:

- Do not define the additional out-of-band blocking requirements in Rel-18

  - Further discuss it, if needed, in the future release in TEI or WI