3GPP TSG-RAN WG4 Meeting #109 														    R4-2321818

# Chicago, US, November 13 – 17, 2023

Agenda item:			8.30.2.1.2 Tx requirements

Source:					LG Electronics

Title:	TP to TR 38.786 Updated PSFCH MPR and A-MPR simulation results

Document for:	Approval

# Introduction

This text proposal updates the LG Electronics PSFCH MPR and A-MPR simulation results in TR 38.786 [2] following the alignment in simulation assumptions during the TSG-RAN WG4 Meeting #109 [1].

# Text proposal

<<<<<<<<<<< Start of changes in section 6 >>>>>>>>>>

##### 6.1.2.1.3	MPR for PSFCH transmission

##### 6.1.2.1.3.1	LG Electronics’ simulation results (R4-2315542 and R4-2321771)

For PSFCH MPR/A-MPR, the following simulation scenarios of Table 6.1.2.1.3.1-1 are considered based on Alt 1-1b.

Table 6.1.2.1.3.1-1: SL-U PSFCH MPR/A-MPR simulation scenarios

| Sub-band RB sets | Scenario | Bitmap | PSFCH RB location index | SCS(kHz) |
| --- | --- | --- | --- | --- |
| 1 (20MHz) | 1 | 1 | {0 10 20 30 40 50 60 70 80 90 100 104} | 15 |
|  | 2 | 1 | {0 5 10 15 20 25 30 35 40 45 49} | 30 |
| 2 (40MHz) | 3 | 11 | {0 10 20 30 40 50 60 70 80 90 100 104 111 121 131 141 151 161 171 181 191 201 211 215} | 15 |
|  | 4 | 10 | {0 10 20 30 40 50 60 70 80 90 100 104} | 15 |
| 3 (60MHz) | 5 | 111 | {0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 6 | 110 | {0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 7 | 100 | {0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 8 | 010 | {56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 9 | 101 | {0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161} | 30 |
| 4 (80MHz) | 10 | 1111 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 11 | 1110 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 12 | 1100 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 13 | 1000 | { 0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 14 | 0110 | { 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 15 | 0100 | { 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 16 | 1101 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 17 | 1010 | { 0 5 10 15 20 25 30 35 40 45 49 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 18 | 1001 | { 0 5 10 15 20 25 30 35 40 45 49 167 172 177 182 187 192 197 202 207 212 216} | 30 |
| 5 (100MHz) | 19 | 11111 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 20 | 11110 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 21 | 11100 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 22 | 11000 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 23 | 10000 | { 0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 24 | 01110 | { 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 25 | 01100 | { 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 26 | 01000 | { 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 27 | 00100 | { 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 28 | 11011 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 29 | 11010 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 30 | 11001 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 31 | 10101 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 32 | 10110 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 33 | 10100 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 34 | 10010 | { 0 5 10 15 20 25 30 35 40 45 49 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 35 | 10001 | { 0 5 10 15 20 25 30 35 40 45 49 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 36 | 01010 | { 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 37 | 11101 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 223 228 233 238 243 248 253 258 263 268 272} | 30 |

Here, for PSFCH RB location index, ‘0’ means that RB index = 0. And, Bitmap ‘1’ means the corresponding RB set is transmitted, and ‘0’ means the corresponding RB set is not transmitted for wide band operation.

Table 6.1.2.1.3.1-2 shows the MPR simulation results for the scenarios.

Table 6.1.2.1.3.1-2: PSFCH MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 7.98 | 7.60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 9.39 | 7.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.65 | 9.05 | 7.67 | 7.43 | 9.31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 9.43 | 9.83 | 9.09 | 7.57 | 9.04 | 7.35 | 11.51 | 9.39 | 9.32 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 9.55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.2.1.3.1-3 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.2.1.3.1-3 : PSFCH MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration5 | Inner RB set configuration5 |
| Contiguous sub-band RB sets | 9.83 | 9.04 |
| Non-contiguous sub-band RB sets | 11.51 | 9.22 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH MPR for SL-U power class 5 can be proposed as Table 6.1.2.1.3.1-4 or Table 6.1.2.1.3.1-5 based on the simulation results when considering implementation margin.

Table 6.1.2.1.3.1-5 PSFCH MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 11.5 | ≤ 10.0 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |

Table 6.1.2.1.3.1-6 PSFCH MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |

### 6.1.3	UE additional maximum output power reduction for NR SL-U

Based on the NR-U agreement for the additional spectrum emission mask in clause 6.5F.2.3 in TS38.101-1, RAN4 do not consider the additional spectrum emission mask.

For the additional spurious emission requirements for SL-U, RAN4 will consider the NR-U additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1.

To derive A-MPR requirements for SL-U operation in single CC, RAN4 will assume the basic simulation parameters and assumptions in section 6.1.2 and the additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1 will be considered.

#### 6.1.3.1	A-MPR for SL-U with NS_28

##### 6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.1.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.1.1.1-1: Simulation results for NS_28 Allocation Full and partial allocations.

| NS_28 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤8.0 | ≤8.1 | ≤7.7 | ≤8.0 | ≤7.1 | ≤7.8 | ≤6.9 | ≤7.8 |
| 16 QAM | ≤8.0 | ≤8.3 | ≤7.4 | ≤8.3 | ≤7.3 | ≤8.1 | ≤6.9 | ≤8.1 |
| 64 QAM | ≤8.1 | ≤8.7 | ≤7.4 | ≤8.7 | ≤7.3 | ≤8.2 | ≤7.0 | ≤8.2 |
| 256 QAM | ≤8.2 | ≤8.8 | ≤7.4 | ≤8.8 | ≤7.3 | ≤8.2 | ≤7.1 | ≤8.1 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

##### 6.1.3.1.2	A-MPR for S-SSB transmission

##### 6.1.3.1.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.2	A-MPR for SL-U with NS_29

##### 6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.2.2	A-MPR for S-SSB transmission

##### 6.1.3.2.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.3	A-MPR for SL-U with NS_30

##### 6.1.3.3.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.3.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.3.1.1-1: Simulation results for NS_30 Allocation Note 2 Full and partial allocations.

| NS_30 A-MPR Allocation Note2 | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤10.7 | ≤10.9 | ≤10.4 | ≤11.0 | ≤9.3 | ≤10.3 | ≤9.1 | ≤10.4 |
| 16 QAM | ≤10.8 | ≤9.0 | ≤10.1 | ≤11.1 | ≤9.6 | ≤10.5 | ≤9.2 | ≤10.5 |
| 64 QAM | ≤10.9 | ≤11.4 | ≤10.1 | ≤11.4 | ≤9.6 | ≤10.6 | ≤9.3 | ≤10.6 |
| 256 QAM | ≤11.0 | ≤11.5 | ≤10.0 | ≤11.5 | ≤9.7 | ≤10.6 | ≤9.4 | ≤10.5 |

Table 6.1.3.3.1.1-2: Simulation results for NS_30 Allocation Note 3 Full and partial allocations.

| NS_30 A-MPR Allocation Note3 | 20M | 20M | 40M | 40M |
| --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial |
| QPSK | ≤8.3 | ≤8.4 | ≤5.9 | ≤6.2 |
| 16 QAM | ≤8.1 | ≤8.7 | ≤5.8 | ≤6.3 |
| 64 QAM | ≤8.3 | ≤9.0 | ≤5.8 | ≤6.3 |
| 256 QAM | ≤8.4 | ≤8.9 | ≤5.8 | ≤6.3 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

##### 6.1.3.3.2	A-MPR for S-SSB transmission

##### 6.1.3.3.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.4	A-MPR for SL-U with NS_31

##### 6.1.3.4.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.4.1.1	LG Electronics’ simulation results (R4-2315542)

Table 6.1.3.4.1.1-6 and Figure 6.1.3.4.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.4.1.1-1: NS_31-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 4.67 | 5.08 | 4.67 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 4.67 | 5.08 | 4.67 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 4.67 | 5.07 | 4.67 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.89 | 5.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 4.28 | 4.67 | 4.66 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 4.28 | 4.67 | 4.67 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.67 | 4.67 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.28 | 4.67 | 5.89 | 4.28 | 4.67 | 3.16 | 5.88 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 4.28 | 4.67 | 5.89 | 4.28 | 4.67 | 3.16 | 5.88 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.29 | 4.66 | 5.89 | 4.27 | 4.67 | 3.15 | 5.88 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.88 | 5.07 | 5.47 | 5.89 | 5.88 | 5.07 | 5.88 | 5.87 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 3.53 | 4.28 | 4.67 | 5.89 | 4.28 | 4.67 | 3.89 | 4.28 | 3.16 | 5.88 | 1.81 | 2.81 |  |  |  |  |  |  |
|  | '16QAM' | 3.53 | 4.28 | 4.67 | 5.90 | 4.28 | 4.67 | 3.90 | 4.28 | 3.16 | 5.87 | 1.81 | 2.80 |  |  |  |  |  |  |
|  | '64QAM' | 3.53 | 4.28 | 4.67 | 5.89 | 4.28 | 4.67 | 3.90 | 4.28 | 3.16 | 5.88 | 3.16 | 2.80 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.90 | 5.47 | 5.07 | 5.47 | 5.07 | 5.46 | 5.87 | 5.88 | 5.06 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 3.17 | 3.90 | 4.66 | 5.89 | 4.27 | 4.67 | 3.89 | 4.28 | 3.53 | 4.28 | 3.16 | 5.88 | 1.81 | 2.80 | 2.47 | 3.16 | 3.16 | 5.88 |
|  | '16QAM' | 3.17 | 3.90 | 4.66 | 5.89 | 4.28 | 4.67 | 3.90 | 4.28 | 3.52 | 4.28 | 3.15 | 5.88 | 1.81 | 2.80 | 2.47 | 3.16 | 3.51 | 5.88 |
|  | '64QAM' | 3.17 | 3.90 | 4.67 | 5.89 | 4.27 | 4.67 | 3.89 | 4.28 | 3.53 | 4.28 | 3.52 | 5.88 | 3.15 | 2.80 | 3.16 | 3.16 | 3.52 | 5.88 |
|  | '256QAM' | 5.88 | 5.06 | 5.47 | 5.88 | 5.47 | 5.07 | 5.88 | 5.06 | 5.88 | 5.06 | 5.46 | 5.88 | 5.88 | 5.06 | 5.88 | 5.06 | 5.88 | 5.88 |

Figure 6.1.3.4.1.1-1: NS_31-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.4.1.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.4.1.1-2 : NS_31-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full(dB) | Partial (dB) |
| CP-OFDM | QPSK | 4.67 | 6.32 | 3.52 | 6.30 |
|  | 16 QAM | 4.68 | 6.32 | 3.52 | 6.30 |
|  | 64 QAM | 4.67 | 6.32 | 3.52 | 6.30 |
|  | 256 QAM | 5.48 | 6.32 | 5.88 | 6.30 |

The inner RB set configuration needs to considered when defining the SL-U A-MPR requirements.

Based on the results, Table 6.1.3.4.1.1-3 can be considered for NS_31 A-MPR for SL-U UE power class 5.

Table 6.1.3.4.1.1-3. NS_31 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 4.5 | ≤ 6.5 | Follow SL-U MPR table |
|  | 16 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1.1-5 apply. |  |  |  |  |  |  |

##### 6.1.3.4.1.2	OPPO’s simulation results (R4-2316119)

For PSD = 10dBm/MHz：

Table 6.1.3.4.1.2-1 A-MPR for single CC PSD=10 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.4.1.2-2 A-MPR for Wideband operation PSD=10 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 2.8 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

For PSD = 7dBm/MHz：

Table 6.1.3.4.1.2-3 A-MPR for single CC PSD=7 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.4.1.2-4 A-MPR for Wideband operation PSD=7 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 3.0 | 2.7 | 2.9 | 2.6 | 2.9 | 2.6 | 2.9 | 2.7 | 2.7 | 2.7 | 2.9 | 2.7 | 2.8 | 3.0 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 2.8 | 3.0 | 3.0 | 3.0 | 3.0 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

For PSD = 4dBm/MHz：

Table 6.1.3.4.1.2-5 A-MPR for single CC PSD=4 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 5.6 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 5.7 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 5.7 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.4.1.2-6 A-MPR for NS_02S Wideband operation PSD=4 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.5 | 3.0 | 3.6 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 2.8 | 2.8 | 3.5 | 2.8 | 2.8 | 3.6 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.6 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 5.9 | 5.9 | 3.1 | 6.0 | 2.9 | 5.9 | 2.6 | 5.9 | 2.9 | 5.9 | 2.9 | 2.7 | 2.7 | 5.9 | 3.0 | 2.8 | 6.0 |
|  | 16QAM | 6.0 | 6.0 | 3.1 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 | 5.9 | 3.0 | 3.0 | 2.8 | 6.0 | 3.0 | 3.0 | 6.0 |
|  | 64QAM | 6.0 | 6.0 | 4.2 | 6.0 | 4.2 | 5.9 | 4.1 | 5.9 | 4.2 | 5.9 | 4.2 | 4.1 | 4.0 | 6.0 | 4.2 | 4.1 | 5.9 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

From the table 6.1.3.4.1.2-1 to table 6.1.3.4.1.2-6 above, it can be found that even though the PSD requirement is different from 10 to 7 to 4 dBm/MHz, the A-MPR simulation result for both single CC and wide-band operation are the same. The reason here is that the PSD requirement is not the dominate factor and it is the SEM and EVM for some of the 256QAM. The figure 6.1.3.4.1.2-1 and figure 6.1.3.4.1.2-2 below show the A-MPR.

Figure 6.1.3.4.1.2-1 Single CC A-MPR

Figure 6.1.3.4.1.2-2 Wideband A-MPR

Note : NS_02S is equal to NS_31.

##### 6.1.3.4.1.3	Qualcomm’s simulation results (R4-2316791)

We performed simulations for the full and partial allocations and computed the A-MPR for NS_31. The results are shown in the tables below.

Table 6.1.3.4.1.3: Simulation results for NS_31 Full and partial allocations.

| NS_31 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤ 5.2 | ≤ 5.7 | ≤ 8.3 | ≤ 8.6 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.1 |
| 16 QAM | ≤ 5.6 | ≤ 5.8 | ≤ 8.0 | ≤ 9.0 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.3 |
| 64 QAM | ≤ 5.5 | ≤ 6.0 | ≤ 8.0 | ≤ 9.3 | ≤ 8.1 | NA | ≤ 8.1 | ≤ 8.5 |
| 256 QAM | ≤ 5.6 | ≤ 5.9 | ≤ 8.0 | ≤ 9.5 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.5 |

The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

##### 6.1.3.4.2	A-MPR for S-SSB transmission

##### 6.1.3.4.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_31, Table 6.1.3.4.2.1-1 and Figure 6.1.3.4.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.4.2.1-1: NS_31-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 7.90 | 6.82 | 6.81 | 6.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 7.45 | 9.20 | 8.17 | 7.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 6.46 | 6.69 | 7.21 | 4.76 | 7.20 | 8.18 | 6.79 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.41 | 8.95 | 6.78 | 7.97 | 6.48 | 6.89 | 7.19 | 5.98 | 4.31 | 4.61 | 7.12 | 9.04 | 6.62 | 7.85 | 6.61 | 7.79 | 6.47 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.91 | 7.19 | 9.59 | 6.61 | 9.54 | 6.67 | 7.99 | 6.53 | 6.80 | 7.20 | 6.12 | 4.95 | 5.66 | 4.09 | 4.70 | 7.28 | 4.56 | 7.16 | 9.97 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 8.85 | 6.62 | 9.14 | 6.54 | 8.62 | 6.08 | 8.80 | 5.83 | 8.04 | 6.48 | 7.86 | 6.46 | 8.13 | 6.49 | 7.49 | 6.35 | 10.15 | 6.31 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.4.2.1-13: NS_31-S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.4.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.4.2.1-2 : NS_31 S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | 11.05 | 7.58 | 6.12 | 7.28 |
| Non-contiguous sub-band RB sets | 10.15 | 7.47 | 7.49 | 6.35 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.4.2.1-3 or Table 6.1.3.4.2.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.4.2.1-3: NS_31 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 | ≤ 10.0 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

Table 6.1.3.4.2.1-4: NS_31 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

##### 6.1.3.4.3	A-MPR for PSFCH transmission

##### 6.1.3.4.3.1	LG Electronics’ simulation results (R4-2315542 and R4-2321771)

For NS_31, Table 6.1.3.4.3.1-1 shows the A-MPR simulation results for the scenarios.

Table 6.1.3.4.3.1-1: NS_31-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 8.38 | 8.31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 9.34 | 8.38 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.65 | 9.05 | 8.27 | 6.51 | 8.96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 9.43 | 9.83 | 9.05 | 8.28 | 8.56 | 6.86 | 9.82 | 9.00 | 8.95 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 9.55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.4.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.4.3.1-2 : NS_31 PSFCH A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous sub-band RB sets | 9.83 | 8.56 |
| Non-contiguous sub-band RB sets | 9.82 | 9.63 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.4.3.1-3 or Table 6.1.3.4.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.4.3.1-3 NS_31 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.5 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 11.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

Table 6.1.3.4.3.1-4 NS_31 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 11.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

<Editor Note> A-MPR requirements will be added

#### 6.1.3.5	A-MPR for SL-U with NS_53

##### 6.1.3.5.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.5.1.1	LG Electronics’ simulation results (R4-2315542)

For NS_53, Table 6.1.3.5.1.1-9 and Figure 6.1.3.5.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.5.1.1-1: NS_53-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 8.50 | 10.84 | 8.51 | 10.85 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 8.50 | 10.84 | 8.51 | 10.84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 8.50 | 10.84 | 8.51 | 10.84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 8.50 | 10.83 | 8.51 | 10.84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.47 | 7.61 | 8.50 | 10.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.47 | 8.06 | 8.50 | 10.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.47 | 8.06 | 8.50 | 10.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 8.05 | 8.50 | 10.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 5.89 | 8.51 | 11.31 | 5.47 | 8.06 | 8.50 | 11.29 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 5.89 | 8.51 | 11.32 | 5.47 | 8.06 | 8.50 | 11.30 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 6.31 | 8.96 | 11.30 | 5.47 | 8.06 | 8.50 | 11.29 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 6.30 | 8.96 | 11.31 | 5.47 | 8.06 | 8.50 | 11.29 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.82 | 4.67 | 8.50 | 11.30 | 5.47 | 8.06 | 3.53 | 6.31 | 8.50 | 11.29 | 5.47 | 8.05 |  |  |  |  |  |  |
|  | '16QAM' | 2.82 | 4.67 | 8.50 | 11.31 | 5.47 | 8.05 | 3.53 | 6.31 | 8.50 | 11.29 | 5.47 | 8.05 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 4.67 | 8.51 | 11.30 | 5.47 | 8.06 | 3.53 | 6.30 | 8.96 | 11.29 | 5.47 | 8.05 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 8.96 | 11.31 | 5.47 | 8.06 | 5.47 | 6.31 | 8.50 | 11.29 | 5.47 | 8.05 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 3.17 | 3.90 | 8.50 | 11.30 | 5.47 | 8.06 | 3.52 | 6.30 | 2.47 | 4.67 | 8.50 | 11.30 | 5.47 | 8.05 | 3.52 | 5.88 | 8.50 | 11.30 |
|  | '16QAM' | 3.17 | 3.90 | 8.50 | 11.32 | 5.47 | 8.06 | 3.52 | 6.31 | 2.47 | 5.06 | 8.50 | 11.29 | 5.47 | 8.05 | 3.52 | 6.30 | 8.96 | 11.30 |
|  | '64QAM' | 3.17 | 3.90 | 8.96 | 11.31 | 5.47 | 8.06 | 3.53 | 6.31 | 2.81 | 4.67 | 8.50 | 11.29 | 5.47 | 8.05 | 3.52 | 5.88 | 8.96 | 11.29 |
|  | '256QAM' | 5.47 | 5.07 | 8.51 | 11.32 | 5.47 | 8.06 | 5.47 | 6.30 | 5.47 | 5.06 | 8.50 | 11.30 | 5.46 | 8.05 | 5.47 | 6.30 | 8.96 | 11.29 |

Figure 6.1.3.5.1.1-1: NS_53-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.5.1.1-2 shows the maximum value of simulation results considering Full/Partial RB allocation.

Table 6.1.3.5.1.1-2 : NS_53-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 8.51 | 11.31 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 4.67 | 3.17 | 3.90 |
|  | 16 QAM | 8.96 | 11.32 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 5.06 | 3.17 | 3.90 |
|  | 64 QAM | 8.96 | 11.31 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 4.67 | 3.17 | 3.90 |
|  | 256 QAM | 8.96 | 11.32 | 5.47 | 8.06 | 5.47 | 6.31 | 5.47 | 5.07 | 5.47 | 5.07 |

Table 6.1.3.5.1.1-3 can be considered for NS_53 A-MPR for SL-U UE power class 5.

Table 6.1.3.5.1.1-3. NS_53 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.5.1.2	OPPO’s simulation results (R4-2316119)

Case 1,2,8,9 are for 20MHz; 3,4,10,11 are for 40MHz; 5,12 are for 60MHz; 6,13 are for 80MHz and 7,14 are for 100MHz.

Table 6.1.3.5.1.2-1 A-MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 8.4 | 8.6 | 5.4 | 5.5 | 3.8 | 3.2 | 3.2 | 9.8 | 10.9 | 6.6 | 7.8 | 6.0 | 4.5 | 3.9 |
| 16QAM | 8.4 | 8.6 | 5.4 | 5.6 | 3.9 | 3.9 | 3.8 | 9.8 | 10.9 | 6.6 | 7.8 | 6.0 | 4.5 | 3.9 |
| 64QAM | 8.3 | 8.5 | 5.5 | 5.4 | 5.6 | 5.5 | 5.5 | 9.8 | 10.9 | 6.7 | 7.8 | 6.1 | 4.6 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 9.8 | 10.9 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.5.1.2-2 A-MPR for Wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.6 | 5.8 | 5.5 | 5.5 | 5.5 | 5.8 | 4.0 | 4.0 | 4.0 | 2.8 |
|  | 16QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.6 | 5.8 | 5.6 | 5.6 | 5.6 | 5.7 | 4.0 | 4.0 | 4.0 | 3.6 |
|  | 64QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 5.8 | 5.5 | 5.5 | 5.5 | 5.7 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.6 | 8.6 | 8.5 | 8.6 | 7.7 | 7.3 | 7.3 | 7.3 | 8.5 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 10.9 | 10.9 | 11.0 | 10.9 | 10.9 | 10.9 | 10.9 | 11.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.2 | 6.4 | 6.4 | 5.1 |
|  | 16QAM | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.3 | 6.5 | 6.3 | 5.1 |
|  | 64QAM | 11.0 | 11.0 | 11.0 | 10.9 | 11.0 | 10.9 | 11.0 | 10.9 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.3 | 6.4 | 6.3 | 5.2 |
|  | 256QAM | 11.0 | 11.0 | 11.0 | 10.9 | 11.0 | 10.9 | 11.0 | 11.0 | 8.1 | 8.0 | 8.0 | 8.0 | 8.0 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3.5.1.2-1 Single CC A-MPR

Figure 6.1.3.5.1.2-2 Wide-band A-MPR

Note : NS_03S is equal to NS_53.

##### 6.1.3.5.1.3	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.5.1.3-1: Simulation results for NS_53 Full and partial allocations.

| NS_53 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤7.5 | ≤7.6 | ≤7.0 | ≤7.3 | ≤6.7 | ≤7.4 | ≤6.4 | ≤7.4 |
| 16 QAM | ≤7.5 | ≤7.8 | ≤6.9 | ≤7.4 | ≤6.8 | ≤7.6 | ≤6.4 | ≤7.6 |
| 64 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.6 | ≤6.8 | ≤7.8 | ≤6.5 | ≤7.8 |
| 256 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.5 | ≤6.8 | ≤7.7 | ≤6.6 | ≤7.7 |

The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

##### 6.1.3.5.2	A-MPR for S-SSB transmission

##### 6.1.3.5.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_53, Table 6.1.3.5.2.1-1 and Figure 6.1.3.5.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.5.2.1-1: NS_53-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 11.16 | 15.09 | 9.13 | 12.21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 12.11 | 9.20 | 11.24 | 15.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 9.19 | 9.12 | 12.22 | 9.09 | 12.22 | 8.18 | 9.12 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.32 | 8.95 | 7.43 | 7.99 | 9.19 | 9.19 | 12.23 | 6.36 | 9.21 | 9.22 | 12.13 | 9.03 | 7.47 | 8.09 | 9.10 | 8.31 | 9.20 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 11.13 | 7.20 | 9.76 | 6.37 | 9.55 | 7.44 | 8.15 | 9.21 | 9.24 | 12.28 | 6.04 | 7.38 | 6.28 | 9.18 | 9.22 | 12.19 | 9.13 | 12.22 | 10.26 | 7.48 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.47 | 7.36 | 8.74 | 7.40 | 8.46 | 7.45 | 8.90 | 7.44 | 8.10 | 9.19 | 8.09 | 9.14 | 8.08 | 9.29 | 7.62 | 9.18 | 10.07 | 6.45 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.5.2.1-1: NS_53 S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.5.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.5.2.1-2 : NS_53 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 11.16 | 15.09 | 11.05 | 15.09 | 9.36 | 12.22 | 9.92 | 12.23 | 11.13 | 11.13 |
| Non-contiguous | - | - | - | - | 8.18 | 9.12 | 9.03 | 9.20 | 10.26 | 9.29 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.5.2.1-3 or Table 6.1.3.5.2.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.5.2.1-3: NS_53 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.5.2.1-4: NS_53 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.5.3	A-MPR for PSFCH transmission

##### 6.1.3.5.3.1	LG Electronics’ simulation results (R4-2315542 and R4-2321771)

For NS_53, Table 6.1.3.5.3.1-1 shows the A-MPR simulation results for the scenarios.

Table 6.1.3.5.3.1-1: NS_53-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 7.98 | 7.60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 9.32 | 7.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.65 | 9.05 | 7.67 | 6.51 | 8.96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 9.43 | 9.83 | 9.01 | 7.57 | 8.56 | 6.86 | 9.82 | 9.00 | 8.95 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 9.55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.5.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.5.3.1-2 : NS_53 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 7.98 | 9.32 | 9.83 | 9.43 | 9.55 |
| Non-contiguous | N/A | 9.0 | 9.82 | 9.63 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.5.3.1-3 or Table 6.1.3.5.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.5.3.1-3 NS_53 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| Non-contiguous | N/A | ≤12.5 | ≤12.5 | ≤12.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 6.1.3.5.3.1-4 NS_53 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### 6.1.3.6	A-MPR for SL-U with NS_54

##### 6.1.3.6.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.6.2	A-MPR for S-SSB transmission

##### 6.1.3.6.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.7	A-MPR for SL-U with NS_58

##### 6.1.3.7.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.7.1.1	LG Electronics’ simulation results (R4-2315542)

For NS_58, Table 6.1.3.7.1.1-1 and Figure 6.1.3.7.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.7.1.1-1: NS_58-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.83 | 2.15 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.83 | 2.15 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.83 | 3.18 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.48 | 5.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.82 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.82 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.82 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.48 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.47 | 2.47 | 2.47 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.16 | 2.48 | 3.16 | 2.48 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.07 | 5.88 | 5.06 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.15 | 2.47 | 2.48 | 2.47 | 2.48 | 2.46 | 0.10 | 2.13 | 0.00 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.15 | 2.47 | 2.47 | 2.47 | 2.48 | 2.46 | 0.36 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.16 | 2.15 | 2.81 | 2.48 | 3.16 | 2.48 | 3.16 | 2.12 | 3.16 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.07 | 5.46 | 5.06 | 5.47 | 5.06 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.48 | 2.47 | 2.48 | 2.47 | 2.47 | 2.13 | 0.10 | 2.46 | 0.00 | 2.47 | 1.20 | 2.13 | 0.10 |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.48 | 2.47 | 2.47 | 2.47 | 2.48 | 2.13 | 0.36 | 2.46 | 0.63 | 2.47 | 1.20 | 2.13 | 0.36 |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.48 | 3.16 | 2.47 | 3.16 | 2.47 | 2.81 | 2.48 | 3.15 | 2.12 | 3.16 | 2.46 | 3.16 | 2.47 | 3.16 | 2.12 |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.06 | 5.46 | 5.06 | 5.46 | 5.06 | 5.47 | 5.06 | 5.46 | 5.05 |

Figure 6.1.3.7.1.1-2: NS_58-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.7.1.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.7.1.1-2 : NS_58-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full(dB) | Partial (dB) |
| CP-OFDM | QPSK | 2.81 | 2.83 | 2.47 | 1.20 |
|  | 16 QAM | 2.81 | 2.83 | 2.47 | 1.20 |
|  | 64 QAM | 3.18 | 2.83 | 3.16 | 2.47 |
|  | 256 QAM | 5.48 | 5.09 | 5.88 | 5.06 |

The inner RB set configuration needs to considered when defining the SL-U A-MPR requirements.

Based on the results, Table 6.1.3.7.1.1-3 can be considered for NS_58 A-MPR for SL-U UE power class 5.

Table 6.1.3.7.1.1-3. NS_58 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |
|  |  | Full (dB)2 | Partial (dB)3 | Full (dB) 2 | Partial (dB) 3 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1.1-5 apply. |  |  |  |  |  |

##### 6.1.3.7.1.2	OPPO’s simulation results (R4-2316119)

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.7.1.2-1 A-MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 5.4 | 5.6 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 6.7 | 8.0 | 3.5 | 4.7 | 3.2 | 3.3 | 3.2 |
| 16QAM | 5.4 | 5.7 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 6.8 | 8.0 | 3.5 | 4.8 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.3 | 5.6 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 6.8 | 8.0 | 4.1 | 4.7 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 8.0 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.7.1.2-2 A-MPR for wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 3.6 | 3.6 | 3.7 | 3.6 | 3.6 | 3.6 | 3.5 | 3.7 | 3.6 |
|  | 64QAM | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 4.8 | 4.7 | 4.9 | 4.7 | 4.8 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 7.4 | 7.5 | 8.5 | 7.4 | 7.5 | 7.4 | 7.4 | 8.5 | 7.7 | 7.3 | 8.5 | 7.3 | 7.7 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 | 3.1 | 3.3 | 3.4 | 2.7 |
|  | 16QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.1 | 5.0 | 5.0 | 5.0 | 5.1 | 3.2 | 3.5 | 3.3 | 2.8 |
|  | 64QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.1 | 5.1 | 5.1 | 5.1 | 5.1 | 4.1 | 4.1 | 4.1 | 4.0 |
|  | 256QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3.7.1.2-1 Single CC A-MPR

Figure 6.1.3.7.1.2-2 Wideband CC A-MPR

Note : NS_04S is equal to NS_58.

##### 6.1.3.7.1.3	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.7.1.3-1: Simulation results for NS_58 Full and partial allocations.

| NS_58 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤5.2 | ≤5.4 | ≤5.9 | ≤6.0 | ≤5.6 | ≤5.8 | ≤5.4 | ≤5.7 |
| 16 QAM | ≤5.3 | ≤5.6 | ≤5.8 | ≤6.2 | ≤5.7 | ≤5.9 | ≤5.6 | ≤5.8 |
| 64 QAM | ≤5.3 | ≤5.8 | ≤5.8 | ≤6.2 | ≤5.6 | ≤5.9 | ≤5.6 | ≤5.8 |
| 256 QAM | ≤5.4 | ≤5.8 | ≤5.8 | ≤6.2 | ≤5.7 | ≤5.9 | ≤5.6 | ≤5.8 |

The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

##### 6.1.3.7.2	A-MPR for S-SSB transmission

##### 6.1.3.7.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_58, Table 6.1.3.7.2.1-1 and Figure 6.1.3.7.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.7.2.1-1: NS_58-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 7.90 | 6.82 | 6.81 | 6.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 7.45 | 9.20 | 8.17 | 7.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 6.46 | 6.69 | 7.21 | 4.76 | 7.20 | 8.18 | 6.79 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.41 | 8.95 | 6.78 | 7.97 | 6.48 | 6.89 | 7.19 | 5.98 | 4.31 | 4.61 | 7.12 | 9.04 | 6.62 | 7.85 | 6.61 | 7.79 | 6.47 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.91 | 7.19 | 9.59 | 6.61 | 9.54 | 6.67 | 7.99 | 6.53 | 6.80 | 7.20 | 6.12 | 4.95 | 5.66 | 4.09 | 4.70 | 7.28 | 4.56 | 7.16 | 9.97 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 8.85 | 6.62 | 9.14 | 6.54 | 8.62 | 6.08 | 8.80 | 5.83 | 8.04 | 6.48 | 7.86 | 6.46 | 8.13 | 6.49 | 7.49 | 6.35 | 10.15 | 6.31 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.7.2.1-1: NS_58-S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.7.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.7.2.1-2 : NS_58 S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | 10.91 | 6.86 | 6.11 | 3.98 |
| Non-contiguous sub-band RB sets | 9.94 | 7.22 | 6.93 | 4.82 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.7.2.1-3 or Table 6.1.3.7.2.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.7.2.1-3: NS_58 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 8.5 | ≤ 6.5 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

Table 6.1.3.7.2.1-4: NS_58 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

##### 6.1.3.7.3	A-MPR for PSFCH transmission

##### 6.1.3.7.3.1	LG Electronics’ simulation results (R4-2315542 and R4-2321771)

For NS_58, Table 6.1.3.7.3.1-1 shows the A-MPR simulation results for the scenarios.

Table 6.1.3.7.3.1-1: NS_58-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 7.98 | 7.60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 9.32 | 7.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.65 | 9.05 | 7.67 | 6.51 | 8.96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 9.43 | 9.83 | 9.01 | 7.57 | 8.56 | 6.86 | 9.82 | 9.00 | 8.95 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 9.55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.7.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.7.3.1-2 : NS_58 PSFCH A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous sub-band RB sets | 9.83 | 8.56 |
| Non-contiguous sub-band RB sets | 9.82 | 8.08 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.7.3.1-3 or Table 6.1.3.7.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.7.3.1-3 NS_58 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.5 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

Table 6.1.3.7.3.1-4 NS_58 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

#### 6.1.3.8	A-MPR for SL-U with NS_59

##### 6.1.3.8.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.8.2	A-MPR for S-SSB transmission

##### 6.1.3.8.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.9	A-MPR for SL-U with NS_60

##### 6.1.3.9.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.9.1.1	LG Electronics’ simulation results (R4-2315542)

For NS_60, Table 6.1.3.9.1.1-1 and Figure 6.1.3.9.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.9.1.1-1: NS_60-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 5.47 | 7.62 | 5.89 | 7.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.47 | 8.07 | 5.89 | 8.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.47 | 8.06 | 5.89 | 7.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 8.06 | 5.89 | 8.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 4.67 | 5.47 | 7.62 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 4.67 | 5.47 | 8.06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 4.68 | 5.47 | 8.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 8.05 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.28 | 5.88 | 8.07 | 2.47 | 5.07 | 5.88 | 8.05 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 4.28 | 5.88 | 8.07 | 2.47 | 5.07 | 5.88 | 8.05 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.28 | 5.88 | 8.52 | 2.82 | 5.06 | 5.88 | 8.05 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.88 | 5.48 | 5.89 | 8.07 | 5.47 | 5.07 | 5.88 | 8.04 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.82 | 3.53 | 5.88 | 8.06 | 2.47 | 5.07 | 2.81 | 3.53 | 5.88 | 8.05 | 2.46 | 5.06 |  |  |  |  |  |  |
|  | '16QAM' | 2.82 | 3.53 | 5.88 | 8.06 | 2.47 | 5.07 | 2.81 | 3.53 | 5.88 | 8.49 | 2.46 | 5.06 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 3.53 | 5.89 | 8.06 | 2.81 | 5.07 | 3.16 | 3.53 | 5.88 | 8.05 | 3.16 | 5.06 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.88 | 8.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 8.51 | 5.46 | 5.06 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 3.17 | 3.90 | 5.89 | 8.07 | 2.47 | 5.07 | 2.47 | 3.16 | 2.47 | 3.17 | 5.88 | 8.04 | 2.46 | 5.05 | 2.47 | 3.16 | 5.88 | 8.05 |
|  | '16QAM' | 3.17 | 3.90 | 5.88 | 8.51 | 2.47 | 5.07 | 2.47 | 3.17 | 2.47 | 3.17 | 5.88 | 8.49 | 2.46 | 5.05 | 2.46 | 3.16 | 5.88 | 8.05 |
|  | '64QAM' | 3.17 | 3.90 | 5.88 | 8.51 | 2.81 | 5.06 | 3.16 | 3.16 | 2.81 | 3.17 | 5.47 | 8.05 | 3.16 | 5.05 | 3.16 | 3.16 | 5.88 | 8.05 |
|  | '256QAM' | 5.47 | 5.06 | 5.89 | 8.52 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.06 | 5.88 | 8.05 | 5.47 | 5.06 | 5.47 | 5.46 | 5.88 | 8.05 |

Figure 6.1.3.9.1.1-1: NS_60-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.9.1.1-2 shows the maximum value of simulation results considering Full/Partial RB allocation.

Table 6.1.3.9.1.1-2 : NS_60-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 5.89 | 8.07 | 2.48 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 16 QAM | 5.89 | 8.51 | 2.48 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 64 QAM | 5.89 | 8.52 | 3.16 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 256 QAM | 5.89 | 8.52 | 5.47 | 5.07 | 5.88 | 5.48 | 5.47 | 5.07 | 5.47 | 5.06 |

Based on the results, Table 6.1.3.9.1.1-3 can be considered for NS_60 A-MPR for SL-U UE power class 5.

Table 6.1.3.9.1.1-3. NS_60 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.9.1.2	OPPO’s simulation results (R4-2316119)

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.9.1.1-1 A-MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 6.4 | 6.6 | 3.4 | 3.5 | 3.5 | 3.5 | 3.5 | 7.8 | 8.9 | 4.5 | 5.8 | 4.0 | 3.6 | 3.5 |
| 16QAM | 6.4 | 6.7 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 7.8 | 9.0 | 4.5 | 5.8 | 4.0 | 3.6 | 3.5 |
| 64QAM | 6.3 | 6.6 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 7.8 | 9.0 | 4.6 | 5.8 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 9.0 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.9.1.1-2 A-MPR for wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 6.5 | 6.5 | 6.6 | 6.5 | 6.5 | 6.5 | 6.5 | 6.6 | 3.7 | 3.5 | 3.7 | 3.5 | 3.7 | 3.0 | 2.8 | 2.8 | 3.2 |
|  | 16QAM | 6.5 | 6.5 | 6.6 | 6.5 | 6.6 | 6.5 | 6.5 | 6.7 | 3.7 | 3.6 | 3.7 | 3.6 | 3.7 | 3.6 | 3.5 | 3.7 | 3.6 |
|  | 64QAM | 6.6 | 6.5 | 6.6 | 6.6 | 6.5 | 6.6 | 6.6 | 6.6 | 4.8 | 4.7 | 4.9 | 4.7 | 4.8 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 7.4 | 7.5 | 8.5 | 7.4 | 7.5 | 7.4 | 7.4 | 8.5 | 7.7 | 7.3 | 8.5 | 7.3 | 7.7 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 8.9 | 9.0 | 9.0 | 9.0 | 8.9 | 9.0 | 8.9 | 9.0 | 6.0 | 5.9 | 5.9 | 5.9 | 6.0 | 4.2 | 4.4 | 4.4 | 3.2 |
|  | 16QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.0 | 6.0 | 5.9 | 6.0 | 6.0 | 4.3 | 4.5 | 4.3 | 3.3 |
|  | 64QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.0 | 6.0 | 6.0 | 6.0 | 6.0 | 4.3 | 4.4 | 4.3 | 4.0 |
|  | 256QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3.9.1.1-1 Single CC A-MPR

Figure 6.1.3.9.1.1-2 Wideband A-MPR

Note : NS_05S is equal to NS_60.

##### 6.1.3.9.1.3	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.9.1.3-1: Simulation results for NS_60 Full and partial allocations.

| NS_53 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤7.5 | ≤7.6 | ≤7.0 | ≤7.3 | ≤6.7 | ≤7.4 | ≤6.4 | ≤7.4 |
| 16 QAM | ≤7.5 | ≤7.8 | ≤6.9 | ≤7.4 | ≤6.8 | ≤7.6 | ≤6.4 | ≤7.6 |
| 64 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.6 | ≤6.8 | ≤7.8 | ≤6.5 | ≤7.8 |
| 256 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.5 | ≤6.8 | ≤7.7 | ≤6.6 | ≤7.7 |

The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

##### 6.1.3.9.2	A-MPR for S-SSB transmission

##### 6.1.3.9.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_60, Table 6.1.3.9.2.1-1 and Figure 6.1.3.9.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.9.2.1-1: NS_60-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.24 | 8.17 | 12.15 | 6.16 | 9.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.31 | 8.63 | 9.09 | 9.02 | 8.17 | 12.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.29 | 7.05 | 7.87 | 6.15 | 6.20 | 9.21 | 6.20 | 9.21 | 7.67 | 6.25 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 10.34 | 6.86 | 8.81 | 5.70 | 7.67 | 6.08 | 6.19 | 9.19 | 6.11 | 6.15 | 6.23 | 9.09 | 8.92 | 6.11 | 7.97 | 6.13 | 7.51 | 6.23 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.92 | 6.67 | 9.86 | 5.49 | 9.44 | 5.56 | 7.59 | 6.17 | 6.22 | 9.19 | 5.89 | 4.50 | 5.39 | 6.18 | 6.21 | 9.20 | 6.14 | 9.18 | 10.54 | 7.21 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.60 | 6.16 | 8.72 | 6.14 | 8.26 | 5.82 | 9.16 | 5.30 | 7.74 | 6.16 | 7.76 | 6.05 | 7.72 | 6.12 | 7.20 | 6.22 | 9.94 | 6.23 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.9.2.1-1: NS_60 S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.9.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.9.2.1-2 : NS_60 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 9.24 | 12.15 | 11.31 | 12.15 | 9.29 | 9.21 | 10.34 | 9.19 | 10.92 | 10.92 |
| Non-contiguous | - | - | - | - | 7.67 | 6.25 | 8.92 | 6.23 | 10.54 | 7.21 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.9.2.1-3 or Table 6.1.3.9.2.1-4 based on the simulation results when considering implementation margin.

Table 2-66: NS_60 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤10.0 | ≤9.0 | ≤11.5 | ≤9.0 | ≤13.0 | ≤10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 2-67: NS_60 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.9.3	A-MPR for PSFCH transmission

##### 6.1.3.9.3.1	LG Electronics’ simulation results (R4-2315542 and R4-2321771)

For NS_60, Table 6.1.3.9.3.1-1 shows the A-MPR simulation results for the scenarios.

Table 6.1.3.9.3.1-1: NS_60-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 8.40 | 7.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 9.32 | 7.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.65 | 9.05 | 7.78 | 7.76 | 8.96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 9.43 | 9.83 | 9.01 | 7.71 | 8.56 | 7.61 | 9.82 | 9.00 | 8.95 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 9.55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.9.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.9.3.1-2 : NS_60 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 8.40 | 9.32 | 9.83 | 9.43 | 9.55 |
| Non-contiguous | N/A | 9.63 | 9.82 | 8.34 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.9.3.1-3 or Table 6.1.3.9.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.9.3.1-3 NS_60 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| Non-contiguous | N/A | ≤12.5 | ≤12.5 | ≤12.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 6.1.3.9.3.1-4 NS_60 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### 6.1.3.10	A-MPR for SL-U with NS_61

##### 6.1.3.10.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.10.1.1	LG Electronics’ simulation results (R4-2315542)

For NS_61, Table 6.1.3.10.1.1-1 and Figure 6.1.3.10.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.10.1.1-1: NS_61-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 6.31 | 8.98 | 6.73 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 6.30 | 8.97 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 6.31 | 8.98 | 6.74 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 6.31 | 8.98 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 4.66 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 4.67 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 4.67 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.06 | 5.47 | 6.73 | 9.44 | 4.67 | 5.89 | 6.73 | 8.95 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.07 | 5.47 | 6.73 | 9.43 | 4.67 | 5.89 | 6.73 | 9.41 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.07 | 5.47 | 6.73 | 9.43 | 4.67 | 5.89 | 6.73 | 9.42 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.47 | 6.73 | 9.43 | 5.47 | 5.88 | 6.73 | 9.42 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 4.67 | 5.47 | 6.74 | 9.44 | 4.67 | 5.89 | 5.06 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '16QAM' | 4.67 | 5.47 | 6.73 | 9.43 | 4.66 | 5.89 | 5.06 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '64QAM' | 4.67 | 5.47 | 6.73 | 9.43 | 4.67 | 5.89 | 4.67 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.47 | 6.73 | 9.43 | 5.47 | 5.89 | 5.47 | 5.47 | 6.73 | 9.42 | 5.47 | 5.88 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 4.67 | 5.47 | 6.73 | 9.43 | 4.67 | 5.89 | 4.66 | 5.47 | 4.66 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 | 2.81 | 3.89 | 6.73 | 8.96 |
|  | '16QAM' | 4.67 | 5.47 | 6.73 | 9.43 | 4.66 | 5.89 | 5.06 | 5.47 | 4.67 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 | 2.81 | 3.89 | 6.73 | 9.41 |
|  | '64QAM' | 4.67 | 5.47 | 6.74 | 9.44 | 4.67 | 6.30 | 4.66 | 5.47 | 4.66 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 | 3.16 | 3.89 | 6.73 | 9.42 |
|  | '256QAM' | 5.47 | 5.47 | 6.73 | 9.43 | 5.47 | 6.31 | 5.47 | 5.47 | 5.47 | 5.47 | 6.73 | 9.42 | 5.47 | 5.88 | 5.47 | 5.47 | 6.73 | 9.41 |

Figure 6.1.3.10.1.1-1: NS_61-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.10.1.1-2 shows the maximum value of simulation results considering Full/Partial RB allocation.

Table 6.1.3.10.1.1-2 : NS_61-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 6.74 | 9.44 | 4.67 | 5.89 | 5.06 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 16 QAM | 6.74 | 9.43 | 4.67 | 5.89 | 5.07 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 64 QAM | 6.74 | 9.44 | 4.67 | 6.30 | 5.07 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 256 QAM | 6.74 | 9.43 | 5.47 | 6.31 | 5.47 | 5.47 | 5.47 | 5.47 | 5.47 | 5.47 |

For NS_61, the maximum output power is limited with 14dBm. Therefore, basically, A-MPR should be met with the following equation considering power class 5, 20dBm.

- NS_61 A-MPR = max{Table 6.1.3.10.1.1-2 + implementation margin, 6 dB}.

Based on the results, Table 6.1.3.10.1.1-3 can be considered for NS_61 A-MPR for SL-U UE power class 5.

Table 6.1.3.10.1.1-3. NS_61 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.10.1.2	OPPO’s simulation results (R4-2316119)

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.10.1.1-1 A-MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 3.2 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.4 | 4.3 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.10.1.1-2 A-MPR for wideband

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Figure 6.1.3.10.1.1-1 Single CC A-MPR

Figure 6.1.3.10.1.1-2  Wideband A-MPR

Note : NS_06S is equal to NS_61.

##### 6.1.3.10.2	A-MPR for S-SSB transmission

##### 6.1.3.10.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_61, Table 6.1.3.10.2.1-1 and Figure 6.1.3.10.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.10.2.1-1: NS_61-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.24 | 9.14 | 13.08 | 7.15 | 10.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.18 | 9.58 | 10.10 | 9.02 | 9.23 | 13.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.84 | 7.13 | 8.68 | 7.13 | 7.13 | 10.21 | 7.12 | 10.20 | 8.88 | 7.69 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 10.20 | 8.27 | 9.35 | 6.64 | 8.49 | 7.23 | 7.20 | 10.22 | 5.69 | 7.14 | 7.21 | 10.21 | 9.98 | 7.65 | 9.34 | 7.52 | 9.37 | 7.37 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 11.00 | 8.15 | 10.53 | 7.12 | 9.82 | 6.71 | 8.24 | 7.20 | 7.14 | 10.13 | 5.89 | 5.43 | 5.38 | 7.07 | 7.21 | 10.23 | 7.22 | 10.11 | 10.29 | 8.04 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.71 | 7.67 | 9.11 | 7.74 | 9.93 | 7.32 | 9.09 | 6.12 | 9.01 | 7.48 | 8.77 | 7.30 | 9.14 | 7.23 | 8.29 | 7.18 | 10.66 | 7.41 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.10.2.1-1: NS_61 S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.10.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.10.2.1-2 : NS_61 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 9.24 | 13.08 | 11.18 | 13.09 | 9.84 | 10.21 | 10.20 | 10.22 | 11.00 | 11.00 |
| Non-contiguous | - | - | - | - | 8.88 | 7.69 | 9.98 | 7.65 | 10.66 | 8.04 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.10.2.1-3 or Table 6.1.3.10.2.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.10.2.1-3: NS_61 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤11.5 | ≤11.0 | ≤12.5 | ≤11.0 | ≤13.0 | ≤11.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.10.2.1-4: NS_61 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.10.3	A-MPR for PSFCH transmission

##### 6.1.3.10.3.1	LG Electronics’ simulation results (R4-2315542 and R4-2321771)

For NS_61, Table 6.1.3.10.3.1-1 shows the A-MPR simulation results for the scenarios.

Table 6.1.3.10.3.1-1: NS_61-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 8.28 | 8.71 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 10.21 | 8.28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 10.63 | 9.76 | 8.68 | 8.62 | 9.55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 10.19 | 10.69 | 9.74 | 8.72 | 8.56 | 8.73 | 10.50 | 9.56 | 9.56 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 10.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.10.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.10.3.1-2 : NS_61 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 8.73 | 10.21 | 10.69 | 10.19 | 10.29 |
| Non-contiguous | N/A | 9.56 | 10.50 | 9.94 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.10.3.1-3 or Table 6.1.3.10.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.10.3.1-3 NS_61 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| Non-contiguous | N/A | ≤11.5 | ≤12.5 | ≤12.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 6.1.3.10.3.1-4 NS_61 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

### 6.1.4	Configured transmitted power for NR SL-U

The configured transmitted power for NR-U is defined in clause 6.2F.4 of TS 38.101-1 and the normal NR requirement in clause 6.2.4 of TS 38.101-1 apply. For NR V2X, PCMAX is defined for PSSCH\PSCCH, S-SSB and PSFCH in clause 6.2E.4 specifically.

It has been agreed to reuse the NR V2X Configured transmitted power requirements in 38.101-1 clause 6.2E.4 for NR SL-U.

<<<<<<<<<<< End of changes in section 6 >>>>>>>>>>

# References

[1] R4-2321771, On Tx requirements for NR sidelink evolution, LG Electronics

[2] TR 38.786 v1.1.0