3GPP TSG-RAN WG4 Meeting #109                                          R4-2321825

Chicago, USA, November 13 – 17, 2023

Title:	[Draft] LS reply on further clarifications on enhancements to realize increasing UE power high limit for CA and DC

Response to:	R4-2318078 (R2-2311611)

Release:	Rel-18

Work Item:	NR_cov_enh2

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact person:

- Name:	Xiang Gao

- E-mail Address:	gaoxiang74@huawei.com

Attachments: None

- 1		Overall description

RAN4 thanks RAN2 for the LS on further clarifications on enhancements to realize increasing UE power high limit for CA and DC in R2-2311611.

Regarding the questions from RAN2, RAN4 would like to share the following answers.

- Q1: What exact information is required to be reported by the UE (i.e., how many bits are required to support the reporting of this information)?

Answer from RAN4:

- For ΔPPowerClass reporting

- Both ΔPPowerClass for a serving cell (the reduction of the maximum output power per serving cell) and ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC for a band combination with more than one CC (reduction of the maximum total output power per band combination) should be considered.

  - For a serving cell, 2 bits are needed to cover 0, 3 and 6 dB for ΔPPowerClass.

  - For a band combination a granularity of 1 bit for ΔPPowerClass, ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC: then 0 dB and ≥[3] dB.

- For full power transmission mode capability reporting, RAN4 has no consensus in Rel-18. No bits are required for this information.

- Q2: What is the granularity of the information to be reported (e.g., per UE / per cell / other option)?

Answer from RAN4:

Option 1

- The capability of ΔPPowerClass reporting is per UE with following options

- UE to report ΔPPowerClass reported per serving cell for only single band single CC operation.

  - UE to report ΔPPowerClass reported per serving cell for only single band single CC operation as well as ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC per configured band combination and ΔPPowerClass per serving cell of configured band combination.

- Network configures the reporting of ΔPPowerClass and/or ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC for the overhead reduction purpose.

Option 2

- The capability of reporting is per UE

- For a band combination, UE can choose the two options below:

  - Reporting of ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC only

  - Reporting of ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC per band combination and ΔPPowerClass per serving cell of band combination configured

- It should also be possible for the UE indicate capability of reporting ΔPPowerClass for single serving cells only (no BC reporting).

- Network configures the reporting of ΔPPowerClass and/or ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC for the overhead reduction purpose.

- Q3: Will RAN4 specification(s) specify the triggering condition(s) when this reporting should be performed by the UE, to which RAN2 specification(s) could then refer to when writing the reporting procedure?

Answer from RAN4:

The conditions to allow UE to set a different ∆PPowerClass, ∆PPowerClass,CA, ∆PPowerClass, EN-DC and ∆PPowerClass,NR-DC values are already specified in RAN4. A UE must be configured by the network to report on this parameter. RAN4 assumes that the triggering is detailed in RAN2 specification, e.g., these changes of the maximum output power can trigger by an aperiodic report similarly to DL pathloss changes for the existing PHR (the DL pathloss also part of the UL power control), it is not specified in RAN4 specifications.

- 2		Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to consider above information for their future work on the implementation of ΔPPowerClass reporting.

- 3		Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #110	     February 26 – March 1, 2024	         Athens, Greek