3GPP TSG-RAN WG4 Meeting # 109						R4-2321856

Chicago, USA, November 13 – November 17, 2023

Title:	TP to TR 38.718-02-01 Addition to CA_n78-n102 and DC_n78-n102

Source:	Nokia, BT

Agenda item:	7.10.2

Document for:	Approval

# 1	Introduction

This is an addition to TR 38.718-02-01 to add CA/DC_78A/(2A)-n102B/C with UL interband CA and variants of non-contiguous and contiguous intra-band carrier aggregation in uplink.

This TP is dependent on fallback also submitted for same meeting in R4-2320035

************************************* Start of TP*****************************************

## 5.31		CA_n78-n102

### 5.31.1	Common for 1 band UL and 2 bands UL CA

#### 5.31.1.1 Operating bands for CA

Table 5.31.1.1-1: CA band combination of band n78+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

#### 5.31.1.2	Channel bandwidths per operating band for CA

Table 5.31.1.2-1: Supported bandwidths per CA band combination of band n78+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n78A-n102A | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78A-n102(2A) | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n78A-n102B | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | CA_n78A-n102B | n102 | CA_n102B_BCS0 |  |
| CA_n78A-n102C | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | CA_n78A-n102C | n102 | CA_n102C_BCS0 |  |
| CA_n78A-n102D | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78A-n102E | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102A | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78(2A)-n102B | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  | CA_n78A-n102B | n102 | CA_n102B_BCS0 |  |
| CA_n78(2A)-n102C | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  | CA_n78A-n102C | n102 | CA_n102C_BCS0 |  |
| CA_n78(2A)-n102D | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78(2A)-n102E | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102(2A) | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |

#### 5.31.1.3	Co-existence studies

Table 5.31.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n78-n102.

Table 5.31.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no uplink harmonic issues.

Table 5.31.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no harmonic mixing issues.

Based on above table, there is no harmonic mixing issues.

Table 5.31.1.3-3 gives a non-contiguous uplink IMD interference analysis

Table 5.31.1.3-3: Non-contiguous uplink IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2800 | 4300 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

Min channel = 10MHz

Min Channel separation = 10MHz

Max channel separation = 500MHz

Based on the table 5.x.2.2-3, there are IMD4 and IMD6 products falling inside n102 DL, which may occur in simultaneous RX/TX configurations.

#### 5.31.1.4	∆TIB and ∆RIB values

For CA_n78-n102, the DTIB,c is based on CA_n78-n79 having n79 as the highest frequency range among similar CA’s. Since this case may have simultaneous RX/TX the values with Note 8 have been used, For CA_n78-n102, the DRIB values are also based on CA_n46-n78.

Table 5.31.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n78-n102 | 1.5 | 1.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.31.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n78-n102 | 0.5 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.31.1.5	REFSENs requirements

Based on the co-existence studies there are a need to define MSD values. MSD values are made in similar way to the NR-U combination of CA_n46A-n78A with CA_n78(2A) in the UL.

Table 5.31.1.5-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n78-n102 | n78 | 3320 | 10 | 1 (RBstart=25) | 3320 | N/A | TDD | N/A |
|  | n78 | 3680 | 10 | 1 (RBstart=25) | 3680 | N/A | TDD | N/A |
|  | n102 | 6280 | 20 | 100 | 6280 | N/A11 | TDD | IMD4 |
| NOTE 11: This is a share spectrum access band, hence no MSD is defined. |  |  |  |  |  |  |  |  |

#### 5.31.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

### 5.31.2		Specific for 2 bands UL CA

#### 5.31.2.1	Maximum output power for inter-band CA

Table 5.31.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n78A-n102A | 23 | +2/-3 |

#### 5.31.2.2		UE co-existence

Table 5.31.2.2-1 gives IMD interference analysis for CA_ n78-n102.

Table 5.31.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 3800 | 5925 | 6425 |
| DL Frequency [MHz] | 3300 | 3800 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2125 | 3125 | 9225 | 10225 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 175 | 1675 | 8050 | 9550 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 12525 | 14025 | 15150 | 16650 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3475 | 5475 | 13975 | 15975 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6250 | 4250 | 18450 | 20450 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 15825 | 17825 | 21075 | 23075 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22400 | 19900 | 9275 | 6775 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 12675 | 10175 | 450 | 2950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 27000 | 29500 | 19125 | 21625 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 24375 | 26875 | 21750 | 24250 |

Based on the table 5.31.2.2-1, there are no IMD interference issues of this band combination

Since this is including a share spectrum access band there is no additional protected bands as compared to n78.

As this is a TDD-TDD combination the triple beat analysis for CA_n78A-n102B and CA_n78A-n102C will never impact any downlink of the two bands, since there’s no triple beat uplink if either band is in downlink operation.

#### 5.31.2.3		REFSENS requirements

There are no additional MSD requirements for this band combination.

********************    Removed sections unaffected         *******************************

## 7.5	DC_n78-n102

### 7.5.1	Configurations for DC_n78-n102

Table 7.5.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n78A-n102A | DC_n78A-n102A |
| DC_n78A-n102B | DC_n78A-n102B |
| DC_n78A-n102C | DC_n78A-n102C |
| DC_n78A-n102D |  |
| DC_n78A-n102E |  |
| DC_n78A-n102(2A) | DC_n78A-n102A |
| DC_n78(2A)-n102A | DC_n78A-n102A |
| DC_n78(2A)-n102B | DC_n78A-n102B |
| DC_n78(2A)-n102C | DC_n78A-n102C |
| DC_n78(2A)-n102D |  |
| DC_n78(2A)-n102E |  |
| DC_n78(2A)-n102(2A) | DC_n78A-n102A |

### 7.5.2	Maximum output power for NR-DC

Table 7.5.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n78A-n102A | 23 | +2/-3 |

************************************* End of TP*****************************************