3GPP TSG-RAN WG4 Meeting #109						     R4-2321877

Chicago, US 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38.101-3 ENDC for FR1 2BLTE1BNR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Softbank Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_2BLTE_1BNR_3DL2UL |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inter-band EN-DC configurations within FR1 (three bands) as follows. DC_3A-8B_n1A DC_8B-11A_n1A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requested Inter-band EN-DC combinations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed CA/DC are not supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.2 Table 5.5B.4.2-1 7.3B.2.3.5.2 Table 7.3B.2.3.5.2-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n1A | DC_1A_n1A2 DC_3A_n1A |
| DC_1A-3A_n3A | DC_1A_n3A DC_3A_n3A2 |
| DC_1A-(n)3AA | DC_1A_n3A |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A |
| DC_1A-3A_n7A DC_1A-3A_n7B DC_1A-3C_n7A DC_1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-1A-3A_n7A DC_1A-1A-3A_n7B DC_1A-1A-3C_n7A DC_1A-1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-3A-3A_n7A DC_1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |
| DC_1A-1A-3A-3A_n7A DC_1A-1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |
| DC_1A-3A_n8A | DC_1A_n8A DC_3A_n8A |
| DC_1A-3A_n26A DC_1A-3C_n26A | DC_1A_n26A DC_3A_n26A |
| DC_1A-3A_n28A DC_1A-3C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A-1A-3A_n28A DC_1A-1A-3C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A |
| DC_1A-3A_n38A | DC_1A_n38A DC_3A_n38A |
| DC_1A_n3A-n38A | DC_1A_n3A DC_1A_n38A |
| DC_1A-3A_n40A | DC_1A_n40A DC_3A_n40A |
| DC_1A-3A_n41A5 DC_1A-3C_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A |
| DC_1A_n3A-n41A5 | DC_1A_n3A DC_1A_n41A |
| DC_1A-3A_n71A DC_1A-3A_n71B | DC_1A_n71A DC_3A_n71A |
| DC_1A-3A_n77A5, 14 DC_1A-3A_n77C5 DC_1A-3C_n77A5,14 | DC_1A_n77A14 DC_3A_n77A14 DC_3C_n77A |
| DC_1A-3A_n77(2A)5,14 DC_1A-3C_n77(2A)5,14 | DC_1A_n77A14 DC_3A_n77A14 DC_3C_n77A |
| DC_1A-3A_n77(3A)5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5,14 DC_1A-3A_n78C5 DC_1A-3C_n78A5,14 | DC_1A_n78A14 DC_3A_n78A14 DC_3C_n78A |
| DC_1A-3A_n78(2A)514 DC_1A-3C_n78(2A)5 | DC_1A_n78A14 DC_3A_n78A14 DC_3C_n78A |
| DC_1A-3A_n78(A-C)5 | DC_1A_n78A DC_3A_n78A |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-3A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A_n3A-n8A | DC_1A_n3A DC_1A_n8A |
| DC_1A_n3A-n75A | DC_1A_n3A |
| DC_1A_n3A-n77A5 | DC_1A_n3A DC_1A_n77A |
| DC_1A_n3A-n77(2A) 5 | DC_1A_n3A DC_1A_n77A |
| DC_1A_n3A-n78A5 | DC_1A_n3A DC_1A_n78A |
| DC_1A_n3A-n78(2A)5 | DC_1A_n3A DC_1A_n78A |
| DC_1A_n3A-n79A | DC_1A_n3A DC_1A_n79A |
| DC_1A-3A_n79A5,14 DC_1A-3A_n79C5 | DC_1A_n79A14 DC_3A_n79A14 |
| DC_1A-3A_n105A | DC_1A_n105A DC_3A_n105A |
| DC_1A-5A_n40A | DC_1A_n40A DC_5A_n40A |
| DC_1A_n5A-n40A | DC_1A_n5A DC_1A_n40A |
| DC_1A-5A_n77A | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n77(2A) DC_1A-5A_n77(3A) | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n78A5 DC_1A-5A_n78C5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n78(2A)5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n78(A-C)5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n79A | DC_1A_n79A DC_5A_n79A |
| DC_1A_n5A-n78A5 | DC_1A_n5A DC_1A_n78A |
| DC_1A-7A_n1A | DC_1A_n1A DC_7A_n1A |
| DC_1A-7A_n3A DC_1A-7C_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A |
| DC_1A-7A_n5A DC_1A-7C_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-(n)7AA | DC_1A_n7A |
| DC_1A-7A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A_n20A | DC_1A_n20A DC_7A_n20A |
| DC_1A-7A_n26A | DC_1A_n26A DC_7A_n26A |
| DC_1A-7C_n26A | DC_1A_n26A DC_7A_n26A DC_7C_n26A |
| DC_1A-7A_n28A5 DC_1A-7C_n28A5 | DC_1A_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-1A-7A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A-7A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n77(2A) DC_1A-7A_n77(3A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A_n77(2A) DC_1A-7A-7A_n77(3A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n78A5 DC_1A-7C_n78A5 DC_1A-7A_n78C5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(2A)5 DC_1A-7C_n78(2A)5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(A-C)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-1A-7A_n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78A5 DC_1A-7A-7A_n78C5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78(2A)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78(A-C)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | DC_1A_n7A DC_1A_n78A |
| DC_1A_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A |
| DC_1A-7A_n105A | DC_1A_n105A DC_7A_n105A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n7A | DC_8A_n7A DC_1A_n7A |
| DC_1A-8A_n20A | DC_1A_n20A DC_8A_n20A |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A |
| DC_1A-8A_n40A | DC_1A_n40A DC_8A_n40A |
| DC_1A_n8A-n40A | DC_1A_n8A DC_1A_n40A |
| DC_1A-8A_n77A5,14 | DC_1A_n77A14 DC_8A_n77A14 |
| DC_1A-8A_n77(2A)5,14 | DC_1A_n77A14 DC_8A_n77A14 |
| DC_1A_n8A-n77A | DC_1A_n8A DC_1A_n77A |
| DC_1A_n8A-n77(2A) | DC_1A_n8A DC_1A_n77A |
| DC_1A-8A_n77(3A)5 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n78A5,14 | DC_1A_n78A14 DC_8A_n78A14 |
| DC_1A-8A_n78(2A)5,14 | DC_1A_n78A14 DC_8A_n78A14 |
| DC_1A_n8A-n78A5 | DC_1A_n8A DC_1A_n78A |
| DC_1A-8A_n79A5 | DC_1A_n79A DC_8A_n79A |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A |
| DC_1A-11A_n28A | DC_1A_n28A DC_11A_n28A |
| DC_1A-11A_n41A5 | DC_1A_n41A DC_11A_n41A |
| DC_1A-11A_n77A5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(2A)5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(3A)5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n78A5 | DC_1A_n78A DC_11A_n78A |
| DC_1A-11A_n78(2A)5 | DC_1A_n78A DC_11A_n78A |
| DC_1A-11A_n79A5 | DC_1A_n79A DC_11A_n79A |
| DC_1A-18A_n3A | DC_1A_n3A DC_18A_n3A |
| DC_1A-18A_n28A | DC_1A_n28A DC_18A_n28A |
| DC_1A-18A_n41A | DC_1A_n41A DC_18A_n41A |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n77(2A)5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n78A5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n78(2A)5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A_n77A5,14 DC_1A-19A_n77C5 | DC_1A_n77A14 DC_19A_n77A14 |
| DC_1A-19A_n77(2A)5,14 | DC_1A_n77A14 DC_19A_n77A14 |
| DC_1A-19A_n78A5 DC_1A-19A_n78C5 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n78(2A)5 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n79A514 DC_1A-19A_n79C5 | DC_1A_n79A14 DC_19A_n79A14 |
| DC_1A-20A_n1A | DC_1A_n1A2 DC_20A_n1A |
| DC_1A-20A_n3A DC_1C-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n7A | DC_1A_n7A DC_20A_n7A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n38A | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n41A | DC_1A_n41A DC_20A_n41A |
| DC_1A-20A_n78A5 DC_1A-20A_n78C5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n78(2A)5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n28A13 | DC_1A_n28A DC_21A_n28A |
| DC_1A-21A_n77A5, 14 DC_1A-21A_n77C5, 14 | DC_1A_n77A14 DC_21A_n77A14 |
| DC_1A-21A_n77(2A)5,14 | DC_1A_n77A14 DC_21A_n77A14 |
| DC_1A-21A_n78A5,14 DC_1A-21A_n78C5 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n78(2A)5,14 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n79A5,14 DC_1A-21A_n79C5 | DC_1A_n79A14 DC_21A_n79A14 |
| DC_1A-26A_n78A | DC_1A_n78A DC_26A_n78A |
| DC_1A-26A_n78(2A) | DC_1A_n78A DC_26A_n78A |
| DC_1A_n26A-n78A | DC_1A_n26A DC_1A_n78A |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-28A_n20A22 | DC_1A_n20A DC_28A_n20A22 |
| DC_1A-28A_n38A | DC_1A_n38A DC_28A_n38A |
| DC_1A_n28A-n38A | DC_1A_n28A DC_1A_n38A |
| DC_1A_n28A-n40A | DC_1A_n28A DC_1A_n40A |
| DC_1A-28A_n40A | DC_1A_n40A DC_28A_n40A |
| DC_1A_n28A-n41A5 | DC_1A_n28A DC_1A_n41A |
| DC_1A_n28A-n75A | DC_1A_n28A |
| DC_1A-28A_n77A5 DC_1A-28A_n77C5 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A_n78A5 DC_1A-28A_n78C5 | DC_1A_n78A DC_28A_n78A |
| DC_1A-1A-28A_n78A | DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n78(2A) | DC_1A_n78A DC_28A_n78A |
| DC_1A_n28A-n77A5 | DC_1A_n28A DC_1A_n77A |
| DC_1A_n28A-n77(2A)5 | DC_1A_n28A DC_1A_n77A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A_n28A-n78(2A)5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A5 DC_1A-28A_n79C5 | DC_1A_n79A DC_28A_n79A |
| DC_1A_n28A-n79A5 | DC_1A_n28A DC_1A_n79A |
| DC_1A-32A_n3A | DC_1A_n3A |
| DC_1A-32A_n8A | DC_1A_n8A |
| DC_1A-32A_n28A | DC_1A_n28A |
| DC_1A-32A_n78A DC_1A-32A_n78C | DC_1A_n78A |
| DC_1A-32A_n78(2A) | DC_1A_n78A |
| DC_1A-38A_n3A | DC_1A_n3A |
| DC_1A-38A_n8A | DC_1A_n8A DC_38A_n8A |
| DC_1A-38A_n28A | DC_1A_n28A DC_38A_n28A |
| DC_1A-(n)38AA | DC_1A_n38A |
| DC_1A_n38A-n78A | DC_1A_n38A DC_1A_n78A |
| DC_1A-38A_n78A | DC_1A_n78A |
| DC_1A-38A_n78(2A) | DC_1A_n78A |
| DC_1A_n40A-n77A | DC_1A_n40A DC_1A_n77A |
| DC_1A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A |
| DC_1A-40A_n78A DC_1A-40C_n78A | DC_1A_n78A DC_40A_n78A |
| DC_1A-40A_n78(2A) DC_1A-40C_n78(2A) | DC_1A_n78A DC_40A_n78A |
| DC_1A_n40A-n78A DC_1A_n40A-n78C | DC_1A_n40A DC_1A_n78A |
| DC_1A_n40A-n78(2A) | DC_1A_n40A DC_1A_n78A |
| DC_1A-41A_n3A5 DC_1A-41C_n3A5 | DC_1A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-41A_n28A5 DC_1A-41C_n28A5 | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-(n)41AA DC_1A-(n)41CA DC_1A-(n)41DA | DC_1A_n41A |
| DC_1A-41A_n41A DC_1A-41C_n41A | DC_1A_n41A |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | DC_1A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A_n41A-n77A | DC_1A_n41A DC_1A_n77A |
| DC_1A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A_n41A-n78A | DC_1A_n41A DC_1A_n78A |
| DC_1A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A |
| DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-41A_n79A5 DC_1A-41C_n79A5 | DC_1A_n79A |
| DC_1A-42A_n3A5 | DC_1A_n3A DC_42A_n3A |
| DC_1A-42C_n3A5 | DC_1A_n3A DC_42A_n3A DC_42C_n3A |
| DC_1A-42A_n28A5 | DC_1A_n28A DC_42A_n28A |
| DC_1A-42C_n28A5 | DC_1A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-42A_n77A14, 15,16 DC_1A-42A_n77C15,16 DC_1A-42C_n77A14, 15,16 DC_1A-42C_n77C15,16 DC_1A-42D_n77A14, 15,16 DC_1A-42D_n77C15,16 DC_1A-42E_n77A14, 15,16 DC_1A-42E_n77C15,16 | DC_1A_n77A14, |
| DC_1A-42A_n77(2A)15,16 DC_1A-42C_n77(2A)15,16 | DC_1A_n77A |
| DC_1A-42A_n78A14,15,16 DC_1A-42A_n78C15,16 DC_1A-42C_n78A14,15,16 DC_1A-42C_n78C15,16 DC_1A-42D_n78A14,15,16 DC_1A-42D_n78C15,16 DC_1A-42E_n78A14,15,16 DC_1A-42E_n78C15,16 | DC_1A_n78A14 |
| DC_1A-42A_n79A14 DC_1A-42A_n79C DC_1A-42C_n79A14 DC_1A-42C_n79C DC_1A-42D_n79A14 DC_1A-42D_n79C DC_1A-42E_n79A14 DC_1A-42E_n79C | DC_1A_n79A14 |
| DC_1A_n75A-n78A | DC_1A_n78A |
| DC_1A_n75A-n78(2A) | DC_1A_n78A |
| DC_1A_n77A-n79A14, 23 | DC_1A_n77A14 DC_1A_n79A14 |
| DC_1A_n77(2A)-n79A23 | DC_1A_n77A DC_1A_n79A |
| DC_1A_SUL_n77A-n80A | DC_1A_n77A DC_1A_n80A |
| DC_1A_SUL_n77A-n84A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |
| DC_1A_n78A-n79A14, 24 | DC_1A_n78A14 DC_1A_n79A14 |
| DC_1A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A |
| DC_1A_SUL_n78A-n84A5 | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A |
| DC_1A_SUL_n79A-n84A | DC_1A_n79A, DC_1A_n84A_ULSUP-TDM_n79A |
| DC_1A_n78A-n105A | DC_1A_n78A DC_1A_n105A |
| DC_2A_n2A-n38A | DC_2A_n38A |
| DC_2A_n2A-n41A | DC_2A_n41A |
| DC_2A_n2A-n66A | DC_2A_n66A |
| DC_2A_n2A-n71A | DC_2A_n71A |
| DC_2A_n2A-n77A14 DC_2A_n2A-n77C14 | DC_2A_n77A14 |
| DC_2A_n2A-n78A | DC_2A_n78A |
| DC_2A-4A_n28A | DC_2A_n28A DC_4A_n28A |
| DC_2A-4A_n38A | DC_2A_n38A DC_4A_n38A |
| DC_2A-4A_n41A | DC_2A_n41A DC_4A_n41A |
| DC_2A-4A_n78A | DC_2A_n78A DC_4A_n78A |
| DC_2A-5A_n2A | DC_5A_n2A DC_2A_n2A2 |
| DC_2A-5B_n2A | DC_5A_n2A |
| DC_2A-5A-5A_n2A | DC_5A_n2A |
| DC_2A-5A_n5A | DC_2A_n5A |
| DC_2A-2A-5A_n5A | DC_2A_n5A |
| DC_2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |
| DC_2A-2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |
| DC_2A-5A_n7A | DC_2A_n7A DC_5A_n7A |
| DC_2A-2A-5A_n7A | DC_2A_n7A DC_5A_n7A |
| DC_2A-5A_n12A | DC_2A_n12A DC_5A_n12A |
| DC_2A-5A_n30A | DC_2A_n30A DC_5A_n30A |
| DC_2A-2A-5A_n30A | DC_2A_n30A DC_5A_n30A |
| DC_2A-5A_n41A | DC_2A_n41A DC_5A_n41A |
| DC_2A-2A-5A_n41A | DC_2A_n41A DC_5A_n41A |
| DC_2A-5A_n48A DC_2A-5A_n48B | DC_2A_n48A DC_5A_n48A |
| DC_2A-5A_n66A DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A_n66A DC_2A-2A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A_n71A | DC_2A_n71A DC_5A_n71A |
| DC_2A-5A_n77A14 DC_2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-5A_n77(2A)14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-2A-5A_n77A14 DC_2A-2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-2A-5A_n77(2A)14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-5A_n78A | DC_2A_n78A DC_5A_n78A |
| DC_2A-2A-5A_n78A | DC_2A_n78A DC_5A_n78A |
| DC_2A-5A_n78(2A) | DC_2A_n78A DC_5A_n78A |
| DC_2A-7A_n2A | DC_7A_n2A |
| DC_2A-7A_n5A DC_2A-7C_n5A | DC_2A_n5A DC_7A_n5A |
| DC_2A-7A-7A_n5A | DC_2A_n5A DC_7A_n5A |
| DC_2A-7A_n7A | DC_2A_n7A DC_7A_n7A2 |
| DC_2A-7A_n12A | DC_2A_n12A DC_7A_n12A |
| DC_2A-2A-7A_n12A | DC_2A_n12A DC_7A_n12A |
| DC_2A-7A_n25A15, 16 DC_2A-7A-7A_n25A15, 16 DC_2A-7C_n25A15, 16 | DC_7A_n25A |
| DC_2A-7A_n28A DC_2A-7C_n28A | DC_2A_n28A DC_7A_n28A |
| DC_2A_n5A-n77A14 DC_2A-2A_n5A-n77A14 DC_2A_n5A-n77C14 DC_2A-2A_n5A-n77C14 | DC_2A_n5A DC_2A_n77A14 |
| DC_2A-7A_n66A DC_2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A_n7A-n66A | DC_2A_n7A DC_7A_n66A |
| DC_2A_n7(2A)-n66A | DC_7A_n66A |
| DC_2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-7A_n77A DC_2A-7C_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-2A-7A_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-7A_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-7A_n77(2A) | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A_n78A5,14 DC_2A-7C_n78A5,14 | DC_2A_n78A14 DC_7A_n78A14 DC_7C_n78A |
| DC_2A-7A_n78(2A) 5,14 DC_2A-7C_n78(2A) 5,14 | DC_2A_n78A14 DC_7A_n78A14 DC_7C_n78A |
| DC_2A-2A-7A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A_n7A-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7A-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A-7A-7A_n78A5,14 | DC_2A_n78A14 DC_7A_n78A14 |
| DC_2A-7A-7A_n78(2A) 5,14 | DC_2A_n78A14 DC_7A_n78A14 |
| DC_2A-8A_n2A | DC_2A_n2A2 DC_8A_n2A |
| DC_2A-12A_n2A | DC_12A_n2A |
| DC_2A-12A_n5A | DC_2A_n5A DC_12A_n5A |
| DC_2A-2A-12A_n5A | DC_2A_n5A DC_12A_n5A |
| DC_2A-12A_n7A | DC_2A_n7A DC_12A_n7A |
| DC_2A-2A-12A_n7A | DC_2A_n7A DC_12A_n7A |
| DC_2A-12A_n7(2A) | DC_2A_n7A DC_12A_n7A |
| DC_2A-(n)12AA | DC_2A_n12A DC_(n)12AA2 |
| DC_2A-12A_n30A | DC_2A_n30A DC_12A_n30A |
| DC_2A-2A-12A_n30A | DC_2A_n30A DC_12A_n30A |
| DC_2A-12A_n41A | DC_2A_n41A DC_12A_n41A |
| DC_2A-2A-12A_n41A | DC_2A_n41A DC_12A_n41A |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-12A_n77A14 DC_2A-2A-12A_n77A14 | DC_2A_n77A14 DC_12A_n77A14 |
| DC_2A-12A_n77(2A)14 DC_2A-2A-12A_n77(2A)14 | DC_2A_n77A14 DC_12A_n77A14 |
| DC_2A_n12A-n77A | DC_2A_n77A DC_2A_n12A |
| DC_2A-2A_n12A-n77A | DC_2A_n12A DC_2A_n77A |
| DC_2A_n12A-n78A | DC_2A_n12A DC_2A_n78A |
| DC_2A-13A_n2A | DC_13A_n2A |
| DC_2A-12A_n78A | DC_2A_n78A DC_12A_n78A |
| DC_2A-12A_n78(2A) | DC_2A_n78A DC_12A_n78A |
| DC_2A-2A-12A_n78A | DC_2A_n78A DC_12A_n78A |
| DC_2A-13A_n5A | DC_2A_n5A |
| DC_2A-2A-13A_n5A | DC_2A_n5A |
| DC_2A-13A_n25A16,20 | DC_13A_n25A |
| DC_2A-13A_n48A DC_2A-13A_n48B | DC_2A_n48A DC_13A_n48A |
| DC_2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-13A_n77A14 DC_2A-13A_n77C14 DC_2A-2A-13A_n77C14 | DC_2A_n77A14 DC_13A_n77A14 |
| DC_2A-2A-13A_n77A | DC_2A_n77A14 DC_13A_n77A14 |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-14A_n30A | DC_2A_n30A DC_14A_n30A |
| DC_2A-2A-14A_n30A | DC_2A_n30A DC_14A_n30A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-14A_n77A14 DC_2A-2A-14A_n77A14 | DC_2A_n77A14 DC_14A_n77A14 |
| DC_2A-14A_n77(2A) 14 DC_2A-2A-14A_n77(2A) 14 | DC_2A_n77A14 DC_14A_n77A14 |
| DC_2A_n25A-n66A | DC_2A_n66A |
| DC_2A-28A_n7A | DC_2A_n7A DC_28A_n7A |
| DC_2A-28A_n66A | DC_2A_n66A DC_28A_n66A |
| DC_2A-28A_n78A | DC_2A_n78A DC_28A_n78A |
| DC_2A-28A_n78(2A) | DC_2A_n78A DC_28A_n78A |
| DC_2A-29A_n30A | DC_2A_n30A |
| DC_2A-2A-29A_n30A | DC_2A_n30A |
| DC_2A-29A_n66A | DC_2A_n66A |
| DC_2A-2A-29A_n66A | DC_2A_n66A |
| DC_2A-29A_n77A14 DC_2A-2A-29A_n77A14 | DC_2A_n77A14 |
| DC_2A-29A_n78A | DC_2A_n78A |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n2A | DC_2A_n2A2 DC_30A_n2A |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-30A_n77A14 DC_2A-2A-30A_n77A14 | DC_2A_n77A14 DC_30A_n77A14 |
| DC_2A-30A_n77(2A) 14 DC_2A-2A-30A_n77(2A) 14 | DC_2A_n77A14 DC_30A_n77A14 |
| DC_2A_n38A-n66A | DC_2A_n38A DC_2A_n66A |
| DC_2A_n38A-n71A | DC_2A_n38A DC_2A_n71A |
| DC_2A-38A_n78A | DC_2A_n78A DC_38A_n78A |
| DC_2A_n38A-n78A | DC_2A_n38A DC_2A_n78A |
| DC_2A_n41A-n66A DC_2A_n41C-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A-2A_n41A-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41(2A)-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41A-n71A DC_2A_n41C-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-2A_n41A-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n2A3 DC_2A-46C_n2A3 DC_2A-46D_n2A3 DC_2A-46E_n2A3 | DC_2A_n2A2 |
| DC_2A-46A_n5A3 DC_2A-46C_n5A3 DC_2A-46D_n5A3 DC_2A-46E_n5A3 DC_2A-2A-46A_n5A3 DC_2A-2A-46C_n5A3 DC_2A-2A-46D_n5A3 | DC_2A_n5A |
| DC_2A-46A_n41A DC_2A-46C_n41A DC_2A-46D_n41A | DC_2A_n41A |
| DC_2A-46A_n41(2A) DC_2A-46C_n41(2A) DC_2A-46D_n41(2A) | DC_2A_n41A |
| DC_2A-46A_n66A DC_2A-46C_n66A DC_2A-46D_n66A DC_2A-46E_n66A | DC_2A_n66A |
| DC_2A-46A_n71A DC_2A-46C_n71A DC_2A-46D_n71A | DC_2A_n71A |
| DC_2A-46A_n77A | DC_2A_n77A |
| DC_2A-46A-46A_n77A | DC_2A_n77A |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | DC_2A_n2A2 DC_48A_n2A21 |
| DC_2A-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-48C_n5A DC_2A-48D_n5A DC_2A-48E_n5A | DC_2A_n5A |
| DC_2A_n48A-n66A | DC_2A_n48A DC_2A_n66A |
| DC_2A-48A_n71A | DC_2A_n71A DC_48A_n71A |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A |
| DC_2A-48A_n48A | DC_2A_n48A |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A DC_2A-48E_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-48A_n77A14,15,16 | DC_2A_n77A14 |
| DC_2A-48A-48A_n77A14,15,16 | DC_2A_n77A DC_48A_n77A |
| DC_2A-48A-48A-48A_n77A14,15,16 | DC_2A_n77A DC_48A_n77A |
| DC_2A-48C_n77A14,15,16 DC_2A-48D_n77A14,15,16 DC_2A-48E_n77A14,15,16 DC_2A-48A_n77C14,15,16 DC_2A-48C_n77C14,15,16 DC_2A-48D_n77C14,15,16 | DC_2A_n77A14 |
| DC_2A-66A_n2A | DC_2A_n2A2 DC_66A_n2A |
| DC_2A-66A-66A_n2A | DC_66A_n2A |
| DC_2A-66A_n5A DC_2A-66B_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-2A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-66A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-66A_n12A | DC_2A_n12A DC_66A_n12A |
| DC_2A-66A_n25A16,20 | DC_66A_n25A |
| DC_2A-66A_n28A | DC_2A_n28A DC_66A_n28A |
| DC_2A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A_n41A14 DC_2A-66A_n41C DC_2C-66A_n41A | DC_2A_n41A DC_66A_n41A14 |
| DC_2A-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-2A-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-66A-66A_n66A DC_2A-66B_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-(n)66AA | DC_2A_n66A |
| DC_2A-2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-2A-66A-66A_n66A | DC_2A_n66A |
| DC_2A-66A_n71A DC_2A-66A_n71B DC_2A-66C_n71A DC_2C-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-66A_n77A14 DC_2A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A_n77(2A)14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A_n77A14 DC_2A-2A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A_n77(2A) 14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A-66A_n77A14 DC_2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A-66A_n77(2A) 14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A-66A_n77A14 DC_2A-2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A_n66A-n77A14 DC_2A_n66A-n77C14 DC_2A-2A_n66A-n77A14 DC_2A-2A_n66A-n77C14 | DC_2A_n77A14 DC_2A_n66A |
| DC_2A-66A_n78A5,14 DC_2A-2A-66A_n78A | DC_2A_n78A14 DC_66A_n78A14 |
| DC_2A-66A_n78(2A) 5,14 | DC_2A_n78A14 DC_66A_n78A14 |
| DC_2A_n66A-n78A DC_2A-2A_n66A-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66A-n78(2A) | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66(2A)-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66(2A)-n78(2A) | DC_2A_n66A DC_2A_n78A |
| DC_2A-66A-66A_n78A5,14 | DC_2A_n78A14 DC_66A_n78A14 |
| DC_2A-66A-66A_n78(2A) 5,14 | DC_2A_n78A14 DC_66A_n78A14 |
| DC_2A-71A_n2A | DC_71A_n2A |
| DC_2A-71A_n7A | DC_2A_n7A DC_71A_n7A |
| DC_2A-2A-71A_n7A | DC_2A_n7A DC_71A_n7A |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-71A_n41A | DC_2A_n41A DC_71A_n41A |
| DC_2A-2A-71A_n41A | DC_2A_n41A DC_71A_n41A |
| DC_2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-71A_n71A | DC_2A_n71A |
| DC_2A-71A_n77A | DC_2A_n77A DC_71A_n77A |
| DC_2A-2A-71A_n77A | DC_2A_n77A DC_71A_n77A |
| DC_2A-71A_n77(2A) | DC_2A_n77A DC_71A_n77A |
| DC_2A_n71A-n77A | DC_2A_n71A DC_2A_n77A |
| DC_2A-2A_n71A-n77A | DC_2A_n71A DC_2A_n77A |
| DC_2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A-71A_n78(2A) | DC_71A_n78A DC_2A_n78A |
| DC_2A-2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A_n71A-n78A | DC_2A_n71A DC_2A_n78A |
| DC_2A-2A_n71A-n78A | DC_2A_n71A DC_2A_n78A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_3A_n1A-n7A | DC_3A_n1A DC_3A_n7A |
| DC_3C_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_3C_n1A DC_3C_n7A |
| DC_3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |
| DC_3A-3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |
| DC_3A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n28A DC_3C_n1A |
| DC_3A_n1A-n38A | DC_3A_n1A DC_3A_n38A |
| DC_3A_n1A-n40A | DC_3A_n1A DC_3A_n40A |
| DC_3A_n1A-n41A | DC_3A_n1A DC_3A_n41A |
| DC_3A_n1A-n75A | DC_3A_n1A |
| DC_3C_n1A-n75A | DC_3C_n1A |
| DC_3A_n1A-n77A5 | DC_3A_n1A DC_3A_n77A |
| DC_3A_n1A-n78A5, 14 DC_3C_n1A-n78A5 | DC_3A_n1A DC_3C_n1A DC_3A_n78A14 DC_3C_n78A |
| DC_3A_n1A-n78(2A)5 DC_3C_n1A-n78(2A)5 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A |
| DC_3A-3A_n1A-n78A5,14 | DC_3A_n1A DC_3A_n78A14 |
| DC_3A_n1A-n79A5 | DC_3A_n1A DC_3A_n79A |
| DC_(n)3AA-n7A | DC_(n)3AA2 DC_3A_n7A |
| DC_3A_n3A-n7A | DC_3A_n3A2 DC_3A_n7A |
| DC_(n)3AA-n8A | DC_(n)3AA2 DC_3A_n8A |
| DC_(n)3AA-n28A | DC_(n)3AA2 DC_3A_n28A |
| DC_3A_n3A-n28A | DC_3A_n3A2 DC_3A_n28A |
| DC_3A_n3A-n41A | DC_3A_n41A DC_3A_n3A2 |
| DC_(n)3AA-n67A | DC_(n)3AA2 |
| DC_3A_n3A-n67A | DC_3A_n3A2 |
| DC_3A_n3A-n77A5 | DC_3A_n77A DC_3A_n3A2 |
| DC_(n)3AA-n77A | DC_(n)3AA2 DC_3A_n77A |
| DC_(n)3AA-n77(2A) | DC_(n)3AA2 DC_3A_n77A |
| DC_(n)3AA-n78A | DC_(n)3AA1 DC_3A_n78A |
| DC_(n)3AA-n78(2A) | DC_(n)3AA1 DC_3A_n78A |
| DC_3A_n3A-n78A5 | DC_3A_n78A DC_3A_n3A2 |
| DC_3A-5A_n40A | DC_3A_n40A DC_5A_n40A |
| DC_3A_n5A-n40A | DC_3A_n5A DC_3A_n40A |
| DC_3A-5A_n77A | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n77(2A) DC_3A-5A_n77(3A) | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n78A5 DC_3C-5A_n78A DC_3A-5A_n78C5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-5A_n78(2A)5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-5A_n78(A-C)5 | DC_3A_n78A DC_5A_n78A |
| DC_3A_n5A-n78A5, 14 DC_3C_n5A-n78A5, 14 | DC_3A_n5A DC_3A_n78A14 DC_3C_n78A14 |
| DC_3A-5A_n79A5 | DC_3A_n79A DC_5A_n79A |
| DC_3A-7A_n1A DC_3A-7C_n1A DC_3C-7A_n1A DC_3C-7C_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A |
| DC_3A-3A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n3A DC_3A-7C_n3A | DC_3A_n3A2 DC_7A_n3A |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_7A_n5A DC_7C_n5A |
| DC_3A-7A_n7A DC_3C-7A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A2 |
| DC_3A-3A-7A_n7A | DC_3A_n7A DC_7A_n7A2 |
| DC_3A-(n)7AA DC_3C-(n)7AA | DC_3A_n7A |
| DC_3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A_n26A DC_3A-7C_n26A DC_3C-7A_n26A DC_3C-7C_n26A | DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_3A-7A-7A_n28A | DC_3A_n28A DC_7A_n28A |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n77(2A) DC_3A-7A_n77(3A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77(2A) DC_3A-7A-7A_n77(3A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n78A5,14 DC_3C-7A_n78A5,14 DC_3A-7C_n78A5,14 DC_3C-7C_n78A5,14 DC_3A-7A_n78C5 | DC_3A_n78A14 DC_3C_n78A14 DC_7A_n78A14 DC_7C_n78A14 |
| DC_3A_n7A-n28A DC_3C_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n28A DC_3C_n7A |
| DC_3A-7A_n78(2A)5 DC_3C-7A_n78(2A)5 DC_3A-7C_n78(2A)5 DC_3C-7C_n78(2A)5 | DC_3A_n78A DC_7A_n78A DC_3C_n78A DC_7C_n78A |
| DC_3A-7A_n78(A-C)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A_n78A5, 14 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A-7A-7A_n78A5, 14 DC_3A-7A-7A_n78C5 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A-7A-7A_n78(2A)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-7A-7A_n78(A-C)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A-7A_n78A5, 14 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A_n7A-n78A5 DC_3A_n7B-n78A5 DC_3C_n7A-n78A5 DC_3C_n7B-n78A5 | DC_3A_n7A DC_3C_n7A DC_3A_n78A DC_3C_n78A |
| DC_3A-3A_n7A-n78A5 DC_3A-3A_n7B-n78A5 | DC_3A_n7A DC_3A_n7B DC_3A_n78A |
| DC_3A_n7A-n78(2A)5 DC_3C_n7A-n78(2A)5 | DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_3A-7A_n105A | DC_3A_n105A DC_7A_n105A |
| DC_3A-8A_n1A DC_3A-8B_n1A DC_3C-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-3A-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A_n7A | DC_3A_n7A DC_8A_n7A |
| DC_3A-3A_n8A-n78A5 | DC_3A_n8A DC_3A_n78A |
| DC_3A_n8A-n40A | DC_3A_n8A DC_3A_n40A |
| DC_3A-8A_n41A | DC_3A_n41A DC_8A_n41A |
| DC_3A_n8A-n41A | DC_3A_n41A DC_3A_n8A |
| DC_3A-8A_n28A DC_3C-8A_n28A | DC_3A_n28A DC_3C_n28A DC_8A_n28A |
| DC_3A-8A_n40A | DC_3A_n40A DC_8A_n40A |
| DC_3A-8A_n77A5,14 DC_3C-8A_n77A5,14 | DC_3A_n77A14 DC_3C_n77A DC_8A_n77A14 |
| DC_3A-8A_n77(2A) 5, 14 DC_3C-8A_n77(2A) 5,14 | DC_3A_n77A14 DC_3C_n77A DC_8A_n77A14 |
| DC_3A-8A_n77(3A) 5 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n78A5, 14 DC_3C-8A_n78A5,14 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-8A_n78(2A) 5,145 DC_3C-8A_n78(2A)5 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-3A-8A_n78A5, 14 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-8B_n78A5 | DC_3A_n78A DC_8A_n78A DC_8B_n78A |
| DC_3A-3A-8B_n78A5 | DC_3A_n78A DC_8A_n78A DC_8B_n78A |
| DC_3A-8A_n79A5 DC_3A-8A_n79C5 | DC_3A_n79A DC_8A_n79A |
| DC_3A_n8A-n77A5 | DC_3A_n8A DC_3A_n77A |
| DC_3A_n8A-n77(2A)5 | DC_3A_n8A DC_3A_n77A |
| DC_3A_n8A-n78A5 | DC_3A_n8A DC_3A_n78A |
| DC_3A-11A_n28A | DC_3A_n28A DC_11A_n28A |
| DC_3A-11A_n77A5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-11A_n77(2A) 5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-11A_n77(3A) 5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-18A_n3A | DC_3A_n3A2 DC_18A_n3A |
| DC_3A-18A_n28A | DC_3A_n28A DC_18A_n28A |
| DC_3A-18A_n41A | DC_3A_n41A DC_18A_n41A |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n77(2A) | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n78(2A) | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n1A | DC_3A_n1A DC_19A_n1A |
| DC_3A-19A_n77A5,14 DC_3A-19A_n77C5 | DC_3A_n77A14 DC_19A_n77A14 |
| DC_3A-19A_n77(2A)5,14 | DC_3A_n77A14 DC_19A_n77A14 |
| DC_3A-19A_n78A5,14 DC_3A-19A_n78C5 | DC_3A_n78A14 DC_19A_n78A14 |
| DC_3A-19A_n78(2A)5,14 | DC_3A_n78A14 DC_19A_n78A14 |
| DC_3A-19A_n79A5,14 DC_3A-19A_n79C5 | DC_3A_n79A14 DC_19A_n79A14 |
| DC_3A-20A_n1A DC_3C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-3A-20A_n1A | DC_3A_n1A DC_20A_n1A |
| DC_3A-20A_n3A | DC_3A_n3A2 DC_20A_n3A |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A |
| DC_3A-20A_n8A | DC_3A_n8A DC_20A_n8A |
| DC_3A-20A_n28A5,6,16,20 DC_3C-20A_n28A5,6,16,20 | DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-20A_n41A | DC_3A_n41A DC_20A_n41A |
| DC_3C-20A_n41A | DC_3C_n41A DC_20A_n41A |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_3A_n20A-n67A DC_3C_n20A-n67A | DC_3A_n20A |
| DC_3A-20A_n78A5 DC_3C-20A_n78A5 DC_3A-20A_n78C5 | DC_3A_n78A DC_3C_n78A DC_20A_n78A |
| DC_3A-3A-20A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A_n78(2A)5 | DC_3A_n78A DC_20A_n78A |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A |
| DC_3A-21A_n1A10,11 | DC_3A_n1A DC_21A_n1A |
| DC_3A-21A_n28A13 | DC_3A_n28A DC_21A_n28A |
| DC_3A-21A_n77A5, 14 DC_3A-21A_n77C5, 14 | DC_3A_n77A14 DC_21A_n77A14 |
| DC_3A-21A_n77(2A)5,14 | DC_3A_n77A14 DC_21A_n77A14 |
| DC_3A-21A_n78A5,14 DC_3A-21A_n78C5 | DC_3A_n78A14 DC_21A_n78A14 |
| DC_3A-21A_n78(2A)5,14 | DC_3A_n78A14 DC_21A_n78A14 |
| DC_3A-21A_n79A5,14 DC_3A-21A_n79C5 | DC_3A_n79A14 DC_21A_n79A14 |
| DC_3A-26A_n78ADC_3C-26A_n78A | DC_3A_n78A DC_26A_n78A |
| DC_3A-26A_n78(2A) DC_3C-26A_n78(2A) | DC_3A_n78A DC_26A_n78A |
| DC_3A_n26A-n78A | DC_3A_n26A DC_3A_n78A |
| DC_3C_n26A-n78A | DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |
| DC_3A-28A_n1A DC_3C-28A_n1A | DC_28A_n1A DC_3A_n1A |
| DC_3A-28A_n3A | DC_3A_n3A2 DC_28A_n3A |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_28A_n5A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A-28A_n40A | DC_3A_n40A DC_28A_n40A |
| DC_3A-3A-28A_n7A DC_3A-3A-28A_n7B | DC_3A_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A-28A_n38A | DC_3A_n38A DC_28A_n38A |
| DC_3A_n28A-n38A | DC_3A_n28A DC_3A_n38A |
| DC_3A_n28A-n40A | DC_3A_n28A DC_3A_n40A |
| DC_3A_n28A-n41A5 | DC_3A_n28A DC_3A_n41A |
| DC_3A-28A_n41A5 | DC_3A_n41A DC_28A_n41A |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | DC_3A_n28A DC_3C_n28A |
| DC_3A-28A_n77A5 DC_3A-28A_n77C5 | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A_n77(2A)5 | DC_3A_n77A DC_28A_n77A |
| DC_3A_n28A-n77A5,14 | DC_3A_n28A DC_3A_n77A14 |
| DC_3A_n28A-n77(2A)5 | DC_3A_n28A DC_3A_n77A |
| DC_3A-28A_n78A5,14 DC_3C-28A_n78A5,14 DC_3A-28A_n78(2A)5,14 DC_3A-28A_n78C5 | DC_3A_n78A14 DC_3C_n78A14 DC_28A_n78A14 |
| DC_3A-3A-28A_n78A | DC_3A_n78A DC_28A_n78A |
| DC_3C-28A_n78(2A)5 | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5, 14 DC_3C_n28A-n78A5, 14 | DC_3A_n28A DC_3C_n28A DC_3A_n78A14 DC_3C_n78A14 |
| DC_3A_n28A-n78(2A)5 DC_3C_n28A-n78(2A)5 | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A |
| DC_3A-28A_n79A5 DC_3A-28A_n79C5 | DC_3A_n79A DC_28A_n79A |
| DC_3A_n28A-n79A5 | DC_3A_n28A DC_3A_n79A |
| DC_3A-32A_n1A DC_3C-32A_n1A | DC_3A_n1A DC_3C_n1A |
| DC_3A-32A_n7A | DC_3A_n7A |
| DC_3A-32A_n28A DC_3C-32A_n28A | DC_3A_n28A DC_3C_n28A |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C | DC_3A_n78A DC_3C_n78A |
| DC_3A-32A_n78(2A) | DC_3A_n78A DC_3C_n78A |
| DC_3A-38A_n28A DC_3C-38A_n28A | DC_3A_n28A DC_3C_n28A DC_38A_n28A |
| DC_3A_n38A-n40A25 | DC_3A_n38A DC_3A_n40A |
| DC_3A-38A_n78A | DC_3A_n78A |
| DC_3A-38A_n78(2A) | DC_3A_n78A |
| DC_3A_n38A-n78A | DC_3A_n38A DC_3A_n78A |
| DC_3C-38A_n78A DC_3C-38A_n78(2A) | DC_3A_n78A DC_3C_n78A DC_38A_n78A |
| DC_3A-40A_n1A DC_3A-40C_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A_n40A-n41A DC_3A_n40A-n41C | DC_3A_n40A DC_3A_n41A |
| DC_3A-40A_n77A DC_3A-40C_n77A | DC_3A_n77A DC_40A_n77A |
| DC_3A_n40A-n77A | DC_3A_n40A DC_3A_n77A |
| DC_3A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A |
| DC_3A-40A_n78A DC_3A-40C_n78A | DC_3A_n78A DC_40A_n78A |
| DC_3A-40A_n78(2A) DC_3A-40C_n78(2A) | DC_3A_n78A DC_40A_n78A |
| DC_3A_n40A-n78A DC_3A_n40A-n78C | DC_3A_n40A DC_3A_n78A |
| DC_3A_n40A-n79A DC_3A_n40A-n79C | DC_3A_n40A DC_3A_n79A |
| DC_3A-41A_n1A | DC_3A_n1A DC_41A_n1A |
| DC_3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-41A_n1A | DC_3A_n1A DC_41A_n1A |
| DC_3A-3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-41A_n3A DC_3A-41C_n3A | DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |
| DC_3A-41A_n28A5 | DC_3A_n28A DC_41A_n28A |
| DC_3A-41C_n28A5 | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A DC_3A-41C_n41A DC_3A-41D_n41A | DC_3A_n41A |
| DC_3A-(n)41AA DC_3A-(n)41CA DC_3A-(n)41DA | DC_3A_n41A DC_(n)41AA |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n78A DC_3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-41A_n78A DC_3A-3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A_n41A-n78A | DC_3A_n41A DC_3A_n78A |
| DC_3A_n41A-n78(2A) | DC_3A_n41A DC_3A_n78A |
| DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-42A_n1A5 DC_3A-42C_n1A5 | DC_3A_n1A DC_42A_n1A |
| DC_3A-42A_n28A5 | DC_3A_n28A DC_42A_n28A |
| DC_3A-42C_n28A5 | DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_3A-41A_n79A5 DC_3A-41C_n79A5 | DC_3A_n79A DC_41A_n79A |
| DC_3A_n41A-n77A | DC_3A_n41A DC_3A_n77A |
| DC_3A_n41A-n77(2A) | DC_3A_n41A DC_3A_n77A |
| DC_3A_n41A-n79A5 DC_3A_n41C-n79A5 DC_3A_n41A-n79C5 DC_3A_n41C-n79C5 | DC_3A_n41A DC_3A_n79A |
| DC_3A_SUL_n41A-n80A DC_3C_SUL_n41A-n80A | DC_3A_n41A DC_3C_n41A DC_3A_n80A_ULSUP-TDM_n41A DC_3C_n80A_ULSUP-TDM_n41A |
| DC_3A-42A_n77A14, 15,16 DC_3A-42A_n77C15,16 DC_3A-42C_n77A14, 15,16 DC_3A-42C_n77C15,16 DC_3A-42D_n77A14, 15,16 DC_3A-42D_n77C15,16 DC_3A-42E_n77A14, 15,16 DC_3A-42E_n77C15,16 | DC_3A_n77A14, |
| DC_3A-42A_n77(2A)15,16 DC_3A-42C_n77(2A)15,16 | DC_3A_n77A |
| DC_3A-42A_n78A14,15,16 DC_3A-42A_n78C15,16 DC_3A-42C_n78A14,15,16 DC_3A-42C_n78C15,16 DC_3A-42D_n78A14,15,16 DC_3A-42D_n78C15,16 DC_3A-42E_n78A14,15,16 DC_3A-42E_n78C15,16 | DC_3A_n78A14 |
| DC_3A-42A_n79A14 DC_3A-42A_n79C DC_3A-42C_n79A14 DC_3A-42C_n79C DC_3A-42D_n79A14 DC_3A-42D_n79C DC_3A-42E_n79A14 DC_3A-42E_n79C | DC_3A_n79A14 |
| DC_3A-67A_n3A | DC_3A_n3A2 |
| DC_3A_n75A-n78A | DC_3A_n78A |
| DC_3A_n75A-n78(2A) | DC_3A_n78A |
| DC_3A_n77A-n79A14, 23 | DC_3A_n77A14 DC_3A_n79A14 |
| DC_3A_n78A-n79A14, 24 DC_3A_n78A-n79C24 | DC_3A_n78A14 DC_3A_n79A14 |
| DC_3A_SUL_n77A-n80A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A |
| DC_3A_SUL_n77A-n84A | DC_3A_n77A DC_3A_n84A |
| DC_3A_SUL_n78A-n80A5 DC_3C_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_3A_SUL_n78A-n82A5 | DC_3A_n78A DC_3A_n82A |
| DC_3A_SUL_n78A-n84A | DC_3A_n78A DC_3A_n84A |
| DC_3A_n78A-n105A | DC_3A_n78A DC_3A_n105A |
| DC_3A_SUL_n79A-n80A5 | DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A |
| DC_4A-7A_n28A | DC_4A_n28A DC_7A_n28A |
| DC_4A-7A_n78A DC_4A-7C_n78A | DC_4A_n78A DC_7A_n78A DC_7C_n78A |
| DC_5A_n1A-n78A | DC_5A_n1A DC_5A_n78A |
| DC_5A_n2A-n41A | DC_5A_n2A DC_5A_n41A |
| DC_5A_n2A-n66A | DC_5A_n2A DC_5A_n66A |
| DC_5A_n2A-n77A14 DC_5A_n2A-n77C14 | DC_5A_n77A14 DC_5A_n2A |
| DC_5A_n2A-n78A | DC_5A_n2A DC_5A_n78A |
| DC_5A_n3A-n78A | DC_5A_n3A DC_5A_n78A |
| DC_5A_n5A-n77A14 DC_5A_n5A-n77C14 | DC_5A_n77A14 |
| DC_5A-7A_n2A | DC_7A_n2A |
| DC_5A-7A_n2(2A) | DC_7A_n2A |
| DC_5A-7A_n7A | DC_5A_n7A DC_7A_n7A2 |
| DC_5A-7A_n25A | DC_5A_n25A DC_7A_n25A |
| DC_5A-7A_n40A | DC_5A_n40A DC_7A_n40A |
| DC_5A-7A-7A_n40A | DC_5A_n40A DC_7A_n40A |
| DC_5A-7A_n66A DC_5A-7C_n66A | DC_5A_n66A DC_7A_n66A |
| DC_5A-7A-7A_n66A | DC_5A_n66A DC_7A_n66A |
| DC_5A-7A_n71A | DC_5A_n71A DC_7A_n71A |
| DC_5A-7A_n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A_n77A | DC_5A_n71A DC_7A_n71A |
| DC_5A-7A_n77(2A) DC_5A-7A_n77(3A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A_n77(2A) DC_5A-7A-7A_n77(3A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7C_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n78(A-C) | DC_5A_n78A DC_7A_n78A |
| DC_5A_n7A-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7A-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A-7A-7A_n78A DC_5A-7A-7A_n78C | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A-7A_n78(A-C) | DC_5A_n78A DC_7A_n78A |
| DC_5A-(n)12AA | DC_5A_n12A DC_(n)12AA2 |
| DC_5A-13A_n2A | DC_5A_n2A DC_13A_n2A |
| DC_5A-13A_n66A | DC_5A_n66A DC_13A_n66A |
| DC_5A-13A_n77A DC_5A-13A_n77C | DC_5A_n77A DC_13A_n77A |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | DC_5A_n77A |
| DC_5A_n28A-n78A DC_5A_n28A-n78C | DC_5A_n78A |
| DC_5A-30A_n2A | DC_5A_n2A DC_30A_n2A |
| DC_5A-30A_n5A | DC_30A_n5A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_5A-30A_n77A14 | DC_5A_n77A14 DC_30A_n77A14 |
| DC_5A-30A_n77(2A) 14 | DC_5A_n77A14 DC_30A_n77A14 |
| DC_5A_n38A-n66A | DC_5A_n38A DC_5A_n66A |
| DC_5A-40A_n77A DC_5A-40C_n77A DC_5A-40A_n77C DC_5A-40C_n77C | DC_5A_n77A DC_40A_n77A |
| DC_5A_n40A-n77A | DC_5A_n40A DC_5A_n77A |
| DC_5A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A |
| DC_5A-40A_n78A DC_5A-40C_n78A DC_5A-40A_n78C DC_5A-40C_n78C | DC_5A_n78A DC_40A_n78A |
| DC_5A_n40A-n78A DC_5A_n40A-n78C | DC_5A_n40A DC_5A_n78A |
| DC_5A_n41A-n66A | DC_5A_n41A DC_5A_n66A |
| DC_5A-41A_n79A | DC_5A_n79A DC_41A_n79A |
| DC_5A-46A_n66A | DC_5A_n66A DC_46A_n66A |
| DC_5A-48A_n5A | DC_48A_n5A |
| DC_5A-48A_n12A | DC_5A_n12A DC_48A_n12A |
| DC_5A-48A_n71A | DC_5A_n71A DC_48A_n71A |
| DC_5A-48A_n77A14,15,16 DC_5A-48C_n77A14,15,16 DC_5A-48D_n77A14,15,16 DC_5A-48A_n77C14,15,16 DC_5A-48C_n77C14,15,16 DC_5A-48D_n77C14,15,16 | DC_5A_n77A14 |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-66B_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-5A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-5A-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A_n2(2A) | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A_n5A | DC_66A_n5A |
| DC_5A-66A-66A_n5A | DC_66A_n5A |
| DC_5A-66A_n7A | DC_5A_n7A DC_66A_n7A |
| DC_5A-66A-66A_n7A | DC_5A_n7A DC_66A_n7A |
| DC_5A-66A_n12A | DC_5A_n12A DC_66A_n12A |
| DC_5A-66A_n25A | DC_5A_n25A DC_66A_n25A |
| DC_5A-66A_n30A | DC_5A_n30A DC_66A_n30A |
| DC_5A-66A-66A_n30A | DC_5A_n30A DC_66A_n30A |
| DC_5A-66A_n41A | DC_5A_n41A DC_66A_n41A |
| DC_5A-66A_n48A DC_5A-66A_n48B | DC_5A_n48A DC_66A_n48A |
| DC_5A-66A-66A_n48A DC_5A-66A-66A_n48B | DC_5A_n48A DC_66A_n48A |
| DC_5A-66A_n66A DC_5B-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A_n66A | DC_5A_n66A |
| DC_5A-66A-66A_n66A DC_5B-66A-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A-66A_n66A | DC_5A_n66A |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A |
| DC_5A-66A_n77A14 DC_5A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A_n77(2A) 14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A-66A_n77A14 DC_5A-66A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A-66A_n77(2A) 14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A_n66A-n77A14 DC_5A_n66A-n77C14 | DC_5A_n66A DC_5A_n77A14 |
| DC_5A-66A_n78A | DC_5A_n78A DC_66A_n78A |
| DC_5A-66A_n78(2A) | DC_5A_n78A DC_66A_n78A |
| DC_5A_n66A-n78A | DC_5A_n66A DC_5A_n78A |
| DC_5A-66A-66A_n78A | DC_5A_n78A DC_66A_n78A |
| DC_7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |
| DC_7A-7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |
| DC_7A_n1A-n28A | DC_7A_n1A DC_7A_n28A |
| DC_7C_n1A-n28A | DC_7A_n1A DC_7A_n28A DC_7C_n1A DC_7C_n28A |
| DC_7A_n1A-n40A | DC_7A_n1A DC_7A_n40A |
| DC_7A_n1A-n75A | DC_7A_n1A |
| DC_7A_n1A-n78A5, 14 DC_7C_n1A-n78A5 | DC_7A_n1A DC_7A_n78A14 DC_7C_n1A DC_7C_n78A |
| DC_7A_n1A-n78(2A)5 DC_7C_n1A-n78(2A)5 | DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_7A-7A_n1A-n78A5, 14 | DC_7A_n1A DC_7A_n78A14 |
| DC_7A_n2A-n66A | DC_7A_n2A DC_7A_n66A |
| DC_7A_n2A-n71A | DC_7A_n2A DC_7A_n71A |
| DC_7A_n2A-n77A | DC_7A_n2A DC_7A_n77A |
| DC_7A_n2A-n78A | DC_7A_n2A DC_7A_n78A |
| DC_7A_n3A-n78A DC_7C_n3A-n78A | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n3A-n78(2A) DC_7C_n3A-n78(2A) | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n5A-n40A | DC_7A_n5A DC_7A_n40A |
| DC_7A_n5A-n78A14 DC_7C_n5A-n78A14 | DC_7A_n5A DC_7C_n5A DC_7A_n78A14 DC_7C_n78A14 |
| DC_7A_n7A-n78A5 | DC_7A_n78A DC_7A_n7A2 |
| DC_7A_n7A-n78(2A) | DC_7A_n78A DC_7A_n7A2 |
| DC_7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A-8A_n7A | DC_7A_n7A DC_8A_n7A |
| DC_7A-8A_n20A | DC_7A_n20A DC_8A_n20A |
| DC_7A-8A_n28A | DC_7A_n28A DC_8A_n28A |
| DC_7A-7A-8A_n28A | DC_7A_n28A DC_8A_n28A |
| DC_7A-8A_n40A | DC_7A_n40A DC_8A_n40A |
| DC_7A_n8A-n40A | DC_7A_n8A DC_7A_n40A |
| DC_7A-8A_n77A5 | DC_7A_n77A DC_8A_n77A |
| DC_7A-8A_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-8A_n78(2A) | DC_7A_n78A DC_8A_n78A |
| DC_7A-7A-8A_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-7A_n8A-n78A5 | DC_7A_n8A DC_7A_n78A |
| DC_7A-8B_n78A5 DC_7A-7A-8B_n78A5 | DC_7A_n78A DC_8A_n78A DC_8B_n78A |
| DC_7A_n8A-n78A5 | DC_7A_n8A DC_7A_n78A |
| DC_7A-12A_n2A | DC_7A_n2A DC_12A_n2A |
| DC_7A-12A_n2(2A) | DC_7A_n2A DC_12A_n2A |
| DC_7A-12A_n25A | DC_7A_n25A DC_12A_n25A |
| DC_7A-12A_n66A | DC_7A_n66A DC_12A_n66A |
| DC_7A-12A_n77A | DC_7A_n77A DC_12A_n77A |
| DC_7A-12A_n77(2A) | DC_7A_n77A DC_12A_n77A |
| DC_7A_n12A-n77A | DC_7A_n12A DC_7A_n77A |
| DC_7A-12A_n78A | DC_7A_n78A DC_12A_n78A |
| DC_7A-12A_n78(2A) | DC_7A_n78A DC_12A_n78A |
| DC_7A_n12A-n78A | DC_7A_n12A DC_7A_n78A |
| DC_7A-13A_n25A DC_7C-13A_n25A | DC_7A_n25A DC_13A_n25A |
| DC_7A-7A-13A_n25A | DC_7A_n25A DC_13A_n25A |
| DC_7A-13A_n66A DC_7C-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-7A-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-20A_n1A DC_7C-20A_n1A | DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_7A-20A_n3A DC_7C-20A_n3A | DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6,16,20 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 DC_7A-20A_n78C5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n78(2A)5 | DC_7A_n78A DC_20A_n78A |
| DC_7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7A-7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7C_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7A-25A_n77A DC_7C-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-7A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-25A-25A_n77A DC_7C-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-7A-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-25A_n78A DC_7C-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-7A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-25A-25A_n78A DC_7C-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-7A-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-26A_n78A DC_7C-26A_n78A | DC_7A_n78A DC_26A_n78A |
| DC_7A-26A_n78(2A) DC_7C-26A_n78(2A) | DC_7A_n78A DC_26A_n78A |
| DC_7A_n26A-n78A DC_7A_n26A-n78(2A) | DC_7A_n26A DC_7A_n78A |
| DC_7C_n26A-n78A DC_7C_n26A-n78(2A) | DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_7A-28A_n1A | DC_28A_n1A DC_7A_n1A |
| DC_7A-7A-28A_n1A | DC_28A_n1A DC_7A_n1A |
| DC_7A-28A_n2A | DC_7A_n2A DC_28A_n2A |
| DC_7A-28A_n3A DC_7C-28A_n3A | DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 DC_7C-28A_n5A6 | DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_7A-28A_n7A | DC_7A_n7A2 DC_28A_n7A |
| DC_7A-28A_n20A | DC_7A_n20A DC_28A_n20A |
| DC_7A_n28A-n40A | DC_7A_n28A DC_7A_n40A |
| DC_7A-28A_n40A | DC_7A_n40A DC_28A_n40A |
| DC_7A-28A_n66A DC_7C-28A_n66A | DC_7A_n66A DC_28A_n66A |
| DC_7A-28A_n78A5,14 DC_7C-28A_n78A5,14 DC_7A-28A_n78(2A)5,14 DC_7C-28A_n78(2A)5,14 | DC_7A_n78A14 DC_7C_n78A14 DC_28A_n78A14 |
| DC_7A_n28A-n78A5,14 DC_7C_n28A-n78A14 | DC_7A_n28A DC_7A_n78A14 DC_7C_n28A DC_7C_n78A14 |
| DC_7A-29A_n78A DC_7C-29A_n78A | DC_7A_n78A |
| DC_7A-7A-29A_n78A | DC_7A_n78A |
| DC_7A-32A_n1A | DC_7A_n1A |
| DC_7A-32A_n3A DC_7C-32A_n3A | DC_7A_n3A |
| DC_7A-32A_n8A | DC_7A_n8A |
| DC_7A-32A_n28A | DC_7A_n28A |
| DC_7A-32A_n78A | DC_7A_n78A |
| DC_7A-40A_n1A DC_7A-40C_n1A | DC_7A_n1A DC_40A_n1A |
| DC_7A_n40A-n77A | DC_7A_n40A DC_7A_n77A |
| DC_7A_n40A-n77(2A) | DC_7A_n40A DC_7A_n77A |
| DC_7A-7A_n40A-n77A | DC_7A_n40A DC_7A_n77A |
| DC_7A-7A_n40A-n77(2A) | DC_7A_n40A DC_7A_n77A |
| DC_7A-40A_n78A DC_7A-40C_n78A | DC_7A_n78A DC_40A_n78A |
| DC_7A-40A_n78(2A) DC_7A-40C_n78(2A) | DC_7A_n78A DC_40A_n78A |
| DC_7A_n40A-n78A DC_7A_n40A-n78C | DC_7A_n40A DC_7A_n78A |
| DC_7A-46A_n78A3 DC_7A-46C_n78A3 DC_7A-46D_n78A3 DC_7A-46E_n78A3 | DC_7A_n78A |
| DC_7A-66A_n2A DC_7A-66A_n2(2A) | DC_7A_n2A DC_66A_n2A |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | DC_7A_n5A DC_66A_n5A |
| DC_7A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| DC_7A-66A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| DC_7A-66A_n12A | DC_7A_n12A DC_66A_n12A |
| DC_7A-66A_n25A DC_7C-66A_n25A | DC_7A_n25A DC_66A_n25A |
| DC_7A-7A-66A_n25A | DC_7A_n25A DC_66A_n25A |
| DC_7A-66A_n28A | DC_7A_n28A DC_66A_n28A |
| DC_7A-66A_n66A DC_7C-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-7A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A_n66A-n71A | DC_7A_n66A DC_7A_n71A |
| DC_7A-66A_n77A DC_7C-66A_n77A | DC_7A_n77A DC_66A_n77A |
| DC_7A-7A-66A_n77A | DC_7A_n66A DC_66A_n77A |
| DC_7A-7A-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |
| DC_7A-66A_n77(2A) DC_7C-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |
| DC_7A_n66A-n77A DC_7C_n66A-n77A | DC_7A_n66A DC_7A_n77A |
| DC_7A-7A_n66A-n77A | DC_7A_n66A DC_7A_n77A |
| DC_7A_n66A-n78A DC_7C_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-7A_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-66A_n78A5,14 DC_7C-66A_n78A5,14 | DC_7A_n78A14 DC_7C_n78A DC_66A_n78A14 |
| DC_7A-66A_n78(2A) 5,14 DC_7C-66A_n78(2A) 5,14 | DC_7A_n78A14 DC_7C_n78A DC_66A_n78A14 |
| DC_7A-7A-66A_n78A5,14 | DC_7A_n78A14 DC_66A_n78A14 |
| DC_7A-7A-66A_n78(2A)5,14 | DC_7A_n78A14 DC_66A_n78A14 |
| DC_7A-7A-66A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-66A-66A_n78A5,14 DC_7C-66A-66A_n78A5,14 | DC_7A_n78A14 DC_66A_n78A14 |
| DC_7A-66A-66A_n78(2A) 5,14 DC_7C-66A-66A_n78(2A) 5,14 | DC_7A_n78A14 DC_66A_n78A14 |
| DC_7A-71A_n2A | DC_7A_n2A DC_71A_n2A |
| DC_7A-71A_n2(2A) | DC_7A_n2A DC_71A_n2A |
| DC_7A-71A_n12A | DC_7A_n12A |
| DC_7A-71A_n25A | DC_7A_n25A DC_71A_n25A |
| DC_7A-71A_n66A | DC_7A_n66A DC_71A_n66A |
| DC_7A-71A_n77A | DC_7A_n77A DC_71A_n77A |
| DC_7A-71A_n77(2A) | DC_7A_n77A DC_71A_n77A |
| DC_7A_n71A-n77A | DC_7A_n71A DC_7A_n77A |
| DC_7A-71A_n78A | DC_7A_n78A DC_71A_n78A |
| DC_7A-71A_n78(2A) | DC_7A_n78A DC_71A_n78A |
| DC_7A_n71A-n78A | DC_7A_n71A DC_7A_n78A |
| DC_7A_n75A-n78A | DC_7A_n78A |
| DC_7A_n78A-n79A24 DC_7A_n78A-n79C24 | DC_7A_n78A DC_7A_n79A |
| DC_7A_SUL_n78A-n80A | DC_7A_n78A DC_7A_n80A |
| DC_7A_n78A-n105A | DC_7A_n78A DC_7A_n105A |
| DC_8A_n1A-n3A | DC_8A_n1A DC_8A_n3A |
| DC_8A_n1A-n28A | DC_8A_n1A DC_8A_n28A |
| DC_8A_n1A-n40A | DC_8A_n1A DC_8A_n40A |
| DC_8A_n1A-n77A5 | DC_8A_n1A DC_8A_n77A |
| DC_8A_n1A-n77(2A)5 | DC_8A_n1A DC_8A_n77A |
| DC_8A_n1A-n78A5,14 | DC_8A_n1A DC_8A_n78A14 |
| DC_8A-(n)3AA | DC_(n)3AA DC_8A_n3A |
| DC_8A_n3A-n28A | DC_8A_n3A DC_8A_n28A |
| DC_8A_n3A-n77A5 | DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n77(2A) 5 | DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n78A | DC_8A_n3A DC_8A_n78A |
| DC_8A_n3A-n79A | DC_8A_n3A DC_8A_n79A |
| DC_8A-11A_n1A DC_8B-11A_n1A | DC_8A_n1A DC_11A_n1A |
| DC_8A-11A_n3A | DC_8A_n3A DC_11A_n3A |
| DC_8A-11A_n28A | DC_8A_n28A DC_11A_n28A |
| DC_8A-11A_n77A5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(2A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(3A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n78A5 | DC_8A_n78A DC_11A_n78A |
| DC_8A-11A_n79A5 | DC_8A_n79A DC_11A_n79A |
| DC_8A-20A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_8A-20A_n3A | DC_8A_n3A DC_20A_n3A |
| DC_8A-20A_n28A6,16,19,20 | DC_8A_n28A DC_20A_n28A |
| DC_8A-20A_n78A | DC_8A_n78A DC_20A_n78A |
| DC_8A-28A_n3A | DC_8A_n3A DC_28A_n3A |
| DC_8A-28A_n78A | DC_8A_n78A DC_28A_n78A |
| DC_8A_n28A-n77A5 | DC_8A_n28A DC_8A_n77A |
| DC_8A_n28A-n77(2A)5 | DC_8A_n28A DC_8A_n77A |
| DC_8A_n28A-n78A5 | DC_8A_n28A DC_8A_n78A |
| DC_8A_n28A-n79A5 | DC_8A_n28A DC_8A_n79A |
| DC_8A-32A_n1A | DC_8A_n1A |
| DC_8A-32A_n3A | DC_8A_n3A |
| DC_8A-32A_n28A | DC_8A_n28A |
| DC_8A-32A_n78A | DC_8A_n78A |
| DC_8A-38A_n1A | DC_8A_n1A DC_38A_n1A |
| DC_8A_n38A-n40A | DC_8A_n38A DC_8A_n40A |
| DC_8A_n39A-n40A | DC_8A_n39A DC_8A_n40A |
| DC_8A_n39A-n41A | DC_8A_n39A DC_8A_n41A |
| DC_8A_n39A-n79A | DC_8A_n39A DC_8A_n79A |
| DC_8A-40A_n1A DC_8A-40C_n1A | DC_8A_n1A DC_40A_n1A |
| DC_8A_n40A-n41A DC_8A_n40A-n41C | DC_8A_n40A DC_8A_n41A |
| DC_8A-40A_n78A DC_8A-40C_n78A | DC_8A_n78A DC_40A_n78A |
| DC_8A-40A_n78(2A) DC_8A-40C_n78(2A) | DC_8A_n78A DC_40A_n78A |
| DC_8A_n40A-n78A | DC_8A_n40A DC_8A_n78A |
| DC_8A_n40A-n79A DC_8A_n40A-n79C | DC_8A_n40A DC_8A_n79A |
| DC_8A-41A_n1A DC_8A-41C_n1A | DC_8A_n1A DC_41A_n1A |
| DC_8A-41A_n3A5 DC_8A-41C_n3A5 | DC_8A_n3A DC_41A_n3A DC_41C_n3A |
| DC_8A-41A_n77A DC_8A-41C_n77A | DC_8A_n77A DC_41A_n77A DC_41C_n77A |
| DC_8A-41A_n78A | DC_8A_n78A DC_41A_n78A |
| DC_8A-41C_n78A | DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_8A_n41A-n79A5 DC_8A_n41A-n79C5 DC_8A_n41C-n79A5 DC_8A_n41C-n79C5 | DC_8A_n41A DC_8A_n79A |
| DC_8A-42A_n1A5 DC_8A-42C_n1A5 | DC_8A_n1A DC_42A_n1A DC_42C_n1A |
| DC_8A-42A_n3A5 | DC_8A_n3A DC_42A_n3A |
| DC_8A-42C_n3A5 | DC_8A_n3A DC_42A_n3A DC_42C_n3A |
| DC_8A-42A_n28A5 | DC_8A_n28A DC_42A_n28A |
| DC_8A-42C_n28A5 | DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n77A15,16 DC_8A-42C_n77A15,16 | DC_8A_n77A |
| DC_8A-42A_n77(2A) 15,16 DC_8A-42C_n77(2A) 15,16 | DC_8A_n77A |
| DC_8A_SUL_n41A-n81A | DC_8A_n41A, DC_8A_n81A_ULSUP-TDM_n41A |
| DC_8A_n77A-n79A23 | DC_8A_n77A DC_8A_n79A |
| DC_8A_n77(2A)-n79A23 | DC_8A_n77A DC_8A_n79A |
| DC_8A_SUL_n78A-n80A | DC_8A_n78A DC_8A_n80A |
| DC_8A_SUL_n78A-n81A5 | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A |
| DC_8A_SUL_n79A-n81A5 | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A |
| DC_11A_n1A-n77A5 | DC_11A_n1A DC_11A_n77A |
| DC_11A_n1A-n77(2A)5 | DC_11A_n1A DC_11A_n77A |
| DC_11A_n3A-n28A | DC_11A_n3A DC_11A_n28A |
| DC_11A_n3A-n77A | DC_11A_n3A DC_11A_n77A |
| DC_11A_n3A-n77(2A) | DC_11A_n3A DC_11A_n77A |
| DC_11A_n3A-n79A5 | DC_11A_n3A DC_11A_n79A |
| DC_11A-18A_n3A | DC_11A_n3A DC_18A_n3A |
| DC_11A-18A_n28A | DC_11A_n28A |
| DC_11A-18A_n41A | DC_11A_n41A DC_18A_n41A |
| DC_11A-18A_n77A | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n77(2A) | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n78A | DC_11A_n78A DC_18A_n78A |
| DC_11A-18A_n78(2A) | DC_11A_n78A DC_18A_n78A |
| DC_11A_n28A-n77A5 | DC_11A_n28A DC_11A_n77A |
| DC_11A_n28A-n77(2A) 5 | DC_11A_n28A DC_11A_n77A |
| DC_11A_n77A-n79A23 | DC_11A_n77A DC_11A_n79A |
| DC_11A_n77(2A)-n79A23 | DC_11A_n77A DC_11A_n79A |
| DC_12A_n2A-n38A | DC_12A_n2A DC_12A_n38A |
| DC_12A_n2A-n41A | DC_12A_n2A DC_12A_n41A |
| DC_12A_n2A-n66A | DC_12A_n2A DC_12A_n66A |
| DC_12A_n2A-n77A | DC_12A_n2A DC_12A_n77A |
| DC_12A_n2A-n78A | DC_12A_n2A DC_12A_n78A |
| DC_12A-(n)5AA | DC_12A_n5A DC_(n)5AA2 |
| DC_12A_n7A-n66A | DC_12A_n7A DC_12A_n66A |
| DC_12A_n7(2A)-n66A | DC_12A_n7A DC_12A_n66A |
| DC_12A_n7A-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7A-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_12A-30A_n5A | DC_12A_n5A DC_30A_n5A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_12A-30A_n77A14 | DC_12A_n77A14 DC_30A_n77A14 |
| DC_12A-30A_n77(2A) 14 | DC_12A_n77A14 DC_30A_n77A14 |
| DC_12A_n41A-n66A | DC_12A_n41A DC_12A_n66A |
| DC_12A-48A_n5A | DC_12A_n5A DC_48A_n5A |
| DC_12A-48A_n12A | DC_48A_n12A |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n2(2A) | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n5A | DC_12A_n5A DC_66A_n5A |
| DC_12A-66A_n7A | DC_12A_n7A DC_66A_n7A |
| DC_12A-66A-66A_n5A | DC_12A_n5A DC_66A_n5A |
| DC_12A-66A_n12A | DC_66A_n12A |
| DC_12A-66A_n25A | DC_12A_n25A DC_66A_n25A |
| DC_12A-66A_n30A | DC_12A_n30A DC_66A_n30A |
| DC_12A-66A-66A_n30A | DC_12A_n30A DC_66A_n30A |
| DC_12A-66A_n41A | DC_12A_n41A DC_66A_n41A |
| DC_12A-66A_n66A | DC_12A_n66A DC_66A_n66A2 |
| DC_12A-66A_n77A14 DC_12A-66A-66A_n77A14 | DC_12A_n77A14 DC_66A_n77A14 |
| DC_12A-66A_n77(2A) 14 DC_12A-66A-66A_n77(2A) 14 | DC_12A_n77A14 DC_66A_n77A14 |
| DC_12A_n66A-n77A | DC_12A_n66A DC_12A_n77A |
| DC_12A-66A_n78A | DC_12A_n78A DC_66A_n78A |
| DC_12A-66A_n78(2A) | DC_12A_n78A DC_66A_n78A |
| DC_12A_n66A-n78A | DC_12A_n66A DC_12A_n78A |
| DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | DC_12A_n66A DC_12A_n78A |
| DC_12A-71A_n2A | DC_12A_n2A DC_71A_n2A |
| DC_12A-71A_n77A | DC_12A_n77A DC_71A_n77A |
| DC_13A_n2A-n77A14 DC_13A_n2A-n77C14 | DC_13A_n2A DC_13A_n77A14 |
| DC_13A_n5A-n48A | DC_13A_n48A |
| DC_13A_n5A-n77A14 DC_13A_n5A-n77C14 | DC_13A_n77A14 |
| DC_13A_n7A-n78A | DC_13A_n7A DC_13A_n78A |
| DC_13A_n25A-n66A | DC_13A_n25A DC_13A_n66A |
| DC_13A-46A_n2A3 | DC_13A_n2A |
| DC_13A-46A_n5A | DC_13A_n5A |
| DC_13A-46A_n66A3 | DC_13A_n66A |
| DC_13A-46A_n77A DC_13A-46A-46A_n77A | DC_13A_n77A |
| DC_13A_n48A-n66A | DC_13A_n48A DC_13A_n66A |
| DC_13A-66A_n2A DC_13A-66B_n2A DC_13A-66C_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A_n5A DC_13A-66A-66A_n5A | DC_13A_n5A DC_66A_n5A |
| DC_13A-66A_n48A DC_13A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A_n66A DC_13A-66B_n66A | DC_13A_n66A |
| DC_13A-66A-66A_n66A | DC_13A_n66A |
| DC_13A-66A_n77A14 DC_13A-66A_n77C14 DC_13A-66A-66A_n77C14 | DC_13A_n77A14 DC_66A_n77A14 |
| DC_13A-66A-66A_n77A | DC_13A_n77A14 DC_66A_n77A14 |
| DC_13A_n66A-n77A14 DC_13A_n66A-n77C14 | DC_13A_n66A DC_13A_n77A14 |
| DC_13A-48A_n2A DC_13A-48B_n2A DC_13A-48C_n2A DC_13A-48D_n2A DC_13A-48E_n2A | DC_13A_n2A |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48C_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A |
| DC_13A-48A_n77A14,15,16 DC_13A-48A_n77C14,15,16 DC_13A-48C_n77A14,15,16 DC_13A-48C_n77C14,15,16 DC_13A-48D_n77A14,15,16 DC_13A-48D_n77C14,15,16 DC_13A-48A-48A_n77A14,15,16 | DC_13A_n77A14 |
| DC_14A-30A_n2A | DC_14A_n2A DC_30A_n2A |
| DC_14A-30A_n5A | DC_14A_n5A DC_30A_n5A |
| DC_14A-30A_n66A | DC_14A_n66A DC_30A_n66A |
| DC_14A-30A_n77A14 | DC_14A_n77A14 DC_30A_n77A14 |
| DC_14A-30A_n77(2A) 14 | DC_14A_n77A14 DC_30A_n77A14 |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A_n30A DC_14A-66A-66A_n30A | DC_14A_n30A DC_66A_n30A |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A2 |
| DC_14A-66A_n77A14 DC_14A-66A-66A_n77A14 | DC_14A_n77A14 DC_66A_n77A14 |
| DC_14A-66A_n77(2A) 14 DC_14A-66A-66A_n77(2A) 14 | DC_14A_n77A14 DC_66A_n77A14 |
| DC_18A_n3A-n41A | DC_18A_n3A DC_18A_n41A |
| DC_18A_n3A-n77A | DC_18A_n3A DC_18A_n77A |
| DC_18A_n3A-n78A | DC_18A_n3A DC_18A_n78A |
| DC_18A_n28A-n41A | DC_18A_n28A DC_18A_n41A |
| DC_18A-28A_n77A5 | DC_18A_n77A DC_28A_n77A |
| DC_18A_n28A-n77A5 | DC_18A_n28A DC_18A_n77A |
| DC_18A_n28A-n77(2A)5 | DC_18A_n28A DC_18A_n77A |
| DC_18A-28A_n78A5 | DC_18A_n78A DC_28A_n78A |
| DC_18A_n28A-n78A5 | DC_18A_n28A DC_18A_n78A |
| DC_18A_n28A-n78(2A)5 | DC_18A_n28A DC_18A_n78A |
| DC_18A-28A_n79A5 | DC_18A_n79A DC_28A_n79A |
| DC_18A-41A_n3A DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_18A_n41A-n77A | DC_18A_n41A DC_18A_n77A |
| DC_18A_n41A-n77(2A) | DC_18A_n41A DC_18A_n77A |
| DC_18A-42A_n77A15,16 DC_18A-42C_n77A15,16 | DC_18A_n77A |
| DC_18A_n41A-n78A | DC_18A_n41A DC_18A_n78A |
| DC_18A_n41A-n78(2A) | DC_18A_n41A DC_18A_n78A |
| DC_18A-42A_n78A15,16 DC_18A-42C_n78A15,16 | DC_18A_n78A |
| DC_18A-42A_n79A DC_18A-42C_n79A | DC_18A_n79A |
| DC_19A-21A_n1A | DC_19A_n1A DC_21A_n1A |
| DC_19A_n1A-n77A5 | DC_19A_n1A DC_19A_n77A |
| DC_19A_n1A-n78A5 | DC_19A_n1A DC_19A_n78A |
| DC_19A_n1A-n79A5 | DC_19A_n1A DC_19A_n79A |
| DC_19A-21A_n77A5,14 DC_19A-21A_n77C5 | DC_19A_n77A14 DC_21A_n77A14 |
| DC_19A-21A_n77(2A)5,14 | DC_19A_n77A14 DC_21A_n77A14 |
| DC_19A-21A_n78A514 DC_19A-21A_n78C5 | DC_19A_n78A14 DC_21A_n78A14 |
| DC_19A-21A_n78(2A)514 | DC_19A_n78A14 DC_21A_n78A14 |
| DC_19A-21A_n79A5,14 DC_19A-21A_n79C5 | DC_19A_n79A14 DC_21A_n79A14 |
| DC_19A-42A_n1A5,10,12 DC_19A-42C_n1A5,10,12 | DC_19A_n1A DC_42A_n1A |
| DC_19A-42A_n77A14,15,16 DC_19A-42A_n77C15,16 DC_19A-42C_n77A14,15,16 DC_19A-42C_n77C15,16 DC_19A-42D_n77A15,16 DC_19A-42D_n77C15,16 | DC_19A_n77A14 |
| DC_19A-42A_n78A14,15,16 DC_19A-42A_n78C15,16 DC_19A-42C_n78A14,15,16 DC_19A-42C_n78C15,16 DC_19A-42D_n78A15,16 DC_19A-42D_n78C15,16 | DC_19A_n78A14 |
| DC_19A-42A_n79A14 DC_19A-42A_n79C DC_19A-42C_n79A14 DC_19A-42C_n79C DC_19A-42D_n79A DC_19A-42D_n79C | DC_19A_n79A14 |
| DC_19A_n77A-n79A14,23 | DC_19A_n77A14 DC_19A_n79A14 |
| DC_19A_n78A-n79A14,24 | DC_19A_n78A14 DC_19A_n79A14 |
| DC_20A_n1A-n7A | DC_20A_n1A DC_20A_n7A |
| DC_20A_n1A-n28A16,20 | DC_20A_n1A DC_20A_n28A |
| DC_20A_n1A-n67A | DC_20A_n1A |
| DC_20A_n1A-n75A | DC_20A_n1A |
| DC_20A_n1A-n78A | DC_20A_n1A DC_20A_n78A |
| DC_20A-(n)3AA | DC_(n)3AA2 DC_20A_n3A |
| DC_20A_n3A-n38A | DC_20A_n3A DC_20A_n38A |
| DC_20A_n3A-n67A | DC_20A_n3A |
| DC_20A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_20A_n7A-n28A, 16, 20 | DC_20A_n7A DC_20A_n28A |
| DC_20A_n7A-n78A | DC_20A_n7A DC_20A_n78A |
| DC_20A_n8A-n75A6 | DC_20A_n8A |
| DC_20A_n8A-n78A | DC_20A_n78A DC_20A_n8A |
| DC_20A-28A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_20A-28A_n3A | DC_20A_n3A DC_28A_n3A |
| DC_20A-28A_n78A | DC_20A_n78A DC_28A_n78A |
| DC_20A_n28A-n75A6,16,20 | DC_20A_n28A |
| DC_20A_n28A-n78A5,6,16,20 | DC_20A_n28A DC_20A_n78A |
| DC_20A-32A_n1A | DC_20A_n1A |
| DC_20A-32A_n3A | DC_20A_n3A |
| DC_20A-32A_n8A | DC_20A_n8A |
| DC_20A-32A_n28A16,20 | DC_20A_n28A |
| DC_20A-32A_n7A | DC_20A_n7A |
| DC_20A-32A_n78A DC_20A-32A_n78C | DC_20A_n78A |
| DC_20A-32A_n78(2A) | DC_20A_n78A |
| DC_20A-38A_n1A | DC_20A_n1A DC_38A_n1A |
| DC_20A-38A_n3A | DC_20A_n3A |
| DC_20A-38A_n8A | DC_38A_n8A |
| DC_20A-(n)38AA | DC_20A_n38A |
| DC_20A-38A_n78A | DC_20A_n78A DC_38A_n78A |
| DC_20A-38A_n78(2A) | DC_20A_n78A |
| DC_20A_n38A-n78A | DC_20A_n38A DC_20A_n78A |
| DC_20A-40A_n1A DC_20A-40C_n1A | DC_20A_n1A DC_40A_n1A |
| DC_20A-40A_n78A DC_20A-40C_n78A | DC_20A_n78A DC_40A_n78A |
| DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | DC_20A_n78A DC_40A_n78A |
| DC_20A-41A_n1A | DC_20A_n1A DC_41A_n1A |
| DC_20A-41C_n1A | DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_20A-41A_n41A DC_20A-41C_n41A | DC_41A_n41A |
| DC_20A-41A_n78A | DC_20A_n78A DC_41A_n78A |
| DC_20A-41C_n78A | DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_20A_n41A-n78A | DC_20A_n41A DC_20A_n78A |
| DC_20A-(n)41AA DC_20A-(n)41CA DC_20A-(n)41DA | DC_20A_n41A |
| DC_20A-67A_n3A | DC_20A_n3A |
| DC_20A_n75A-n78A5 | DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_20A_SUL_n78A-n80A | DC_20A_n78A DC_20A_n80A |
| DC_20A_SUL_n78A-n82A5 | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |
| DC_20A_SUL_n78A-n83A5 | DC_20A_n78A DC_20A_n83A |
| DC_20A_n78A-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |
| DC_20A_n78(2A)-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |
| DC_21A_n1A-n77A5 | DC_21A_n1A DC_21A_n77A |
| DC_21A_n1A-n78A5 | DC_21A_n1A DC_21A_n78A |
| DC_21A_n1A-n79A5 | DC_21A_n1A DC_21A_n79A |
| DC_21A-28A_n77A5 DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A |
| DC_21A_n28A-n77A5,13 | DC_21A_n28A DC_21A_n77A |
| DC_21A-28A_n78A5 DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A |
| DC_21A_n28A-n78A5,13 | DC_21A_n28A DC_21A_n78A |
| DC_21A-28A_n79A5 DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A |
| DC_21A_n28A-n79A5,13 | DC_21A_n28A DC_21A_n79A |
| DC_21A-42A_n1A510,12 DC_21A-42C_n1A510,12 | DC_21A_n1A DC_42A_n1A |
| DC_21A-42A_n77A14, 15,16 DC_21A-42A_n77C15,16 DC_21A-42C_n77A14, 15,16 DC_21A-42C_n77C15,16 DC_21A-42D_n77A15,16 DC_21A-42D_n77C15,16 DC_21A-42E_n77A15,16 DC_21A-42E_n77C15,16 | DC_21A_n77A14, |
| DC_21A-42A_n78A14,15,16 DC_21A-42A_n78C15,16 DC_21A-42C_n78A14,15,16 DC_21A-42C_n78C15,16 DC_21A-42D_n78A14,15,16 DC_21A-42D_n78C15,16 DC_21A-42E_n78A14,15,16 DC_21A-42E_n78C15,16 | DC_21A_n78A14 |
| DC_21A-42A_n79A14 DC_21A-42A_n79C DC_21A-42C_n79A14 DC_21A-42C_n79C DC_21A-42D_n79A DC_21A-42D_n79C DC_21A-42E_n79A DC_21A-42E_n79C | DC_21A_n79A14 |
| DC_28A-(n)7AA | DC_28A_n7A |
| DC_28A-32A_n1A | DC_28A_n1A |
| DC_28A-32A_n3A | DC_28A_n3A |
| DC_28A-38A_n1A | DC_28A_n1A DC_38A_n1A |
| DC_28A-38A_n78A | DC_28A_n78A DC_38A_n78A |
| DC_28A-66A_n7A | DC_28A_n7A DC_66A_n7A |
| DC_28A-66A_n66A | DC_28A_n66A DC_66A_n66A2 |
| DC_21A_n77A-n79A14, 23 | DC_21A_n77A14 DC_21A_n79A14 |
| DC_21A_n78A-n79A14, 24 | DC_21A_n78A14 DC_21A_n79A14 |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-(n)41CA DC_25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_25A-66A_n77A | DC_25A_n77A DC_66A_n77A |
| DC_25A-25A-66A_n77A | DC_25A_n77A DC_66A_n77A |
| DC_25A-66A_n78A | DC_25A_n78A DC_66A_n78A |
| DC_25A-25A-66A_n78A | DC_25A_n78A DC_66A_n78A |
| DC_28A_n5A-n40A | DC_28A_n5A DC_28A_n40A |
| DC_28A-40A_n78A DC_28A-40C_n78A | DC_28A_n78A DC_40A_n78A |
| DC_28A-41A_n77A DC_28A-41C_n77A | DC_28A_n77A DC_41A_n77A |
| DC_28A-41A_n78A DC_28A-41C_n78A | DC_28A_n78A DC_41A_n78A |
| DC_28A-41A_n79A5 DC_28A-41C_n79A5 | DC_28A_n79A DC_41A_n79A |
| DC_28A_n1A-n40A | DC_28A_n1A DC_28A_n40A |
| DC_28A_n1A-n78A5 | DC_28A_n1A DC_28A_n78A |
| DC_28A_n3A-n77A5 | DC_28A_n3A DC_28A_n77A |
| DC_28A_n3A-n78A5 | DC_28A_n3A DC_28A_n78A |
| DC_28A_n5A-n78A5 | DC_28A_n5A DC_28A_n78A |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A_n7B-n78A | DC_28A_n7A DC_28A_n7B DC_28A_n78A |
| DC_28A_n8A-n78A5 | DC_28A_n8A DC_28A_n78A |
| DC_28A_n38A-n78A | DC_28A_n38A DC_28A_n78A |
| DC_28A_n40A-n78A | DC_28A_n40A DC_28A_n78A |
| DC_28A_SUL_n41A-n83A5 | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41A |
| DC_28A-42A_n77A15,16 DC_28A-42A_n77C15,16 DC_28A-42C_n77A15,16 DC_28A-42C_n77C15,16 | DC_28A_n77A |
| DC_28A-42A_n78A15,16 DC_28A-42A_n78C15,16 DC_28A-42C_n78A15,16 DC_28A-42C_n78C15,16 | DC_28A_n78A |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A DC_28A-42C_n79C | DC_28A_n79A |
| DC_28A_SUL_n78A-n83A5 | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |
| DC_29A-30A_n2A | DC_30A_n2A |
| DC_29A-30A_n66A | DC_30A_n66A |
| DC_29A-30A_n77A14 | DC_30A_n77A14 |
| DC_29A-66A_n2A | DC_66A_n2A |
| DC_29A-66A-66A_n2A | DC_66A_n2A |
| DC_29A-66A_n30A | DC_66A_n30A |
| DC_29A-66A-66A_n30A | DC_66A_n30A |
| DC_29A-66A_n77A14 DC_29A-66A-66A_n77A14 | DC_66A_n77A14 |
| DC_29A-66A_n78A | DC_66A_n78A |
| DC_30A-(n)5AA | DC_30A_n5A DC_(n)5AA2 |
| DC_30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A_n66A | DC_30A_n66A DC_66A_n66A2 |
| DC_30A-66A_n77A14 DC_30A-66A-66A_n77A14 | DC_30A_n77A14 DC_66A_n77A14 |
| DC_30A-66A_n77(2A) 14 DC_30A-66A-66A_n77(2A) 14 | DC_30A_n77A14 DC_66A_n77A14 |
| DC_32A-38A_n1A | DC_38A_n1A |
| DC_32A-38A_n28A | DC_38A_n28A |
| DC_38A_n3A-n78A | DC_38A_n3A DC_38A_n78A |
| DC_38A_n28A-n78A | DC_38A_n28A DC_38A_n78A |
| DC_39A_n40A-n41A DC_39A_n40A-n41C | DC_39A_n40A DC_39A_n41A |
| DC_39A_n40A-n79A DC_39A_n40A-n79C | DC_39A_n40A DC_39A_n79A |
| DC_39A_n41A-n79A DC_39A_n41A-n79C DC_39A_n41C-n79A DC_39A_n41C-n79C | DC_39A_n41A DC_39A_n79A |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | DC_40A_n1A DC_40A_n78A |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A |
| DC_40A-42A_n77A DC_40A-42A_n77C | DC_40A_n77A |
| DC_40A-42A_n78A | DC_40A_n78A |
| DC_41A_n1A-n3A | DC_41A_n1A DC_41A_n3A |
| DC_41C_n1A-n3A | DC_41A_n1A DC_41A_n3A |
| DC_41A_n1A-n77A | DC_41A_n1A DC_41A_n77A |
| DC_41C_n1A-n77A | DC_41A_n1A DC_41A_n77A DC_41C_n77A |
| DC_41A_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41C_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41A_n3A-n41A | DC_41A_n3A DC_41A_n41A |
| DC_41A_n3A-n77A | DC_41A_n3A DC_41A_n77A |
| DC_41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_41A_n3A-n78A | DC_41A_n3A DC_41A_n78A |
| DC_41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_41A_n28A-n41A | DC_41A_n28A |
| DC_41A_n28A-n77A | DC_41A_n28A DC_41A_n77A |
| DC_41C_n28A-n77A | DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_41A_n28A-n78A | DC_41A_n28A DC_41A_n78A |
| DC_41C_n28A-n78A | DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_(n)41AA-n78A DC_(n)41CA-n78A DC_(n)41DA-n78A | DC_41A_n78A |
| DC_41A_n41A-n77A | DC_41A_n77A |
| DC_41A_n41A-n78A DC_41C_n41A-n78A | DC_41A_n78A |
| DC_41A-42A_n77A15,16 DC_41A-42C_n77A15,16 DC_41C-42A_n77A15,16 DC_41C-42C_n77A15,16 | DC_41A_n77A |
| DC_41A-42A_n77(2A)15,16 DC_41A-42C_n77(2A)15,16 | DC_41A_n77A |
| DC_41A-42A_n78A15,16 DC_41A-42C_n78A15,16 DC_41C-42A_n78A15,16 DC_41C-42C_n78A15,16 | DC_41A_n78A |
| DC_41A-42A_n79A DC_41A-42C_n79A DC_41C-42A_n79A DC_41C-42C_n79A | DC_41A_n79A |
| DC_42A_n1A-n3A5 | DC_42A_n1A DC_42A_n3A |
| DC_42C_n1A-n3A5, | DC_42A_n1A DC_42A_n3A DC_42C_n1A DC_42C_n3A |
| DC_42A_n1A-n77A15,16 | DC_42A_n1A |
| DC_42C_n1A-n77A15,16 | DC_42A_n1A DC_42C_n1A |
| DC_42A_n1A-n78A15,16 DC_42C_n1A-n78A15,16 | N/A |
| DC_42A_n1A-n79A DC_42C_n1A-n79A | N/A |
| DC_42A_n3A-n28A | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A | DC_42A_n3A DC_42A_n28A DC_42C_n28A |
| DC_42A_n3A-n77A15,16 | DC_42A_n3A |
| DC_42A_n3A-n77(2A)15,16 | DC_42A_n3A |
| DC_42C_n3A-n77A15,16 | DC_42A_n3A DC_42C_n3A |
| DC_42C_n3A-n77(2A)15,16 | DC_42A_n3A DC_42C_n3A |
| DC_42A_n28A-n77A15,16 | DC_42A_n28A |
| DC_42A_n28A-n77(2A)15,16 | DC_42A_n28A |
| DC_42C_n28A-n77A15,16 | DC_42A_n28A DC_42C_n28A |
| DC_42C_n28A-n77(2A)15,16 | DC_42A_n28A DC_42C_n28A |
| DC_46A-48A_n2A3 DC_46C-48A_n2A3 DC_46D-48A_n2A3 DC_46E-48A_n2A3 | DC_48A_n2A |
| DC_46A-48A_n5A3 DC_46C-48A_n5A3 DC_46D-48A_n5A3 DC_46E-48A_n5A3 | DC_48A_n5A |
| DC_46A-48A_n66A3 DC_46C-48A_n66A3 DC_46D-48A_n66A3 DC_46E-48A_n66A3 | DC_48A_n66A |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | DC_66A_n5A |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |
| DC_46A-66A_n41A DC_46C-66A_n41A DC_46D-66A_n41A | DC_66A_n41A |
| DC_46A-66A_n41(2A) DC_46C-66A_n41(2A) DC_46D-66A_n41(2A) | DC_66A_n41A |
| DC_46A-66A_n71A DC_46C-66A_n71A DC_46D-66A_n71A | DC_66A_n71A |
| DC_46A-66A_n77A DC_46A-46A-66A_n77A | DC_66A_n77A |
| DC_48A-(n)5AA | DC_48A_n5A DC_(n)5AA2 |
| DC_48A-(n)12AA | DC_48A_n12A DC_(n)12AA2 |
| DC_48A_n25A-n48A | DC_48A_n25A |
| DC_48A_n48A-n66A | DC_48A_n66A |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | DC_66A_n2A DC_48A_n2A |
| DC_48A-66A_n5A DC_48B-66A_n5A DC_48C-66A_n5A DC_48D-66A_n5A DC_48E-66A_n5A | DC_66A_n5A |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | DC_48A_n25A DC_66A_n25A |
| DC_48A-66A_n48A | DC_66A_n48A |
| DC_48A-66A_n66A DC_48C-66A_n66A DC_48D-66A_n66A DC_48E-66A_n66A | DC_66A_n66A2 DC_48A_n66A |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A |
| DC_48A-66A_n77A14,15,16 DC_48A-66A_n77C14,15,16 DC_48C-66A_n77A14,15,16 DC_48C-66A_n77C14,15,16 DC_48D-66A_n77A14,15,16 DC_48D-66A_n77C14,15,16 DC_48E-66A_n77A14,15,16 | DC_66A_n77A14 |
| DC_48A-48A-66A_n77A14,15,16 | DC_66A_n77A |
| DC_67A-(n)3AA | DC_(n)3AA2 |
| DC_66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |
| DC_66A-66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |
| DC_66A_n2A-n38A | DC_66A_n2A DC_66A_n38A |
| DC_66A_n2A-n41A | DC_66A_n2A DC_66A_n41A |
| DC_66A_n2A-n66A | DC_66A_n2A |
| DC_66A_n2A-n71A | DC_66A_n2A DC_66A_n71A |
| DC_66A_n2A-n77A14 DC_66A_n2A-n77C14 DC_66A-66A_n2A-n77C14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A-66A_n2A-n77A14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A_n2A-n78A | DC_66A_n2A DC_66A_n78A |
| DC_66A_n5A-n48A | DC_66A_n5A DC_66A_n48A |
| DC_66A_n5A-n77A14 DC_66A_n5A-n77C14 DC_66A-66A_n5A-n77C14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A-66A_n5A-n77A14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n12A-n77A | DC_66A_n77A DC_66A_n12A |
| DC_66A_n12A-n78A | DC_66A_n12A DC_66A_n78A |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_66A_n38A-n66A | DC_66A_n38A DC_66A_n66A2 |
| DC_66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_66A_n66A-n77A14 DC_66A_n66A-n77C14 | DC_66A_n77A14 |
| DC_66A_n66A-n78A | DC_66A_n66A2 DC_66A_n78A |
| DC_66A-(n)12AA | DC_66A_n12A DC_(n)12AA2 |
| DC_66A-(n)71AA DC_66C-(n)71AA | DC_66A_n71A DC_(n)71AA |
| DC_66A_n25A-n41A DC_66A_n25A-n41C | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n41(2A) | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n48A | DC_66A_n25A DC_66A_n48A |
| DC_66A_n25A-n66A | DC_66A_n25A DC_66A_n66A2 |
| DC_66A_n38A-n71A | DC_66A_n38A DC_66A_n71A |
| DC_66A_n41A-n66A | DC_66A_n41A DC_66A_n66A2 |
| DC_66A_n41A-n71A DC_66A_n41C-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n66A-n71A | DC_66A_n66A DC_66A_n71A |
| DC_66A-71A_n2A | DC_71A_n2A DC_66A_n2A |
| DC_66A-71A_n2(2A) | DC_66A_n2A DC_71A_n2A |
| DC_66A-71A_n7A | DC_66A_n7A DC_71A_n7A |
| DC_66A-71A_n12A | DC_66A_n12A |
| DC_66A-71A_n25A | DC_66A_n25A DC_71A_n25A |
| DC_66A-71A_n38A | DC_71A_n38A DC_66A_n38A |
| DC_66A-71A_n41A | DC_66A_n41A DC_71A_n41A |
| DC_66A-71A_n66A | DC_71A_n66A DC_66A_n66A2 |
| DC_66A-71A_n71A | DC_66A_n71A |
| DC_66A-71A_n77A | DC_66A_n77A DC_71A_n77A |
| DC_66A-71A_n77(2A) | DC_66A_n77A DC_71A_n77A |
| DC_66A_n71A-n77A | DC_66A_n71A DC_66A_n77A |
| DC_66A-71A_n78A | DC_71A_n78A DC_66A_n78A |
| DC_66A-71A_n78(2A) | DC_71A_n78A DC_66A_n78A |
| DC_66A_n71A-n78A | DC_66A_n71A DC_66A_n78A |
| DC_66A_SUL_n78A-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| DC_66A_SUL_n78(2A)-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| DC_71A_n2A-n41A | DC_71A_n2A DC_71A_n41A |
| DC_71A_n2A-n66A | DC_71A_n2A DC_71A_n66A |
| DC_71A_n2A-n77A | DC_71A_n77A DC_71A_n2A |
| DC_71A_n2A-n78A | DC_71A_n2A DC_71A_n78A |
| DC_71A_n38A-n66A | DC_71A_n38A DC_71A_n66A |
| DC_71A_n38A-n78A | DC_71A_n38A DC_71A_n78A |
| DC_71A_n41A-n66A | DC_71A_n41A DC_71A_n66A |
| DC_71A_n66A-n77A | DC_71A_n66A DC_71A_n77A |
| DC_71A_n66A-n78A | DC_71A_n66A DC_71A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: N/A NOTE 7: Void. NOTE 8: Void NOTE 9: Void NOTE 10: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE 11: The frequency range in band 3 is restricted for this band combination to 1765 - 1785 MHz for the UL and 1860-1880 MHz for the DL. NOTE 12: The frequency range in band 42 is restricted for this band combination to 3440 - 3520 MHz. NOTE 13: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. NOTE 14: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 17: Void. NOTE 18: Void. NOTE 19: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 20: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 21: The downlink DC_2_n2 RESSENS requirements only apply when the band n2 downlink carrier is configured closer to the uplink operating band than the E-UTRA Band 2 downlink carrier. NOTE 22: The frequency range in band 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758 - 788 MHz for the DL. NOTE 23: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. NOTE 24: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. NOTE 25: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |

--Unaffected parts omitted—

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB |  | DL Fc (MHz) | MSD (dB) | IMD order |  |
| DC_1A-3A_n1A | n1 | 1950 | 5 |  | 25 | 2140 | N/A | N/A |
|  | 3 | 1750 | 5 |  | 25 | 1845 | N/A | N/A |
|  | 1 | N/A | 5 |  | N/A | 2150 | 23 | IMD3 |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1818.5 | 4.0 | IMD5 |  |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2139 | 11.0 | IMD4 |  |
|  | 3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | N/A | 5 | N/A |  | 2150 | 5 | IMD4 |  |
|  | 3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | n71 | 675 | 5 | 25 |  | 629 | N/A | N/A |  |
| DC_1A_n3A-n28A | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1818.5 | 4.0 | IMD5 |  |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
| DC_1A_n3A-n41A | 1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | n3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A |  | 2507.5 | 5.0 | IMD5 |  |
| DC_1A_n3A-n75A | n75 | N/A | 5 | N/A |  | 1480 | 15.2 | IMD34 |  |
|  | n3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | 1 | 1960 | 5 | 25 |  | 2150 | N/A | N/A |  |
| DC_1A_n3A-n79A | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4950 | 4.7 | IMD5 |  |
| DC_1A_n5A-n40A | 1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | n5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
|  | n40 | N/A | 10 | N/A |  | 2305 | 9.0 | IMD4 |  |
|  | 1 | 1945 | 5 | 25 |  | 2135 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A |  | 880 | 8.5 | IMD4 |  |
|  | n40 | 2385 | 5 | 25 |  | 2385 | N/A | N/A |  |
| DC_1A-7A_n28A DC_1A-7C_n28A DC_1A-7A-7A_n28A | 1 | 1935 | 5 | 25 |  | 2125 | N/A | N/A |  |
|  | n28 | 718 | 5 | 25 |  | 773 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2653 | 30.0 | IMD2 |  |
| DC_1A-7A_n40A | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
| DC_1A-7A-7A_n40A | 7 | N/A | 5 | N/A |  | 2630 | 23 | IMD3 |  |
|  | n40 | 2390 | 5 | 25 |  | 2390 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2120 | 16.4 | IMD3 |  |
|  | 7 | 2530 | 5 | 25 |  | 2650 | N/A | N/A |  |
|  | n40 | 2310 | 5 | 25 |  | 2310 | N/A | N/A |  |
| DC_1A_n8A-n77A | 1 | 1955 | 5 | 25 |  | 2145 | N/A | N/A |  |
| DC_1A_n8A-n77(2A) | n8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3410 | 1.5 | IMD5 |  |
| DC_1A_n8A-n77A | n8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
| DC_1A_n8A-n77(2A) | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3960 | 8.8 | IMD3 |  |
| DC_1A_n8A-n77A | 1 | 1955 | 5 | 25 |  | 2145 | N/A | N/A |  |
| DC_1A_n8A-n77(2A) | n77 | 3410 | 10 | 50 |  | 3410 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A |  | 955 | 3.3 | IMD5 |  |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_1A_n8A-n77(2A) | 8 | N/A | N/A | N/A |  | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_1A-3A_n77A DC_1A-3A_n77(2A) DC_1A-3A_n77(3A) DC_1A-3C_n77A DC_1A-3A_n77C DC_1A-3C_n77(2A) | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1807.5 | 31.5 | IMD2 |  |
|  | n77 | 3757.5 | 10 | 50 |  | 3757.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1870 | 8.5 | IMD4 |  |
|  | n77 | 3980 | 10 | 50 |  | 3980 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 31.0 | IMD2 |  |
|  | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | n77 | 3915 | 10 | 50 |  | 3915 | N/A | N/A |  |
| DC_1A-3A_n78A DC_1A-3A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78C DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) DC_1A-3A_n78(A-C) | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1807.5 | 31.2 | IMD2 |  |
|  | n78 | 3757.5 | 10 | 50 |  | 3757.5 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2125 | 2.8 | IMD5 |  |
|  | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | n78 | 3725 | 10 | 50 |  | 3725 | N/A | N/A |  |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3700 | 28.4 | IMD2 |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3360 | 11.2 | IMD4 |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1807.5 | 31.5 | IMD2 |  |
|  | n77 | 3757.5 | 10 | 50 |  | 3757.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1870 | 8.5 | IMD4 |  |
|  | n77 | 3980 | 10 | 50 |  | 3980 | N/A | N/A |  |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3700 | 28.4 | IMD2 |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1830 | 27.9 | IMD2 |  |
|  | n78 | 3780 | 10 | 50 |  | 3780 | N/A | N/A |  |
| DC_1A-3A_n105A | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1855 | 4 | IMD5 |  |
|  | n105 | 695 | 5 | 25 |  | 644 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2160 | 5 | IMD4 |  |
|  | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | n105 | 695 | 5 | 25 |  | 644 | N/A | N/A |  |
| DC_1A-5A_n77A DC_1A-5A_n77(2A) DC_1A-5A_n77(3A) | 1 | N/A | 5 | N/A |  | 2122 | 18.1 | IMD3 |  |
|  | 5 | 829 | 5 | 25 |  | 874 | N/A | N/A |  |
|  | n77 | 3780 | 10 | 50 |  | 3780 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 885 | 3.1 | IMD5 |  |
|  | n77 | 3405 | 10 | 50 |  | 3405 | N/A | N/A |  |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1807.5 | 37.5 | IMD21 |  |
|  | n77 | 3757.5 | 10 | 50 |  | 3757.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1870 | 20.5 | IMD41 |  |
|  | n77 | 3980 | 10 | 50 |  | 3980 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 37.0 | IMD21 |  |
|  | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | n77 | 3915 | 10 | 50 |  | 3915 | N/A | N/A |  |
| DC_1A-5A_n78A DC_1A-5A_n78C DC_1A-5A_n78(A-C) | 1 | N/A | 5 | N/A |  | 2122 | 18.1 | IMD3 |  |
|  | 5 | 829 | 5 | 25 |  | 874 | N/A | N/A |  |
|  | n78 | 3780 | 10 | 50 |  | 3780 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 |  | 3405 | N/A | N/A |  |
| DC_1A-7A_n77A DC_1A-7A_n77(2A) DC_1A-7A_n77(3A) DC_1A-7A-7A_n77A DC_1A-7A-7A_n77(2A) DC_1A-7A-7A_n77(3A) | 1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2627.5 | 9.1 | IMD44 |  |
|  | n77 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 8.7 | IMD4 |  |
|  | 7 | 2510 | 10 | 50 |  | 2630 | N/A | N/A |  |
|  | n77 | 3580 | 10 | 50 |  | 3580 | N/A | N/A |  |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) DC_1A-7A_n78C DC_1A-7A_n78(A-C) DC_1A-1A-7A_n78A DC_1A-7A-7A_n78C DC_1A-7A-7A_n78(A-C) | 1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2627.5 | 9.1 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 8.7 | IMD4 |  |
|  | 7 | 2510 | 10 | 50 |  | 2630 | N/A | N/A |  |
|  | n78 | 3580 | 10 | 50 |  | 3580 | N/A | N/A |  |
| DC_1A_n7A-n78A DC_1A_n7B-n78A DC_1A_n7A-n78(2A) | 1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A |  | 2627.5 | 9.1 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3390 | 10.1 | IMD4 |  |
| DC_1A-3A_n79A | 1 | N/A | 5 | N/A |  | 2140 | 3.6 | IMD5 |  |
|  | 3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | n79 | 4860 | 40 | 216 |  | 4860 | N/A | N/A |  |
| DC_1A-5A_n40A | 1 | N/A | 5 | N/A |  | 2144 | 4.0 | IMD5 |  |
|  | 5 | 832 | 5 | 25 |  | 877 | N/A | N/A |  |
|  | n40 | 2320 | 5 | 25 |  | 2320 | N/A | N/A |  |
|  | 1 | 1945 | 5 | 25 |  | 2135 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 880 | 8.0 | IMD4 |  |
|  | n40 | 2385 | 5 | 25 |  | 2385 | N/A | N/A |  |
| DC_1A-5A_n79A | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 882.5 | 18.3 | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 |  | 4782.5 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 882.5 | 8.9 | IMD4 |  |
|  | n79 | 4907.5 | 40 | 216 |  | 4907.5 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 8.1 | IMD4 |  |
|  | 5 | 837.5 | 5 | 25 |  | 882.5 | N/A | N/A |  |
|  | n79 | 4652.5 | 40 | 216 |  | 4652.5 | N/A | N/A |  |
| DC_1A-7A_n105A | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2673 | 30 | IMD2 |  |
|  | n105 | 698 | 5 | 25 |  | 647 | N/A | N/A |  |
| DC_1A-8A_n7A | 1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | n7 | 2502.5 | 5 | 25 |  | 2622.5 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 927.5 | 1.0 | IMD5 |  |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n28 | 730 | 5 | 25 |  | 785 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 950 | 3.3 | IMD5 |  |
| DC_1A-8A_n40A | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 930 | 8.0 | IMD4 |  |
|  | n40 | 2395 | 5 | 25 |  | 2395 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2135 | 5.3 | IMD5 |  |
|  | 8 | 885 | 5 | 25 |  | 930 | N/A | N/A |  |
|  | n40 | 2395 | 5 | 25 |  | 2395 | N/A | N/A |  |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 |  | 2145 | N/A | N/A |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 | 3410 | 10 | 50 |  | 3410 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 955 | 3.3 | IMD5 |  |
| DC_1A-8A_n77A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 | 3960 | 10 | 50 |  | 3960 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 14.4 | IMD3 |  |
| DC_1A-8A_n79A | 1 | 1935 | 5 | 25 |  | 2125 | N/A | N/A |  |
|  | n79 | 4815 | 40 | 216 |  | 4815 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 945 | 15.8 | IMD3 |  |
| DC_1A-8A_n79A | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n79 | 4845 | 40 | 216 |  | 4845 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2145 | 8.2 | IMD4 |  |
| DC_1A_n8A-n40A | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A |  | 930 | 8.0 | IMD4 |  |
|  | n40 | 2395 | 5 | 25 |  | 2395 | N/A | N/A |  |
| DC_1A_n8A-n78A | 1 | 1945 | 5 | 25 |  | 2135 | N/A | N/A |  |
|  | n8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3745 | 14.9 | IMD3 |  |
|  | 1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A |  | 940 | 3.3 | IMD5 |  |
|  | n78 | 3380 | 10 | 50 |  | 3330 | N/A | N/A |  |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 |  | 2150 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1480 | 15.2 | IMD3 |  |
| DC_1A-11A_n28A | 11 | 1440 | 5 | 25 |  | 1488 | N/A | N/A |  |
|  | n28 | 710 | 5 | 25 |  | 765 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2150 | 28.3 | IMD21 |  |
| DC_1A-11A_n41A | 11 | 1442 | 5 | 25 |  | 1490 | N/A | N/A |  |
|  | n41 | 2520 | 10 | 50 |  | 2520 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2156 | 10.2 | IMD4 |  |
|  | 1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 |  | 2685 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1490 | 10.6 | IMD4 |  |
| DC_1A-11A_n77A DC_1A-11A_n77(2A) DC_1A-11A_n77(3A) | 1 | 1955 | 5 | 25 |  | 2145 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1486 | 31.4 | IMD2 |  |
|  | n77 | 3441 | 10 | 50 |  | 3441 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 30.8 | IMD2 |  |
|  | 11 | 1438 | 5 | 25 |  | 1486 | N/A | N/A |  |
|  | n77 | 3578 | 10 | 50 |  | 3578 | N/A | N/A |  |
| DC_1A-11A_n78A DC_1A-11A_n78(2A) | 1 | 1955 | 5 | 25 |  | 2145 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1486 | 31.4 | IMD2 |  |
|  | n78 | 3441 | 10 | 50 |  | 3441 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 30.8 | IMD2 |  |
|  | 11 | 1438 | 5 | 25 |  | 1486 | N/A | N/A |  |
|  | n78 | 3578 | 10 | 50 |  | 3578 | N/A | N/A |  |
| DC_1A-11A_n79A | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1483 | 10.2 | IMD4 |  |
|  | n79 | 4427 | 40 | 216 |  | 4427 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2118 | 15.6 | IMD3 |  |
|  | 11 | 1431 | 5 | 25 |  | 1479 | N/A | N/A |  |
|  | n79 | 4980 | 40 | 216 |  | 4980 | N/A | N/A |  |
| DC_1A-18A_n77A DC_1A-18A_n77(2A) | 1 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 18 | N/A | N/A | N/A |  | N/A | N/A | IMD5 |  |
|  | n77 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2120 | 16.4 | IMD3 |  |
|  | 18 | 825 | 5 | 25 |  | 870 | N/A | N/A |  |
|  | n77 | 3770 | 10 | 50 |  | 3770 | N/A | N/A |  |
| DC_1A-18A_n78A DC_1A-18A_n78(2A) | 1 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 18 | N/A | N/A | N/A |  | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2120 | 16.4 | IMD3 |  |
|  | 18 | 819 | 5 | 25 |  | 864 | N/A | N/A |  |
|  | n78 | 3758 | 10 | 50 |  | 3758 | N/A | N/A |  |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 |  | 2125 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A |  | 867.5 | 18.3 | IMD3 |  |
|  | n79 | 4737.5 | 40 | 216 |  | 4737.5 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A |  | 865 | 8.9 | IMD4 |  |
|  | n79 | 4925 | 40 | 216 |  | 4925 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2125 | 8.1 | IMD4 |  |
|  | 18 | 822.5 | 5 | 25 |  | 867.5 | N/A | N/A |  |
|  | n79 | 4592.5 | 40 | 216 |  | 4592.5 | N/A | N/A |  |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | N/A | 5 | N/A |  | 2130 | 17.8 | IMD3 |  |
|  | 19 | 832.5 | 5 | 25 |  | 877.5 | N/A | N/A |  |
|  | n77, n78 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
|  | 1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | 19 | N/A | 5 | N/A |  | 880 | 5.1 | IMD5 |  |
|  | n77, n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 19 | N/A | 5 | N/A |  | 882.5 | 18.3 | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 |  | 4782.5 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 8.1 | IMD4 |  |
|  | 19 | 837.5 | 5 | 25 |  | 882.5 | N/A | N/A |  |
|  | n79 | 4652.5 | 40 | 216 |  | 4652.5 | N/A | N/A |  |
| DC_1A-20A_n1A | n1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 20 | 850 | 5 | 25 |  | 809 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2160 | 6 | IMD4 |  |
| DC_1A_n28A-n41A | 1 | 1935 | 5 | 25 |  | 2125 | N/A | N/A |  |
|  | n28 | 718 | 5 | 25 |  | 773 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A |  | 2653 | 30.1 | IMD2 |  |
|  | 1 | 1923 | 5 | 25 |  | 2113 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 762 | 29.3 | IMD2 |  |
|  | n41 | 2685 | 10 | 50 |  | 2685 | N/A | N/A |  |
|  | 1 | 1935 | 5 | 25 |  | 2125 | N/A | N/A |  |
|  | n28 | N/A | 10 | N/A |  | 785 | 4.5 | IMD5 |  |
|  | n41 | 2510 | 10 | 50 |  | 2510 | N/A | N/A |  |
| DC_1A-20A_n7A | 1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A |  | 800 | 4.5 | IMD5 |  |
|  | n7 | 2510 | 10 | 50 |  | 2630 | N/A | N/A |  |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 805 | 11.5 | IMD4 |  |
|  | n8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
| DC_1A-20A_n38A | 1 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 20 | N/A | N/A | N/A |  | N/A | N/A | IMD5 |  |
|  | n38 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_1A-20A_n78A DC_1A-1A-20A_n78A | 1 | N/A | 5 | N/A |  | 2120 | 20.3 | IMD3 |  |
| DC_1A-20A_n78(2A) | 20 | 835 | 5 | 25 |  | 794 | N/A | N/A |  |
| DC_1A-20A_n78C | n78 | 3790 | 10 | 50 |  | 3790 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 810 | 3.0 | IMD5 |  |
|  | n78 | 3330 | 10 | 50 |  | 3330 | N/A | N/A |  |
| DC_1A-21A_n28A10 | 1 | N/A | 5 | N/A |  | 2165.3 | 16.1 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n28 | 735.5 | 5 | 25 |  | 790.5 | N/A | N/A |  |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | N/A | 5 | N/A |  | 2154.6 | 30.6 | IMD2 |  |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3605 | 10 | 50 |  | 3605 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2154.6 | 3.6 | IMD5 |  |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3647 | 10 | 50 |  | 3647 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A |  | 1500 | 31.5 | IMD2 |  |
|  | n77, n78 | 3450 | 10 | 50 |  | 3450 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A |  | 1500 | 2.9 | IMD5 |  |
|  | n77, n78 | 3675 | 10 | 50 |  | 3675 | N/A | N/A |  |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A |  | N/A | N/A | IMD4 |  |
|  | n79 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_1A-26A_n78A | 1 | N/A | 5 | N/A |  | 2122 | 18.1 | IMD3 |  |
| 829 | 26 |  | 5 | 25 |  | 874 | N/A | N/A |  |
| 3780 | n78 |  | 10 | 50 |  | 3780 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 26 | N/A | 5 | N/A |  | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 |  | 3405 | N/A | N/A |  |
| DC_1A_n26A-n78A | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n26 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3610 | 15.7 | IMD3 |  |
|  | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | n26 | N/A | 5 | N/A |  | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 |  | 3405 | N/A | N/A |  |
| DC_1A-28A_n3A | 1 | N/A | 5 | N/A |  | 2139 | 11.0 | IMD4 |  |
|  | 28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
|  | n3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 | 5 | 25 |  | 2125 | N/A | N/A |  |
|  | 28 | N/A | 10 | N/A |  | 785 | 4.5 | IMD5 |  |
|  | n7 | 2510 | 10 | 50 |  | 2630 | N/A | N/A |  |
| DC_1A-28A_n40A | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
| DC_1A-28A_n38A | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 765 | 4.5 | IMD5 |  |
|  | n38 | 2580 | 5 | 25 |  | 2580 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 780 | 8.9 | IMD4 |  |
|  | n40 | 2340 | 5 | 25 |  | 2340 | N/A | N/A |  |
| DC_1A-28A_n77A | 1 | N/A | 5 | N/A |  | 2150 | 15.7 | IMD3 |  |
|  | 28 | 740 | 5 | 25 |  | 795 | N/A | N/A |  |
|  | n77 | 3630 | 10 | 50 |  | 3630 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 794 | 4.2 | IMD5 |  |
|  | n77 | 3352 | 10 | 50 |  | 3352 | N/A | N/A |  |
| DC_1A-28A_n78A | 1 | N/A | 5 | N/A |  | 2150 | 15.7 | IMD3 |  |
|  | 28 | 740 | 5 | 25 |  | 795 | N/A | N/A |  |
|  | n78 | 3630 | 10 | 50 |  | 3630 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 794 | 4.2 | IMD5 |  |
|  | n78 | 3352 | 10 | 50 |  | 3352 | N/A | N/A |  |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 788 | 15.2 | IMD3 |  |
|  | n79 | 4648 | 40 | 216 |  | 4648 | N/A | N/A |  |
|  | 1 | 1925 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 795 | 10.0 | IMD4 |  |
|  | n79 | 4980 | 40 | 216 |  | 4980 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2167.5 | 1.2 | IMD4 |  |
|  | 28 | 745.5 | 5 | 25 |  | 800.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 |  | 4420 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2125 | 4.5 | IMD5 |  |
|  | 28 | 718 | 5 | 25 |  | 773 | N/A | N/A |  |
|  | n79 | 4807 | 40 | 216 |  | 4807 | N/A | N/A |  |
| DC_1A_n28A-n40A | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2374 | 10.1 | IMD4 |  |
|  | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 768 | 8.6 | IMD4 |  |
|  | n40 | 2314 | 5 | 25 |  | 2314 | N/A | N/A |  |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n28 | 733 | 5 | 25 |  | 788 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3416 | 15.7 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | 3320 | 10 | 50 |  | 3320 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 790 | 4.2 | IMD5 |  |
| DC_1A_n28A-n79A | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 788 | 15.2 | IMD39 |  |
|  | n79 | 4648 | 40 | 216 |  | 4648 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n28 | 730 | 5 | 25 |  | 785 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4630 | 14.9 | IMD34 |  |
| DC_1A-32A_n3A | n3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1480 | 15.2 | IMD34 |  |
|  | 1 | 1960 | 5 | 25 |  | 2150 | N/A | N/A |  |
| DC_1A-32A_n78A DC_1A-32A_n78C DC_1A-32A_n78(2A) | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1470 | 31.8 | IMD2 |  |
|  | n78 | 3400 | 10 | 50 |  | 3400 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1470 | 0 | IMD5 |  |
|  | n78 | 3630 | 10 | 50 |  | 3630 | N/A | N/A |  |
| DC_1A-38A_n78A DC_1A-38A_n78(2A) | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | 38 | N/A | 5 | N/A |  | 2590 | 12.7 | IMD4 |  |
|  | n78 | 3320 | 10 | 50 |  | 3320 | N/A | N/A |  |
| DC_1A_n38A-n78A | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A |  | 2590 | 12.7 | IMD4 |  |
|  | n78 | 3320 | 10 | 50 |  | 3320 | N/A | N/A |  |
| DC_1A_n40A-n77A | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
| DC_1A_n40A-n77(2A) | n40 | 2340 | 5 | 25 |  | 2340 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3450 | 9.8 | IMD4 |  |
|  | 1 | 1960 | 5 | 25 |  | 2150 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2360 | 10.6 | IMD4 |  |
|  | n77 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A |  | 2340 | 10.6 | IMD4 |  |
|  | n78 | 3450 | 10 | 50 |  | 3450 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 9.1 | IMD4 |  |
|  | 40 | 2360 | 5 | 25 |  | 2360 | N/A | N/A |  |
|  | n78 | 3430 | 10 | 50 |  | 3430 | N/A | N/A |  |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
| DC_1A_n40A-n78C | n40 | 2340 | 5 | 25 |  | 2340 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3450 | 9.8 | IMD4 |  |
|  | 1 | 1960 | 5 | 25 |  | 2150 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2360 | 10.6 | IMD4 |  |
|  | n78 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2507.5 | 5.0 | IMD5 |  |
|  | n3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 |  | 2125 | N/A | N/A |  |
|  | 41 | N/A | 10 | N/A |  | 2653 | 30 | IMD2 |  |
|  | n28 | 718 | 5 | 25 |  | 773 | N/A | N/A |  |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | 41 | 2510 | 5 | 25 |  | 2510 | N/A | IMD4 |  |
|  | n77 | 3400 | 10 | 50 |  | 3400 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 9.3 | IMD4 |  |
|  | 41 | 2640 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | n77 | 3710 | 10 | 50 |  | 3710 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 |  | 2120 | 11.0 | N/A |  |
|  | 41 | 2510 | 5 | 25 |  | 2510 | N/A | IMD5 |  |
|  | n77 | 4150 | 10 | 50 |  | 4150 | N/A |  |  |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 | N/A | 5 | N/A |  | 2140 | 9.3 | IMD4 |  |
|  | 41 | 2640 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | n78 | 3710 | 10 | 50 |  | 3710 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2515 | 12 | IMD4 |  |
|  | n78 | 3410 | 10 | 50 |  | 3410 | N/A | N/A |  |
| DC_1A_n41A-n77A DC_1A_n41A-n77(2A) DC_1A_n41A-n78A DC_1A_n41A-n78(2A) | 1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A |  | 2515 | 11.5 | IMD4 |  |
|  | n78 | 3410 | 10 | 50 |  | 3410 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n41 | 2650 | 10 | 25 |  | 2650 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3330 | 19.6 | IMD3 |  |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2530 | 29.4 | IMD2 |  |
|  | n79 | 4500 | 40 | 216 |  | 4500 | N/A | N/A |  |
| DC_1A-42A_n3A | 1 | 1922.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
|  | n3 | 1782.5 | 5 | 25 |  | 1877.5 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A |  | 3425 | 13.0 | IMD4 |  |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n28 | 733 | 5 | 25 |  | 788 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A |  | 3416 | 15.7 | IMD3 |  |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 |  | 3580 | N/A | N/A |  |
|  | n28 | 723 | 5 | 25 |  | 778 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2134 | 15.7 | IMD3 |  |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 |  | 4420 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A |  | 3490 | 4.8 | IMD5 |  |
|  | 42 | 3402.5 | 5 | 25 |  | 3402.5 | N/A | N/A |  |
|  | n79 | 4640 | 40 | 216 |  | 4640 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2165 | 15.5 | IMD3 |  |
|  | 42 | 3450 | 5 | 25 |  | 3450 | N/A | N/A |  |
|  | n79 | 4520 | 40 | 216 |  | 4520 | N/A | N/A |  |
|  | 1 | N/A | 5 | N/A |  | 2140 | 9.3 | IMD4 |  |
| DC_1A_SUL_n77A-n80A | 1 | N/A | 5 | N/A |  | 2140 | 23 | IMD3 |  |
|  | n80 | 1760 | 5 | 25 |  |  | N/A | N/A |  |
| DC_1A_SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
|  | n80 | 1782.5 | 5 | 25 |  |  | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3425 | 13.0 | IMD4 |  |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | 1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n75 | N/A | 5 | N/A |  | 1470 | 30.4 | IMD2 |  |
|  | n78 | 3400 | 10 | 50 |  | 3400 | N/A | N/A |  |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | 3410 | 10 | 50 |  | 3410 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4870 | 15.9 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n79 | 4670 | 40 | 216 |  | 4670 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3490 | 4.6 | IMD5 |  |
| DC_1A_SUL_n78A-n80A | 1 | 1950 | 5 | 25 |  | 2140 | 23 | IMD3 |  |
|  | n80 | 1760 | 5 | 25 |  |  | N/A | N/A |  |
|  | 1 | 1922.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
|  | n80 | 1782.5 | 5 | 25 |  |  | N/A | N/A |  |
|  | n78 | 3425 | 10 | 50 |  | 3425 | 13.0 | IMD4 |  |
| DC_1_n78-n105 | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | n105 | 686 | 5 | 25 |  | 635 | 15.2 | IMD3 |  |
|  | 1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3342 | 15.7 | IMD3 |  |
|  | n105 | 686 | 5 | 25 |  | 635 | N/A | N/A |  |
| DC_2A-(n)66AA | 2 | 1883.3 | 5 | 25 |  | 1963.3 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2145 | 2.8 | IMD5 |  |
|  | n66 | 1750 | 5 | 25 |  | 2150 | 4 | IMD5 |  |
| DC_2A_n2A-n66A | 2 | 1875 | 5 | 25 |  | 1955 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1975 | 20 | IMD3 |  |
|  | n66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
| DC_2A_n2A-n77A | 2 | 1875 | 5 | 25 |  | 1955 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1935 | 26 | IMD2 |  |
|  | n77 | 3810 | 10 | 50 |  | 3810 | N/A | N/A |  |
|  | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1965 | 8.0 | IMD44 |  |
|  | n77 | 3735 | 10 | 50 |  | 3735 | N/A | N/A |  |
| DC_2A_n2A-n78A | 2 | 1852.5 | 5 | 25 |  | 1932.5 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1942.5 | 26 | IMD24 |  |
|  | n78 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
| DC_2A-4A_n28A | 2 | N/A | 5 | N/A |  | 1960 | 11.0 | IMD4 |  |
|  | 4 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n28 | 740 | 5 | 25 |  | 795 | N/A | N/A |  |
| DC_2A-4A_n41A | 2 | N/A | 5 | N/A |  | 1940 | 11.0 | IMD4 |  |
|  | 4 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 |  | 2685 | N/A | N/A |  |
| DC_2A-4A_n78A | 2 | 1875 | 5 | 25 |  | 1955 | N/A | N/A |  |
|  | 4 | N/A | 5 | N/A |  | 2145 | 10.3 | IMD4 |  |
|  | n78 | 3480 | 10 | 50 |  | 3480 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1960 | 32.1 | IMD2 |  |
|  | 4 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | 3700 | 10 | 50 |  | 3700 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1940 | 9.1 | IMD4 |  |
|  | 4 | 1750 | 5 | 25 |  | 2150 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1950 | 2.1 | IMD5 |  |
|  | 4 | 1750 | 5 | 25 |  | 2150 | N/A | N/A |  |
|  | n78 | 3600 | 10 | 50 |  | 3600 | N/A | N/A |  |
| DC_2A-5A_n12A8 | 2 | N/A | 5 | N/A |  | 1980 | 5.9 | IMD5 |  |
|  | 5 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
|  | n12 | 705 | 5 | 25 |  | 735 | N/A | N/A |  |
| DC_2A-5A_n30A | 2 | 1870 | 5 | 25 |  | 1959 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 880 | 9.7 | IMD4 |  |
|  | n30 | 2310 | 10 | 50 |  | 2355 | N/A | N/A |  |
| DC_2A-5A_n48A DC_2A-5A_n48B | 2 | N/A | 5 | N/A |  | 1962 | 15.6 | IMD3 |  |
|  | 5 | 839 | 5 | 25 |  | 884 | N/A | N/A |  |
|  | n48 | 3640 | 5 | 25 |  | 3640 | N/A | N/A |  |
| DC_2A-5A_n71A | 2 | 1855 | 5 | 25 |  | 1935 | N/A | N/A |  |
|  | n71 | 686.5 | 5 | 25 |  | 640.5 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 891.5 | 4.2 | IMD5 |  |
| DC_2A_n5A-n77A | 2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | n5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3540 | 16.0 | IMD3 |  |
| DC_2A_n5A-n77A11 | 2 | 1907 | 5 | 25 |  | 1987 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A |  | 889 | 3.8 | IMD5 |  |
|  | n77 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
| DC_2A-5A_n77A11 | 2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
| DC_2A-5A_n77C11 DC_2A-5A_n77(2A)11 DC_2A-2A-5A_n77A11 | 5 | N/A | 5 | N/A |  | 887.5 | 3.8 | IMD5 |  |
| DC_2A-2A-5A_n77C11 DC_2A-2A-5A_n77(2A)11 | n77 | 3305 | 5 | 25 |  | 3305 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1987 | 16.5 | IMD3 |  |
|  | 5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
|  | n77 | 3680 | 5 | 25 |  | 3680 | N/A | N/A |  |
| DC_2A-5A_n78A DC_2A-2A-5A_n78A DC_2A-5A_n78(2A) | 2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 887.5 | 3.8 | IMD5 |  |
|  | n78 | 3305 | 5 | 25 |  | 3305 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1987 | 16.5 | IMD3 |  |
|  | 5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
|  | n78 | 3680 | 5 | 25 |  | 3680 | N/A | N/A |  |
| DC_2A-7A_n5A DC_2A-7C_n5A DC_2A-7A-7A_n5A | 2 | 1855 | 10 | 50 |  | 1935 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2685 | 30.0 | IMD2 |  |
|  | n5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
| DC_2A-7A_n12A | 2 | 1907.5 | 5 |  | 25 | 1987.5 | N/A | N/A |
| DC_2A-2A-7A_n12A | 7 | 2502.5 | 5 |  | 25 | 2622.5 | 30.8 | IMD2 |
|  | n12 | 713.5 | 5 |  | 25 | 743.5 | N/A | N/A |
| DC_2A-7A_n28A DC_2A-7C_n28A | 2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2120 | 29.0 | IMD2 |  |
|  | n28 | 740 | 5 | 25 |  | 795 | N/A | N/A |  |
| DC_2A-7A_n77A DC_2A-2A-7A_n77A DC_2A-7C_n77A DC_2A-7A-7A_n77A DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) DC_2A-7A-7A_n77(2A) | 2 | N/A | 5 | N/A |  | 1950 | 8.6 | IMD4 |  |
|  | 7 | 2550 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n77 | 3525 | 10 | 50 |  | 3475 | N/A | N/A |  |
|  | 2 | 1860 | 5 | 25 |  | 1940 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2660 | 3.4 | IMD5 |  |
|  | n77 | 4120 | 10 | 50 |  | 4120 | N/A | N/A |  |
| DC_2A-7A_n78A DC_2A-2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 | N/A | 5 | N/A |  | 1950 | 8.6 | IMD4 |  |
|  | 7 | 2550 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n78 | 3525 | 10 | 50 |  | 3475 | N/A | N/A |  |
| DC_2A_n7A-n78A, DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n7 | 2525 | 5 | 25 |  | 2645 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3775 | 4.2 | IMD5 |  |
| DC_2-8_n2 | 2 | N/A | 5 | N/A |  | 1940 | 4 | IMD4 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
| DC_2A-12A_n5A DC_2A-2A-12A_n5A | 2 | N/A | 5 | N/A |  | 1980 | 5.9 | IMD5 |  |
|  | 12 | 705 | 5 | 25 |  | 735 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
| DC_2A-12A_n7A DC_2A-12A_n7(2A) | 2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
| DC_2A-2A-12A_n7A | 12 | N/A | 5 | N/A |  | 731.5 | 4.5 | IMD5 |  |
|  | n7 | 2502.5 | 5 | 25 |  | 2622.5 | N/A | N/A |  |
| DC_2A-12A_n41A DC_2A-2A-12A_n41A | 2 | N/A | 5 | N/A |  | 1952 | 26 | IMD2 |  |
|  | 12 | 708 | 5 | 50 |  | 738 | N/A | N/A |  |
|  | n41 | 2660 | 10 | 50 |  | 2660 | N/A | N/A |  |
|  | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A |  | 738 | 28.7 | IMD24 |  |
|  | n41 | 2638 | 10 | 50 |  | 2638 | N/A | N/A |  |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A |  | N/A | N/A | IMD4 |  |
|  | 12 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | n66 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_2A-12A_n77A DC_2A-12A_n77(2A) | 2 | N/A | 5 | N/A |  | 1960 | 16.5 | IMD39,11 |  |
| DC_2A-2A-12A_n77A DC_2A-2A-12A_n77(2A) | 12 | 707.5 | 5 | 25 |  | 737.5 | N/A | N/A |  |
|  | n77 | 3375 | 10 | 50 |  | 3375 | N/A | N/A |  |
| DC_2A_n12A-n77A DC_2A-2A_n12A-n77A | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n12 | 707.5 | 5 | 25 |  | 737.5 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3315 | 16.0 | IMD34,9,11 |  |
| DC_2A-12A_n78A DC_2A-2A-12A_n78A DC_2A-12A_n78(2A) | 2 | N/A | 5 | N/A |  | 1954 | 16.5 | IMD3 |  |
|  | 12 | 708 | 5 | 25 |  | 738 | N/A | N/A |  |
|  | n78 | 3370 | 10 | 50 |  | 3370 | N/A | N/A |  |
| DC_2A_n12A-n78A | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n12 | 707.5 | 5 | 25 |  | 737.5 | N/A | N/A |  |
|  | n78 | 3315 | 10 | 50 |  | 3315 | 16.0 | IMD3 |  |
| DC_2A-13A_n48A DC_2A-13A_n48B | 2 | N/A | 5 | N/A |  | 1983.5 | 15.6 | IMD3 |  |
|  | 13 | 784.5 | 5 | 25 |  | 753.5 | N/A | N/A |  |
|  | n48 | 3552.5 | 5 | 25 |  | 3552.5 | N/A | N/A |  |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 | N/A | 5 | N/A |  | 1940 | 6.2 | IMD4 |  |
|  | 13 | 780 | 10 | 50 |  | 749 | N/A | N/A |  |
|  | n66 | 1750 | 5 | 25 |  | 2150 | N/A | N/A |  |
| DC_2A-13A_n77A | 2 | N/A | 5 | N/A |  | 1944 | 16.0 | IMD3 |  |
| DC_2A-13A_n77C | 13 | 783 | 5 | 25 |  | 752 | N/A | N/A |  |
| DC_2A-2A-13A_n77A DC_2A-2A-13A_n77C | n77 | 3510 | 5 | 25 |  | 3510 | N/A | N/A |  |
| DC_2A-14A_n77A DC_2A-14A_n77(2A) | 2 | N/A | 5 | N/A |  | 1954 | 16.5 | IMD3 |  |
| DC_2A-2A-14A_n77A DC_2A-2A-14A_n77(2A) | 14 | 793 | 5 | 25 |  | 763 | N/A | N/A |  |
|  | n77 | 3540 | 10 | 50 |  | 3540 | N/A | N/A |  |
| DC_2_n25-n66 | 2 | 1855 | 5 | 25 |  | 1935 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A |  | 1935 | 20 | IMD3 |  |
|  | n66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
|  | 2 | 1883.3 | 5 | 25 |  | 1963.3 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A |  | 1963.3 | 4 | IMD5 |  |
|  | n66 | 1750 | 5 | 25 |  | 2150 | N/A | N/A |  |
| DC_2A_n38A-n71A | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A |  | 2586 | 29.2 | IMD2 |  |
|  | n71 | 686 | 5 | 25 |  | 640 | N/A | N/A |  |
| DC_2A_n38A-n78A | 2 | 1870 | 5 | 25 |  | 1950 | N/A | N/A |  |
|  | n38 | 2610 | 10 | 50 |  | 2610 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3350 | 14.8 | IMD3 |  |
| DC_2A-14A_n66A | 2 | N/A | 5 | N/A |  | 1954 | 7.2 | IMD4 |  |
|  | 14 | 793 | 5 | 25 |  | 763 | N/A | N/A |  |
|  | 66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
| DC_2A-28A_n66A | 2 | N/A | 5 | N/A |  | 1980 | 11 | IMD4 |  |
|  | 28 | 730 | 5 | 25 |  | 785 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
| DC_2A-30A_n77A DC_2A-30A_n77(2A) | 2 | N/A | 5 | N/A |  | 1986 | 8.6 | IMD411 |  |
| DC_2A-2A-30A_n77A DC_2A-2A-30A_n77(2A) | 30 | 2312 | 5 | 25 |  | 2357 | N/A | N/A |  |
|  | n77 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | 2 | 1905 | 5 | 25 |  | 1985 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2354 | 10.6 | IMD411 |  |
|  | n77 | 3361 | 10 | 50 |  | 3361 | N/A | N/A |  |
|  | 2 | 1860 | 5 | 25 |  | 1940 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2354 | 3.4 | IMD5 |  |
|  | n77 | 3967 | 10 | 50 |  | 3967 | N/A | N/A |  |
| DC_2A-38A_n78A | 2 | N/A | 5 | N/A |  | 1932.5 | 16 | IMD39 |  |
|  | 38 | 2617.5 | 5 | 25 |  | 2617.5 | N/A | N/A |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
| DC_2A_n41A-n71A DC_2A-2A_n41A-n71A | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n41 | 2530 | 10 | 50 |  | 2530 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A |  | 630 | 28.7 | IMD2 |  |
|  | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A |  | 2586 | 29.2 | IMD2 |  |
|  | n71 | 686 | 5 | 50 |  | 640 | N/A | N/A |  |
| DC_2A-46A_n5A5 DC_2A-46C_n5A5 DC_2A-46D_n5A5 DC_2A-46E_n5A5 | 2 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_2A-2A-46A_n5A5 DC_2A-2A-46C_n5A5 DC_2A-2A-46D_n5A5 | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD4, IMD5 |  |
|  | n5 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_2A-46A_n66A5 DC_2A-46C_n66A5 DC_2A-46D_n66A5 DC_2A-46E_n66A5 | 2 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD3, IMD5 |  |
|  | n66 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_2A-46A_n77A5 DC_2A-46A-46A_n77A5 | 2 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD2, IMD3 |  |
|  | n77 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | n2 | 1853 | 5 | 25 |  | 1933 | N/A | N/A |  |
|  | 48 | 3590 | 20 | 100 |  | 3590 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1969 | 12 | IMD4 |  |
| DC_2A-48A_n5A | 2 | N/A | 5 | N/A |  | 1950 | 16.9 | IMD3 |  |
| DC_2A-48C_n5A | 48 | 3610 | 10 | 50 |  | 3610 | N/A | N/A |  |
| DC_2A-48D_n5A | n5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
| DC_2A-48E_n5A | 2 | 1890 | 5 | 25 |  | 1970 | N/A | N/A |  |
|  | 48 | N/A | 5 | N/A |  | 3570 | 16.2 | IMD3 |  |
|  | n5 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A | 2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | 48 | N/A | 10 | N/A |  | 3620 | 29.4 | IMD2 |  |
|  | n66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1960 | 28.3 | IMD2 |  |
|  | 48 | 3695 | 5 | 25 |  | 3695 | N/A | N/A |  |
|  | n66 | 1735 | 5 | 25 |  | 2135 | N/A | N/A |  |
| DC_2A_n48A-n66A | 2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
| DC_2A-48E_n66A | n48 | N/A | 10 | N/A |  | 3620 | 29.4 | IMD2 |  |
|  | n66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1980 | 20 | IMD3 |  |
| DC_2A-66A_n2A | 66 | 1730 | 5 | 25 |  | 2130 | N/A | N/A |  |
| DC_2A-66A-66A_n2A | n2 | 1855 | 5 | 25 |  | 1935 | N/A | N/A |  |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2140 | 7.2 | IMD4 |  |
|  | n5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
| DC_2A-66A_n25A | 2 | N/A | 5 | N/A |  | 1935 | 20 | IMD3 |  |
|  | 66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A |  | 1935 | 20 | IMD3 |  |
|  | 2 | 1883.3 | 5 | 25 |  | 1963.3 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2150 | 4 | IMD5 |  |
|  | n25 | 1883.3 | 5 | 25 |  | 1963.3 | N/A | N/A |  |
|  | 2 | 1883.3 | 5 | 25 |  | 1963.3 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2112.5 | 23 | IMD3 |  |
|  | n25 | 1912.5 | 5 | 25 |  | 1992.5 | N/A | N/A |  |
| DC_2A-66A_n28A | 2 | N/A | 5 | N/A |  | 1960 | 11.0 | IMD4 |  |
|  | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n28 | 740 | 5 | 25 |  | 795 | N/A | N/A |  |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2A-66A_n41(2A) | 2 | N/A | 5 | N/A |  | 1940 | 11.0 | IMD4 |  |
|  | 66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | n41 | 2685 | 5 | 25 |  | 2685 | N/A | N/A |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 | 5 | 25 |  | 1985 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2155 | 12.1 | IMD4 |  |
|  | n48 | 3560 | 5 | 25 |  | 3560 | N/A | N/A |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | N/A | 5 | N/A |  | 1960 | 28.3 | IMD5 |  |
|  | 66 | 1735 | 5 | 25 |  | 2135 | N/A | N/A |  |
|  | n48 | 3695 | 5 | 25 |  | 3695 | N/A | N/A |  |
| DC_2A-66A_n77A | 2 | 1855 | 5 | 25 |  | 1935 | N/A | N/A |  |
| DC_2A-66A_n77C DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77C DC_2A-2A-66A_n77(2A) DC_2A-66A-66A_n77A DC_2A-66A-66A_n77C DC_2A-66A-66A_n77(2A) DC_2A-2A-66A-66A_n77A DC_2A-2A-66A-66A_n77C | 66 | N/A | 5 | N/A |  | 2115 | 29.2 | IMD2 |  |
|  | n77 | 3970 | 5 | 25 |  | 3970 | N/A | N/A |  |
|  | 2 | 1880 | 5 | 25 |  | 1960 | M/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2140 | 10.4 | IMD4 |  |
|  | n77 | 3500 | 5 | 25 |  | 3500 | N/A | N/A |  |
|  | 2 | 1885 | 5 | 25 |  | 1965 | M/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2175 | 4.0 | IMD5 |  |
|  | n77 | 3915 | 5 | 25 |  | 3915 | N/A | N/A |  |
|  | 2 | N/A | 5 | N/A |  | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n77 | 3720 | 5 | 25 |  | 3720 | N/A | N/A |  |
| DC_2A-66A_n77A11 DC_2A-66A_n77C11 DC_2A-66A_n77(2A)11 DC_2A-2A-66A_n77A11 DC_2A-2A-66A_n77C11 DC_2A-66A-66A_n77A11 DC_2A-66A-66A_n77C11 DC_2A-2A-66A-66A_n77A11 DC_2A-2A-66A-66A_n77C11 | 2 | N/A | 5 | N/A |  | 1940 | 9.1 | IMD4 |  |
|  | 66 | 1775 | 5 | 25 |  | 2195 | N/A | N/A |  |
|  | n77 | 3385 | 5 | 25 |  | 3385 | N/A | N/A |  |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1855 | 5 | 25 |  | 1935 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2115 | 29.2 | IMD2 |  |
|  | n77 | 3970 | 10 | 50 |  | 3970 | N/A | N/A |  |
|  | 2 | 1853 | 5 | 25 |  | 1933 | N/A | N/A |  |
|  | n66 | 1713 | 5 | 25 |  | 2113 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3566 | 29.4 | IMD2 |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78A | 2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
| DC_2A-2A-66A_n78A | 66/n66 | N/A | 5 | N/A |  | 2160 | 10.3 | IMD4 |  |
|  | n78 | 3480 | 10 | 50 |  | 3480 | N/A | N/A |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | N/A | 5 | N/A |  | 1960 | 32.1 | IMD2 |  |
| DC_2A-2A-66A_n78A | 66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | 3700 | 10 | 50 |  | 3700 | N/A | N/A |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | N/A | 5 | N/A |  | 1960 | 9.1 | IMD4 |  |
| DC_2A-2A-66A_n78A | 66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | N/A | 5 | N/A |  | 1960 | 2.1 | IMD5 |  |
| DC_2A-2A-66A_n78A | 66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n78 | 3620 | 10 | 50 |  | 3620 | N/A | N/A |  |
| DC_2A_n66A-n78A DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3620 | 29.4 | IMD2 |  |
|  | 2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3340 | 8.9 | IMD4 |  |
| DC_2A-71A_n7A | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
| DC_2A-2A-71A_n7A | 71 | N/A | 5 | N/A |  | 630 | 28.7 | IMD24 |  |
|  | n7 | 2530 | 10 | 50 |  | 2650 | N/A | N/A |  |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | N/A | 5 | N/A |  | 1942 | 26 | IMD2 |  |
|  | 71 | 668 | 5 | 25 |  | 622 | N/A | N/A |  |
|  | n38 | 2610 | 10 | 50 |  | 2610 | N/A | N/A |  |
| DC_2A-71A_n41A DC_2A-2A-71A_n41A | 2 | N/A | 5 | N/A |  | 1942 | 26 | IMD2 |  |
|  | 71 | 668 | 5 | 25 |  | 622 | N/A | N/A |  |
|  | n41 | 2610 | 10 | 50 |  | 2610 | N/A | N/A |  |
|  | 2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A |  | 630 | 28.7 | IMD24 |  |
|  | n41 | 2530 | 10 | 50 |  | 2530 | N/A | N/A |  |
| DC_2A-71A_n77A DC_2A-2A-71A_n77A DC_2A-71A_n77(2A) | 2 | N/A | 5 | N/A |  | 1954 | 16.5 | IMD39 |  |
|  | 71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n77 | 3340 | 10 | 50 |  | 3340 | N/A | N/A |  |
| DC_2A_n71A-n77A | 2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
| DC_2A-2A_n71A-n77A | n71 | 695.5 | 5 | 25 |  | 649.5 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3305 | 8 | IMD3 |  |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 | N/A | 5 | N/A |  | 1954 | 16.5 | IMD3 |  |
|  | 71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n78 | 3340 | 10 | 50 |  | 3340 | N/A | N/A |  |
| DC_2A_n71A-n78A DC_2A-2A_n71A-n78A | 2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
|  | n71 | 695.5 | 5 | 25 |  | 649.5 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3305 | 8 | IMD3 |  |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2139 | 11.0 | IMD4 |  |
| DC_3A_n1A-n40A | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 3 | 1735 | 5 | 25 |  | 1830 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A |  | 2380 | 8.0 | IMD5 |  |
| DC_3A_n1A-n41A | 3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
|  | n1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A |  | 2507.5 | 5.0 | IMD5 |  |
| DC_3A_n1A-n75A | n75 | N/A | 5 | N/A |  | 1480 | 15.2 | IMD34,19 |  |
| DC_3C_n1A-n75A | n1 | 1960 | 5 | 25 |  | 2150 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
| DC_3A_n1A-n77A | 3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3700 | 28.4 | IMD2 |  |
|  | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2140 | 31.0 | IMD2 |  |
|  | n77 | 3915 | 10 | 50 |  | 3915 | N/A | N/A |  |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | 3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3700 | 28.4 | IMD2 |  |
|  | 3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2130 | 3.5 | IMD5 |  |
|  | n78 | 3720 | 10 | 50 |  | 3720 | N/A | N/A |  |
| DC_(n)3AA-n8A | 8 | 897.5 | 5 | 25 |  | 942.5 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1837.5 | 4.5 | IMD5 |  |
|  | n3 | 1747.5 | 5 | 25 |  | 1842.5 | 6.4 | IMD5 |  |
| DC_3A_n3A-n41A | 3 | 1725 | 5 | 25 |  | 1820 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1865 | 8.2 | IMD4 |  |
|  | n41 | 2657.5 | 5 | 25 |  | 2657.5 | N/A | N/A |  |
| DC_(n)3AA-n78A | 3 | 1740 | 5 | 25 |  | 1835 | 31.9 | IMD24 |  |
| DC_(n)3AA-n78(2A) | n3 | N/A | 5 | N/A |  | 1840 | [28.9] | IMD24 |  |
|  | n78 | 3575 | 5 | 25 |  | 3575 | N/A | N/A |  |
| DC_3A-5A_n77A DC_3A-5A_n77(2A) DC_3A-5A_n77(3A) | 3 | N/A | 5 | N/A |  | 1820 | 17.3 | IMD3 |  |
|  | 5 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n77 | 3510 | 10 | 50 |  | 3510 | N/A | N/A |  |
| DC_3A-5A_n78A DC_3A-5A_n78(A-C) | 3 | N/A | N/A | N/A |  | N/A | N/A | IMD3 |  |
|  | 5 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_3A-5A_n79A | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 885 | 18.5 | IMD3 |  |
|  | n79 | 4435 | 40 | 216 |  | 4435 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1877.5 | 0.2 | IMD4 |  |
|  | 5 | 842.5 | 5 | 25 |  | 887.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 |  | 4420 | N/A | N/A |  |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 |  | 1875 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2625 | 30.0 | IMD21 |  |
|  | n5 | 845 | 5 | 25 |  | 890 | N/A | N/A |  |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2647.5 | 6.9 | IMD4 |  |
|  | n7 | 2535 | 10 | 50 |  | 2655 | 10.2 | IMD4 |  |
| DC_3A-7A_n8A | 3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
|  | n8 | 890 | 5 | 25 |  | 935 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2670 | 29.0 | IMD2 IMD33 |  |
| DC_3A-7A_n26A | 3 | 1780 | 10 | 50 |  | 1875 | N/A | N/A |  |
| DC_3A-7C_n26A | 7 | N/A | 10 | N/A |  | 2625 | 30.0 | IMD2 |  |
| DC_3C-7A_n26A DC_3C-7C_n26A | n26 | 845 | 5 | 25 |  | 890 | N/A | N/A |  |
|  | 3 | 1760 | 5 | 25 |  | 1855 | N/A | N/A |  |
|  | 7 | 2555 | 10 | N/A |  | 2675 | 16.9 | IMD319 |  |
|  | n26 | 845 | 5 | 25 |  | 890 | N/A | N/A |  |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
| DC_3A-7A-7A_n28A | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2682 | 16.9 | IMD3 |  |
|  | 7 | 2543 | 10 | 50 |  | 2663 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1832.5 | 26.0 | IMD2 |  |
| DC_3A-18A_n3A | 3 | N/A | 5 | N/A |  | 1814 | 4 | IMD4 ¦2*fn3-2*fB18¦ |  |
|  | 18 | 823 | 5 | 25 |  | 868 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
| DC_3-18_n41 | 18 | N/A | 5 | N/A |  | 865 | 28.9 | IMD2 |  |
|  | 3 | 1765 | 5 | 25 |  | 1860 | N/A | N/A |  |
|  | n41 | 2630 | 10 | 50 |  | 2630 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A |  | 865 | 19.0 | IMD3 |  |
|  | 3 | 1725 | 5 | 25 |  | 1820 | N/A | N/A |  |
|  | n41 | 2585 | 5 | 25 |  | 2585 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1850 | 28.8 | IMD2 |  |
|  | n41 | 2670 | 10 | 50 |  | 2670 | N/A | N/A |  |
|  | 18 | 820 | 5 | 25 |  | 865 | MSD | N/A |  |
| DC_3A-18A_n77A DC_3A-18A_n77(2A) DC_3A-18A_n78A DC_3A-18A_n78(2A) | 3 | N/A | N/A | N/A |  | N/A | N/A | IMD3 |  |
|  | 18 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | n77, n78 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_3A-19A_n77A DC_3A-19A_n78A | 3 | N/A | 5 | N/A |  | 1850 | 17.3 | IMD3 |  |
|  | 19 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | n77, n78 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
| DC_3A_n7A-n28A | 3 | 1747 | 5 | 25 |  | 1842 | N/A | N/A |  |
| DC_3C_n7A-n28A | n7 | 2543 | 5 | 25 |  | 2663 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 796.0 | 20.0 | IMD2 |  |
|  | 3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A |  | 2682 | 17.0 | IMD3 |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
| DC_3A-7A_n40A | 3 | N/A | 5 | N/A |  | 1866.6 | 3.4 | IMD5 |  |
| DC_3A-7A-7A_n40A | 7 | 2530 | 5 | 25 |  | 2650 | N/A | N/A |  |
|  | n40 | 2310 | 5 | 25 |  | 2310 | N/A | N/A |  |
| DC_3A-7A_n77A | 3 | N/A | 5 | N/A |  | 1820 | 17.6 | IMD3 |  |
| DC_3A-7A_n77(2A) DC_3A-7A_n77(3A) | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
| DC_3A-7A-7A_n77(2A) | n77 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
| DC_3A-7A-7A_n77(3A) | 3 | N/A | 5 | N/A |  | 1820 | 8.6 | IMD4 |  |
|  | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n77 | 3475 | 10 | 50 |  | 3475 | N/A | N/A |  |
|  | 3 | 1715 | 5 | 25 |  | 1810 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2670 | 5.2 | IMD5 |  |
|  | n77 | 4190 | 10 | 50 |  | 4190 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2640 | 3.4 | IMD5 |  |
|  | n77 | 3900 | 10 | 50 |  | 3900 | N/A | N/A |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) DC_3A-7A_n78C DC_3A-7A_n78(A-C) DC_3A-7A-7A_n78C | 3 | N/A | 5 | N/A |  | 1820 | 17.6 | IMD3 |  |
| DC_3A-7A-7A_n78(A-C) | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1820 | 8.6 | IMD4 |  |
|  | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n78 | 3475 | 10 | 50 |  | 3475 | N/A | N/A |  |
| DC_3A-7A_n105A | 3 | N/A | 5 | N/A |  | 1875 | 16.5 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | n105 | 675 | 5 | 25 |  | 624 | N/A | N/A |  |
| DC_3A-8A_n7A | 3 | 1735 | 5 | 25 |  | 1830 | N/A | N/A |  |
|  | n7 | 2530 | 10 | 50 |  | 2650 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 940 | 18.0 | IMD3 |  |
| DC_3A-8A_n40A | 3 | N/A | 5 | N/A |  | 1874 | 4 | IMD5 |  |
|  | 8 | 912 | 5 | 25 |  | 957 | N/A | N/A |  |
|  | n40 | 2305 | 5 | 25 |  | 2305 | N/A | N/A |  |
| DC_3A-8A_n41A | 3 | 1725 | 5 | 25 |  | 1820 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 945 | 26.0 | IMD215 |  |
|  | n41 | 2670 | 10 | 50 |  | 2670 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1807.5 | 25 | IMD2x |  |
|  | 8 | 882.5 | 5 | 25 |  | 927.5 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 |  | 2685 | N/A | N/A |  |
| DC_3A_n8A-n41A | 3 | 1722.5 | 5 | 25 |  | 1817.5 | N/A | N/A |  |
|  | n8 | 887.5 | 5 | 25 |  | 932.5 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A |  | 2610 | 28.0 | IMD216 |  |
|  | 3 | 1725 | 5 | 25 |  | 1820 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A |  | 945 | 26.0 | IMD216 |  |
|  | n41 | 2516 | 10 | 50 |  | 2516 | N/A | N/A |  |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 3 | 1715 | 5 | 25 |  | 1810 | N/A | N/A |  |
|  | n77 | 4190 | 10 | 50 |  | 4190 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 955 | 9.7 | IMD4 |  |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n77 | 3640 | 10 | 50 |  | 3640 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1820 | 16.5 | IMD3 |  |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
| DC_3A-8B_n78A DC_3A-3A-8B_n78A DC_3A-8A_n78(2A) DC_3C-8A_n78(2A) | n78 | 3640 | 10 | 50 |  | 3640 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1820 | 16.5 | IMD319 |  |
| DC_3A_n8A-n78A | 3 | 1740 | 5 | 25 |  | 1835 | N/A | N/A |  |
|  | n8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3540 | 16.3 | IMD3 |  |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 |  | 1850 | N/A | N/A |  |
|  | n79 | 4465 | 40 | 216 |  | 4465 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 955 | 15.3 | IMD3 |  |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n79 | 4580 | 40 | 216 |  | 4580 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1850 | 8.8 | IMD4 |  |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
| DC_3A_n7A-n78(2A) | n7 | 2560 | 5 | 25 |  | 2680 | N/A | N/A |  |
| DC_3C_n7A-n78(2A) | n78 | N/A | 10 | N/A |  | 3390 | 16.1 | IMD3 |  |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n77 | 3675 | 10 | 50 |  | 3675 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1491 | 8.8 | IMD4 |  |
|  | 11 | 1435.4 | 5 | 25 |  | 1483.4 | N/A | N/A |  |
|  | n77 | 3905 | 10 | 50 |  | 3905 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1848 | 3.4 | IMD57 |  |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | 19 | N/A | 5 | N/A |  | 885 | 18.5 | IMD3 |  |
|  | n79 | 4435 | 40 | 216 |  | 4435 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1877.5 | 0.2 | IMD4 |  |
|  | 19 | 842.5 | 5 | 25 |  | 887.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 |  | 4420 | N/A | N/A |  |
| DC_3A-20A_n3A | 3 | N/A | 5 | N/A |  | 1870 | 4 | IMD4 |  |
|  | 20 | 835 | 5 | 25 |  | 794 | N/A | N/A |  |
|  | n3 | 1765 | 5 | 25 |  | 1860 | N/A | N/A |  |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 | 5 | 25 |  | 1832 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A |  | 806 | 10.5 | IMD2 |  |
|  | n7 | 2543 | 10 | 50 |  | 2663 | N/A | N/A |  |
| DC_3A-20A_n8A | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 810 | 27 | IMD2 |  |
| DC_3A-20A_n8A | 3 | N/A | 5 | N/A |  | 1860 | 14.5 | IMD4 |  |
|  | n8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | 20 | 840 | 5 | 25 |  | 799 | N/A | N/A |  |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 |  | 811 | N/A | N/A |  |
|  | n28 | 728 | 5 | 25 |  | 783 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1828 | 9.4 | IMD4 |  |
| DC_3A-20A_n38A | 3 | 1779 | 5 | 25 |  | 1874 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A |  | 811 | 26.0 | IMD21 |  |
|  | n38 | 2590 | 10 | 50 |  | 2590 | N/A | N/A |  |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | N/A | 5 | N/A |  | 1839 | 26.0 | IMD2 |  |
|  | n41 | 2680 | 10 | 50 |  | 2680 | N/A | N/A |  |
|  | 20 | 841 | 10 | 50 |  | 800 | N/A | N/A |  |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1779 | 5 | 25 |  | 1874 | N/A | N/A |  |
|  | n41 | 2590 | 10 | 50 |  | 2590 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A |  | 811 | 26.0 | IMD2 |  |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n41 | 2660 | 10 | 50 |  | 2660 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 800 | 12.5 | IMD3 |  |
| DC_3_n20-n67 | 3 | 1765 | 5 | 25 |  | 1860 | N/A | N/A |  |
|  | n20 | 837 | 5 | 25 |  | 796 | N/A | N/A |  |
|  | n67 | N/A | 5 | N/A |  | 746 | 10.1 | IMD4 |  |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | 3 | N/A | 5 | N/A |  | 1820 | 17.3 | IMD3 |  |
|  | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n78 | 3510 | 10 | 50 |  | 3510 | N/A | N/A |  |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3420 | 16.1 | IMD3 |  |
| DC_3A-20A_n78A DC_3C-20A_n78A DC_3A-20A_n78(2A) | 3 | N/A | 5 | N/A |  | 1820 | 17.3 | IMD3 |  |
|  | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n78 | 3510 | 10 | 50 |  | 3510 | N/A | N/A |  |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 |  | 1862.5 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A |  | 1507.5 | 8.8 | IMD4 |  |
|  | n77, n78 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1862.5 | 30.8 | IMD2 |  |
|  | 21 | 1459.5 | 5 | 25 |  | 1507.5 | N/A | N/A |  |
|  | n77, n78 | 3322 | 10 | 50 |  | 3322 | N/A | N/A |  |
| DC_3A-21A_n77A | 3 | N/A | 5 | N/A |  | 1866.6 | 3.4 | IMD5 |  |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n77 | 3935 | 10 | 50 |  | 3935 | N/A | N/A |  |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A |  | N/A | N/A | IMD3 |  |
|  | n79 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1869.2 | 17.8 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 |  | 4770 | N/A | N/A |  |
| DC_3A-26A_n78A DC_3C-26A_n78A | 3 | N/A | 5 | N/A |  | 1862 | 15.7 | IMD3 |  |
|  | 26 | 839 | 5 | 25 |  | 884 | N/A | N/A |  |
|  | n78 | 3540 | 10 | 50 |  | 3540 | N/A | N/A |  |
| DC_3A-28A_n1A DC_3C-28A_n1A | 3 | N/A | 5 | N/A |  | 1820 | 4 | IMD5 |  |
|  | 28 | 710 | 5 | 25 |  | 765 | N/A | N/A |  |
|  | n1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | N/A | 5 | N/A |  | 1830 | 8.7 | IMD4 |  |
|  | 28 | 705 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n5 | 845 | 5 | 25 |  | 874 | N/A | N/A |  |
|  | 3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 785 | 9.4 | IMD4 |  |
|  | n5 | 845 | 5 | 25 |  | 874 | N/A | N/A |  |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | N/A | 5 | N/A |  | 1832.5 | 26.0 | IMD2 |  |
|  | 28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
|  | n7 | 2543 | 10 | 50 |  | 2663 | N/A | N/A |  |
|  | 3 | 1747 | 5 | 25 |  | 1842 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 796.0 | 20.0 | IMD2 |  |
|  | n7 | 2543 | 5 | 25 |  | 2663 | N/A | N/A |  |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 770 | 15.3 | IMD3 |  |
|  | n77 | 4195 | 10 | 50 |  | 4195 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1850 | 17.0 | IMD3 |  |
|  | 28 | 735 | 5 | 25 |  | 790 | N/A | N/A |  |
|  | n77 | 3320 | 10 | 50 |  | 3320 | N/A | N/A |  |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | B3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
|  | n28 | 708 | 5 | 25 |  | 763 | N/A | N/A |  |
|  | n75 | N/A | - | N/A |  | 1436 | 3.3 | IMD5 |  |
| DC_3A_n28A-n77A | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | 28 | 733 | 5 | 25 |  | 788 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 4173 | 15.9 | IMD3 |  |
|  | 3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 770 | 15.3 | IMD3 |  |
|  | n77 | 4195 | 10 | 50 |  | 4195 | N/A | N/A |  |
| DC_3A-28A_n38A | 3 | N/A | 5 | N/A |  | 1870 | 26.0 | IMD2 |  |
|  | 28 | 710 | 5 | 25 |  | 765 | N/A | N/A |  |
|  | n38 | 2580 | 5 | 25 |  | 2580 | N/A | N/A |  |
|  | 3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 800 | 20.0 | IMD21 |  |
|  | n38 | 2580 | 5 | 25 |  | 2580 | N/A | N/A |  |
| DC_3A-28A_n41A | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n41 | 2510 | 5 | 25 |  | 2510 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 790 | 26.0 | IMD21 |  |
|  | 3 | N/A | 5 | N/A |  | 1832.5 | 26.0 | IMD2 |  |
|  | n41 | 2543 | 10 | 50 |  | 2543 | N/A | N/A |  |
|  | 28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
| DC_3A_n28A-n41A | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 790 | 261 | IMD2 ¦fn41-fB3¦ |  |
|  | n41 | 2510 | 5 | 25 |  | 2510 | N/A | N/A |  |
|  | 3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 |  | 793 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A |  | 2518 | 27.4 | IMD2 ¦fB3+fn28¦ |  |
|  | 3 | 1715 | 5 | 25 |  | 1810 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A |  | 2687 | 15.9 | IMD3 ¦2*fB3-fn28¦ |  |
| DC_3A_n26A-n78A | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
| DC_3C_n26A-n78A | n26 | 839 | 5 | 25 |  | 884 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3408 | 16.1 | IMD3 |  |
|  | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n26 | 839 | 5 | 25 |  | 884 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3512 | 4.5 | IMD5 |  |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A | 3 | N/A | 5 | N/A |  | 1870 | 17.3 | IMD3 |  |
|  | 28 | 740 | 5 | 25 |  | 760 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 25 |  | 3350 | N/A | N/A |  |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 780 | 10.3 | IMD4 |  |
|  | n79 | 4530 | 40 | 216 |  | 4530 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1870 | 5.7 | IMD5 |  |
|  | 28 | 725 | 5 | 25 |  | 780 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 |  | 4770 | N/A | N/A |  |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3764 | 4.5 | IMD5 |  |
| DC_3A_n28A-n79A | 3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 780 | 10.3 | IMD4 |  |
|  | n79 | 4530 | 40 | 216 |  | 4530 | N/A | N/A |  |
|  | 3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n28 | 725 | 5 | 25 |  | 780 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4585 | 9.4 | IMD44 |  |
| DC_3A_n40A-n77A | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
| DC_3A_n40A-n77(2A) | n40 | 2350 | 5 | 25 |  | 2350 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 4070 | 30.3 | IMD2 |  |
|  | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n40 | 2360 | 5 | 25 |  | 2360 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3620 | 4.8 | IMD5 |  |
|  | 3 | 1745 | 5 | 25 |  | 1840 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2355 | 29,2 | IMD2 |  |
|  | n77 | 4100 | 10 | 50 |  | 4100 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2360 | 4.4 | IMD5 |  |
|  | n77 | 3760 | 10 | 50 |  | 3760 | N/A | N/A |  |
| DC_3A_SUL_n77A-n84A | 3 | 1782.5 | 5 | 25 |  | 1877.5 | N/A | N/A |  |
|  | n84 | 1922.5 | 5 | 25 |  |  | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3425 | 13.0 | IMD4 |  |
| DC_3A_n40A-n78A | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
| DC_3A_n40A-n78C | n40 | 2360 | 5 | 25 |  | 2360 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3620 | 4.8 | IMD5 |  |
|  | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2360 | 4.4 | IMD5 |  |
|  | n78 | 3760 | 10 | 50 |  | 3760 | N/A | N/A |  |
| DC_3A_n40A-n79A | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n40 | 2330 | 5 | 25 |  | 2330 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4550 | 4.7 | IMD5 |  |
|  | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2330 | 3.2 | IMD5 |  |
|  | n79 | 4550 | 40 | 216 |  | 4550 | N/A | N/A |  |
| DC_3A_n41A-n79A | 3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n41 | 2670 | 10 | 50 |  | 2670 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4440 | 30.8 | IMD24 |  |
| DC_3A-42A_n1A DC_3A-42C_n1A | 3 | 1782.5 | 5 | 25 |  | 1877.5 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A |  | 3425 | 13.0 | IMD4 |  |
|  | n1 | 1922.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
| DC_3A_n75A-n78A DC_3A_n75A-n78(2A) | 3 | 1782.5 | 5 | 25 |  | 1877.5 | N/A | N/A |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | n75 | N/A | - | N/A |  | 1514.5 | 10.0 | IMD2 |  |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n78 | 3340 | 10 | 50 |  | 3340 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4910 | 16.3 | IMD3 |  |
|  | 3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n79 | 4510 | 40 | 216 |  | 4510 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3710 | 4.2 | IMD5 |  |
| DC_3A_SUL_n78A-n82A | 3 | N/A | 5 | N/A |  | 1870 | 4 | IMD4 |  |
|  | n82 | 840 | 5 | 25 |  |  | N/A | N/A |  |
| DC_3A_SUL_n78A-n84A | 3 | 1782.5 | 5 | 25 |  | 1877.5 | N/A | N/A |  |
|  | n84 | 1922.5 | 5 | 25 |  |  | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3425 | 13.0 | IMD4 |  |
| DC_3A-21A_n79A | 3 | N/A | 5 | N/A |  | 1869.2 | 17.8 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 |  | 4770 | N/A | N/A |  |
| DC_3A-32A_n1A | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
| DC_3C-32A_n1A | 32 | N/A | 5 | N/A |  | 1480 | 15.2 | IMD34, 19 |  |
|  | n1 | 1960 | 5 | 25 |  | 2150 | N/A | N/A |  |
| DC_3A-32A_n7A | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1470 | 10.5 | IMD4 |  |
|  | n7 | 2510 | 10 | 50 |  | 2630 | N/A | N/A |  |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C DC_3A-32A_n78(2A) | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1470 | 4.9 | IMD4 |  |
|  | n78 | 3720 | 10 | 50 |  | 3720 | N/A | N/A |  |
|  | 3 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1475 | 0 | IMD5 |  |
|  | n78 | 3400 | 10 | 50 |  | 3400 | N/A | N/A |  |
| DC_3A-38A_n28A DC_3C-38A_n28A | 38 | 2575 | 5 | 25 |  | 2575 | N/A | N/A |  |
|  | n28 | 725 | 5 | 25 |  | 780 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1850 | 26 | IMD2 |  |
| DC_3A-38A_n78A DC_3C-38A_n78A | 3 | N/A | 5 | N/A |  | 1830 | 16.4 | IMD35 |  |
|  | 38 | 2615 | 5 | 25 |  | 2615 | N/A | N/A |  |
|  | n78 | 3400 | 10 | 50 |  | 3400 | N/A | N/A |  |
| DC_3A-40A_n1A DC_3A-40C_n1A | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 3 | 1735 | 5 | 25 |  | 1830 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A |  | 2380 | 8.0 | IMD5 |  |
| DC_3A-40A_n77A DC_3A-40C_n77A | 3 | 1720 | 5 |  | 25 | 1815 | N/A | N/A |
|  | 40 | 2310 | 5 |  | 25 | 2310 | 29.4 | IMD2 |
|  | n77 | 4030 | 10 |  | 50 | 4030 | N/A | N/A |
|  | 3 | 1720 | 5 |  | 25 | 1815 | N/A | N/A |
|  | 40 | 2350 | 5 |  | 25 | 2350 | 5.3 | IMD5 |
|  | n77 | 3755 | 10 |  | 50 | 3755 | N/A | N/A |
|  | 3 | 1725 | 5 |  | 25 | 1820 | 29.9 | IMD29 |
|  | 40 | 2310 | 5 |  | 25 | 2310 | N/A | N/A |
|  | n77 | 4130 | 10 |  | 50 | 4130 | N/A | N/A |
| DC_3A-40A_n78A DC_3A-40C_n78A | 3 | N/A | 5 | N/A |  | 1870 | 9.1 | IMD4 |  |
|  | 40 | 2390 | 5 | 25 |  | 2390 | N/A | N/A |  |
|  | n78 | 3325 | 10 | 50 |  | 3325 | N/A | N/A |  |
|  | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A |  | 2360 | 4.4 | IMD5 |  |
|  | n78 | 3760 | 10 | 50 |  | 3760 | N/A | N/A |  |
| DC_3A-41A_n1A DC_3A-41C_n1A DC_3A-3A-41A_n1A DC_3A-3A-41C_n1A | n1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | 3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2507.5 | 5.0 | IMD5 |  |
| DC_3A-41A_n3A DC_3A-41C_n3A | 3 | N/A | 5 | N/A |  | 1865 | 8.2 | IMD4 ¦2*fB41-2*fn3¦ |  |
|  | 41 | 2657.5 | 5 | 25 |  | 2657.5 | N/A | N/A |  |
|  | n3 | 1725 | 5 | 25 |  | 1820 | N/A | N/A |  |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 |  | 2543 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1832.5 | 26 | IMD2 |  |
|  | 3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 |  | 793 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2518 | 27.4 | IMD2 |  |
|  | 3 | 1715 | 5 | 25 |  | 1810 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2687 | 15.9 | IMD3 |  |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) DC_3A_n41A-n77A DC_3A_n41A-n77(2A) | 3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n77 | 3900 | 10 | 50 |  | 3900 | N/A | N/A |  |
|  | 41/n41 | N/A | 5 | N/A |  | 2640 | 5.3 | IMD5 |  |
|  | 41/n41 | 2620 | 5 | 25 |  | 2620 | N/A | N/A |  |
|  | n77 | 3400 | 10 | 50 |  | 3400 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1840 | 16.4 | IMD3 |  |
| DC_3A-41A_n78A DC_3A-41C_n78A DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | 41 | 2620 | 5 | 25 |  | 2620 | N/A | N/A |  |
|  | n78 | 3400 | 10 | 50 |  | 3400 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1840 | 16.4 | IMD3 |  |
| DC_3A_n41A-n78A DC_3A_n41A-n78(2A) | 3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n41 | 2560 | 10 | 50 |  | 2560 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3390 | 16.4 | IMD3 |  |
| DC_3A-41A_n79A | 3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n79 | 4440 | 40 | 216 |  | 4440 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2670 | 30.2 | IMD2 |  |
|  | 41 | 2570 | 5 | 25 |  | 2570 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 |  | 4420 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1850 | 29.4 | IMD2 |  |
| DC_3_n78-n105 | 3 | 1715 | 5 | 25 |  | 1810 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3725 | 13 | IMD44 |  |
|  | n105 | 670 | 5 | 25 |  | 619 | N/A | N/A |  |
| DC_4A-7A_n28A | 4 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2685 | 18.0 | IMD3 |  |
|  | n28 | 745 | 5 | 25 |  | 800 | N/A | N/A |  |
| DC_4A-7A_n78A | 4 | N/A | 5 | N/A |  | 2150 | 8.7 | IMD4 |  |
|  | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | n78 | 3625 | 10 | 50 |  | 3625 | N/A | N/A |  |
| DC_5_n1-n78 | 5 | 829 | 5 | 25 |  | 874 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2122 | 18.1 | IMD3 |  |
|  | n78 | 3780 | 10 | 50 |  | 3780 | N/A | N/A |  |
|  | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3610 | 15.7 | IMD3 |  |
| DC_5A_n2A-n41A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n2 | 1855 | 10 | 50 |  | 1935 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 |  | 2685 | 30.0 | IMD2 |  |
| DC_5A_n2A-n66A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 |  | 2140 | 7.2 | IMD4 |  |
| DC_5A_n2A-n77A11 | n2 | N/A | 5 | N/A |  | 1987 | 16.5 | IMD3 |  |
|  | 5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
|  | n77 | 3680 | 5 | 25 |  | 3680 | N/A | N/A |  |
| DC_5A_n5A-n77A11 | 5 | 834 | 5 | 25 |  | 879 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A |  | 889 | 8.3 | IMD4 |  |
|  | n77 | 3391 | 10 | 50 |  | 3391 | N/A | N/A |  |
|  | 5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A |  | 882 | 5.5 | IMD5 |  |
|  | n77 | 4188 | 10 | 50 |  | 4188 | N/A | N/A |  |
| DC_5A-7A_n7A | 5 | N/A | 5 | N/A |  | 879 | 12 | IMD34 |  |
|  | 7 | 2527 | 10 | 50 |  | 2647 | N/A | N/A |  |
|  | n7 | 2547 | 10 | 50 |  | 2667 | N/A | N/A |  |
| DC_5A_n2A-n78A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3540 | 16.0 | IMD3 |  |
|  | 5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1987 | 16.5 | IMD3 |  |
|  | n78 | 3680 | 10 | 25 |  | 3680 | N/A | N/A |  |
| DC_5A_n3A-n78A | 5 | 839 | 5 | 25 |  | 884 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3408 | 16.1 | IMD3 |  |
|  | 5 | 839 | 5 | 25 |  | 884 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3512 | 4.5 | IMD5 |  |
|  | 5 | 839 | 5 | 25 |  | 884 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1862 | 15.7 | IMD3 |  |
|  | n78 | 3540 | 10 | 50 |  | 3540 | N/A | N/A |  |
| DC_5A-7A_n25A | 5 | 1855 | 5 |  | 25 | 1935 | N/A | N/A |
|  | 7 | 2565 | 5 |  | 25 | 2685 | 30.0 | IMD2 |
|  | n25 | 830 | 5 |  | 25 | 875 | N/A | N/A |
| DC_5A-7A_n66A DC_5A-7C_n66A DC_5A-7A-7A_n66A | 5 | N/A | 5 | N/A |  | 880 | 17.8 | IMD3 |  |
|  | 7 | 2560 | 5 | 25 |  | 2680 | N/A | N/A |  |
|  | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2624 | 29.0 | IMD21 |  |
|  | 66 | 1777.5 | 5 | 25 |  | 2177.5 | N/A | N/A |  |
| DC_5A-7A_n71A | 5 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2660 | 6.5 | IMD5 |  |
|  | n71 | 680 | 5 | 25 |  | 634 | N/A | N/A |  |
|  | 5 | 844 | 5 | 25 |  | 889 | N/A | N/A |  |
| DC_5A-7A_n77A | 7 | N/A | 5 | N/A |  | 2645 | 30.1 | IMD2 |  |
| DC_5A-7A_n77(2A) DC_5A-7A_n77(3A) | n77 | 3489 | 10 | 50 |  | 3489 | N/A | N/A |  |
| DC_5A-7A-7A_n77A | 5 | N/A | 5 | N/A |  | 879 | 30.2 | IMD21 |  |
| DC_5A-7A-7A_n77(2A) DC_5A-7A-7A_n77(3A) | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | n77 | 3429 | 10 | 50 |  | 3429 | N/A | N/A |  |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7A_n78(A-C) DC_5A-7A-7A_n78C | 5 | 844 | 5 | 25 |  | 889 | N/A | N/A |  |
| DC_5A-7A-7A_n78(A-C) | 7 | N/A | 5 | N/A |  | 2645 | 30.1 | IMD2 |  |
|  | n78 | 3489 | 10 | 50 |  | 3489 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 879 | 30.2 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | n78 | 3429 | 10 | 50 |  | 3429 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 875 | 3.3 | IMD5 |  |
|  | 7 | 2525 | 5 | 25 |  | 2645 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
| DC_5A_n7A-n78A, DC_5A_n7(2A)-n78A DC_5A_n7A-n78(2A) DC_5A_n7(2A)-n78(2A) | 5 | 844 | 5 | 25 |  | 889 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A |  | 2645 | 30.1 | IMD2 |  |
|  | n78 | 3489 | 10 | 50 |  | 3489 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | n7 | 2540 | 5 | 25 |  | 2660 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3375 | 29.7 | IMD2 |  |
| DC_5A-13A_n66A | 5 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
|  | 13 | N/A | 5 | N/A |  | 750 | 9.4 | IMD4 |  |
|  | n66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
| DC_5A-13A_n77A11 | 5 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
| DC_5A-13A_n77C11 | n77 | 4110 | 10 | 50 |  | 4110 | N/A | N/A |  |
|  | 13 | N/A | 5 | N/A |  | 750 | 4.4 | IMD5 |  |
|  | 13 | 782 | 5 | 20 |  | 751 | N/A | N/A |  |
|  | n77 | 4013 | 10 | 50 |  | 4013 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 885 | 4.5 | IMD5 |  |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | 5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 | 11.6 | IMD4 |  |
|  | n77 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | n28 | 710 | 5 | 25 |  | 765 | 4.4 | IMD5 |  |
|  | n77 | 4105 | 10 | 50 |  | 4105 | N/A | N/A |  |
| DC_5A_n28A-n78A DC_5A_n28A-n78C | 5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
|  | n28 | 715.5 | 5 | 25 |  | 765.5 | 11.6 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
| DC_5A-30A_n2A | 5 | N/A | 5 | N/A |  | 880 | 8 | IMD4 |  |
|  | 30 | 2310 | 5 | 25 |  | 2355 | N/A | N/A |  |
|  | n2 | 1870 | 5 | 25 |  | 1950 | N/A | N/A |  |
| DC_5A-30A_n77A DC_5A-30A_n77(2A) | 5 | 835 | 5 | 25 |  | 880 | 15.2 | IMD34 |  |
|  | 30 | N/A | 5 | N/A |  | 2355 | N/A | N/A |  |
|  | n77 | 3740 | 10 | 50 |  | 3740 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2355 | 13.2 | IMD311 |  |
|  | n77 | 4025 | 10 | 50 |  | 4025 | N/A | N/A |  |
| DC_5A_n38A-n66A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A |  | 2590 | 28.9 | IMD2 |  |
| DC_5A-40A_n77A | 5 | 835 | 5 |  | 25 | 880 | N/A | N/A |
| DC_5A-40C_n77A | 40 | 2355 | 5 |  | 25 | 2355 | 13.2 | IMD3 |
| DC_5A-40A_n77C | n77 | 4025 | 10 |  | 50 | 4025 | N/A | N/A |
| DC_5A-40C_n77C | 5 | 835 | 5 |  | 25 | 880 | 15.2 | IMD34 |
|  | 40 | 2310 | 5 |  | 25 | 2310 | N/A | N/A |
|  | n77 | 3740 | 10 |  | 50 | 3740 | N/A | N/A |
| DC_5A_n41A-n66A | 5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
|  | n41 | 2624 | 10 | 50 |  | 2624 | 29.0 | IMD2 |  |
|  | n66 | 1777.5 | 5 | 25 |  | 2177.5 | N/A | N/A |  |
|  | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n41 | 2600 | 10 | 50 |  | 2600 | 18 | IMD3 |  |
|  | n66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
| DC_5A_n40A-n77A | 5 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
| DC_5A_n40A-n77(2A) | n40 | 2310 | 5 | 25 |  | 2310 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3780 | 16.1 | IMD3 |  |
|  | 5 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2355 | 13.2 | IMD3 |  |
|  | n77 | 4025 | 10 | 50 |  | 4025 | N/A | N/A |  |
| DC_5A-40A_n78A DC_5A-40C_n78A | 5 | 835 | 5 | 25 |  | 880 | 15.2 | IMD3 |  |
| DC_5A-40A_n78C | 40 | 2310 | 5 | 25 |  | 2310 | N/A | N/A |  |
| DC_5A-40C_n78C | n78 | 3740 | 10 | 50 |  | 3740 | N/A | N/A |  |
| DC_5A_n40A-n78A | 5 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
| DC_5A_n40A-n78C | n40 | 2310 | 5 | 25 |  | 2310 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3780 | 16.1 | IMD3 |  |
| DC_5A_41A_n78A | 5 | N/A | 5 | N/A |  | 885 | 30.2 | IMD2 |  |
|  | 41 | 2615 | 5 | 25 |  | 2615 | N/A | N/A |  |
|  | n78 | 3500 | 10 | 50 |  | 3500 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 881.5 | 3.1 | IMD5 |  |
|  | 41 | 2620.5 | 5 | 25 |  | 2620.5 | N/A | N/A |  |
|  | n78 | 3490 | 10 | 50 |  | 3490 | N/A | N/A |  |
| DC_5A-41A_n79A | 5 | N/A | 5 | N/A |  | 880 | 23.9 | IMD3 |  |
|  | 41 | 2665 | 5 | 25 |  | 2665 | N/A | N/A |  |
|  | n79 | 4450 | 40 | 216 |  | 4450 | N/A | N/A |  |
|  | 5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2517.5 | 1.8 | IMD4 |  |
|  | n79 | 4980 | 40 | 216 |  | 4980 | N/A | N/A |  |
| DC_5A-46A_n66A | 5 | 847 | 5 | 25 |  | 892 | N/A | N/A |  |
|  | 46 | N/A | 10 | N/A |  | 5163 | 9.04 | IMD4 ¦2*fB5+2*fn66¦ |  |
|  | n66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
| DC_5A-48A_n12A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | 48 | N/A | 5 | N/A |  | 3650 | 4.4 | IMD5 |  |
|  | n12 | 705 | 5 | 25 |  | 735 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 875 | 5.9 | IMD5 |  |
|  | 48 | 3695 | 5 | 25 |  | 3695 | N/A | N/A |  |
|  | n12 | 705 | 5 | 25 |  | 735 | N/A | N/A |  |
| DC_5A-48A_n71A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | 48 | N/A | 5 | N/A |  | 3590 | 4.4 | IMD5 |  |
|  | n71 | 690 | 5 | 25 |  | 644 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 880 | 5.9 | IMD5 |  |
|  | 48 | 3600 | 5 | 25 |  | 3600 | N/A | N/A |  |
|  | n71 | 680 | 5 | 25 |  | 634 | N/A | N/A |  |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 | 5 | 25 |  | 879 | N/A | N/A |  |
| DC_5A-66B_n2A DC_5A-66A_n2(2A) | 66 | N/A | 5 | N/A |  | 2132 | 7.2 | IMD4 |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | 5 | N/A | 5 | N/A |  | 880 | 18.0 | IMD3 |  |
|  | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n7 | 2560 | 5 | 25 |  | 2680 | N/A | N/A |  |
| DC_5A-66A_n25A | 5 | 834 | 5 | 25 |  | 879 | N/A | N/A |  |
|  | 66 | 1732 | 5 | 25 |  | 2132 | 7.2 | IMD4 |  |
|  | n25 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
| DC_5A-66A_n30A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2125 | 4 | IMD5 |  |
|  | n30 | 2307.5 | 5 | 50 |  | 2352.5 | N/A | N/A |  |
| DC_5A-66A_n41A | 5 | 830 | 5 | 25 |  | 875 | 28.9 | IMD2 |  |
|  | 66 | 1765 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | n41 | 2640 | 10 | 50 |  | 2640 | N/A | N/A |  |
|  | 5 | 835 | 5 | 25 |  | 880 | 18.0 | IMD3 |  |
|  | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n41 | 2560 | 10 | 50 |  | 2560 | N/A | N/A |  |
| DC_5A-66A_n71A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2161 | 13 | IMD3 |  |
|  | n71 | 665.5 | 5 | 25 |  | 619.5 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 891.5 | 4.2 | IMD5 |  |
|  | 66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
|  | n71 | 665.5 | 5 | 25 |  | 619.5 | N/A | N/A |  |
| DC_5A-66A_n77A | 5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
| DC_5A-66A_n77C DC_5A-66A_n77(2A) DC_5A-66A-66A_n77A DC_5A-66A-66A_n77C | 66 | N/A | 5 | N/A |  | 2142 | 13.2 | IMD3 |  |
| DC_5A-66A-66A_n77(2A) | n77 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
| DC_5A-66A-66A_n78A | 66 | N/A | 5 | N/A |  | 2142 | 13.2 | IMD3 |  |
|  | n78 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
| DC_5A_n66A-n78A | 5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
|  | n66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3420 | 16.6 | IMD3 |  |
|  | 5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2142 | 13.2 | IMD3 |  |
|  | n78 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
| DC_5A_n66A-n77A | 5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2142 | 13.2 | IMD3 |  |
|  | n77 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
|  | 5 | 845 | 5 | 25 |  | 890 | N/A | N/A |  |
|  | n66 | 1785 | 5 | 25 |  | 2185 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3475 | 16.1 | IMD3 |  |
| DC_7A_n1A-n28A | 7 | 2535 | 5 | 25 |  | 2655 | N/A | N/A |  |
| DC_7C-n1A-n28A | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 780 | 4.3 | IMD5 |  |
| DC_7A_n1A-n40A | 7 | 2540 | 5 | 25 |  | 2660 | N/A | N/A |  |
|  | n40 | 2335 | 5 | 25 |  | 2335 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2130 | 15.2 | IMD3 |  |
| DC_7A_n1A-n75A | n1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | 7 | 2502.5 | 5 | 25 |  | 2622.5 | N/A | N/A |  |
|  | 75 | N/A | 5 | N/A |  | 1454.5 | 15.2 | IMD3 |  |
| DC_7A_n1A-n78A | 7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
| DC_7C_n1A-n78A | n1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
| DC_7A_n1A-n78(2A) | n78 | N/A | 10 | N/A |  | 3390 | 10.1 | IMD4 |  |
| DC_7C_n1A-n78(2A) | 7 | 2530 | 5 | 25 |  | 2650 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2160 | 9.0 | IMD4 |  |
|  | n78 | 3610 | 10 | 50 |  | 3610 | N/A | N/A |  |
| DC_7A_n2A-n71A | 7 | 2530 | 5 | 25 |  | 2530 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A |  | 630 | 28.7 | IMD2 |  |
| DC_7A_n2A-n77A | 7 | 2550 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n2 | 1870 | 5 | 25 |  | 1950 | 8.6 | IMD4 |  |
|  | n77 | 3525 | 10 | 50 |  | 3525 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 |  | 2645 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n77 | 3775 | 10 | 50 |  | 3775 | 4.2 | IMD5 |  |
| DC_7A_n2A-n78A | 7 | 2550 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1950 | 8.6 | IMD4 |  |
|  | n78 | 3525 | 10 | 50 |  | 3525 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 |  | 2645 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3775 | 4.2 | IMD5 |  |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 |  | 2680 | N/A | N/A |  |
| DC_7C_n3A-n78A | n3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
| DC_7A_n3A-n78(2A) | n78 | N/A | 10 | N/A |  | 3390 | 16.1 | IMD3 |  |
| DC_7C_n3A-n78(2A) | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1820 | 15.6 | IMD3 |  |
|  | n78 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
| DC_7A_n8A-n40A | 7 | 2530 | 5 | 25 |  | 2650 | N/A | N/A |  |
|  | n8 | 905 | 5 | 25 |  | 950 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2345 | 3.0 | IMD5 |  |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 |  | 1830 | N/A | N/A |  |
|  | 7 | 2530 | 10 | 50 |  | 2650 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 940 | 18.0 | IMD3 |  |
| DC_7A-8A_n3A | n3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
|  | 8 | 890 | 5 | 25 |  | 935 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2670 | 29.0 | IMD2+IMD33 |  |
| DC_7A-8A_n20A | 7 | N/A | 5 | N/A |  | 2640 | 21.1 | IMD34,15 |  |
|  | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n20 | 840 | 5 | 25 |  | 799 | N/A | N/A |  |
|  | 7 | 2503 | 5 | 25 |  | 2623 | N/A | N/A |  |
|  | n20 | 859 | 5 | 25 |  | 818 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 933 | 4.4 | IMD5 |  |
| DC_7A-8A_n77A | 7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 940 | 3.1 | IMD5 |  |
|  | n77 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
| DC_7A-8A_n77A | 7 | N/A | 5 | N/A |  | 2650 | 28 | IMD2 |  |
|  | 8 | 895 | 5 | 25 |  | 940 | N/A | N/A |  |
|  | n77 | 3545 | 10 | 50 |  | 3545 | N/A | N/A |  |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 |  | 2650 | N/A | N/A |  |
| DC_7A-8B_n78A | 8 | N/A | 5 | N/A |  | 940 | 30.5 | IMD2 |  |
| DC_7A-7A-8B_n78A | n78 | 3470 | 10 | 50 |  | 3470 | N/A | N/A |  |
| DC_7A-8A_n78A | 7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
| DC_7A-8B_n78A | 8 | N/A | 5 | N/A |  | 940 | 3.1 | IMD5 |  |
| DC_7A-7A-8B_n78A | n78 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
| DC_7A-8A_n78A | 7 | N/A | 5 | N/A |  | 2650 | 28 | IMD2 |  |
| DC_7A-8B_n78A | 8 | 895 | 5 | 25 |  | 940 | N/A | N/A |  |
| DC_7A-7A-8B_n78A | n78 | 3545 | 10 | 50 |  | 3545 | N/A | N/A |  |
| DC_7A_n8A-n78A | 7 | 2555 | 5 | 25 |  | 2675 | N/A | N/A |  |
|  | n8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3455 | 28.5 | IMD2 |  |
|  | 7 | 2555 | 5 | 25 |  | 2675 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A |  | 945 | 29.7 | IMD2 |  |
|  | n78 | 3500 | 10 | 50 |  | 3500 | N/A | N/A |  |
| DC_7A-12A_n2A | 7 | 2502.5 | 5 | 25 |  | 2622.5 | N/A | N/A |  |
| DC_7A-12A_n2(2A) | 12 | N/A | 5 | N/A |  | 731.5 | 5.3 | IMD5 |  |
|  | n2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2621 | 30.8 | IMD2 |  |
|  | 12 | 713.5 | 5 | 25 |  | 743.5 | N/A | N/A |  |
|  | n2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
| DC_7A-12A_n25A | 7 | 2502.5 | 5 | 25 |  | 2622.5 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A |  | 731.5 | 5.3 | IMD5 |  |
|  | n25 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2622.5 | 30.8 | IMD2 |  |
|  | 12 | 713.5 | 5 | 25 |  | 743.5 | N/A | N/A |  |
|  | n25 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
| DC_7A-12A_n66A | 7 | 2515 | 5 | 25 |  | 2635 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A |  | 742 | 31 | IMD2 |  |
|  | n66 | 1773 | 5 | 25 |  | 2173 | N/A | N/A |  |
| DC_7A_n12A-n77A | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n12 | N/A | 5 | N/A |  | 740 | 30.8 | IMD2 |  |
|  | n77 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | 7 | 2505 | 5 | 25 |  | 2625 | N/A | N/A |  |
|  | n12 | 702 | 5 | 25 |  | 732 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3909 | 16.0 | IMD3 |  |
| DC_7A-12A_n77A DC_7A-12A_n77(2A) | 7 | N/A | 5 | N/A |  | 2662 | 29.6 | IMD21 |  |
|  | 12 | 708 | 5 | 25 |  | 738 | N/A | N/A |  |
|  | n77 | 3370 | 10 | 50 |  | 3370 | N/A | N/A |  |
|  | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A |  | 740 | 30.8 | IMD2 |  |
|  | n77 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
| DC_7A-12A_n78A DC_7A-12A_n78(2A) | 7 | N/A | 5 | N/A |  | 2662 | 29.6 | IMD2 |  |
|  | 12 | 708 | 5 | 25 |  | 738 | N/A | N/A |  |
|  | n78 | 3370 | 10 | 50 |  | 3370 | N/A | N/A |  |
|  | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A |  | 740 | 30.8 | IMD24 |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
| DC_7A_n12A-n78A | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n12 | 710 | 5 | 25 |  | 740 | 30.8 | IMD24 |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
|  | 7 | 2505 | 5 | 25 |  | 2625 | N/A | N/A |  |
|  | n12 | 673 | 5 | 25 |  | 732 | N/A | N/A |  |
|  | n78 | 3664 | 10 | 50 |  | 3664 | 10.3 | IMD4 |  |
| DC_7A-13A_n66A | 7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | 13 | N/A | 5 | N/A |  | 750 | 31 | IMD2 |  |
|  | n66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
| DC_7A-13A_n66A | 7 | N/A | 5 | N/A |  | 2660 | 18 | IMD3 |  |
|  | 13 | 780 | 5 | 25 |  | 749 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
| DC_7A-13A_n25A DC_7A-7A-13A_n25A DC_7C-13A_n25A | 7 | N/A | 10 | N/A |  | 2662 | 27.6 | IMD2 |  |
|  | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n25 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 | 2510 | 10 | 50 |  | 2630 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A |  | 800 | 4.5 | IMD5 |  |
|  | n1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 |  | 2663 | N/A | N/A |  |
|  | 20 | N/A | 10 | N/A |  | 806 | 10.5 | IMD2 |  |
|  | n3 | 1737 | 5 | 25 |  | 1832 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2630 | 26.0 | IMD21 |  |
|  | 20 | 855 | 5 | 25 |  | 896 | N/A | N/A |  |
|  | n3 | 1775 | 10 | 50 |  | 1870 | N/A | N/A |  |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n8 | 885 | 5 | 25 |  | 930 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 795 | 17.4 | IMD3 |  |
| DC_7A-20A_n8A | 7 | N/A | 5 | N/A |  | 2640 | 21.1 | IMD3 |  |
|  | n8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | 20 | 840 | 5 | 25 |  | 799 | N/A | N/A |  |
| DC_7A-20A_n8A | 7 | N/A | 5 | N/A |  | 2624 | 18.8 | IMD3 |  |
|  | n8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | 20 | 857 | 5 | 25 |  | 816 | N/A | N/A |  |
| DC_7A-20A_n28A | 20 | 842 | 5 | 25 |  | 801 | N/A | N/A |  |
|  | n28 | 728 | 5 | 25 |  | 783 | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2640 | 5.9 | IMD5 |  |
| DC_7A-20A_n78A DC_7A-7A-20A_n78A | 7 | 2560 | 5 | 25 |  | 2680 | N/A | N/A |  |
| DC_7A-20A_n78(2A) | 20 | N/A | 5 | N/A |  | 810 | 30.5 | IMD2 |  |
| DC_7A-20A_n78C | n78 | 3370 | 10 | 50 |  | 3370 | N/A | N/A |  |
|  | 7 | 2560 | 5 | 25 |  | 2680 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 810 | 3.0 | IMD5 |  |
|  | n78 | 3435 | 10 | 50 |  | 3435 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2675 | 30.8 | IMD2 |  |
|  | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n78 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
| DC_7A-25A_n77A DC_7A-7A-25A_n77A DC_7C-25A_n77A DC_7C-25A-25A_n77A DC_7A-25A-25A_n77A DC_7A-7A-25A-25A_n77A | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A |  | 1950 | 8.6 | IMD4 |  |
|  | n77 | 3525 | 10 | 50 |  | 3525 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2660 | 3.4 | IMD5 |  |
|  | 25 | 1860 | 5 | 25 |  | 1940 | N/A | N/A |  |
|  | n77 | 4120 | 10 | 50 |  | 4120 | N/A | N/A |  |
| DC_7A-25A_n78A DC_7A-7A-25A_n78A DC_7C-25A_n78A DC_7A-25A-25A_n78A DC_7A-7A-25A-25A_n78A DC_7C-25A-25A_n78A | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A |  | 1950 | 8.6 | IMD4 |  |
|  | n78 | 3525 | 10 | 50 |  | 3525 | N/A | N/A |  |
| DC_7A-26A_n78A | 7 | 2525 | 5 | 25 |  | 2645 | 30.1 | IMD2 |  |
| DC_7C-26A_n78A | 26 | 844 | 5 | 25 |  | 889 | N/A | N/A |  |
|  | n78 | 3489 | 10 | 50 |  | 3489 | N/A | N/A |  |
|  | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | 26 | 834 | 5 | 25 |  | 879 | 30.2 | IMD2 |  |
|  | n78 | 3429 | 10 | 50 |  | 3429 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 |  | 2645 | N/A | N/A |  |
|  | 26 | 830 | 5 | 25 |  | 875 | 3.3 | IMD5 |  |
|  | n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
| DC_7A_n26A-n78A | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
| DC_7C_n26A-n78A | n26 | N/A | 5 | N/A |  | 879 | 30.2 | IMD2 |  |
|  | n78 | 3429 | 10 | 50 |  | 3429 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 |  | 2645 | N/A | N/A |  |
|  | n26 | N/A | 5 | N/A |  | 875 | 3.3 | IMD5 |  |
|  | n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
|  | 7 | 2540 | 5 | 25 |  | 2660 | N/A | N/A |  |
|  | n26 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3375 | 29.7 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | n26 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3430 | 9.7 | IMD4 |  |
| DC_7A-28A_n1A | 7 | 2535 | 5 | 25 |  | 2655 | N/A | N/A |  |
| DC_7A-7A-28A_n1A | 28 | N/A | 5 | N/A |  | 780 | 4.3 | IMD5 |  |
|  | n1 | 1950 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2665 | 29.0 | IMD2 |  |
|  | 28 | 730 | 5 | 25 |  | 785 | N/A | N/A |  |
|  | n1 | 1935 | 5 | 25 |  | 2125 | N/A | N/A |  |
| DC_7A-28A_n2A | 7 | N/A | 10 | N/A |  | 2630 | 27.6 | IMD2 |  |
|  | 28 | 730 | 5 | 25 |  | 785 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 | 2543 | 5 | 25 |  | 2663 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 796.0 | 20.0 | IMD2 |  |
|  | n3 | 1747 | 5 | 25 |  | 1842 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2685 | 18 | IMD3 |  |
|  | 28 | 745 | 5 | 25 |  | 800 | N/A | N/A |  |
|  | n3 | 1715 | 5 | 25 |  | 1810 | N/A | N/A |  |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 |  | 2725 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 776 | 4.4 | IMD5 |  |
|  | n5 | 829 | 5 | 25 |  | 854 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2630 | 5.9 | IMD5 |  |
|  | 28 | 730 | 5 | 25 |  | 785 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 |  | 874 | N/A | N/A |  |
| DC_7A-28A_n20A | 7 | N/A | 5 | N/A |  | 2640 | 5.9 | IMD5 |  |
|  | 28 | 728 | 5 | 25 |  | 783 | N/A | N/A |  |
|  | n20 | 842 | 5 | 25 |  | 801 | N/A | N/A |  |
|  | 7 | 2505 | 5 | 25 |  | 2625 | N/A | N/A |  |
|  | n20 | 859 | 5 | 25 |  | 818 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 787 | 17.4 | IMD3 |  |
| DC_7A-28A_n40A | 7 | N/A | 5 | N/A |  | 2630 | 5.9 | IMD5 |  |
|  | 28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n40 | 2310 | 5 | 25 |  | 2310 | N/A | N/A |  |
| DC_7A-28A_n66A DC_7C-28A_n66A | 7 | N/A | 10 | N/A |  | 2682 | 16.9 | IMD3 |  |
|  | 28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n66 | 1712.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
|  | 7 | 2543 | 5 | 25 |  | 2663 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 796 | 20.0 | IMD2 |  |
|  | n66 | 1747 | 5 | 25 |  | 2147 | N/A | N/A |  |
| DC_7A-28A_n78A | 7 | 2567.5 | 5 | 25 |  | 2687.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 782.5 | 28.8 | IMD2 |  |
|  | n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
|  | 7 | 2567.5 | 5 | 25 |  | 2687.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 782.5 | 3.0 | IMD5 |  |
|  | n78 | 3460 | 10 | 50 |  | 3460 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2650 | 30.5 | IMD2 |  |
|  | 28 | 740 | 5 | 25 |  | 795 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 |  | 3390 | N/A | N/A |  |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 |  | 800 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3310 | 29.7 | IMD2 |  |
|  | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n78 | 3365 | 10 | 50 |  | 3365 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 800 | 28.8 | IMD2 |  |
| DC_7A-29A_n78A DC_7C-29A_n78A DC_7A-7A-29A_n78A | 7 | 2540 | 5 | 25 |  | 2660 | N/A | N/A |  |
|  | 29 | N/A | N/A | N/A |  | 720 | 3.0 | IMD5 |  |
|  | n78 | 3450 | 10 | 50 |  | 3450 | N/A | N/A |  |
| DC_7A-32A_n1A | n1 | 1977.5 | 5 | 25 |  | 2167.5 | N/A | N/A |  |
|  | 7 | 2502.5 | 5 | 25 |  | 2622.5 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1454.5 | 15.2 | IMD3 |  |
| DC_7A-32A_n3A | 7 | 1775 | 5 | 25 |  | 1870 | N/A | N/A |  |
|  | n3 | 2510 | 10 | 50 |  | 2630 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1470 | 10.5 | IMD4 |  |
| DC_7A-32A_n78A | n78 | 3560.5 | 10 | 50 |  | 3560.5 | N/A | N/A |  |
|  | 7 | 2517.5 | 5 | 25 |  | 2637.5 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1474.5 | 17.6 | IMD3 |  |
|  | n78 | 3311 | 10 | 50 |  | 3311 | N/A | N/A |  |
|  | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1492 | 4.9 | IMD4 |  |
| DC_7A-40A_n1A DC_7A-40C_n1A | n1 | 1970 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2650 | 32.1 | IMD3 |  |
|  | 40 | 2310 | 5 | 25 |  | 2310 | N/A | N/A |  |
| DC_7A_n40A-n77A | 7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
| DC_7A_n40A-n77(2A) | n40 | N/A | 5 | N/A |  | 2360 | 9.2 | IMD4 |  |
| DC_7A-7A_n40A-n77A DC_7A-7A_n40A-n77(2A) | n77 | 3700 | 10 | 50 |  | 3700 | N/A | N/A |  |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 | N/A | 5 | N/A |  | 2630 | 10.1 | IMD4 |  |
|  | 40 | 2310 | 5 | 25 |  | 2310 | N/A | N/A |  |
|  | n78 | 3625 | 10 | 50 |  | 3625 | N/A | N/A |  |
|  | 7 | 2510 | 5 | 25 |  | 2630 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A |  | 2310 | 8.7 | IMD4 |  |
|  | n78 | 3785 | 10 | 50 |  | 3785 | N/A | N/A |  |
| DC_7A_n40A-n78A | 7 | 2520 | 10 | 50 |  | 2640 | N/A | N/A |  |
| DC_7A_n40A-n78C | n40 | N/A | 5 | N/A |  | 2360 | 8.7 | IMD4 |  |
|  | n78 | 3700 | 10 | 50 |  | 3700 | N/A | N/A |  |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD2, IMD5 |  |
|  | n78 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | 7 | N/A | 10 | N/A |  | 2625 | 30.0 | IMD26 |  |
|  | 66 | 1775 | 10 | 50 |  | 2175 | N/A | N/A |  |
|  | n5 | 846.5 | 5 | 25 |  | 891.5 | N/A | N/A |  |
| DC_7A-66A_n7A DC_7A-66A-66A_n7A | 7 | N/A | 10 | N/A |  | 2675 | 15 | IMD4 |  |
|  | 66 | 1730 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | n7 | 2515 | 10 | 50 |  | 2635 | N/A | N/A |  |
| DC_7A-66A_n28A | 7 | N/A | 5 | N/A |  | 2685 | 18.0 | IMD3 |  |
|  | 66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 |  | 800 | N/A | N/A |  |
| DC_7A-66A_n77A DC_7A-7A-66A_n77A DC_7A-7A-66A_n77(2A) DC_7A-66A_n77(2A) DC_7C-66A_n77A DC_7C-66A_n77(2A) | 7 | 2550 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2150 | 8.7 | IMD4 ¦2*fB7-2*fn77¦ |  |
|  | n77 | 3625 | 10 | 50 |  | 3475 | N/A | N/A |  |
|  | 66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2670 | 5.2 | IMD5 |  |
|  | n77 | 4190 | 10 | 50 |  | 4190 | N/A | N/A |  |
|  | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 7 | N/A | 5 | N/A |  | 2640 | 3.4 | IMD5 |  |
|  | n77 | 3900 | 10 | 50 |  | 3900 | N/A | N/A |  |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2550 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2150 | 8.7 | IMD4 |  |
|  | n77 | 3625 | 10 | 50 |  | 3625 | N/A | N/A |  |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2542 | 5 | 25 |  | 2662 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3344 | 16.0 | IMD3 |  |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | n66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 4040 | 4.2 | IMD5 |  |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 | 2550 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | 66/n66 | N/A | 5 | N/A |  | 2150 | 8.7 | IMD4 |  |
|  | n78 | 3625 | 10 | 50 |  | 3475 | N/A | N/A |  |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | 2542 | 5 | 25 |  | 2662 | N/A | N/A |  |
|  | n66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3344 | 16.0 | IMD3 |  |
| DC_7A-71A_n2A | n2 | 1859 | 5 | 25 |  | 1933 | N/A | N/A |  |
| DC_7A-71A_n2(2A) | 7 | 2505 | 5 | 25 |  | 2625 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A |  | 646 | 30.8 | IMD2 |  |
| DC_7A-71A_n25A | 7 | 2530 | 5 | 25 |  | 2530 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A |  | 630 | 28.7 | IMD24 |  |
|  | n25 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
| DC_7A-71A_n77A DC_7A-71A_n77(2A) | 7 | N/A | 5 | N/A |  | 2670 | 29.6 | IMD21 |  |
|  | 71 | 680 | 5 | 25 |  | 634 | N/A | N/A |  |
|  | n77 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
| DC_7A_n71A-n77A | 7 | 2505 | 5 | 25 |  | 2625 | N/A | N/A |  |
|  | n71 | 666 | 5 | 25 |  | 620 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3837 | 16.0 | IMD3 |  |
| DC_7A-71A_n78A DC_7A-71A_n78(2A) | 7 | N/A | 5 | N/A |  | 2670 | 29.6 | IMD2 |  |
|  | 71 | 680 | 5 | 25 |  | 634 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
|  | 7 | 2540 | 5 | 25 |  | 2660 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A |  | 640 | 3.0 | IMD5 |  |
|  | n78 | 3490 | 10 | 50 |  | 3490 | N/A | N/A |  |
| DC_7A_n71A-n78A | 7 | 2550 | 5 | 25 |  | 2670 | N/A | N/A |  |
|  | n71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3714 | 9.7 | IMD4 |  |
|  | 7 | 2555 | 5 | 25 |  | 2675 | N/A | N/A |  |
|  | n78 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A |  | 625 | 3.9 | IMD5 |  |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | 7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | n78 | 3600 | 10 | 50 |  | 3600 | N/A | N/A |  |
|  | n79 | N/A | 10 | N/A |  | 4680 | 20.6 | IMD34,9,13 |  |
|  | 7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3770 | 6.4 | IMD413 |  |
|  | n79 | 4450 | 10 | 50 |  | 4450 | N/A | N/A |  |
| DC_7A_SUL_n78A-n80A | n80 | 1730 | 5 | 25 |  |  | N/A | N/A |  |
|  | 7 | N/A | 10 | N/A |  | 2655 | 13 | IMD4 |  |
| DC_7_n78-n105 | 7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3700 | 9.7 | IMD4 |  |
|  | n105 | 670 | 5 | 25 |  | 619 | N/A | N/A |  |
|  | 7 | 2555 | 5 | 25 |  | 2675 | N/A | N/A |  |
|  | n78 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
|  | n105 | N/A | 5 | N/A |  | 625 | 3.9 | IMD5 |  |
| DC_8A_n1A-n28A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n1 | 1965 | 5 | 25 |  | 2155 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 765 | 11.6 | IMD4 |  |
| DC_8A_n1A-n40A | 8 | 885 | 5 | 25 |  | 930 | N/A | N/A |  |
|  | n40 | 2395 | 5 | 25 |  | 2395 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2135 | 3.3 | IMD5 |  |
| DC_8A_n1A-n77A | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n1 | 1945 | 5 | 25 |  | 2135 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3745 | 14.9 | IMD31 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n77 | 3960 | 10 | 50 |  | 3960 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2140 | 14.4 | IMD3 |  |
| DC_8A_n1A-n78A | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n1 | 1945 | 5 | 25 |  | 2135 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3745 | 14.9 | IMD3 |  |
| DC_8A-(n)3AA | 8 | 897.5 | 5 | 25 |  | 942.5 | N/A | N/A |  |
|  | 3 | N/A | 5 | N/A |  | 1835 | 4.5 | IMD5 |  |
|  | n3 | 1747.5 | 10 | 50 |  | 1842.5 | 6.4 | IMD5 |  |
| DC_8A_n3A-n28A | 8 | 912.5 | 5 | 25 |  | 957.5 | N/A | N/A |  |
|  | n3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 800 | 30.4 | IMD2 |  |
| DC_8A_n3A-n77A DC_8A_n3A-n77(2A) | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n3 | 1740 | 5 | 25 |  | 1835 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3540 | 16.3 | IMD3 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n77 | 3640 | 10 | 50 |  | 3640 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1820 | 16.5 | IMD3 |  |
| DC_8A_n3A-n78A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3550 | 16.1 | IMD3 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3370 | 4.5 | IMD5 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1820 | 15.7 | IMD3 |  |
|  | n78 | 3640 | 10 | 50 |  | 3640 | N/A | N/A |  |
| DC_8A_n3A-n79A | 8 | 885 | 5 | 25 |  | 930 | N/A | N/A |  |
|  | n3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4425 | 15.7 | IMD39 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n79 | 4580 | 40 | 216 |  | 4580 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1850 | 8.8 | IMD4 |  |
| DC_8A-11A_n1A DC_8B-11A_n1A | 11 | 1435 | 5 | 25 |  | 1483 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 930 | 16.6 | IMD35 |  |
| DC_8A-11A_n77A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n77 | 3311 | 10 | 50 |  | 3311 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1491 | 18.8 | IMD3 |  |
| DC_8A-11A_n77A | 11 | 1430.5 | 5 | 25 |  | 1478.5 | N/A | N/A |  |
|  | n77 | 3791 | 10 | 50 |  | 3791 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 930 | 18.2 | IMD3 |  |
| DC_8A-11A_n78A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n78 | 3311 | 10 | 50 |  | 3311 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1491 | 18.8 | IMD3 |  |
| DC_8A-11A_n78A | 11 | 1430.5 | 5 | 25 |  | 1478.5 | N/A | N/A |  |
|  | n78 | 3791 | 10 | 50 |  | 3791 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 930 | 18.2 | IMD3 |  |
| DC_8A-11A_n79A | 8 | 882.5 | 5 | 25 |  | 927.5 | N/A | N/A |  |
|  | n79 | 4980 | 40 | 216 |  | 4980 | N/A | N/A |  |
|  | 11 | N/A | 5 | N/A |  | 1478.4 | 1.2 | IMD5 |  |
| DC_8A-11A_n79A | 11 | 1435 | 5 | 25 |  | 1483 | N/A | N/A |  |
|  | n79 | 4810 | 40 | 216 |  | 4810 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 930 | 2.8 | IMD5 |  |
| DC_8-20_n1 | n1 | 1925 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 805 | 11.5 | IMD4 |  |
| DC_8-20_n3 | n3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 810 | 27 | IMD24 |  |
|  | n3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | 8 | 890 | 5 | 25 |  | 930 | 27 | IMD24 |  |
|  | 20 | 840 | 5 | 25 |  | 799 | N/A | N/A |  |
| DC_8A-20A_n28A | 8 | N/A | 5 | N/A |  | 946 | [23.5] | IMD3 |  |
|  | 20 | 837 | 5 | 25 |  | 796 | N/A | N/A |  |
|  | n28 | 728 | 5 | 25 |  | 773 | N/A | N/A |  |
| DC_8A-20A_n78A | 8 | 890 | 5 | 25 |  | 935 | N/A | N/A |  |
|  | n78 | 3470 | 10 | 50 |  | 3470 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 800 | 12.1 | IMD4 |  |
|  | 8 | N/A | 5 | N/A |  | 940 | 12.1 | IMD4 |  |
|  | n78 | 3481 | 10 | 50 |  | 3481 | N/A | N/A |  |
|  | 20 | 847 | 5 | 25 |  | 806 | N/A | N/A |  |
| DC_8A-28A_n3A | 8 | 912.5 | 5 | 25 |  | 957.5 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 800 | 30.4 | IMD24 |  |
|  | n3 | 1712.5 | 5 | 25 |  | 1807.5 | N/A | N/A |  |
| DC_8A_n28A-n77A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3473 | 10.3 | IMD4 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 765 | 11.6 | IMD4 |  |
|  | n77 | 3495 | 10 | 50 |  | 3495 | N/A | N/A |  |
| DC_8A_n28A-n78A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n28 | 725 | 5 | 25 |  | 780 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3455 | 10.3 | IMD4 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 765 | 11.6 | IMD4 |  |
|  | n78 | 3495 | 10 | 50 |  | 3495 | N/A | N/A |  |
| DC_8A_n28A-n79A | 8 | 912.5 | 5 | 25 |  | 957.5 | N/A | N/A |  |
|  | n28 | 745.5 | 5 | 25 |  | 800.5 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4420 | 0.0 | IMD5 |  |
|  | 8 | 905 | 5 | 25 |  | 950 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 |  | 4420 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 800 | 3.9 | IMD5 |  |
| DC_8A-32A_n78A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n78 | 3311 | 10 | 50 |  | 3311 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1491 | 18.8 | IMD3 |  |
| DC_8A_n39A-n79A | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n39 | 1890 | 10 | 50 |  | 1890 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4680 | 15.9 | IMD3 |  |
| DC_8A_n39A-n79A | 8 | 890 | 5 | 25 |  | 935 | N/A | N/A |  |
|  | n39 | 1890 | 10 | 50 |  | 1890 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4560 | 12.1 | IMD4 |  |
| DC_8A_n39A-n79A | 8 | 897.5 | 5 | 25 |  | 942.5 | N/A | N/A |  |
|  | n39 | N/A | 10 | N/A |  | 1907.5 | 13.8 | IMD4 |  |
|  | n79 | 4600 | 40 | 216 |  | 4600 | N/A | N/A |  |
| DC_8A-40A_n1A DC_8A-40C_n1A | 8 | N/A | 5 | N/A |  | 930 | 8.0 | IMD4 |  |
|  | 40 | 2395 | 5 | 25 |  | 2395 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
| DC_8A-40A_n78A DC_8A-40C_n78A | 8 | N/A | 5 | N/A |  | 950 | 30.5 | IMD2 |  |
|  | 40 | 2380 | 5 | 25 |  | 2380 | N/A | N/A |  |
|  | n78 | 3330 | 10 | 50 |  | 3330 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 935 | 19.8 | IMD3 |  |
|  | 40 | 2320 | 5 | 25 |  | 2320 | N/A | N/A |  |
|  | n78 | 3705 | 10 | 50 |  | 3705 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A |  | 2395 | 28 | IMD2 |  |
|  | n78 | 3305 | 10 | 50 |  | 3305 | N/A | N/A |  |
| DC_8A_n40A-n79A | 8 | 885 | 5 | 25 |  | 930 | N/A | N/A |  |
|  | n40 | 2305 | 5 | 25 |  | 2305 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4960 | 10.7 | IMD4 |  |
|  | 8 | 885 | 5 | 25 |  | 930 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2305 | 9.2 | IMD4 |  |
|  | n79 | 4960 | 40 | 216 |  | 4960 | N/A | N/A |  |
| DC_8A-41A_n1A | 41 | 2500 | 5 | 25 |  | 2500 | N/A | N/A |  |
| DC_8A-41C_n1A | n1 | 1977 | 5 | 25 |  | 2167 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 931 | 4.5 | IMD5 |  |
| DC_8A-41A_n3A | n3 | 1780 | 5 | 25 |  | 1875 | N/A | N/A |  |
| DC_8A-41C_n3A | 8 | 885 | 5 | 25 |  | 930 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2665 | 27.4 | IMD21 |  |
|  | n3 | 1715 | 5 | 25 |  | 1810 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 950 | 28.9 | IMD21 |  |
|  | 41 | 2665 | 5 | 25 |  | 2665 | N/A | N/A |  |
| DC_8A-41A_n77A | 8 | N/A | 5 | N/A |  | 950 | 29.1 | IMD21, 4 |  |
| DC_8A-41C_n77A | 41 | 2630 | 10 | 50 |  | 2630 | N/A | N/A |  |
|  | n77 | 3580 | 10 | 50 |  | 3580 | N/A | N/A |  |
|  | 8 | 895 | 5 | 25 |  | 940 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2650 | 28.0 | IMD2 |  |
|  | n77 | 3545 | 10 | 50 |  | 3545 | N/A | N/A |  |
| DC_8A-41A_n78A | 8 | N/A | 5 | N/A |  | 950 | 29.1 | IMD24 |  |
| DC_8A-41C_n78A | 41 | 2630 | 5 | 25 |  | 2630 | N/A | N/A |  |
|  | n78 | 3580 | 10 | 50 |  | 3580 | N/A | N/A |  |
|  | 8 | 895 | 5 | 25 |  | 940 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2650 | 28.0 | IMD2 |  |
|  | n78 | 3545 | 10 | 50 |  | 3545 | N/A | N/A |  |
| DC_8A_n41A-n79A | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n41 | 2650 | 10 | 50 |  | 2650 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4470 | 16.3 | IMD3 |  |
|  | 8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A |  | 2650 | 15.5 | IMD3 |  |
|  | n79 | 4470 | 40 | 216 |  | 4470 | N/A | N/A |  |
| DC_8A-42A_n1A | 42 | 3405 | 10 | 50 |  | 3405 | N/A | N/A |  |
| DC_8A-42C_n1A | n1 | 1955 | 5 | 25 |  | 2145 | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 945 | 3.3 | IMD5 |  |
| DC_8A-42A_n3A | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n3 | 1740 | 5 | 25 |  | 1835 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A |  | 3540 | 16.3 | IMD3 |  |
| DC_8A-42A_n28A | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | 42 | N/A | 5 | N/A |  | 3443 | 8.7 | IMD4 |  |
| DC_8A_SUL_n78A-n80A | n80 | 1755 | 10 | 50 |  |  | N/A | N/A |  |
|  | 8 | N/A | 5 | N/A |  | 945 | 8 | IMD4 |  |
|  | n80 | 1750 | 10 | 50 |  |  | N/A | N/A |  |
|  | 8 | 900 | 5 | 25 |  | 945 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3550 | 8 | IMD33 |  |
| DC_11A_n1A-n77A DC_11A_n1A-n77(2A) | 11 | 1435 | 5 | 25 |  | 1483 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3375 | 29.6 | IMD21 |  |
|  | 11 | 1438 | 5 | 25 |  | 1486 | N/A | N/A |  |
|  | n77 | 3578 | 10 | 50 |  | 3578 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2140 | 30.8 | IMD21 |  |
| DC_11A_n3A-n28A | 11 | 1435 | 5 | 25 |  | 1483 | N/A | N/A |  |
|  | n3 | 1753 | 5 | 25 |  | 1848 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 800 | 3.0 | IMD5 |  |
| DC_11A_n3A-n77A DC_11A_n3A-n77(2A) | 11 | 1440 | 5 | 25 |  | 1488 | N/A | N/A |  |
|  | n3 | 1740 | 5 | 25 |  | 1835 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3780 | 10.8 | IMD4 |  |
|  | 11 | 1440 | 5 | 25 |  | 1488 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1870 | 29.0 | IMD2 |  |
|  | n77 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
| DC_11A_n3A-n79A | 11 | 1435 | 5 | 25 |  | 1483 | N/A | N/A |  |
|  | n3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4640 | 16.2 | IMD3 |  |
|  | 11 | 1435 | 5 | 25 |  | 1483 | N/A | N/A |  |
|  | n79 | 4735 | 40 | 216 |  | 4735 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1865 | 17.8 | IMD3 |  |
| DC_11A-18A_n77A | 11 | 1443 | 5 | 25 |  | 1491 | N/A | N/A |  |
| DC_11A-18A_n77(2A) | n77 | 3706 | 10 | 50 |  | 3706 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A |  | 865 | 18.7 | IMD3 |  |
| DC_11A-18A_n78A | 11 | 1443 | 5 | 25 |  | 1491 | N/A | N/A |  |
| DC_11A-18A_n78(2A) | n78 | 3706 | 10 | 50 |  | 3706 | N/A | N/A |  |
|  | 18 | N/A | 5 | N/A |  | 865 | 18.7 | IMD3 |  |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | 1443 | 5 | 25 |  | 1491 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3629 | 17.5 | IMD3 |  |
|  | 11 | 1443 | 5 | 25 |  | 1491 | N/A | N/A |  |
|  | n77 | 3684 | 10 | 50 |  | 3684 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 798 | 15.8 | IMD3 |  |
| DC_12A_n2A-n38A | 12 | 708 | 5 | 25 |  | 738 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A |  | 2608 | 28.7 | IMD2 |  |
| DC_12A_n2A-n41A | 12 | 708 | 5 | 25 |  | 738 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A |  | 2608 | 28.7 | IMD2 |  |
| DC_12A_n2A-n66A | 12 | 713.5 | 5 | 25 |  | 743.5 | N/A | N/A |  |
|  | n2 | 1907.5 | 5 | 25 |  | 1987.5 | 2 | IMD4 |  |
|  | n66 | 1712.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
| DC_12A_n2A-n77A | 12 | 707.5 | 5 | 25 |  | 737.5 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 |  | 1960 | 16.5 | IMD3 |  |
|  | n77 | 3375 | 10 | 50 |  | 3375 | N/A | N/A |  |
|  | 12 | 710 | 5 | 25 |  | 740 | N/A | N/A |  |
|  | n2 | 1890 | 5 | 25 |  | 1970 | 12 | IMD4 |  |
|  | n77 | 4100 | 10 | 50 |  | 4100 | N/A | N/A |  |
|  | 12 | 707.5 | 5 | 25 |  | 737.5 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n77 | 3315 | 10 | 50 |  | 3315 | 16.0 | IMD34, |  |
|  | 12 | 710 | 5 | 25 |  | 740 | N/A | N/A |  |
|  | n2 | 1870 | 5 | 25 |  | 1950 | N/A | N/A |  |
|  | n77 | 4000 | 10 | 50 |  | 4000 | 12 | IMD4 |  |
| DC_12_n2-n78 | 12 | 707.5 | 5 | 25 |  | 737.5 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1960 | 16.5 | IMD3 |  |
|  | n78 | 3375 | 10 | 50 |  | 3375 | N/A | N/A |  |
|  | 12 | 707.5 | 5 | 25 |  | 737.5 | N/A | N/A |  |
|  | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3315 | 16.0 | IMD3 |  |
| DC_12A_n7A-n78A, DC_12A_n7(2A)-n78A DC_12A_n7A-n78(2A) DC_12A_n7(2A)-n78(2A) | 12 | 708 | 5 | 25 |  | 738 | N/A | N/A |  |
|  | n7 | 2520 | 5 | 25 |  | 2640 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3624 | 9 | IMD4 |  |
|  | 12 | 708 | 5 | 25 |  | 738 | N/A | N/A |  |
|  | n78 | 3370 | 10 | 50 |  | 3370 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A |  | 2662 | 29.6 | IMD2 |  |
| DC_12A-30A_n2A | 12 | 708.5 | 5 | 25 |  | 738.5 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2353 | 12.0 | IMD4 |  |
|  | n2 | 1885 | 5 | 25 |  | 1965 | N/A | N/A |  |
| DC_12A-30A_n5A | 12 | 702 | 5 | 25 |  | 732 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2355 | 18.8 | IMD3 |  |
|  | n5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
| DC_12A-30A_n77A DC_12A-30A_n77(2A) | 12 | N/A | 5 | N/A |  | 740 | 15.2 | IMD34 |  |
|  | 30 | 2310 | 5 | 25 |  | 2355 | N/A | N/A |  |
|  | n77 | 3880 | 10 | 50 |  | 3880 | N/A | N/A |  |
|  | 12 | 707.5 | 5 | 25 |  | 737.5 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2355 | 13.2 | IMD3 |  |
|  | n77 | 3770 | 10 | 50 |  | 3770 | N/A | N/A |  |
| DC_12A_n41A-n66A | 12 | 713.5 | 5 | 25 |  | 743.5 | N/A | N/A |  |
|  | n41 | 2501 | 10 | 50 |  | 2501 | 20.0 | IMD218 |  |
|  | n66 | 1777.5 | 5 | 25 |  | 2177.5 | N/A | N/A |  |
| DC_12A-66A_n5A DC_12A-66A-66A_n5A | 12 | N/A | 5 | N/A |  | 742 | 9.4 | IMD4 |  |
|  | 66 | 1745 | 5 | 25 |  | 2145 | N/A | N/A |  |
|  | n5 | 829 | 5 | 25 |  | 874 | N/A | N/A |  |
| DC_12A-66A_n7A | 12 | N/A | 5 | N/A |  | 742 | 31 | IMD2 |  |
|  | 66 | 1773 | 5 | 25 |  | 2173 | N/A | N/A |  |
|  | n7 | 2515 | 5 | 25 |  | 2635 | N/A | N/A |  |
| DC_12A-66A_n77A DC_12A-66A_n77(2A) | 12 | N/A | 5 | N/A |  | 740 | 15.2 | IMD311 |  |
| DC_12A-66A-66A_n77A | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
| DC_12A-66A-66A_n77(2A) | n77 | 4180 | 10 | 50 |  | 4180 | N/A | N/A |  |
|  | 12 | 707 | 5 | 25 |  | 737 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2126 | 13.2 | IMD3 |  |
|  | n77 | 3540 | 10 | 50 |  | 3540 | N/A | N/A |  |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n2 | 1896 | 5 | 25 |  | 1976 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3460 | 17.3 | IMD3 |  |
|  | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1960 | 16.0 | IMD3 |  |
|  | n77 | 3524 | 10 | 50 |  | 3524 | N/A | N/A |  |
| DC_13A_n5A-n77A11 | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n77 | 4013 | 10 | 50 |  | 4013 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A |  | 885 | 4.5 | IMD5 |  |
| DC_13A_n25A-n66A | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n25 | 1860 | 5 | 25 |  | 1940 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2156 | 7.2 | IMD4 |  |
| DC_13A_n25A-n66A | 13 | 780 | 5 | 25 |  | 749 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A |  | 1940 | 6.2 | IMD4 |  |
|  | n66 | 1750 | 5 | 25 |  | 2150 | N/A | N/A |  |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n48 | N/A | 5 | N/A |  | 3584 | 2.8 | IMD5 |  |
|  | n66 | 1716 | 5 | 25 |  | 2116 | N/A | N/A |  |
|  | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n48 | 3695 | 5 | 25 |  | 3695 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2131 | 17.1 | IMD3 |  |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
| DC_13A-66B_n2A | 66 | N/A | 5 | N/A |  | 2156 | 7..2 | IMD4 |  |
| DC_13A-66C_n2A | n2 | 1860 | 5 | 25 |  | 1940 | N/A | N/A |  |
| DC_13A-66A_n5A | 13 | N/A | 5 | N/A |  | 750 | 9.4 | IMD4 |  |
| DC_13A-66A-66A_n5A | 66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
| DC_12A-66A_n25A | 12 | 708.5 | 5 | 25 |  | 738.5 | N/A | N/A |  |
|  | 66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A |  | 1935 | 20 | IMD3 |  |
|  | 12 | 708.5 | 5 | 25 |  | 738.5 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2150 | 4 | IMD5 |  |
|  | n25 | 1883.3 | 5 | 25 |  | 1963.3 | N/A | N/A |  |
|  | 12 | 708.5 | 5 | 25 |  | 738.5 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2112.5 | 23 | IMD3 |  |
|  | n25 | 1912.5 | 5 | 25 |  | 1992.5 | N/A | N/A |  |
| DC_12A-66A_n41A | 12 | N/A | 5 | N/A |  | 742 | 31 | IMD2 |  |
|  | 66 | 1773 | 5 | 25 |  | 2173 | N/A | N/A |  |
|  | n41 | 2515 | 5 | 25 |  | 2515 | N/A | N/A |  |
| DC_12A_n66A-n77A | 12 | 707 | 5 | 25 |  | 737 | N/A | N/A |  |
|  | n66 | 1726 | 5 | 25 |  | 2126 | 13.2 | IMD3 |  |
|  | n77 | 3540 | 10 | 50 |  | 3540 | N/A | N/A |  |
|  | 12 | 704 | 5 | 25 |  | 734 | N/A | N/A |  |
|  | n66 | 1723 | 5 | 25 |  | 2123 | N/A | N/A |  |
|  | n77 | 4150 | 10 | 50 |  | 4150 | 16.0 | IMD32,4 |  |
|  | 12 | 709 | 5 | 25 |  | 739 | N/A | N/A |  |
|  | n66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | n77 | 3842 | 10 | 50 |  | 3842 | 9 | IMD4 |  |
| DC_12A-66A_n78A | 12 | 710 | 5 | 25 |  | 740 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2160 | 17.1 | IMD3 |  |
|  | n78 | 3580 | 5 | 25 |  | 3580 | N/A | N/A |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 |  | 733 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2140 | 16.5 | IMD3 |  |
|  | n78 | 3546 | 10 | 50 |  | 3546 | N/A | N/A |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 |  | 733 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3754 | 4.1 | IMD5 |  |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n78 | 3432 | 10 | 50 |  | 3432 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A |  | 2650 | 27.9 | IMD2 |  |
| DC_13A_n7A-n78A | 13 | 749 | 5 | 25 |  | 780 | N/A | N/A |  |
|  | n7 | 2560 | 5 | 25 |  | 2680 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3622 | 9 | IMD4 |  |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | n7 | 2530 | 5 | 25 |  | 2650 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3312 | 29.0 | IMD2 |  |
| DC_13A-46A_n2A5 | 13 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD4 |  |
|  | n2 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_13A-46A_n66A5 | 13 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD4, IMD5 |  |
|  | n66 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_13A-46A_n77A5 DC_13A-46A-46A_n77A5 | 13 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD3, IMD4, IMD5 |  |
|  | n77 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2131 | 17.1 | IMD3 |  |
|  | n48 | 3695 | 5 | 25 |  | 3695 | N/A | N/A |  |
| DC_13A-66A_n77A | 13 | 782 | 5 | 25 |  | 751 | N/A | N/A |  |
| DC_13A-66A_n77C DC_13A-66A-66A_n77A DC_13A-66A-66A_n77C | 66 | N/A | 5 | N/A |  | 2156 | 17.1 | IMD3 |  |
|  | n77 | 3720 | 10 | 50 |  | 3720 | N/A | N/A |  |
| DC_13A-66A_n77A11 | 13 | N/A | 5 | N/A |  | 750 | 15.2 | IMD3 |  |
| DC_13A-66A_n77C11 DC_13A-66A-66A_n77A11 DC_13A-66A-66A_n77C11 | 66 | 1710 | 5 | 25 |  | 2110 | N/A | N/A |  |
|  | n77 | 4170 | 10 | 50 |  | 4170 | N/A | N/A |  |
| DC_14A-30A_n5A | 14 | 795 | 5 | 25 |  | 765 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2353 | 5.9 | IMD5 |  |
|  | n5 | 827 | 5 | 25 |  | 872 | N/A | N/A |  |
| DC_14A-30A_n77A DC_14A-30A_n77(2A) | 14 | N/A | 5 | N/A |  | 763 | 15.2 | IMD34 |  |
|  | 30 | 2310 | 5 | 25 |  | 2355 | N/A | N/A |  |
|  | n77 | 3857 | 10 | 50 |  | 3857 | N/A | N/A |  |
|  | 14 | 793 | 5 | 25 |  | 763 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2355 | 13.2 | IMD3 |  |
|  | n77 | 3941 | 10 | 50 |  | 3941 | N/A | N/A |  |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 |  | 763 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2162 | 7.6 | IMD4 |  |
|  | n2 | 1874 | 5 | 25 |  | 1954 | N/A | N/A |  |
| DC_14A-66A_n5A | 14 | N/A | 5 | N/A |  | 762 | 9.4 | IMD4 |  |
|  | 66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n5 | 834 | 5 | 25 |  | 879 | N/A | N/A |  |
| DC_14A-66A_n77A DC_14A-66A_n77(2A) | 14 | N/A | 5 | N/A |  | 763 | 15.2 | IMD311 |  |
| DC_14A-66A-66A_n77A | 66 | 1712.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
| DC_14A-66A-66A_n77(2A) | n77 | 4188 | 10 | 50 |  | 4188 | N/A | N/A |  |
|  | 14 | 793 | 5 | 25 |  | 763 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2155 | 13.2 | IMD3 |  |
|  | n77 | 3741 | 10 | 50 |  | 3741 | N/A | N/A |  |
| DC_18A_n3A-n41A | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A |  | 2540 | 29.4 | IMD2 |  |
|  | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n41 | 2670 | 10 | 50 |  | 2670 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1850 | 28.2 | IMD2 |  |
| DC_18A_n3A-n77A | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n3 | 1770 | 5 | 25 |  | 1865 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3410 | 16.3 | IMD3 |  |
|  | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1865 | 15.7 | IMD3 |  |
|  | n77 | 3505 | 10 | 50 |  | 3505 | N/A | N/A |  |
| DC_18A_n3A-n78A | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3390 | 15.2 | IMD33 |  |
| DC_18A-28A_n77A DC_18A_n28A-n77A | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | 28/n28 | N/A | 5 | N/A |  | 778 | 4.4 | IMD5 |  |
|  | n77 | 4058 | 10 | 50 |  | 4058 | N/A | N/A |  |
| DC_18A-28A_n77A | 18 | N/A | 5 | N/A |  | 865 | 3.9 | IMD5 |  |
|  | 28 | 723 | 5 | 25 |  | 778 | N/A | N/A |  |
|  | n77 | 3757 | 10 | 50 |  | 3757 | N/A | N/A |  |
| DC_18A-28A_n78A | 18 | N/A | 5 | N/A |  | 864 | 3.8 | IMD5 |  |
|  | 28 | 723 | 5 | 25 |  | 778 | N/A | N/A |  |
|  | n78 | 3756 | 10 | 50 |  | 3756 | N/A | N/A |  |
| DC_18A_n28A-n77A DC_18A_n28A-n78A | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n28 | 710 | 5 | 25 |  | 765 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A |  | 3770 | 4.0 | IMD5 |  |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n3 | 1725 | 5 | 25 |  | 1820 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2630 | 16.0 | IMD3 |  |
|  | 18 | N/A | 5 | N/A |  | 865 | 28.9 | IMD21 |  |
|  | n3 | 1765 | 5 | 25 |  | 1860 | N/A | N/A |  |
|  | 41 | 2630 | 5 | 25 |  | 2630 | N/A | N/A |  |
| DC_18A_n41A-n77A DC_18A_n41A-n77(2A) DC_18A_n41A-n78A DC_18A_n41A-n78(2A) | 18 | N/A | 5 | N/A |  | 865 | 3.4 | IMD5 |  |
|  | n77 | 3527.5 | 10 | 50 |  | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 |  | 2640 | N/A | N/A |  |
| DC_18A_n41A-n77A DC_18A_n41A-n78A | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n41 | 2570 | 5 | 25 |  | 2570 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A |  | 3390 | 30.1 | IMD2 |  |
|  | 18 | 820 | 5 | 25 |  | 865 | N/A | N/A |  |
|  | n77/n78 | 3450 | 10 | 50 |  | 3450 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A |  | 2630 | 28.5 | IMD2 |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | N/A | 5 | N/A |  | 865 | 3.4 | IMD5 |  |
|  | n78 | 3527.5 | 10 | 50 |  | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 |  | 2640 | N/A | N/A |  |
| DC_19A_n1A-n77A DC_19A_n1A-n78A | 19 | 840 | 5 | 25 |  | 885 | N/A | N/A |  |
|  | n1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A |  | 3655 | [21.4] | IMD3 |  |
|  | 19 | 832.5 | 5 | 25 |  | 877.5 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2130 | 17.8 | IMD3 |  |
|  | n77/n78 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | N/A | 5 | N/A |  | 882.5 | 18.7 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3783.3 | 10 | 50 |  | 3783.3 | N/A | N/A |  |
|  | 19 | N/A | 5 | N/A |  | 882.5 | 13.2 | IMD4 |  |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3468.7 | 10 | 50 |  | 3468.7 | N/A | N/A |  |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 |  | 882.5 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A |  | 1502.5 | 9.0 | IMD4 |  |
|  | n77 | 4015 | 10 | 50 |  | 4015 | N/A | N/A |  |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A |  | N/A | N/A | IMD5 |  |
|  | 21 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | n79 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 19 | 837.5 | 5 | 25 |  | 882.2 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A |  | 1500 | 3.8 | IMD5 |  |
|  | n79 | 4850 | 40 | 216 |  | 4850 | N/A | N/A |  |
| DC_20A-n1A_n75A | n1 | 1950.5 | 5 | 50 |  | 2140.5 | N/A | N/A |  |
|  | 20 | 852.5 | 5 | 25 |  | 811.5 | N/A | N/A |  |
|  | n75 | N/A | 5 | N/A |  | 1459.5 | 4.0 | IMD5 |  |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3630 | 16.0 | IMD3 |  |
|  | 20 | 835 | 5 | 25 |  | 794 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2120 | 15.3 | IMD3 |  |
|  | n78 | 3790 | 10 | 50 |  | 3790 | N/A | N/A |  |
| DC_20_n3-n67 | 20 | 837 | 5 | 25 |  | 796 | N/A | N/A |  |
|  | n3 | 1765 | 5 | 25 |  | 1860 | N/A | N/A |  |
|  | n67 | N/A | 5 | N/A |  | 746 | 9.4 | IMD4 |  |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 |  | 1825 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3420 | 16.1 | IMD3 |  |
|  | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1860 | 15.7 | IMD3 |  |
|  | n78 | 3550 | 10 | 50 |  | 3550 | N/A | N/A |  |
| DC_20A_n7A-n78A | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A |  | 2675 | 30.8 | IMD2 |  |
|  | n78 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
|  | 20 | 850 | 5 | 25 |  | 809 | N/A | N/A |  |
|  | n7 | 2550 | 10 | 50 |  | 2675 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3400 | 28.8 | IMD21 |  |
| DC_20A_n8A-n78A | n8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | 20 | 837 | 5 | 25 |  | 796 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3567 | 10.3 | IMD4 |  |
|  | n8 | N/A | 5 | N/A |  | 940 | 12.1 | IMD4 |  |
|  | n78 | 3481 | 10 | 50 |  | 3481 | N/A | N/A |  |
|  | 20 | 847 | 5 | 25 |  | 806 | N/A | N/A |  |
| DC_20A-38A_n1A | n1 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 20 | N/A | N/A | N/A |  | N/A | N/A | IMD5 |  |
|  | 38 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_20A-38A_n78A DC_20A-38A_n78(2A | 20 | N/A | N/A | N/A |  | N/A | N/A | IMD2 |  |
|  | 38 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 20 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | 38 | N/A | N/A | N/A |  | N/A | N/A | IMD2 |  |
|  | n78 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_20A_n38A-n78A | 20 | 850 | 5 | 25 |  | 809 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A |  | 2600 | 30.9 | IMD2 |  |
|  | n78 | 3450 | 10 | 50 |  | 3450 | N/A | N/A |  |
| DC_20A_n7A-n28A | 20 | 857 | 5 | 25 |  | 816 | N/A | N/A |  |
|  | n7 | 2512 | 5 | 25 |  | 2632 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 798 | 13.9 | IMD3 |  |
|  | 20 | 852 | 5 | 25 |  | 811 | N/A | N/A |  |
|  | n7 | N/A | 10 | N/A |  | 2670 | 5.9 | IMD5 |  |
|  | n28 | 738 | 5 | 25 |  | 793 | N/A | N/A |  |
| DC_20A_SUL_n78A-n80A | 20 | N/A | 5 | N/A |  | 806 | 9 | IMD4 |  |
|  | n80 | 1735 | 5 | 25 |  |  | N/A | N/A |  |
| DC_20A_n41A-n78A | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A |  | 2675 | 29.8 | IMD2 |  |
|  | n78 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
|  | 20 | 850 | 5 | 25 |  | 809 | N/A | N/A |  |
|  | n41 | 2550 | 10 | 50 |  | 2550 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3400 | 28.8 | IMD2 |  |
| DC_20A-67A_n3A | 20 | 837 | 5 | 25 |  | 796 | N/A | N/A |  |
|  | 67 | N/A | 5 | N/A |  | 746 | 9.4 | IMD4 |  |
|  | n3 | 1765 | 5 | 25 |  | 1860 | N/A | N/A |  |
| DC_21A_n1A-n77A DC_21A_n1A-n78A | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2154.6 | 30.6 | IMD24 |  |
|  | n77/n78 | 3605 | 10 | 50 |  | 3605 | N/A | N/A |  |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 |  | 1500 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 785.5 | 16.9 | IMD3 |  |
|  | n77 | 3689.5 | 10 | 50 |  | 3689.5 | N/A | N/A |  |
|  | 21 | N/A | 5 | N/A |  | 1498.5 | 9.9 | IMD4 |  |
|  | 28 | 730.5 | 5 | 25 |  | 785.5 | N/A | N/A |  |
|  | n77 | 3690 | 10 | 50 |  | 3690 | N/A | N/A |  |
| DC_21A-28A_n79A | 21 | N/A | 5 | N/A |  | 1498 | 5.2 | IMD5 |  |
|  | 28 | 730.5 | 5 | 25 |  | 785.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 |  | 4420 | N/A | N/A |  |
| DC_21A_n28A-n77A | 21 | 1452 | 5 | 25 |  | 1500 | N/A | N/A |  |
| DC_21A_n28A-n78A | n28 | N/A | 5 | N/A |  | 785.5 | 16.9 | IMD39 |  |
|  | n77/n78 | 3689.5 | 10 | 50 |  | 3689.5 | N/A | N/A |  |
|  | 21 | 1452 | 5 | 25 |  | 1500 | N/A | N/A |  |
|  | n28 | 730.5 | 5 | 25 |  | 785.5 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A |  | 3634.5 | 17.3 | IMD39 |  |
| DC_21A_n28A-n79A 17 | 21 | 1450.4 | 5 | 25 |  | 1498.4 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 790.5 | 2.8 | IMD5 |  |
|  | n79 | 4980 | 40 | 216 |  | 4980 | N/A | N/A |  |
|  | 21 | 1460.4 | 5 | 25 |  | 1508.4 | N/A | N/A |  |
|  | n28 | 735.5 | 5 | 25 |  | 790.5 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4420 | [6.3] | IMD44 |  |
| DC_21A-42A_n1A | 21 | N/A | 5 | N/A |  | 1500 | 31.4 | IMD2 |  |
|  | 42 | 3450 | 10 | 50 |  | 3450 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
| DC_28A_n1A-n40A | 28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2374 | 10.1 | IMD4 |  |
| DC_28A_n1A-n78A | 28 | 733 | 5 | 25 |  | 788 | N/A | N/A |  |
|  | n1 | 1950 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3416 | 15.7 | IMD3 |  |
|  | 28 | 740 | 5 | 25 |  | 795 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2150 | 15.7 | IMD3 |  |
|  | n78 | 3630 | 10 | 50 |  | 3630 | N/A | N/A |  |
| DC_28A_n3A-n77A | 28 | 735 | 5 | 25 |  | 790 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1850 | 17.0 | IMD3 |  |
|  | n77 | 3320 | 10 | 50 |  | 3320 | N/A | N/A |  |
|  | 28 | 733 | 5 | 25 |  | 788 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 4173 | 15.9 | IMD3 |  |
| DC_28A_n5A-n40A | 28 | 712 | 5 | 25 |  | 767 | N/A | N/A |  |
|  | n5 | 826.5 | 5 | 25 |  | 871.5 | N/A | N/A |  |
|  | n40 | N/A | 5 | N/A |  | 2365 | 18.8 | IMD3 |  |
|  | 28 | 720 | 5 | 25 |  | 775 | N/A | N/A |  |
|  | n5 | N/A | 5 | N/A |  | 880 | 17.0 | IMD3 |  |
|  | n40 | 2320 | 5 | 25 |  | 2320 | N/A | N/A |  |
| DC_28A_n7A-n78A DC_28A_n7B-n78A | 28 | 745 | 5 | 25 |  | 800 | N/A | N/A |  |
|  | n7 | 2565 | 5 | 25 |  | 2685 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3310 | 29.7 | IMD2 |  |
|  | 28 | 740 | 5 | 25 |  | 795 | N/A | N/A |  |
|  | n7 | N/A | 5 | N/A |  | 2650 | 30.5 | IMD2 |  |
|  | n78 | 3390 | 10 | 50 |  | 3390 | N/A | N/A |  |
| DC_28A-38A_n1A | n1 | 1975 | 5 | 25 |  | 2165 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 775 | 4.5 | IMD5 |  |
|  | 38 | 2575 | 5 | 25 |  | 2575 | N/A | N/A |  |
| DC_28A-38A_n78A | 28 | 738 | 5 | 25 |  | 793 | N/A | N/A |  |
|  | 38 | N/A | 5 | N/A |  | 2582 | 29.5 | IMD2 |  |
|  | n78 | 3320 | 10 | 50 |  | 3320 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 793 | 30.8 | IMD24 |  |
|  | 38 | 2582 | 5 | 25 |  | 2582 | N/A | N/A |  |
|  | n78 | 3375 | 10 | 50 |  | 3375 | N/A | N/A |  |
| DC_28A-41A_n77A | 28 | 738 | 5 | 25 |  | 793 | N/A | N/A |  |
|  | n77 | 3380 | 10 | 50 |  | 3380 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2642 | 29.5 | IMD2 |  |
| DC_28A-41A_n77A | 41 | 2642 | 5 | 25 |  | 2642 | N/A | N/A |  |
|  | n77 | 3440 | 10 | 50 |  | 3440 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 798 | 30.8 | IMD2 |  |
| DC_28A-41A_n77A | 41 | 2567.5 | 10 | 50 |  | 2567.5 | N/A | N/A |  |
|  | n77 | 3460 | 10 | 50 |  | 3460 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 782.5 | 3.0 | IMD5 |  |
| DC_28A-41A_n78A | 28 | 738 | 5 | 25 |  | 793 | N/A | N/A |  |
|  | n78 | 3380 | 10 | 50 |  | 3380 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2642 | 29.5 | IMD2 |  |
| DC_28A-41A_n78A | 41 | 2642 | 5 | 25 |  | 2642 | N/A | N/A |  |
|  | n78 | 3440 | 10 | 50 |  | 3440 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 798 | 30.8 | IMD2 |  |
| DC_28A-41A_n79A | 28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n79 | 4739 | 40 | 216 |  | 4739 | N/A | N/A |  |
|  | 41 | N/A | 5 | N/A |  | 2510 | 8.6 | IMD4 |  |
| DC_28A-41A_n79A | 41 | 2650 | 5 | 25 |  | 2650 | N/A | N/A |  |
|  | n79 | 4502 | 40 | 216 |  | 4502 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 798 | 15.9 | IMD3 |  |
| DC_28A-42A_n79A | 28 | 730 | 5 | 25 |  | 785 | N/A | N/A |  |
| DC_28A-42A_n79C | 42 | N/A | 5 | N/A |  | 3420 | 15.3 | IMD3 |  |
| DC_28A-42C_n79A | n79 | 4880 | 40 | 216 |  | 4880 | N/A | N/A |  |
| DC_28A-42C_n79C | 28 | N/A | 5 | N/A |  | 800 | 16.2 | IMD2 |  |
|  | 42 | 3597.5 | 5 | 25 |  | 3597.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 |  | 4420 | N/A | N/A |  |
| DC_28A-66A_n7A | 28 | N/A | 5 | N/A |  | 790 | 27.6 | IMD2 |  |
|  | 66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | n7 | 2505 | 5 | 50 |  | 2625 | N/A | N/A |  |
| DC_28A-66A_n66A | 28 | 710.5 | 5 | 25 |  | 765.5 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2129 | 11.0 | IMD4 |  |
|  | n66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | n78 | 3680 | 10 | 50 |  | 3680 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4515 | 29.3 | IMD2 |  |
|  | 19 | 835 | 5 | 25 |  | 880 | N/A | N/A |  |
|  | n79 | 4550 | 40 | 216 |  | 4550 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3715 | 28.8 | IMD2 |  |
| DC_20A-(n)3AA | 3 | N/A | 5 | N/A |  | 1865 | 3 | IMD4 |  |
|  | n3 | 1775 | 5 | 25 |  | 1870 | 4 | IMD4 |  |
|  | 20 | 840 | 5 | 25 |  | 799 | N/A | N/A |  |
| DC_20A-28A_n3A | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 785 | 9.4 | IMD4 |  |
|  | n3 | 1750 | 5 | 25 |  | 1845 | N/A | N/A |  |
| DC_20A-28A_n78A | 20 | 837 | 5 | 25 |  | 796 | N/A | N/A |  |
|  | 28 | N/A | 5 | N/A |  | 799 | 9.4 | IMD4 |  |
|  | n78 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 808 | 3.8 | IMD5 |  |
|  | 28 | 705.5 | 5 | 25 |  | 760.5 | N/A | N/A |  |
|  | n78 | 3630 | 10 | 50 |  | 3630 | N/A | N/A |  |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 |  | 816 | N/A | N/A |  |
|  | n28, n83 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3314 | 8.7 | IMD4 |  |
|  | 20 | 837 | 5 | 25 |  | 796 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 |  | 3310 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 799 | 9.4 | IMD4 |  |
| DC_20A-32A_n1A | n1 | 1950.5 | 5 | 50 |  | 2140.5 | N/A | N/A |  |
|  | 20 | 852.5 | 5 | 25 |  | 811.5 | N/A | N/A |  |
|  | 32 | N/A | 5 | N/A |  | 1459.5 | 4.0 | IMD5 |  |
| DC_20A-38A_n3A | 20 | 850 | 5 | 25 |  | 809 | N/A | N/A |  |
|  | 38 | N/A | 5 | N/A |  | 2610 | 28.4 | IMD21 |  |
|  | n3 | 1760 | 5 | 25 |  | 1855 | N/A | N/A |  |
| DC_20A-40A_n1A DC_20A-40C_n1A | 20 | N/A | 5 | N/A |  | 800 | 8.0 | IMD4 |  |
|  | 40 | 2330 | 5 | 25 |  | 2330 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
| DC_20A-41A_n1A | 20 | N/A | 5 | N/A |  | 800 | 4.5 | IMD5 |  |
| DC_20A-41C_n1A | 41 | 2510 | 10 | 50 |  | 2510 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 |  | 2130 | N/A | N/A |  |
| DC_20A-41A_n78A | 20 | 845 | 5 | 25 |  | 804 | N/A | N/A |  |
| DC_20A-41C_n78A | 41 | N/A | 10 | N/A |  | 2675 | 29.8 | IMD2 |  |
|  | n78 | 3520 | 10 | 50 |  | 3520 | N/A | N/A |  |
|  | 20 | N/A | 5 | N/A |  | 798 | 30.8 | IMD24 |  |
|  | 41 | 2642 | 10 | 50 |  | 2642 | N/A | N/A |  |
|  | n78 | 3440 | 10 | 50 |  | 3440 | N/A | N/A |  |
| DC_20A-40A_n78A DC_20A-40C_n78A DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | 20 | N/A | 5 | N/A |  | 815 | 19.8 | IMD3 |  |
|  | 40 | 2302.5 | 5 | 25 |  | 2302.5 | N/A | N/A |  |
|  | n78 | 3790 | 10 | 50 |  | 3790 | N/A | N/A |  |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 |  | 1501 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 50 |  | 3420 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4873 | 30.1 | IMD2 |  |
|  | 21 | 1453 | 5 | 25 |  | 1501 | N/A | N/A |  |
|  | n79 | 4940 | 40 | 216 |  | 4940 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3487 | 29.8 | IMD2 |  |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | 25 | N/A | 5 | N/A |  | 1992.5 | 8.5 | IMD7 |  |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | 41 | 2502.5 | 5 | 1 (RBstart=0) |  | 2502.5 | N/A | N/A |  |
| DC_25A-(n)41CA DC_25A-(n)41DA DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | n41 | 2670 | 5 | 1 (RBstart=9) |  | 2670 | N/A | N/A |  |
| DC_25A-66A_n77A DC_25A-25A-66A_n77A | 25 | 1855 | 5 | 25 |  | 1935 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2115 | 29.2 | IMD2 |  |
|  | n77 | 3970 | 10 | 25 |  | 3970 | N/A | N/A |  |
|  | 25 | 1880 | 5 | 25 |  | 1960 | M/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2140 | 10.4 | IMD4 |  |
|  | n77 | 3500 | 10 | 25 |  | 3500 | N/A | N/A |  |
|  | 25 | 1885 | 5 | 25 |  | 1965 | M/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2175 | 4.0 | IMD5 |  |
|  | n77 | 3915 | 10 | 25 |  | 3915 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A |  | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n77 | 3720 | 10 | 25 |  | 3720 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A |  | 1940 | 9.1 | IMD411 |  |
|  | 66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
|  | n77 | 3385 | 10 | 25 |  | 3385 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A |  | 1935 | 4.2 | IMD5 |  |
|  | 66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | n77 | 3540 | 10 | 25 |  | 3540 | N/A | N/A |  |
| DC_25A-66A_n78A DC_25A-25A-66A_n78A | 25 | 1880 | 5 | 25 |  | 1960 | M/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2160 | 10.4 | IMD4 |  |
|  | n78 | 3480 | 10 | 50 |  | 3480 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A |  | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n78 | 3700 | 10 | 50 |  | 3700 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A |  | 1960 | 9.1 | IMD4 |  |
|  | 66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
|  | 25 | N/A | 5 | N/A |  | 1980 | 4.2 | IMD5 |  |
|  | 66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
|  | n78 | 3645 | 10 | 25 |  | 3645 | N/A | N/A |  |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 |  | 783 | N/A | N/A |  |
|  | n8 | 910 | 5 | 25 |  | 955 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3458 | 9.1 | IMD4 |  |
|  | 28 | 713 | 5 | 25 |  | 768 | N/A | N/A |  |
|  | n8 | N/A | 5 | N/A |  | 935 | 4.3 | IMD5 |  |
|  | n78 | 3787 | 10 | 50 |  | 3787 | N/A | N/A |  |
| DC_28A-40A_n78A DC_28A-40C_n78A | 28 | N/A | 5 | N/A |  | 800.5 | 11 | IMD3 |  |
|  | 40 | 2302.5 | 5 | 25 |  | 2302.5 | N/A | N/A |  |
|  | n78 | 3795 | 10 | 50 |  | 3795 | N/A | N/A |  |
|  | 28 | 715 | 5 | 25 |  | 770 | N/A | N/A |  |
|  | 40 | N/A | 5 | N/A |  | 2320 | 15.7 | IMD3 |  |
|  | n78 | 3750 | 10 | 50 |  | 3750 | N/A | N/A |  |
| DC_29A-30A_n66A | 29 | N/A | 5 | 25 |  | 719.5 | 4.5 | IMD5 |  |
|  | 30 | 2307.5 | 5 | 25 |  | 2352.5 | N/A | N/A |  |
|  | n66 | 1777.5 | 5 | 25 |  | 2177.5 | N/A | N/A |  |
| DC_29A-30A_n77A | 29 | N/A | 5 | N/A |  | 722 | 15.2 | IMD34 |  |
|  | 30 | 2310 | 5 | 25 |  | 2355 | N/A | N/A |  |
|  | n77 | 3898 | 10 | 50 |  | 3898 | N/A | N/A |  |
| DC_29A-66A_n77A | 29 | N/A | 5 | N/A |  | 722 | 15.2 | IMD311 |  |
| DC_29A-66A-66A_n77A | 66 | 1734 | 5 | 25 |  | 2134 | N/A | N/A |  |
|  | n77 | 4190 | 10 | 50 |  | 4190 | N/A | N/A |  |
| DC_30A-66A_n5A, DC_30A-66A-66A_n5A, DC_30A-66A-66A-66A_n5A | 30 | 2310 | 5 | 25 |  | 2355 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2130 | 2.5 | IMD5 |  |
|  | n5 | 830 | 5 | 25 |  | 875 | N/A | N/A |  |
| DC_30A-66A_n77A DC_30A-66A_n77(2A) | 30 | N/A | 5 | N/A |  | 2355 | 29.2 | IMD211 |  |
| DC_30A-66A-66A_n77A | 66 | 1745 | 5 | 25 |  | 2145 | N/A | N/A |  |
| DC_30A-66A-66A_n77(2A) | n77 | 4100 | 10 | 50 |  | 4100 | N/A | N/A |  |
|  | 30 | N/A | 5 | N/A |  | 2355 | 3.4 | IMD5 |  |
|  | 66 | 1735 | 5 | 25 |  | 2135 | N/A | N/A |  |
|  | n77 | 3780 | 10 | 50 |  | 3780 | N/A | N/A |  |
|  | 30 | 2310 | 5 | 25 |  | 2355 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2160 | 8.7 | IMD411 |  |
|  | n77 | 3390 | 10 | 50 |  | 3390 | N/A | N/A |  |
| DC_38A_n28A-n78A | 38 | 2615 | 5 | 25 |  | 2615 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3360 | 28.2 | IMD29 |  |
|  | 38 | 2615 | 5 | 25 |  | 2615 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 785 | 30.8 | IMD24 |  |
|  | n78 | 3400 | 10 | 25 |  | 3400 | N/A | N/A |  |
| DC_39A_n40A-n79A | 39 | 1917.5 | 5 | 25 |  | 1917.5 | N/A | N/A |  |
|  | n40 | 2302.5 | 5 | 25 |  | 2302.5 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4980 | 5.8 | IMD4 |  |
| DC_39A_n41A-n79A | 39 | 1900 | 5 | 25 |  | 1900 | N/A | N/A |  |
|  | n41 | 2620 | 10 | 50 |  | 2620 | N/A | N/A |  |
|  | n79 | N/A | 40 | N/A |  | 4520 | 29.8 | IMD24 |  |
|  | 39 | 1900 | 5 | 25 |  | 1900 | N/A | N/A |  |
|  | n41 | N/A | 10 | N/A |  | 2620 | 30.2 | IMD24 |  |
|  | n79 | 4520 | 40 | 216 |  | 4520 | N/A | N/A |  |
| DC_40A_n1A-n78A | 40 | 2340 | 5 | 25 |  | 2340 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3450 | 9.8 | IMD4 |  |
|  | 40 | 2360 | 5 | 25 |  | 2360 | N/A | N/A |  |
|  | n1 | N/A | 5 | N/A |  | 2140 | 9.1 | IMD4 |  |
|  | n78 | 3430 | 10 | 50 |  | 3430 | N/A | N/A |  |
| DC_41A_n1A-n77A | 41 | 2650 | 5 | 25 |  | 2650 | N/A | TDD |  |
| DC_41C_n1A-n77A | n1 | 1970 | 5 | 25 |  | 2160 | N/A | FDD |  |
|  | n77 | N/A | 10 | N/A |  | 3330 | 19.6 | TDD |  |
|  | 41 | 2510 | 5 | 25 |  | 2510 | N/A | TDD |  |
|  | n77 | 4150 | 10 | 50 |  | 4150 | N/A | TDD |  |
|  | n1 | N/A | 5 | N/A |  | 2120 | 11.0 | FDD |  |
| DC_41A_n3A-n77A DC_41C_n3A-n77A DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 | 2620 | 5 | 25 |  | 2620 | N/A | N/A |  |
|  | n3 | N/A | 5 | N/A |  | 1840 | 16.4 | IMD3 |  |
|  | n77/n78 | 3400 | 10 | 50 |  | 3400 | N/A | N/A |  |
|  | 41 | 2580 | 5 | 25 |  | 2580 | N/A | N/A |  |
|  | n3 | 1720 | 5 | 25 |  | 1815 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A |  | 3440 | 16.8 | IMD34 |  |
| DC_41A_n28A-n77A DC_41C_n28A-n77A DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | 2580 | 5 | 25 |  | 2580 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 |  | 798 | N/A | N/A |  |
|  | n77/n78 | N/A | 10 | N/A |  | 3323 | 28.2 | IMD21 |  |
|  | 41 | 2642 | 5 | 25 |  | 2642 | N/A | N/A |  |
|  | n28 | N/A | 5 | N/A |  | 798 | 30.8 | IMD21 |  |
|  | n77/n78 | 3440 | 10 | 50 |  | 3440 | N/A | N/A |  |
| DC_46A-48A_n5A5 DC_46C-48A_n5A5 DC_46D-48A_n5A5 DC_46E-48A_n5A5 | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD2, IMD3 |  |
|  | 48 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | n5 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_46A-48A_n66A5 DC_46C-48A_n66A5 DC_46D-48A_n66A5 DC_46E-48A_n66A5 | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD2, IMD3 |  |
|  | 48 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | n66 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_46A-66A_n5A | 46 | N/A | 10 | N/A |  | 5163 | 9.0 | IMD4 |  |
| DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | 66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
|  | n5 | 847 | 5 | 25 |  | 892 | N/A | N/A |  |
| DC_46A-66A_n25A4 DC_46C-66A_n25A4 DC_46D-66A_n25A4 | 46 | N/A | 10 | N/A |  | 5505 | 16.1 | IMD3 |  |
|  | 66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A |  | 1935 | 20 | IMD3 |  |
|  | 46 | N/A | 10 | N/A |  | 5505 | 16.1 | IMD3 |  |
|  | 66 | N/A | 5 | N/A |  | 2150 | 4 | IMD5 |  |
|  | n25 | 1883.3 | 5 | 25 |  | 1963.3 | N/A | N/A |  |
|  | 46 | N/A | 10 | N/A |  | 5505 | 16.1 | IMD3 |  |
|  | 66 | N/A | 5 | N/A |  | 2112.5 | 23 | IMD3 |  |
|  | n25 | 1912.5 | 5 | 25 |  | 1992.5 | N/A | N/A |  |
| DC_46A-66A_n77A5 DC_46A-46A-66A_n77A5 | 46 | N/A | N/A | N/A |  | N/A | N/A | IMD2, IMD3 |  |
|  | 66 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
|  | n77 | N/A | N/A | N/A |  | N/A | N/A | N/A |  |
| DC_48A-(n)12AA | 48 | 3557.5 | 10 | 50 |  | 3557.5 | N/A | N/A |  |
|  | 12 | N/A | 5 | N/A |  | 740.5 | 5.5 | IMD5 |  |
|  | n12 | 705.5 | 5 | 25 |  | 735.5 | 5.5 | IMD5 |  |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | n2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | 48 | N/A | 10 | N/A |  | 3620 | 29.4 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | 48 | 3560 | 5 | 25 |  | 3560 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2155 | 12.1 | IMD4 |  |
|  | n2 | 1905 | 5 | 25 |  | 1985 | N/A | N/A |  |
| DC_48A-66A_n12A | 48 | 3580 | 5 | 25 |  | 3580 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2160 | 17.1 | IMD3 |  |
|  | n12 | 710 | 5 | 25 |  | 740 | N/A | N/A |  |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | 48 | 3630 | 20 | 100 |  | 3630 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2130 | 8.3 | IMD4 |  |
|  | n25 | 1883.3 | 5 | 25 |  | 1963.3 | N/A | N/A |  |
|  | 48 | N/A | 10 | N/A |  | 3620 | 29.4 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n25 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
| DC_48A-66A_n66A DC_48C-66A_n66A | 48 | 3660 | 20 | 100 |  | 3660 | N/A | N/A |  |
| DC_48D-66A_n66A | 66 | N/A | 5 | N/A |  | 2175 | 4.0 | IMD5 |  |
| DC_48E-66A_n66A | n66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
| DC_48A-66A_n71A | 48 | 3560 | 5 | 25 |  | 3560 | N/A | N/A |  |
|  | 66 | N/A | 5 | N/A |  | 2174 | 15.8 | IMD3 |  |
|  | n71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | 48 | N/A | 5 | N/A |  | 3697.5 | 13.0 | IMD4 |  |
|  | 66 | 1712.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
|  | n71 | 665.5 | 5 | 25 |  | 619.5 | N/A | N/A |  |
| DC_66A_n2A-n41A | 66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | n2 | 1860 | 5 | 25 |  | 1940 | 11.0 | IMD4 |  |
|  | n41 | 2685 | 5 | 25 |  | 2685 | N/A | N/A |  |
| DC_66A_n2A-n66A | 66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1935 | 20 | IMD3 |  |
|  | n66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n2 | 1870 | 5 | 25 |  | 1950 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2170 | 4.0 | IMD5 |  |
| DC_66A_n2A-n77A | n2 | N/A | 5 | N/A |  | 1960 | 32.1 | IMD2 |  |
|  | 66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n77 | 3720 | 10 | 50 |  | 3720 | N/A | N/A |  |
| DC_66_n2-n78 | 66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1960 | 32.1 | IMD2 |  |
|  | n78 | 3720 | 10 | 50 |  | 3720 | N/A | N/A |  |
|  | 66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3620 | 34.9 | IMD2 |  |
|  | 66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n2 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3340 | 20.9 | IMD4 |  |
|  | 66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1960 | 21.1 | IMD4 |  |
|  | n78 | 3350 | 10 | 50 |  | 3350 | N/A | N/A |  |
|  | 66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1960 | 2.1 | IMD5 |  |
|  | n78 | 3620 | 10 | 50 |  | 3620 | N/A | N/A |  |
| DC_66A-(n)5AA | 66 | 1721 | 5 | 25 |  | 2121 | N/A | N/A |  |
|  | 5 | N/A | 5 | N/A |  | 878 | 25 | IMD2 |  |
|  | n5 | 838 | 5 | 25 |  | 883 | 30 | IMD2 |  |
| DC_66A_n5A-n48A | 66 | 1750 | 5 | 25 |  | 2150 | N/A | N/A |  |
|  | n5 | 834 | 5 | 25 |  | 879 | N/A | N/A |  |
|  | n48 | N/A | 5 | N/A |  | 3582 | 3.3 | IMD5 |  |
| DC_66A_n5A-n77A | 66 | 1770 | 5 | 25 |  | 2170 | N/A | N/A |  |
|  | n5 | 845 | 5 | 25 |  | 890 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3460 | 16.6 | IMD39 |  |
| DC_66A_n7A-n78A, DC_66A-66A_n7A-n78 DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | 66 | 1730 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | n7 | 2560 | 5 | 25 |  | 2680 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3390 | 16.1 | IMD3 |  |
| DC_66A_n12A-n77A | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n12 | N/A | 5 | N/A |  | 740 | 15.2 | IMD311 |  |
|  | n77 | 4180 | 10 | 50 |  | 4180 | N/A | N/A |  |
|  | 66 | 1723 | 5 | 25 |  | 2123 | N/A | N/A |  |
|  | n12 | 704 | 5 | 25 |  | 734 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 4150 | 16.0 | IMD34,9,11 |  |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 |  | 2115 | N/A | N/A |  |
|  | n41 | 2685 | 10 | 50 |  | 2685 | N/A | N/A |  |
|  | n25 | 1860 | 5 | 25 |  | 1940 | 5 | 11.0 |  |
| DC_66A_n25A-n48A | 66 | 1740 | 5 | 25 |  | 2140 | N/A | N/A |  |
|  | n25 | 1880 | 5 | 25 |  | 1960 | N/A | N/A |  |
|  | n48 | N/A | 10 | N/A |  | 3620 | 29.4 | IMD2 |  |
|  | 66 | 1735 | 5 | 25 |  | 2135 | N/A | N/A |  |
|  | n25 | N/A | 5 | N/A |  | 1960 | 28.3 | IMD2 |  |
|  | n48 | 3695 | 5 | 25 |  | 3695 | N/A | N/A |  |
| DC_66A_n25A-n66A | 66 | 1712.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
|  | n25 | 1912.5 | 5 | 25 |  | 1992.5 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2117.5 | 23 | IMD3 |  |
|  | 66 | 1750 | 5 | 25 |  | 2150 | N/A | N/A |  |
|  | n25 | 1873 | 5 | 25 |  | 1953 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2119 | 4 | IMD5 |  |
| DC_66A_n38A-n78A | 66 | 1760 | 5 | 25 |  | 2160 | N/A | N/A |  |
|  | n38 | 2610 | 10 | 50 |  | 2610 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3460 | 15.0 | IMD3 |  |
| DC_66A_n66A-n71A | 66 | 1752 | 5 | 25 |  | 2152 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2118 | 5.0 | IMD4 |  |
|  | n71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
| DC_66A_n66A-n77A | 66 | 1730 | 5 | 25 |  | 2130 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2170 | 31 | IMD2 |  |
|  | n77 | 3900 | 10 | 50 |  | 3900 | N/A | N/A |  |
| DC_66A_n66A-n78A | 66 | 1775 | 5 | 25 |  | 2175 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2125 | 2.8 | IMD5 |  |
|  | n78 | 3725 | 10 | 50 |  | 3725 | N/A | N/A |  |
| DC_66A-71A_n77A DC_66A-71A_n77(2A) | 66 | N/A | 5 | N/A |  | 2160 | 15.5 | IMD39 |  |
|  | 71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n77 | 3546 | 10 | 50 |  | 3546 | N/A | N/A |  |
|  | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | 71 | N/A | 5 | N/A |  | 640 | 15.3 | IMD311 |  |
|  | n77 | 4080 | 10 | 50 |  | 4080 | N/A | N/A |  |
| DC_66A_n71A-n77A | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n71 | 668 | 5 | 25 |  | 622 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 4108 | 15.9 | IMD34,9,11 |  |
|  | 66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n71 | N/A | 5 | N/A |  | 640 | 15.3 | IMD311 |  |
|  | n77 | 4080 | 10 | 50 |  | 4080 | N/A | N/A |  |
| DC_66A_n71A-n78A | 66 | 1712.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
|  | n71 | 665.5 | 5 | 25 |  | 619.5 | N/A | N/A |  |
|  | n78 | N/A | 5 | N/A |  | 3709 | 13.0 | IMD4 |  |
| DC_71A_n2A-n41A | n2 | 1900 | 5 | 25 |  | 1980 | N/A | N/A |  |
|  | n41 | N/A | 5 | N/A |  | 2586 | 29.2 | IMD2 |  |
|  | 71 | 686 | 5 | 50 |  | 640 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1942 | 26 | IMD2 |  |
|  | n41 | 2610 | 5 | 25 |  | 2610 | N/A | N/A |  |
|  | 71 | 668 | 5 | 25 |  | 622 | N/A | N/A |  |
| DC_71A_n2A-n78A | n2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
|  | 71 | 695.5 | 5 | 25 |  | 649.5 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3305 | 8.0 | IMD3 |  |
|  | n2 | N/A | 5 | N/A |  | 1954 | 16.5 | IMD3 |  |
|  | 71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n78 | 3340 | 10 | 50 |  | 3340 | N/A | N/A |  |
| DC_71A_n2A-n77A | 71 | 695.5 | 5 | 25 |  | 649.5 | N/A | N/A |  |
|  | n2 | 1907.5 | 5 | 25 |  | 1987.5 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 3305 | 8.0 | IMD34,9,11 |  |
|  | 71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n2 | N/A | 5 | N/A |  | 1954 | 16.5 | IMD39,11 |  |
|  | n77 | 3340 | 10 | 50 |  | 3340 | N/A | N/A |  |
| DC_71A_n38A-n78A | 71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n38 | 2615 | 10 | 50 |  | 2615 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3308 | 29.1 | IMD2 |  |
|  | 71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n78 | 3308 | 10 | 50 |  | 3308 | N/A | N/A |  |
|  | n38 | N/A | 10 | N/A |  | 2615 | 28.7 | IMD2 |  |
| DC_71A_n66A-n77A | 71 | 668 | 5 | 25 |  | 622 | N/A | N/A |  |
|  | n66 | 1720 | 5 | 25 |  | 2120 | N/A | N/A |  |
|  | n77 | N/A | 10 | N/A |  | 4108 | 15.9 | IMD34,9,11 |  |
|  | 71 | 690 | 5 | 25 |  | 644 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2150 | 15.5 | IMD39,11 |  |
|  | n77 | 3530 | 10 | 50 |  | 3530 | N/A | N/A |  |
| DC_71A_n66A-n78A | 71 | 693 | 5 | 25 |  | 647 | N/A | N/A |  |
|  | n78 | 3546 | 10 | 50 |  | 3546 | N/A | N/A |  |
|  | n66 | N/A | 5 | N/A |  | 2160 | 15.5 | IMD3 |  |
|  | 71 | 665.5 | 5 | 25 |  | 619.5 | N/A | N/A |  |
|  | n78 | N/A | 10 | N/A |  | 3697.5 | 13.0 | IMD4 |  |
|  | n66 | 1712.5 | 5 | 25 |  | 2112.5 | N/A | N/A |  |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_2A-46A_n77A; DC_2A_n77A; fc_2A + fc_n77A; BW_2A + BW_n77A. DC_2A-46A_n77A; DC_2A_n77A; -fc_2A + 2*fc_n77A; -BW_2A + 2*BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 2*fc_13A + fc_n77A; 2*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 3*fc_13A + fc_n77A; 3*BW_13A + BW_n77A. DC_13A-46A_n2A; DC_13A_n2A; 2*fc_n2A + 2*fc_13A; 2*BW_n2A+2*BW_13A. DC_13A-46A_n77A; DC_13A_n77A; -3*fc_13A + 2*fc_n77A; -3*BW_13A + 2*BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; fc_66A + fc_n77A; BW_66A + BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; -fc_66A + 2*fc_n77A; -BW_66A + 2*BW_n77A. DC_13A-46A_n66A; DC_13A_n66A; 3*fc_13A + fc_n66A; BW_13A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 2*fc_13A + 3*fc_n66A; BW_13A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; fc_48A + fc_n66A; BW_48A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; 2*fc_48A + fc_n66A; 2*BW_48A + BW_n66A. DC_2A-46_n5A; DC_2A_n5A; 2*fc_2A + 2*fc_n5A; BW_2A + 2*BW_n5A. DC_2A-46_n5A; DC_2A_n5A; fc_2A + 4*fc_n5A; BW_2*2A + BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + fc_n5A; BW_48A + 2*BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + 2*fc_n5A; BW_2*48A + BW_n5A. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. NOTE 8: Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 10: The frequency range in band n28 is restricted for this band combination to 728 – 738 MHz for the UL and 783 – 793 MHz for the DL. This band is subject to IMD2 fall in B1 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 12: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 13: Void NOTE 14: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 15: This band is subject to additional IMD3 for which MSD is not specified. NOTE 16: This band is subject to IMD3 also which MSD is not specified. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 – 738 MHz for the UL and 783 – 793 MHz for the DL. NOTE 18: In the MSD test configuration, the IMD center does not fall into the DL victim Fc. NOTE 19: This band is subject to 1st order triple-beat IMD3 where MSD is not specified when the UL configuration includes intra-band uplink CCs. |

---End of changes---