3GPP TSG-RAN WG4 Meeting # 109                                R4-2321912

# Chicago, US, November 13 – 17, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to maintain the Tx RF requirements for ATG UE

Agenda Item:	8.13.3.1

Document for:	Approval

Introduction

It’s proposed to capture update the content for the doppler shift compensation based on RAN2’s latest agreement and remove brackets for configured output power.

- References

[1]

[2]

- Text proposal

- <<Start of Change for TR 38.876>>

- 7.1.2.1 Frequency error

The doppler frequency for ATG UE can be determined by,

fD =  cosθ*vc*f_c

Where,

-	fD is Doppler frequency, f_c is the carrier frequency, which is set as 5GHz for n79.

-	v is the speed of the aircraft which is 1200km/h.

-	θ is the elevation angle between UE and gNB.

Since the potential maximum cell range is up to 200-300km [2] and the normal commercial airplane altitude is 10km, the elevation angle is only about 2~3 degrees when UE doing initial access at cell edge. Then the maximum DL doppler frequency is approaching 5.5kHz and the UL doppler frequency compared to the configured UL central frequency is approaching 5.5kHz if without any compensation which may cause not negligible impact for link performance and access successful rate. ATG UE needs to perform frequency compensation. The frequency accuracy requirement will be written assuming Doppler frequency pre-compensation.

The ATG BS coarse location information captured in new SIB specified in TS 38.331 for ATG could be broadcasted for ATG UE to do frequency compensation if necessary. [UE shall rely on the ATG BS location broadcasted by the IE EphemerisInfo in NTN SIB 19 in 38.331 if pre-compensation is based on SIB19. Where, Ephemeris may be expressed in format of position and velocity state vector].

- <<Next of Change>>

- 7.1.2.4 Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, PMaxOutputPower}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PMaxOutputPower is the maximum UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE;

- <<End of Change>>