3GPP TSG-RAN WG4 Meeting #109                                                               R4-2321921

Chicago, US, November 13 - 17, 2023

Source:	ZTE Corporation

Title:	TP for TR 38.846: On spec structure for inter-band CA configuration tables

Agenda Item:	8.1.3

Document for:	Approval

- Introduction

In current RAN4 specs, the inter-band CA configuration tables have different spec structure in the specifications. For example, in TS 38.101-1 the inter-band CA configurations within FR1 are organized by the band number in the combinations, while in TS 38.101-3 the inter-band CA configurations between FR1 and FR2 are organized in one clause without further categorizing into more sub-clauses. This will lead the side effect that section 5.5A.1 in TS 38.101-3 have as many as 400 pages in one section which makes people too difficult to retrieve a certain CA configuration.

Furthermore, if we look at inter-band CA configuration within FR1, we see that in 5.5A.3.1 the two-band inter-band configuration table has further split the table into many sub-tables, such as Table 5.5A.3.1-1a ~ 5.5A.3.1-1n. But for other configuration tables in 5.5A.3.2 ~ 5.5A.3.5, it has only one big configuration table for each band number category respectively.

It can also be seen that, even like sub-tables used in two-band CA configuration, it is still difficult to retrieve the sub-tables in the sub-clause.

This TP is to collect the rules in TR 38.846 for spec structure for inter-band CA configuration tables.

- Reference

- TR 38.846, Study on simplification of band combination specification for NR and LTE v1.3.0

- Text Proposal

----- Start of TP -----

6.8	Guidelines on configuration tables

6.8.1	CA configuration table

The CA configuration table in TS 38.101-1/-2/-3 provides the information of channel bandwidth, SCS and bandwidth combination set of the bands for each CA configuration.  The uplink CA configuration information is also included in the configuration tables for the allowed UL CA configurations supported by the specification.

For inter-band CA configuration table, considering the huge number of configurations introduced into the spec, the guideline for the spec structure is as below:

-	All combinations having the same number of constituent bands are categorized into one sub-clause, see Fig 6.8.1-1.

-	For two bands inter-band CA configuration table in TS 38.101-1 5.5A.3.1, a “sub-table-group” tag is suggested to be applied for the purpose of easier retrieval, see Fig 6.8.1-2.

-	For the other inter-band CA configuration tables which have huge configurations of more than 50 pages, the big table could be split to a limited number of up to three smaller sub-tables. The sub-tables should not have less than 40 pages and the maximum number should be 3 sub-tables after the split.

Figure 6.8.1-1 Example for sub-clauses for inter-band CA configurations

Figure 6.8.1-2 Example for “sub-table-group” tag for two bands inter-band CA configurations

For CA within FR1 bands, TS 38.101-1 [4] provides the CA configuration tables for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA for NR FR1 bands. The additional information for maximum aggregated bandwidth is set for intra-band contiguous and intra-band non-contiguous CA configuration tables for FR1.

For CA within FR2 bands, TS 38.101-2 [5] provides the CA configuration tables for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA for NR FR2 bands. The additional information for maximum aggregated bandwidth is set for intra-band contiguous and intra-band non-contiguous CA configuration tables in TS 38.101-2 [5]. For intra-band non-contiguous CA, the concept of sub-block is applied to FR2. Sub-blocks belonging to a CA configuration can be in any order. This means a certain CA configuration acronym includes all sub-block arrangements which have exactly the same sub-block set. As an example, CA_n260(2G-3O) denotes CA_n260(2O-2G-O), CA_n260(G-3O-G) etc, but these are not listed in tables separately.

For CA between FR1 and FR2 bands, TS 38.101-3 [6] provides the inter-band CA configuration tables for NR bands between FR1 and FR2.

For the uplink CA configuration, “-” in the configuration table denotes that non-CA operation is supported in this configuration, i.e. only single carrier operation for the constituent band is used for uplink.

For the sake of brevity and to reduce the size of CA configuration tables, instead of showing explicitly in the CA configuration tables, the SCS info for each NR band in the configuration is referred to the channel bandwidths for each NR band in clause 5.3.5 of TS 38.101-1 [4] and TS 38.101-2 [5]. For configurations including intra-band contiguous part, the detail configuration for this part is referred to the corresponding intra-band contiguous CA configuration table. Examples:

For NR inter-band CA configuration with two bands in FR1, Table 6.8.1-1 illustrates that,

-	CA_n1A-n3A consists of two NR bands n1 and n3 whose SCS values are defined in Table 6.8.1-2. For example, for NR band n1, the supported channel bandwidth in BCS0 is 5MHz, 10MHz, 15MHz and 20MHz where channel bandwidth 5MHz supports SCS with only 15kHz, channel bandwidths 10MHz, 15MHz and 20MHz support all SCS of {15kHz, 30kHz, 60kHz}.

-	CA_n1B-n3A having intra-band contiguous part CA_n1B, the configuration of band n1 for the corresponding CA part is referred to CA_n1B_BCS0 defined in intra-band contiguous CA configuration table.

-	CA_n2A-n66A consists of two BCSs. The UL CA configurations denote the allowed UL CA configurations supported by the specification. For BCS0, the uplink configuration “-” indicates that non-CA operation is supported and only single carrier operation is used in uplink. For BCS1, the uplink configuration supports CA configuration CA_n2A-n66A.

-	For some configurations, there are regional spectrum limitations to the corresponding bands and the notes can be found in the configuration table, such as for CA_n2A-n48A_BCS0, the channel bandwidths 50MHz, 60MHz, 80MHz, 90MHz and 100MHz are applicable only to downlink.

For channel bandwidth per operating band defined in clause 5.3.5 of TS 38.101-1/-2 and TS 38.104, Table 6.8.1-2 illustrates that,

-	The requirements for each configuration should be complied with the combination of channel bandwidths, SCS for each operating band defined in the table.

-	For some bands the limitations to the bandwidth may be captured with notes in the table, such as for NR band n48, the channel bandwidth 5MHz is restricted to operation when carrier is configured as an SCell part of DC or CA configuration.

Table 6.8.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
| CA_n1B-n3A | CA_n1A-n3A | n1 | CA_n1B_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | CA_n1B_BCS0 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| … | … | … | … | … |
| CA_n2A-n48A | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701, 801, 901, 1001 |  |
| … | … | … | … | … |
| CA_n2A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| … | … | … | … | … |
| NOTE 1: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |

Table 6.8.1-2: Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n1 | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n2 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |
| n3 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
| … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … |
| n41 | 15 | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … |
| n48 | 15 | 55 | 10 | 15 | 20 |  | 30 |  | 40 |  | 506 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 506 | 606 | 706 | 806 | 906 | 1006 |
|  | 60 |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 506 | 606 | 706 | 806 | 906 | 1006 |
| … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … |
| n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |
| … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … |
| n78 | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
| … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … |
| n80 | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |
| … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … |
| n86 | 15 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
| … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … | … |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-720.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: Void. NOTE 10: For this band, UE channel bandwidths which are applicable to sidelink operation are specified in Table 5.3E.1-1. NOTE 11: Not all frequency positions of 5 MHz carriers are possible due limitations of the SSB position relative to the 5 MHz channels. 5 MHz channels with Fc such that 2499+N*1.2 ≤Fc<2499.3+N*1.2MHz for 0≤N<157 are not compatible with SSB positions and cannot be used for 5 MHz n41. NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

----- End of TP -----