3GPP TSG-RAN4 Meeting #109	 R4-2321923

# Chicago, USA, November 13 –17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0147 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for adding RedCap UE for release independent feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FDD_bands_R18_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additonal band to support RedCap UE is added in Rel-18 and RedCap is agreed to be one release independent feature. The general rule in 38.107 is “The latest release of release independent spec refers to the release which the new feature is introduced in. If an RF feature introduced in the same release as the release which the feature is independent from, (i.e. M=N), the common UE RF requirements table in annex B.4 is specified from release N+1, otherwise the common UE RF requirements table is specified from release N.”, in this CR, the interpretation of this rule is that this additional band for RedCap feature is Rel-18 feature as it is Rel-18 WI, and as such, the common UE RF requirement table is specified from release Rel-17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the release independent clause for RedCap |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap is not release independent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- START CHANGE ------------------------------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BW	Bandwidth

CA	Carrier Aggregation

CC	Component carrier

DL	Downlink

EN-DC	Dual connectivity between E-UTRA and NR

FDD	Frequency Division Duplex

FR1	Frequency range 1

FR2	Frequency range 2

NR	New radio

RedCap	Reduced Capability

REL	Release

SDL	Supplementary downlink

SUL	Supplementary uplink

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

------------------------------------------------------------- NEXT OF CHANGES ------------------------------------------------------

## 5.9	Additional operating bands for Redcap for NR frequency range 1

Requirements for a Rel-18 Redcap UE for additional operating bands within FR1 compared to TS 38.101-1 of Rel-18 [2] are introduced via this clause.

Table 5.9-1: NR RedCap operating band

| Feature | Duplex-mode | Release independent from |  | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- | --- |
| Redcap operating band | FDD | Rel-17 |  | Table B.4.14-1 |

------------------------------------------------------------- NEXT OF CHANGES ------------------------------------------------------

## B.4.14	Common UE RF requirements for RedCap

The requirements and test cases listed in Table B.4.14-1 are specified in REL-18 version of TS 38.101-1 [2].

Table B.4.14-1: Common UE RF requirements for RedCap UE in FR1

| Clause | Description |
| --- | --- |
| 5.3 | UE channel bandwidth |
| 5.3I | Channel bandwidth for RedCap |
| 5.4 | Channel arrangement |
| 6.1 | General |
| 6.2 | Transmitter power |
| 6.2I | Transmitter power for RedCap |
| 6.3 | Output power dynamics |
| 6.4 | Transmit signal quality |
| 6.5 | Output RF spectrum emissions |
| 7.1 | General |
| 7.1I | General |
| 7.2 | Diversity characteristics |
| 7.3I | Reference sensitivity for RedCap |
| 7.4 | Maximum input level |
| 7.5 | Adjacent channel selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------