# 3GPP TSG-RAN WG4 Meeting # 109		                           							R4-2321925

# Incheon, KR, May 22 – May 26, 2023

Source:	Huawei, HiSilicon, Skyworks Solutions Inc

Title:	TP for TR 38.718-02-01 to introduce CA_n34A-n40A_BCS4 and 5 with MSD analysis

Agenda item:	7.1.1.2

Document for:	Approval

1 Background

This contribution provides some technical text proposals on the introduction BCS4 and BCS5 of the NR CA band combination CA_n34-n40 based on WF [1] as this combo was captured in Revised WID: NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1,2) [2].

2 Text Proposal

---Start of changes---

5.X		CA_n34-n40

5.X.1	Common for 1 band UL and 2 bands UL CA

5.X.1.1 Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n34+n40

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

5.X.1.2	Channel bandwidths per operating band for CA

Table 5.14.1.2-1: Supported bandwidths per CA band combination of band n78+n79

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n34A-n40A | CA_n34A-n40A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |

5.X.1.3	REFSENS exception requirements

The MSD test point due to cross band isolation are introduced as below for BCS4 and 5 cases.

Table 5.X.1.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n40 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2302.5 | 5 | 6 | >ACLR2 |
| n40 | n34 | 2350 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 17.9 | >ACLR2 |

---End of changes---

Reference

[1] R4-2317657, WF on CA_n34-n40 MSD, Skyworks Solutions Inc, Huawei

[2]	RP-231840, “Revised WID: Rel-18 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation