3GPP TSG-RAN WG4 Meeting #109	R4-2321926

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1948 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for introducing NR-U uplink CA for NS_53 and NS_54 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Charter Communications, Inc., Skyworks Solutions, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic_enh-Core. |  | Date: |  |  | 2023-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce additional requirements of intra-band contiguous CA operation in shared spectrum channel access in order to use already added band combinations in the specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing additional requirements for intra-band contiguous CA operation in shared spectrum channel access and A-MPR table for bandwidth classes B and C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some band combinations already in the specifications do not have their corresponding maximum output power reduction and therefore they cannot be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.1A.2.1, 6.2F.1A.2.2, 6.2F.3A.2, 6.2F.3A.2.1, 6.2F.3A.2.2, 6.2F.3A.2.3, 6.5F.3A, 6.5F.3A.0, 6.5F.3A.1, 6.5F.3A.2, 6.5F.3A.3, 6.5F.3A.3.0, 6.5F.3A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

#### 6.2F.1A.2	UE maximum output power for intra-band contiguous CA

For uplink intra-band contiguous carrier aggregation, the maximum output power is specified in Table 6.2F.1A.2-1. For downlink intra-band contiguous carrier aggregation with a single uplink component carrier configured in the NR-U band, the maximum output power is specified in Table 6.2F.1-1 for power class 5.

Table 6.2F.1A.2-1: UE Power Class for intra-band contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n96B |  |  |  |  |  |  | 20 | +2/-3 |
| CA_n96C |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

##### 6.2F.1A.2.1	Additional requirements for transmit power density for intra-band contiguous CA for CA_NS_53

The UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1A.2.1-1 when CA_NS_53 is signalled.

Table 6.2F.1A.2.1-1: Additional requirements for transmit power density for CA_NS_53

| NR CA Band | NS value | Aggregated channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| CA_n96 | CA_NS_53 | 40, 60, 80, 100, 120, 140, 160 | 5925 – 7125 | -1 |

##### 6.2F.1A.2.2	Additional requirements for transmit power density for intra-band contiguous CA for CA_NS_54

The UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1A.2.2-1 when CA_NS_54 is signalled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Table 6.2F.1A.2.2-1: Additional requirements for transmit power density for CA_NS_54

| NR CA Band | NS value | Aggregated channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| CA_n96 | CA_NS_54 | 40, 60, 80, 100, 120, 140, 160 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

<<< Next changed section >>>

### 6.2F.3A	UE additional maximum output power reduction for CA

#### 6.2F.3A.1	UE additional maximum output power reduction for inter-band CA

For inter-band carrier aggregation with uplink assigned to two bands and including one of the bands listed in Table 6.2F.1-1, the requirements in clause 6.2.3 apply for the NR uplink carrier and clause 6.2F.3 for the carrier operating with shared spectrum access.

#### 6.2F.3A.2	 UE additional maximum output power reduction for Intra-band CA

##### 6.2F.3A.2.1	UE additional maximum output power reduction for Intra-band contiguous CA

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7]. Relation between NR CA band and NR frequency band is specified in Table 5.2A.1-1.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2F.1A.2-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2F.2A.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2F.3A.2.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NS value. The CA_NS_xy value indicates the additional unwanted emissions requirements that apply for intra-band contiguous CA bands with NS_xy indicated or configured in multiple uplinks serving cells, except CA_NS_01 that indicates the general emission requirements for intra-band contiguous CA bands. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2F.3A.2.1-2. For any NR CA band not listed in Table 6.2F.3A.2.1-2 the network signalling label CA_NS_01 applies.

Table 6.2F.3A.2.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR CA Band | Aggregated channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_53 | 6.5F.3A.3.1 6.2F.1A.2.1 | CA_n96 | 40, 60, 80, 100, 120, 140, 160 |  | 6.2F.3A.2.2 |
| CA_NS_54 | 6.5F.3A.3.1 6.2F.1A.2.2 | CA_n96 | 40, 60, 80, 100, 120, 140, 160 |  | 6.2F.3A.2.3 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

Table 6.2F.3A.2.1-2: Mapping of network signalling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n96 | CA_NS_01 | CA_NS_53 | CA_NS_54 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

6.2F.3A.2.2 	A-MPR for CA_NS_53

### When "CA_NS_53" is indicated in the cell, the A-MPR is specified in Table 6.2F.3A.2.2-1.

Table 6.2F.3A.2.2-1: A-MPR for CA_NS_53 power class 5

| Pre-coding | Modulation | Bandwidth of contiguously transmitted sub-bands / RB Allocation / A-MPR (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  | 120MHz |  | 140MHz |  |  | 160MHz |  |  |
|  |  | Full | Partial | Full | Partial | Full | Partial | Full | Partial | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| DFT-s-OFDM | PI/2 BPSK2 | ≤ 9.0 | ##### ≤ 11.5 | ##### ≤ 5.5 | ≤ 8.5 | ##### ≤ 4.0 | ##### ≤ 6.5 | ##### ≤ 3.5 | ##### ≤ 5.5 | ##### ≤ 3.5 | ##### ≤ 5.0 | ##### ≤ 3.5 | ##### ≤ 4.0 | ##### ≤ 3.5 | ##### ≤ 4.0 | ##### ≤ 3.5 | ##### ≤ 4.0 |
|  | QPSK | ##### ≤ 9.0 | ##### ≤ 11.5 | ##### ≤ 5.5 | ##### ≤ 8.5 | ##### ≤ 4.0 | ##### ≤ 6.5 | ##### ≤ 3.5 | ##### ≤ 5.5 | ##### ≤ 3.5 | ##### ≤ 5.0 | ##### ≤ 3.5 | ##### ≤ 4.0 | ##### ≤ 3.5 | ##### ≤ 4.0 | ##### ≤ 3.5 | ##### ≤ 4.0 |
|  | 16 QAM | ##### ≤ 9.0 | ##### ≤ 11.5 | ##### ≤ 5.5 | ##### ≤ 8.5 | ##### ≤ 4.0 | ##### ≤ 6.5 | ##### ≤ 3.5 | ##### ≤ 5.5 | ##### ≤ 3.5 | ##### ≤ 5.0 | ##### ≤ 3.5 | ##### ≤ 4.5 | ##### ≤ 3.5 | ##### ≤ 4.0 | ##### ≤ 3.5 | ##### ≤ 4.0 |
|  | 64 QAM | ##### ≤ 9.0 | ##### ≤ 11.5 | ##### ≤ 5.5 | ##### ≤ 8.5 | ##### ≤ 4.5 | ##### ≤ 6.5 | ##### ≤ 4.5 | ##### ≤ 5.5 | ##### ≤ 4.5 | ##### ≤ 5.0 | ##### ≤ 4.5 | ##### ≤ 4.5 | ##### ≤ 4.5 | ##### ≤ 4.5 | ##### ≤ 4.5 | ##### ≤ 4.5 |
|  | 256 QAM | ##### ≤ 9.0 | ##### ≤ 11.5 | ##### ≤ 6.0 | ##### ≤ 8.5 | ##### ≤ 6.0 | ##### ≤ 6.5 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 11.0 | ≤ 5.5 | ≤ 8.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 |  | ≤ 5.0 |  |
|  | 16 QAM | ≤ 9.0 | ≤ 11.0 | ≤ 5.5 | ≤ 8.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 |
|  | 64 QAM | ##### ≤ 9.0 | ##### ≤ 11.0 | ##### ≤ 6.0 | ##### ≤ 8.5 | ##### ≤ 6.0 | ##### ≤ 6.5 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 | ##### ≤ 6.0 |
|  | 256 QAM | ##### ≤ 9.0 | ##### ≤ 11.0 | ##### ≤ 7.0 | ##### ≤ 8.5 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 | ##### ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.2F.3A.2.3 	A-MPR for CA_NS_54

When “CA_NS_54” is indicated in the cell, the A-MPR is specified in Table 6.2F.3A.2.3-1.

Table 6.2F.3A.2.3-1: A-MPR for CA_NS_54 power class 5

| Pre-coding | Modulation | Bandwidth of contiguously transmitted sub-bands / RB Allocation / A-MPR (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RB Allocation (Note 4) |  | 40MHz – 80MHz (Note 5) |  | 100MHz – 120MHz (Note 5) |  | 140MHz – 160MHz (Note 5) |  |
|  |  | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 |
| DFT-s-OFDM |  | PI/2 BPSK7 |  | ≤ 3.0 |  |  | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 3.0 |  | ≤ 4.0 |  | ≤ 3.0 |  | ≤ 3.5 |  |
|  |  | QPSK |  | ≤ 3.0 |  |  | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 3.0 |  | ≤ 4.0 |  | ≤ 3.0 |  | ≤ 3.5 |  |
|  |  | 16 QAM |  | ≤ 3.5 |  |  | ≤ 4.0 |  | ≤ 4.5 |  | ≤ 3.5 |  | ≤ 4.0 |  | ≤ 3.5 |  | ≤ 3.5 |  |
|  |  | 64 QAM |  | ≤ 4.5 |  |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  |
|  |  | 256 QAM |  | ≤ 6.0 |  |  | ≤ 6.0 |  | ≤ 6.0 |  | ≤ 6.0 |  | ≤ 6.0 |  | ≤ 6.0 |  | ≤ 6.0 |  |
| CP-OFDM |  | QPSK |  | ≤ 4.0 |  |  | ≤ 4.5 |  | ≤ 5.0 |  | ≤ 4.0 |  | ≤ 4.5 |  | ≤ 4.0 |  | ≤ 4.0 |  |
|  |  | 16 QAM |  | ≤ 4.5 |  |  | ≤ 4.5 |  | ≤ 5.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.0 |  |
|  |  | 64 QAM |  | ≤ 6.0 |  |  | ≤ 5.5 |  | ≤ 6.0 |  | ≤ 6.0 |  | ≤ 6.0 |  | ≤ 6.0 |  | ≤ 6.0 |  |
|  |  | 256 QAM |  | ≤ 7.0 |  |  | ≤ 7.0 |  | ≤ 7.0 |  | ≤ 7.0 |  | ≤ 7.0 |  | ≤ 7.0 |  | ≤ 7.0 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated. NOTE 2: The A-MPR for Full allocation applies to all RBs in all contiguously transmitted sub-bands for operation that are fully allocated. NOTE 3: The A-MPR for Partial RB allocation applies to all contiguously transmitted sub-bands with interlaced allocations with uplink resource allocation type 2 as specified in TS38.214 [10]. NOTE 4: Applicable for all valid channels and bandwidth of contiguously transmitted sub-bands other than those enumerated in NOTE 5. NOTE 5: Applicable to aggregated channel’s lower edge at 5945MHz. NOTE 6: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |

<<< Next changed section >>>

##### 6.5F.3.3.8	Requirements for network signalling value “NS_63” or “NS_69”

When "NS_63" or “NS_69” is indicated in the cell, the power of any UE emission for channels assigned within 5945-6425 MHz shall not exceed the levels specified in Table 6.5F.3.3.8-1. These requirements also apply for frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.8-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| 6425 ≤ f ≤ X | -13 | 1 MHz |
| f ≥ X | -19 | 1 MHz |
| Note 1: In case of NS_63 the parameter X is defined as 6435.9MHz for 20MHz channel, 6440.1MHz for 40MHz channel, and 6440.4MHz for 80MHz. Note 2: In case of NS_69 the parameter X is defined as 6425.5MHz for 20MHz channel, 6425.4MHz for 40MHz channel, and 6425.2MHz for 80MHz. |  |  |

ACLR is specified for the first adjacent channel (ACLR1) which centre frequency is ±CBW from assigned channel centre and for the 2nd adjacent channel (ACLR2) which centre frequency is ±2*CBW from assigned channel centre. The assigned channel power and ACLR1/ACLR2 are measured with rectangular filters with measurement bandwidth of CBW.

Instead of the general ACLR requirement in clause 6.5.2.4 and 6.5F.2.4.1, if the measured adjacent channel power is greater than –47 dBm then the ACLR shall be higher than the value specified in Table 6.5F.3.3.10-2.

Table 6.5F.3.3.10-2: Shared spectrum channel access ACLR requirement

|  | ACLR1 | ACLR2 |
| --- | --- | --- |
| Spectrum emission limit (dB) | 25 | 40 |

##### 6.5F.3.3.9	Requirements for network signalling value “NS_64”

When "NS_64" is indicated in the cell, the power of any UE emission for channels assigned within 5945-6425 MHz shall not exceed the levels specified in Table 6.5F.3.3.9-1. These requirements also apply for frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.9-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 694 | -54 | 100 kHz |
| f ≤ 5935 | -45 | 1 MHz |

### 6.5F.3A	Spurious emissions for CA

#### 6.5F.3A.0	General

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

#### 6.5F.3A.1	General spurious emissions

For intra-band contiguous carrier aggregation, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5F.3A.1-1 from the edge of the aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB as specified in Table 6.5F.3A.1-1 the spurious emission requirements in Table 6.5.3.1-2 are applicable.

Table 6.5F.3A.1-1: Boundary between out of band and spurious emission domain for intra-band contiguous carrier aggregation

| Aggregated channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel_CA | BWChannel_CA + 5 |

#### 6.5F.3A.2	Spurious emissions for UE co-existence

Spurious emissions requirements for UE coexistence are not applicable to bands restricted to stand-alone operation with shared spectrum channel access as identified in Table 5.2-1.

#### 6.5F.3A.3	Additional spurious emissions

##### 6.5F.3A.3.0	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

##### 6.5F.3A.3.1	Requirements for network signalling value "CA_NS_53" or "CA_NS_54"

When "CA_NS_53" or "CA_NS_54" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3A.3.1-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5F.3A.3.1-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

<<< End of changed sections >>>