3GPP TSG-RAN WG4 Meeting #109    	R4-2321928

Chicago, US, 13 – 17 November, 2023

Title:	WF on nominal channel spacing for NR-U

Agenda item:	7.1.2

Source:	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

For NR-U intra-band contiguous CA, a way forward (WF) was assigned during the Ad-Hoc meeting [1] for [109][104], to study the observations of discussion paper R4-2321677 [2], i.e.:

- for some intra-band CA configurations, the CC channel spacing may exceed the nominal channel spacing (NCS) defined in clause 5.4.4. For example, it was observed that for the case of 20MHz+20MHz CA, the NCS may be exceeded by 60kHz for band n96 [2],

- According to clause 5.4.4,

  - a CC spacing greater than NCS is allowed only for non-contiguous intra-band CA,

  - for intra-band contiguous CA, it is allowed to adjust the CC channel spacing to a value <= NCS.

It is likely that for other CA configurations, the NCS excess may reach up to [240kHz] for the example of 60MHz+20MHz. It was also observed in [2] that this issue applies equally to downlink CA and to uplink CA.

These deviations may occur because the list of NR-ARFCN for NR-U was designed to fulfil three challenging criteria:

- the NR-U channels need to align with WiFi channels,

- the NR-U channels need to meet the intra-band CA NCS,

- the NR-U channels need to enable wideband operation.

In this respect, NR-U has constraints on NR-ARFCN channels that may require exceptions to requirements of clause 5.4.4. The following two solutions were briefly discussed in the ad-hoc:

In clause 5.4.4, consider either of the following options as exceptions applicable only to NR-U intra-band CA operation:

Option 1:

As exception

- for NR-U intra-band contiguous CA, it is allowed to exceed the nominal channel spacing (NCS) by [TBD] kHz, i.e. CC channel spacing <= NCS+[TBD]kHz.

- For CC channel spacing > NCS + [TBD] kHz, the configuration is a non-contiguous intra-band CA configuration.

The excess may be expressed in units of common largest Mu (numerology) or in kHz.

Option 2: As exception for NR-U intra-band CA:

For

CC channel spacing >= NCS+ smallest supported CBW specified in Table 5.3.5-1,

- the configuration is a non-contiguous intra-band CA configuration.

For

CC channel spacing > NCS

AND

CC channel spacing < NCS + smallest supported CBW specified in Table 5.3.5-1,

- the configuration is a contiguous intra-band CA configuration.

# 2 	Way forward

- Interested companies are invited to study ways to address NR-U exceptions to the nominal channel spacing requirements for intra-band CA.

# 3 	Reference

[1]	R4-2321924	Ad hoc minutes on NR_Baskets_Part_1, 3GPP TSG-RAN WG4 Meeting #109, Chicago, US, 13 – 17 November, 2023, Skyworks Solutions, Inc.

[2]		R4-2321677	On nominal channel spacing for NR-U, 3GPP TSG-RAN WG4 Meeting #109, Chicago, US, 13 – 17 November, 2023, Skyworks Solutions, Inc.