3GPP TSG-RAN WG4 Meeting #109	R4-2321929

# Chicago, US, November 13 – 17, 2023

Source: 	Huawei, HiSilicon, Skyworks Solutions

Title: 	WF on clarifying the single UL configuration for NR CA

Agenda Item:	7.1.2

Document for:	Approval

- Way forward

Based on [1,2], it’s proposed to improve the indication about single UL configurations for NR CA band combinations.

Proposal:

Introduce the following new text in the core requirements of clause 5.5A.0 in TS38.101-1.

In the CA configuration tables of clause 5.5A.1 and clause 5.5A.2:

- UL CA configuration entries with "-" mean singe UL carrier is valid for intra-band CA

"In the CA configuration tables of clause 5.5A.3:

- UL CA configuration entries with "-" mean that any constituent band of the inter-band DL CA combination can be configured as a valid single uplink carrier

- No other single uplink carrier configurations than those specified are valid UL configurations

- If an Uplink CA configuration is supported, its fallback single uplink also is supported

- Unless otherwise noted, the default power class applies to all valid single uplink carriers or uplink combination(s) specified configurations. The default power class is:

  - PC5 for shared spectrum channel access frequency bands,

  - PC3 for any NR FR1 band other than shared spectrum channel access,

  - PC3 for intra-band UL CA and inter-band UL CA configurations".

Reference

[1] R4-2316773, Resolving valid 1 UL carrier configurations for DL CA_n7-n38 CA_n5-n8, Skyworks Solutions, RAN4#108bis, October, 2023

[2] R4-2318506, Discussion on clarifying the 1 UL configuration for NR CA,Huawei, HiSilicon RAN4#109, November, 2023