3GPP TSG-RAN WG4 Meeting #109	R4-2321930

Chicago, Nov 13th – Nov 17th , 2023

Title: 	WF on CA_n78A-n104A

Agenda Item:	7.1.1.2

Source: 	Qualcomm France

Document for:	Approval

# TX/RX concurrency

Based on the Ad-Hoc discussion both simultaneous TX/RX and non-simultaneous TX/RX were briefly discussed. It was commented by one of the proponent operators that simultaneous TX/RX is more preferred option.

Potential RF performance impacts, i.e. UL harmonics, Harmonic mixing, and Cross-band MSD are present in simultaneous TX/RX so they should be analysed. Companies should also evaluate ΔTIB and ΔRIB impacts using different RF filtering attenuation assumptions and the difference between Simultaneous and non-simultaneous TX/RX.

<Way forward>:

- Companies are encouraged to provide analysis for Harmonic MSD, Harmonic Mixing MSD, and Cross-band MSD for simultaneous TX/RX

- Companies are encouraged to evaluate ΔTIB and ΔRIB impacts using different RF filtering attenuation assumptions and the ΔTIB and ΔRIB difference between Simultaneous and non-simultaneous TX/RX

# RF filter frequency ranges

The frequency range for n77 is 3300-3800MHz. n78 is widely supported NR band and it is co-banded with n77 whose frequency range is 3300-4200MHz. Thus it is practical to assume that for the analysis the n78 RF filter frequency range is 3300-4200 MHz.

The frequency range of n104 is 6425-7125MHz. There are other bands around same frequencies i.e. n46 whose frequency range is 5150-5925MHz, n96 whose frequency range is 5925-7125MHz, and n102 whose frequency range is 5925-6425MHz. To address different implementation options, it is suggested to evaluate three frequency ranges for n104 RF filter.

<Way forward>:

- n78 RF filter frequency range is 3300-4200 MHz in this analysis

- n104 RF filter frequency range has three alternatives in this analysis

  - 5150-7125 MHz

  - 5925-7125 MHz

  - 6425-7125 MHz

- Companies are encouraged to also discuss potential limitations in features and use cases related to antenna and RF front end architectures