3GPP TSG-RAN4 Meeting #109	R4-2321952

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0047 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_NTN_solutions-Core] CR for 38.101-5 to update the clause of Transmit modulation quality (R17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Phase continuity requirements for DMRS bundling is newly added in Rel-18, but it can’t be supported for NTN UE in Rel-17. The original wordings didn’t reflect the current situation. The content in clause 6.4.2.5 from TS 38.101-1 should not be captured in Rel-17 TS 38.101-5 spec. - The restriction about Pi/2 BPSK should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Phase continuity requirements for DMRS bundling is newly added in Rel-18, but it can’t be supported for NTN UE in Rel-17. The original wordings didn’t reflect the current situation. The content in clause 6.4.2.5 from TS 38.101-1 should not be captured in Rel-17 TS 38.101-5 spec. - The restriction about Pi/2 BPSK is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current Phase continuity requirements specified in clause 6.4.2.5 of TS 38.101-1 shouldn’t have been applicable to NTN UE in Rel-17 TS 38.101-5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-5>>

6.4.2	Transmit modulation quality

The requirements for transmit modulation quality defined in 3GPP TS 38.101-1 [5] clause 6.4.2 shall apply for NTN satellite UE except for the Phase continuity requirements for DMRS bundling specified in clause 6.4.2.5 of TS 38.101-1 [5].

<<End of Change>>