# 3GPP TSG-RAN WG4 Meeting # 109				R4-2321956

# Chicago, US, November 13 – 17, 2023

Agenda item:	10

Source: 	CMCC

Title: 	Rel-18 RAN4 UE feature list for NR (version 2)

Document for:	Approval

- Introduction

This contribution includes the RAN4 UE feature list for Rel-18 NR. The previous Rel-18 RAN4 UE feature list is R4-2321797.

- NR_ENDC_RF_FR1_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_ENDC_RF_FR1_enh2 | [27-1] | TxDiversity for 4Tx | Indicates UE supports Tx diversity for 4Tx for the band configured. |  | Yes | N/A | UE doesn’t support 4Tx Tx diversity for the band configured | Per FS | No | FR1 only | N/A |  | Optional with capability signalling |
| 27. NR_ENDC_RF_FR1_enh2 | 27-2 | LowerMSD for inter-band NR CA and EN-DC | Capability to indicate better MSD performance than the specified minimum requirements. [The essential information of this capability includes: - victim band and aggressor band(s) of the band combination - MSD type - Lower-MSD capability class - power class] |  | Yes |  | The UE shall comply with the minimum requirements for MSD. | Per band | No | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |

- NR_channel_raster_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_channel_raster_enh | 28-1 | Enhanced channel raster | The UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1, 38.101-4, TS 38.101-5 [and in TS38.133] | N/A | Yes |  | [N/A (not defined)] | Per Band | No | FR1 only | The feature is supported for applicable bands in FDD-TDD and FR1/FR2 combinations | Applies only for bands with a 100 kHz channel raster for both TN and NTN. | FFS |

- NR_RF_FR2_req_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_RF_FR2_req_Ph3 | 29-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_FR2_multiRX_DL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_RRM_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | Enhanced L3 measurement reporting for unknown SCell activation if the valid L3 measurement results are available | Support of reporting valid L3 measurement results triggered by the SCell activation command |  | Yes | N/A | UE does not support reporting valid L3 measurement results triggered by SCell activation command | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported, including single SCell activation, single PUCCH SCell activation, and multiple SCell activation with/without PUCCH SCell. | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-2 | Beam sweeping factor reduction for FR2 unknown SCell activation | Support of reducing beam sweeping factor for cell detection if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure Support of reducing beam sweeping factor for SSB based L1-RSRP measurement if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure |  | Yes | N/A | UE does not support beam sweeping factor reduction for cell detection during FR2-1 unknown SCell activation. UE does not support beam sweeping factor reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation. | Per Band | TDD onl | FR2-1 only | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. Candidate values for beam sweeping reduction for cell detection during FR2-1 unknown SCell activation are 1,2,4, or 6. [Agreed in WF R4-2310081] Candidate values for beam sweeping reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation are 0,1,2,3,4,5,6, or 7. [Agreed in WF R4-2310081] | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-3 | Shorter measurement interval for unknown SCell activation | (1) Support of using SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in measurement object for enhanced unknown SCell activation requirement. (2) Support of performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation |  | Yes | N/A | UE does not use SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in MO for enhanced unknown Scell activation requirement. UE does not support performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signaling |

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NonCol_intraB_ENDC_NR_CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NonCol_intraB_ENDC_NR_CA | 33-1 | Support of intra-band non-collocated NR CA operation | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 [2]. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 [2]. |  | Yes | N/A | Intra-band non-collocated NR CA operation is not supported. The UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1. | Per BC | N/A | FR1 only | N/A | Supported for band n77/n78 only | Optional with capability signaling |
| 33. NonCol_intraB_ENDC_NR_CA | 33-2 | Support network control of requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 | For Rel-18, introduce a new UE capability which indicates support network control of requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16.. | 2-19 (Rel-16 RAN4 feature) | Yes | N/A | The UE will support “interBandMRDC-WithOverlapDL-Bands-r16” only which means in Rel-18 the network can’t enforce modification on the UE types | [per UE] | N/A | FR1 Only | N/A |  | Optional with capability signalling |

- NR_HST_FR2_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34.NR_HST_FR2_enh | 34-1 | Support of NR FR2 HST with simultaneous DL reception with [two different QCL TypeD RSs] | [1) Support of enhanced RF requirement to support FR2-1 PC6 UEs with simultaneous DL reception with two different QCL TypeD RSs] 2) Support of enhanced RRM requirement to support FR2-1 PC6 UEs with simultaneous DL reception with [two different QCL TypeD RSs] [3) Support of enhanced demodulation processing to support FR2-1 PC6 UEs with simultaneous DL reception with two different QCL TypeD RSs] | 22-1, [16-2c] | Yes | N/A | UE does not support FR2 high speed train scenario with simultaneous DL reception with [two different QCL TypeD RSs] | [Per Band] | N/A No | FR2 only | N/A | FFS how to give the condition of bi-directional deployment | Optional with capability signaling |
| 34.NR_HST_FR2_enh | 34-2 | Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode | [1) Support of the RRM requirement for intra-band CA in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 2) Support of the RRM requirement for inter-frequency measurements in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133] | [22-1] | [Yes] | [N/A] | [The performance of intra-frequency measurement on SCC and/or inter-frequency measurements in connected mode for NR FR2 HST scenario cannot be guaranteed] | [Per UE] | No | FR2 only | N/A |  | Optional with capability signaling |
| 34.NR_HST_FR2_enh | 34-3 | Enhanced FR2 HST RRM requirements for inter-frequency measurement in Idle and Inactive mode | [Support of the RRM requirement for inter-frequency measurements in idle and Inactive mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133] | [22-1] | [No] | [N/A] | [The performance of inter-frequency measurement in idle and Inactive mode for FR2 HST scenario cannot be guaranteed] | [Per UE] | No | FR2 only | N/A |  | Optional without capability signaling |
| 34.NR_HST_FR2_enh | 34-4 | [Support of enhanced MAC CE for TCI state switch indication for FR2 HST] | [1. Support of enhanced large one-shot UL transmit timing adjustment as specified in TS 38.133 based on the new MAC CE named as [TBA] 2. Support of Power Class 6 UE requirements for TCI state switching delay requirement as specified in TS 38.133 based on the new MAC CE named as [TBA] ] | FFS: Option 1: [22-2] Option 2: [22-1] | Yes | N/A | [UE does not support enhanced MAC CE for TCI state switch indication for FR2 HST] | [Per Band] | No | FR2 only | N/A |  | Optional with capability signalli |

- NR_ATG

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-1 | Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | Indicate the support of enhanced inter-frequency cell re-selection requirements for ATG (as specific in TS 38.133 Table 4.2D.2.4-2) |  | no | N/A | If UE does not support the capability, legacy measurement requirements (as specified in TS 38.133, Table 4.2D.2.4-1) are applied. | Per UE | No | FR1 only | N/A |  | Optional without capability signaling |
| 35. NR_ATG | 35-2 | Antenna type | Indicate whether UE supports the RF and RRM requirements with antenna array as specified in TS 38.101-1 section 6.1J, 7.1J and TS 38.133. If the field is absent, the RF and RRM requirements with omni-directional antenna applies as specified in TS 38.101-1 section 6.1J, 7.1J and TS 38.133. |  | yes | N/A | If UE does not support this feature group, performance cannot be guaranteed. | Per Band | No | FR1 only | N/A |  | Mandatory with capability signaling for UE supports NR communication via ATG |
| 35. NR_ATG | 35-3 | Rated max output power | Indicate the support of rated maximum output power at maximum modulation order and full PRB configurations |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s maximum output power. | Per band | No | FR1 only | N/A | Value range from 23dBm to 40dBm with 1dB as granularity | Mandatory with capability signaling for UE supports NR communication via ATG |
| 35. NR_ATG | 35-4 | ATG specific P-max | Indicate the support of ATG specific P-max configured by network. |  | no | N/A | If UE does not support ATG specific P-max value, ATG UE can’t identify configured maximum output power PCMAX,f,c | Per UE | No | FR1 only | N/A | Value range from -21dBm to 42dBm | Mandatory without capability signaling for UE supports NR communication via ATG |

- NR_demod_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | [1) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for total 2 layers across target and co-scheduled UEs with 2 RX antennas 2) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for up to 2,3, or 4 total layers across target and co-scheduled UEs with 4 RX antennas.] | 3-4 | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | TBD | No | FR1 only | N/A |  | Optional with capability signaling |

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-1 | Support of reduced number of samples for PRS based positioning measurements with frequency hopping for RRC_CONNECTED | 1. Support of reduced number of samples in PRS based positioning measurements with frequency hopping | RAN1 feature 28-1, 27-3-1, 45-5-1 | No | NA | RedCap UE does not support reduced number of samples for PRS based positioning measurements with frequency hopping | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-1A | Support of reduced number of samples for PRS based positioning measurements with frequency hopping for RRC_IDLE and RRC_INACTIVE | 1. Support of reduced number of samples in PRS based positioning measurements with frequency hopping | RAN1 feature 28-1, 27-3-1, 45-5-1 | No | NA | RedCap UE does not support reduced number of samples for PRS based positioning measurements with frequency hopping | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-2 | Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_CONNECTED | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 | No | NA | UE does not support reduced number of samples in positioning measurements with PRS bandwidth aggregation | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-2A | Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_IDLE and RRC_INACTIVE | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 | No | NA | UE does not support reduced number of samples in positioning measurements with PRS bandwidth aggregation | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_NTN_enh

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 41 NR_cov_enh2 | [41-1] | MPR reduction for pi/2 BPSK and QPSK | Indicates whether UE supports power boosting for pi/2 BPSK and QPSK for MPR reduction without transparent scheme | None | Yes | N/A | MPR reduction for pi/2 BPSK and QPSK without transparent schemes for MPR reduction is not possible | Per FS | No | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 41 NR_cov_enh2 | [41-2] | MPR reduction for pi/2 BPSK and QPSK | Indicates whether UE supports power boosting for pi/2 BPSK and QPSK with transparent schemes for MPR reduction | None | Yes | N/A | MPR reduction for pi/2 BPSK and QPSK with transparent schemes for MPR reduction is not possible | Per FS | No | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_DualTxRx_MUSIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC | [43-1] | TxDiversity for 2Tx | Indicates UE supports 2Tx Tx diversity for the band configured |  | Yes | N/A | UE doesn’t support 2Tx Tx diversity for the band configured | Per FS | No | FR1 only | N/A | . | Optional with capability signalling |

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. # NR_SL_enh2 | 44-1 | Power class for sidelink CA | [Indicates power class the UE supports when operating according to this band combination used for sidelink. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassSidelink-r16 in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1.] |  | Yes | Yes | UE cannot transmit in proper power class as specified in 38.101-1 | Per BC | No | FR1 only | N/A |  | Optional with capability signalling |
| 44. NR_SL_enh2 | 44-2 | SL reception in intra-carrier guard band | Capability of reception in the non-zero intra-cell guardband between contiguous RB sets in SL wideband carrier operation wider than 20MHz when LBT is successful only in a subset of RB sets |  | Yes | Yes | UE cannot receive in the intra-cell guard band specified in 38.101-1 | Per band | No | FR1 only | N/A |  | Optional with capability signalling |