3GPP TSG-RAN WG4 Meeting #109                                      	                        R4-2318675

# Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.3.3.1.2

Source:	Apple Inc.

Title:	Summary of Simulation Results for 8Rx Demodulation Requirements

Release:	Rel-18

Document for:	Discussion

# Introduction

Overall, this WI has been focusing on:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices

  - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx

    - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers

  - Specify the SDR requirements with 8 MIMO layers

In addition, as per the revised WID RP-231490 in RAN#100, we wait for RAN4 RF to specify UE RF requirements for both single carrier and CA/DC for 8Rx. About specific band combos and configurations, these are FFS.

  - Specify the UE RF requirements to support 8Rx for both single carrier and CA/DC

    - Example band combos and configurations need to be defined

The main aim of this paper is to summarize the status and still open discussions related to PDSCH demodulation requirements for 8Rx UEs.

# Simulation Results

Table 1: FDD Results, TDLC300-100, Rank 2

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.5 |
| 10 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.6 |
| 15 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.6 |
| 20 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.6 |
| 25 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.6 |
| 30 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.7 |
| 35 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.8 |
| 40 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.7 |
| 45 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.8 |
| 50 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.9 |

Table 2: TDD Results, TDLC300-100, Rank 2.

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.0 |
| 10 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.0 |
| 15 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.2 |
| 20 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.1 |
| 25 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.2 |
| 30 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.3 |
| 40 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.2 |
| 50 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.2 |
| 60 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.3 |
| 70 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.3 |
| 80 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.4 |
| 90 | MCS19 | TDLA30-10 | 2x8, ULA Medium B | 70 | 11.5 |
| 100 | MCS19 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.4 |

Table 3: FDD Results, TDLA30-10, Rank 8

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.1 |
| 10 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.4 |
| 15 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.7 |
| 20 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 21.2 |
| 25 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 21.4 |
| 30 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 35 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 21.8 |
| 40 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 21.9 |
| 50 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 22.0 |

Table 4: TDD Results, TDLA30-10, Rank 8

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.8 |
| 10 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |
| 15 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.7 |
| 20 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.5 |
| 25 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.4 |
| 30 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.8 |
| 40 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 50 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 20.8 |
| 60 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 21.3 |
| 70 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 21.3 |
| 80 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 21.6 |
| 90 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 21.8 |
| 100 | MCS17 | TDLA30-10 | 8x8, ULA Low | 70 | 22.1 |

Table 5: FDD Results, TDLA30-10, Rank 2

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.7 |
| 10 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.5 |
| 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.7 |
| 20 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.6 |
| 25 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.6 |
| 30 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.6 |
| 35 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.7 |
| 40 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.8 |
| 45 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.7 |
| 50 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.9 |

Table 6: TDD Results, TDLA30-10, Rank 2

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.6 |
| 10 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.6 |
| 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.7 |
| 20 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.7 |
| 25 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.7 |
| 30 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.8 |
| 40 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.7 |
| 50 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.8 |
| 60 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.9 |
| 70 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 6.9 |
| 80 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 7.1 |
| 90 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 7.0 |
| 100 | MCS17 | TDLA30-10 | 2x8, ULA Low | 70 | 7.1 |