3GPP TSG-RAN WG4 Meeting #109	R4-2318011

Chicago, USA, 13 – 17 November 2023

3GPP TSG RAN WG1 Meeting #114bis		                                                                  R1-2310595

Xiamen, China, October 9th – October 13th, 2023

Title:	LS on PSFCH power control

Response to:

Release:	Rel-18

Work Item:	NR_SL_enh2-Core

Source:	RAN1, Huawei

To:	RAN4

Cc:	-

Contact person:	Xiang Mi

shawn.mixiang@huawei.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# Overall description

Regarding PSFCH power control for “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”, RAN1#114bis made the following working assumption.

Working assumption - In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”: - Assume the UE transmits N PSFCH - Denote the final Tx power on one common PRB is P_common - Denote the final Tx power on one dedicated PRB is P_dedicated - P_common <= P_dedicated - (pre-)configure an offset between P_common and P_dedicated - Send an LS to RAN4 asking whether there is any difficulty for supporting the following cases - P_common < P_dedicated - P_common = P_dedicated

# Actions:

To RAN4:

RAN1 respectfully requests RAN4 to take the above information into account, and provide feedback to RAN1 whether there is any difficulty for supporting the following cases

- P_common < P_dedicated

- P_common = P_dedicated

# Date of Next TSG-RAN1 Meetings:

RAN1#115			13 - 17 Nov 2023				Chicago, US

RAN1#116			26 Feb – 01 Mar 2024				Athens, GR