3GPP TSG-RAN WG4 Meeting #109	R4-2318025

Chicago, USA, 13 – 17 November 2023

3GPP TSG-RAN WG2 #123bis	R2-2311505

# Xiamen, China, 9 – 13 Oct 2023

Title:	LS reply to RAN4 LS R4-2314351

Response to:	R4-2314351

Release:	Rel-18

Work Item:	NR_SL_enh2

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact person:	Min Wang

min.w.wang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

In RAN4 LS R4-2314351, RAN4 has discussed RRM requirements for following sidelink procedure:

- Procedure for initiation/cease of SLSS transmissions

For the three options listed in the RAN4 LS R4-2314351

  - Option 1: cease all SLSS transmissions,

  - Option 2: initiate SLSS transmissions,

  - Option 3: UE keeps current SLSS transmission status,

based on the current RRC spec (i.e., clause 5.8.6.2 of TS 38.331), RAN2 has the following understanding.

When the maximum evaluation period for initiation/cease of SLSS transmission measurement has been reached and the number of available SL SS transmission occasions during this maximum evaluation period is not enough to derive an accurate PSBCH-RSRP measurement result to compare with syncTxThreshOoC, the PSBCH-RSRP of the current SyncRef UE is lower than the minimum requirement defined in TS 38.133 ( i.e., number of available SLSS transmission occasions due to its SL LBT failure is not sufficient during the PSBCH-RSRP measurement period). In this case, the current SyncRefUE is considered invalid, and the UE is expected to reselect a different SyncRefUE per TS38.331.

Therefore, none of the above options is suitable to what’s described above based on the TS 38.331. That is, if the number of LBT failures exceeds the maximum allowed LBT failures during the evaluation period for initiation/cease of SLSS transmission, the UE is expected to reselect to a new synchronization reference source.  RAN2 would like to inform RAN4 of the above RAN2’s understanding, and respectfully suggest RAN4 to consider this into RAN4’s further decision.

# 2	Actions

To RAN4:

ACTION: 	RAN2 respectfully asks RAN4 to consider the above RAN2 understanding in the further decision.

# 3	Dates of next TSG-RAN WG2 meetings

TSG RAN WG2 Meeting #124	November 13 – November 17, 2023	Chicago, US

TSG RAN WG2 Meeting #125	February 26 – March 1, 2024	             Athens, GR