3GPP TSG RAN WG 4 Meeting #109		R4-2321388

Chicago, USA, 13 Nov. 2023 – 17 Nov. 2023

Title:	Reply LS on L1 measurements for LTM

Response to:	R2-2311333

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact person:	Venkatarao Gonuguntla

Venkatarao.gonuguntla@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

RAN4 would like to thank RAN2 for the LS R2-2311333. RAN4 discussed whether SMTC needs to be conveyed in RS configuration or not and made following agreements.

RAN4 agreed to define requirements only for known cell measurements in Rel-18 and the relevant agreements are given below.

- UE is not required to perform L1 measurements on unknown cell. Known cell definition: - In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell, and - FFS whether to add time constraint e.g., during the last [5] seconds - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3. - Otherwise, it is unknown

Based on the above agreement, for LTM L1 measurement on known cell，UE needs at least L3 measurement object for the LTM target cell, which already includes SMTC. If the UE is not provided with the L3 measurement object, LTM RRM requirements are not applied in Rel-18. It is RAN4 understanding that LTM UE needs SMTC information of the target cell, but there is, in RAN4, no discussion on set of parameters and configurations to make the requirement applicable when L3 measurement object for the LTM cell is not configured in Rel-18. It is up to RAN2 to include SMTC in LTM RS configuration or not.

# 2	Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take the above into their work

# 3	Dates of next TSG RAN WG 4 meetings

TSG RAN WG4 meeting #110			26 Feb. 2023 to 01 Mar. 2023		Athens, Greece

TSG RAN WG4 meeting #111			15 Apr. 2023 to 19 Apr. 2023		TBC, China