3GPP TSG-RAN WG4 Meeting #110						                 					     R4-2400379

Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for NeedForGaps requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Interruption requirements for R18 NeedForGaps were introduced in RAN4#109. However, the requirements are still incomplete. Besides, total interruption D is not specified correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that requirements apply only when gap is not configured, or gap is fully non-overlapped with SMTC on any carriers that are measured with interruption. - Update total interruption D. - Add requirements for DRX >= 320ms. - Algin Tcycle with measurement requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements for R18 NeedForGaps would still be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

##### 8.2.2.2.19	Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

When a UE supports [NeedForInterruptionInfoNR-R18] measurements and indicates [no-gap-with-interruption] on intra-frequency SSB-based or inter-frequency SSB-based measurements, the UE is allowed to cause interruptions while performing measurements on the frequency layers of the bands for which [no-gap-with-interruption] is indicated. Requirements in this section apply only when

- the UE is in SA operation mode.

- Gap is not configured, or gap is fully non-overlapped with SMTC on any carriers on which UE indicates [no-gap-with-interruption]

The UE is allowed to cause interruption with interruption ratio no more than the requirements specified below upon UE measurements on a specific frequency layer that corresponds to the configured MO, where Tcycle,i is the interruption cycle on a certain frequency layer i, specified in Table 8.2.2.2.19-1, Table 8.2.2.2.19-2, Table 8.2.2.2.19-3 and Table 8.2.2.2.19-4, where CSSFoutside_gap,i is defined in clause 9.1.5.1 for measurement conducted outside measurement gaps.

Table 8.2.2.2.19-1: Tcycle,i length for inter/intra-frequency measurement target carrier i in FR1

| DRX cycle | TCycle,i |
| --- | --- |
| No DRX | max (80ms, SMTC period) x CSSFoutside_gap,i |
| [DRX cycle ≤ 320ms] | 1.5 x max (80ms, SMTC period) x CSSFoutside_gap,i |
| [DRX cycle>320ms] | DRX cycle x CSSFoutside_gap,i |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 8.2.2.2.19-2: Tcycle,i length for inter/intra-frequency measurement target carrier i in FR2

| DRX cycle | TCycle,i |
| --- | --- |
| No DRX | Klayer1_measurement x max (80ms, SMTC period) x CSSFoutside_gap,i |
| [DRX cycle ≤ 320ms] | 1.5 x Klayer1_measurement x max (80ms, SMTC period) x CSSFoutside_gap,i |
| [DRX cycle>320ms] | DRX cycle x Klayer1_measurement x CSSFoutside_gap,i |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 8.2.2.2.19-3: Tcycle,i length for inter/intra-frequency measurement target carrier i in FR1 when highSpeedMeasFlag-r16 and/or highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX Note 2 | max (80ms, SMTC period) x CSSFoutside_gap,i |
| DRX cycle≤ 320ms | M2 Note 2 x max(80ms, SMTC period,DRX cycle) x CSSFoutside_gap,i |
| DRX cycle>320ms | DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC period > 40 ms, otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to measurements of secondary component carrier with active SCell. |  |

Table 8.2.2.2.19-4: Tcycle,i length for inter/intra-frequency measurement target carrier i in FR2 when highSpeedMeasFlagFR2-r17 is configured and SMTC period <= 40ms

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Klayer1_measurement x max(80ms, SMTC period) x CSSFoutside_gap,i |
| DRX cycle≤ 80ms | Klayer1_measurement x max(80ms, SMTC period,DRX cycle) x CSSFoutside_gap,i |
| 80ms< DRX cycle≤ 320ms | 1.5 x Klayer1_measurement x max(80ms, SMTC period,DRX cycle) x CSSFoutside_gap,i |
| DRX cycle>320ms | Klayer1_measurement x max(80ms, SMTC period,DRX cycle) x CSSFoutside_gap,i |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

UE is allowed to cause interruption on a certain frequency layer i with the maximum interruption ratio that equals 2LTcycle,i.

The total allowed maximum interruption ratio (D) on each of the active serving cells due to UE measurements without gap applied in this sub-clause is specified as

D= 1Mi=1N2LTcycle,i

Where,

-	N is the total number of configured SSB based frequency layers to be measured outside gap including intra-frequency and inter-frequency target carriers where UE indicates that interruption is needed through [no-gap-with-interruption], and

-    M is total number of carriers which are measured outside gap, including carriers that are measured with and without interruption, and

-	L is the maximum interruption length for each interruption occasion specified in the Table 8.2.2.2.19-2 and 8.2.2.2.19-3.

The interruptions are allowed for all the active serving cells in the same FR as NR MO being measured if UE supports per-FR measurement gaps, and all the active serving cells if UE does not support per-FR measurement gaps.

Table 8.2.2.2.19-2: Interruption length L in FR1

|  | SCS (kHz) of victim cell | NR Slot length (ms) of victim cell | Interruption length L (slots) |
| --- | --- | --- | --- |
| 0 | 15 | 1 | [1] |
| 1 | 30 | 0.5 | [2] |
| 2 | 60 | 0.25 | [4] |

Table 8.2.2.2.19-3: Interruption length L in FR2

|  | SCS (kHz) of victim cell | NR Slot length (ms) of victim cell | Interruption length L (slots) |
| --- | --- | --- | --- |
| 2 | 60 | 0.25 | [3] |
| 3 | 120 | 0.125 | [6] |

Editors’ note: Discussion is ongoing on cases where DRX is configured. Further update to this sub-clause subjects to the conclusions of the discussion.

End of Change 1

Start of Change 1

### 9.2.5	Intrafrequency measurements without measurement gaps

#### 9.2.5.1	Intrafrequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TPSS/SSS_sync_intra is given in Table 9.2.5.1-11; otherwise, TPSS/SSS_sync_intra is given in Table 9.2.5.1-2.

-	For UE indicating [no gap without interruption], TPSS/SSS_sync_intra is given in Table 9.2.5.1-1 for FR1 and  Table 9.2.5.1-2 for FR2. For UE indicating [no gap with interruption], TPSS/SSS_sync_intra is given in Table  9.2.5.1-X1 for FR1 and  Table 9.2.5.1-X2 for FR2.

-	Otherwise, TPSS/SSS_sync_intra is given in table 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-4 (deactivated SCell) or 9.2.5.1-5 (deactivated SCell) or 9.2.5.1-9 (deactivated SCell) or 9.2.5.1-11 or 9.2.5.1-12 (deactivated PSCell) or 9.2.5.1-13 (deactivated PSCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured

-	For UE indicatting [no gap without interruption], TSSB_time_index_intra is given in Table 9.2.5.1-3 for FR1 and Table 9.2.5.1-15 for FR2-2. For UE indicating [no gap with interruption], TSSB_time_index_intra is given in Table 9.2.5.1-X3 for FR1 and Table 9.2.5.1-X4 for FR2-2.

-	Otherwise, TSSB_time_index_intra is given in table 9.2.5.1-3, 9.2.5.1-15 (FR2-2), 9.2.5.1-6 (deactivated SCell), 9.2.5.1-10(deactivated SCell) or 9.2.5.1-14 (deactivated PSCell).

-	T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TSSB_measurement_period_intra is given in Table 9.2.5.2-7; otherwise, T SSB_measurement_period_intra is given in Table 9.2.5.2-2.

-	For UE indicating [no gap without interruption], TSSB_measurement_period_intra is given in Table 9.2.5.2-1 for FR1 and  Table 9.2.5.2-2 for FR2. For UE indicating [no gap with interruption], TSSB_measurement_period_intra is given in Table 9.2.5.2-Y1 for FR1 and  Table Table 9.2.5.2-Y2 for FR2.

-	For UE indicating [no gap without interruption], TSSB_measurement_period_intra is given in Table 9.2.5.2-1 for FR1 and  Table 9.2.5.2-2 for FR2. For UE indicating [no gap with interruption], TSSB_measurement_period_intra is given in Table 9.2.5.2-Y1 for FR1 and  Table Table 9.2.5.2-Y2 for FR2.

-	For power class 6 UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured, the T SSB_measurement_period_intra given in Table 9.2.5.2-7 (if SMTC <= 40ms) and Table 9.2.5.2-2 (if SMTC > 40ms) shall apply for SCC.

-	Otherwise, T SSB_measurement_period_intra is given in table 9.2.5.2-1, table 9.2.5.2-2 table 9.2.5.2-3 (deactivated SCell), 9.2.5.2-4(deactivated SCell), 9.2.5.2-5 or 9.2.5.2-6(deactivated SCell), 9.2.5.2-8(deactivated PSCell) or 9.2.5.2-9(deactivated PSCell).

-	CSSFintra: it is a carrier specific scaling factor and is determined

-	according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e.

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP, or

-	 when intra-frequency SMTC is fully non overlapping with GAP for UE indicating [no gap with interruption],  or

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP for UE indicating [no gap without interruption], or

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24.	For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

When UE supports concurrent measurement GAPs, i.e., supports the following capability or capabilities’ combination:

-	concurrentMeasGap-r17, or

-	or musim-GapPreference-r17, or

-	both concurrent measurement gap and musim-GapPreference-r17, or

-	[concurrent gap with Pre-MG capability], or

-	[concurrent gap with NCSG capability],

and UE concurrent measurement GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and/or per-FR measurement GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in section 9.1.8.3 and 9.1.10.6, respectively.

Kp = 1 when Navailable = 0.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Editor Note: FSS for the case when Navailable = 0 due to fully overlapping between SMTC occasions and the union of MUSIM gap and measurement gap occasions within the window W.

When UE supports [MUSIM-GapConfig-17] and the SMTC occasion of the target frequency layer is overlapping with the configured aperiodic MUSIM gap, longer cell identification period for the target frequency layer is expected.

-	Otherwise, when the UE is not configured with or UE does not support concurrent measurement GAPs: and the UE is not configured with periodic MUSIM gaps or UE does not support MUSIM gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.	If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in Table 9.2.5.1-1, Table 9.2.5.1-2, Table 9.2.5.1-3, Table 9.2.5.1-4, Table 9.2.5.1-5 and Table 9.2.5.1-6 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in Table 9.2.5.1-1, Table 9.2.5.1-2, Table 9.2.5.1-3, Table 9.2.5.1-4, Table 9.2.5.1-5, Table 9.2.5.1-6, Table 9.2.5.1-12, Table 9.2.5.1-13 and Table 9.2.5.1-14 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

-	When the target SSB is completely contained in active BWP of UE or the active downlink BWP is initial BWP, the intra-frequency measurement should be without gap without interruption regardless of the NeedForGaps’ status reporting.

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-5: Time period for PSS/SSS detection, deactivated SCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-6: Time period for time index detection, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-7: Void

Table 9.2.5.1-8: Void

Table 9.2.5.1-9: Time period for PSS/SSS detection, deactivated SCell (FR1), when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-10: Time period for time index detection, deactivated SCell (FR1)，when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3 x Kp)x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-11: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (Frequency range FR2) when SMTC period <= 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80ms | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | ceil(1.5 x Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

Table 9.2.5.1-12: Time period for PSS/SSS detection, deactivated PSCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-13: Time period for PSS/SSS detection, deactivated PSCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-14: Time period for time index detection, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-15: Time period for time index detection (Frequency range FR2-2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(200ms, ceil(MSSB_index_intra x Kp x SMTC period) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5 x MSSB_index_intra x Kp) x max(SMTC period, DRX cycle) x CSSFintra) |
| DRX cycle>320ms | Ceil(MSSB_index_intra x Kp )x DRX cycle x CSSFintra |

Table 9.2.5.1-16: Time period for time index detection (FR1) for less_than_5Mhz channel bandwidth UE

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, ceil( [X] x Kp ) Note 2 x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x [Y] x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_less_than_5Mhz |
| DRX cycle>320ms | Ceil([Z] x Kp) Note 2 x DRX cycle x CSSFintra_less_than_5Mhz |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.1-17: Time period for PSS/SSS detection for UE indicating [no gap with interruption], (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max (80ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320ms] | max( 600ms, ceil(M2 Note 2x 5) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320ms] | 5 x [DRX cycle x] CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 5: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.1-18: Time period for PSS/SSS detection for UE indicating [no gap with interruption], (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x max (80ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320ms] | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320ms] | [ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x DRX cycle] x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. NOTE 3: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.1-19: Time period for time index detection for UE indicating [no gap with interruption] (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 3 x max (80ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320ms] | max(120ms, ceil (M2 Note 2 x 3) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320ms] | 3 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 NOTE 5: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.1-20: Time period for time index detection  for UE indicating [no gap with interruption] (Frequency range FR2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(200ms, ceil(MSSB_index_intra x Kp x max(80ms, SMTC period )) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5 x MSSB_index_intra x Kp) x max(80ms, SMTC period, DRX cycle) x CSSFintra) |
| DRX cycle>320ms | Ceil(MSSB_index_intra x Kp )x DRX cycle x CSSFintra |
| NOTE 1: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.1-21: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (Frequency range FR2) when SMTC period <= 40ms, UE indicating [no gap with interruption]

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Klayer1_measurement) x max (80ms, SMTC period))Note 1 x CSSFintra |
| [DRX cycle≤ 80ms] | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x [max(80ms,SMTC period,DRX cycle)]) x CSSFintra |
| [80ms< DRX cycle≤ 320ms] | ceil(1.5 x Mpss/sss_sync_w/o_gaps x Klayer1_measurement) x max(80ms, SMTC period,DRX cycle) x CSSFintra |
| [DRX cycle>320ms] | ceil(Mpss/sss_sync_w/o_gaps x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.1-22: Time period for time index detection (FR1) for less_than_5Mhz channel bandwidth UE, UE indicating [no gap with interruption]

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, ceil( [X] ) x max (80ms,SMTC period))Note 1 x CSSFintra_less_than_5Mhz |
| [DRX cycle≤ 320ms] | max(120ms, ceil (1.5 x [Y]) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra_less_than_5Mhz |
| [DRX cycle>320ms] | [Z] x DRX cycle x CSSFintra_less_than_5Mhz |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. NOTE 2: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.1-23: Time period for time index detection (Frequency range FR1) [for a target cell with 12 or 15 PRB SSB]

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, [7] x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x [7]) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | [7] x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 |  |

#### 9.2.5.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 9.2.5.2-1, 9.2.5.2-2, 9.2.5.2-3 (deactivated SCell), 9.2.5.2-4 (deactivated SCell), 9.2.5.2-8 (deactivated SCG applicable for PSCell) or 9.2.5.2-9 (deactivated SCG applicable for PSCell). When highSpeedMeasFlag-r16 is configured, T SSB_measurement_period_intra is specified in Table 9.2.5.2-5. When UE highSpeedMeasFlagFR2-r17 is configured, if SMTC <= 40ms, TSSB_measurement_period_intra is given in Table 9.2.5.2-7; otherwise, T SSB_measurement_period_intra is given in Table 9.2.5.2-2. For power class 6 UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured, the T SSB_measurement_period_intra given in Table 9.2.5.2-7 (if SMTC <= 40ms) and Table 9.2.5.2-2 (if SMTC > 40ms) shall apply for SCC.

If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

If MCG DRX is in use, measurement period requirements for intra-frequency measurement in MCG specified in Table 9.2.5.2-1, Table 9.2.5.2-2, Table 9.2.5.2-3 and Table 9.2.5.2-4 shall depend on the MCG DRX cycle. If SCG DRX is in use, measurement period requirements for intra-frequency measurement in SCG specified in Table 9.2.5.2-1, Table 9.2.5.2-2, Table 9.2.5.2-3, Table 9.2.5.2-4, Table 9.2.5.2-8 and Table 9.2.5.2-9, shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

For FR2, a longer measurement period is allowed, if aperiodic CSI-RS resource is measured for L1-RSRP measurement on any FR2 serving frequency in the same band, and the CSI-RS resource is outside measurement gap and overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols. If SSB-ToMeasure or SS-RSSI-Measurement is configured, the SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same band which can be merged and the RSSI symbols are indicated by SS-RSSI-Measurement.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCel |  |

Table 9.2.5.2-4: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(Mmeas_period_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.2-5: T SSB_measurement_period_intra When highSpeedMeasFlag-r16 and/or highSpeedMeasCA-Scell-r17 is configured (Frequency range FR1

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX Note 2 | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 160ms | max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms < DRX cycle≤ 320ms | ceil(4 x M2 Note 2 x Kp) x DRX cycle x CSSFintra |
| DRX cycle>320ms | ceil( Y Note 3 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC period > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC period <= 40ms, Y=5 when SMTC period > 40ms NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 5: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to measurements of secondary component carrier with active SCell. |  |

Table 9.2.5.2-6: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1), when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | ceil( 5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 160ms | ceil(5 x Kp) x max(measCycleSCell, M2 Note 1 x DRX cycle) x CSSFintra |
| 160ms < DRX cycle≤ 320ms | ceil(4 x Kp) x max(measCycleSCell, M2 Note 1 x DRX cycle) |
| DRX cycle>320ms | ceil( Y Note 2 x Kp ) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 2: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

Table 9.2.5.2-7: Measurement period for intra-frequency measurements without gaps when highSpeedMeasFlagFR2-r17 is configured (FR2) when SMTC period <= 40ms

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80ms | max(400ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | ceil(1.5 x Mmeas_period_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps Note 3 xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.2.5.2-8 Measurement period for intra-frequency measurements without gaps (deactivated SCG applicable for PSCell) (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.2-9: Measurement period for intra-frequency measurements without gaps (deactivated SCG applicable for PSCell) (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(Mmeas_period_w/o_gaps x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.2-10: Measurement period for intra-frequency measurements without gaps for UE indicating [no gap with interruption] (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x (80ms, SMTC period ))Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| DRX cycle>320ms | 5 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.2-11: Measurement period for intra-frequency measurements without gaps for UE indicating [no gap with interruption] (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Klayer1_measurement) x (80ms, SMTC period ))Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Klayer1_measurement) x [max(80, SMTC period,DRX cycle)]) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. NOTE 2: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.2-12: T SSB_measurement_period_intra When highSpeedMeasFlag-r16 and/or highSpeedMeasCA-Scell-r17 is configured (Frequency range FR1, UE indicating [no gap with interruption]

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX Note 2 | max(200ms, 5 x max(80ms,SMTC period))Note 1 x CSSFintra |
| DRX cycle≤ 160ms | max(200ms, ceil(5 x M2 Note 2) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| 160ms < DRX cycle≤ 320ms | ceil(4 x M2 Note 2) x DRX cycle x CSSFintra |
| DRX cycle>320ms | ceil( Y Note 3) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC period > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC period <= 40ms, Y=5 when SMTC period > 40ms NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 5: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to measurements of secondary component carrier with active SCell. NOTE 6: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 9.2.5.2-13: Measurement period for intra-frequency measurements without gaps when highSpeedMeasFlagFR2-r17 is configured (FR2) when SMTC period <= 40ms, UE indicating [no gap with interruption]

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(M1Note 2 x Klayer1_measurement) x max(80ms, SMTC period))Note 1 x CSSFintra |
| DRX cycle≤ 80ms | max(400ms, ceil(M1Note 2 x Klayer1_measurement) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | ceil(1.5 x Mmeas_period_w/o_gaps x Klayer1_measurement) x max(80ms,SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

End of Change 2