3GPP TSG-RAN WG4 Meeting #110	R4-2401403

# Athens, Greece, February 26th – March 1st 2024

Source:	Ericsson

Title:	Simulation results for NR NTN enhancement SAN demodulation requirements

Agenda item:	8.18.8.1

Document for:	Information

# Introduction

According to the WF in RAN4#109 [1], some simulation assumptions are agreed for initial PUSCH, PUCCH and PRACH simulations.

In this contribution, simulation results are provided for further discussion.

# Simulation results

## 2.1	FR2 normal PUSCH

Table 2.1-1 Simulation parameters for normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120 kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth 120kHz SCS: 50MHz |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission (If 2Tx is agreed) |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, Disabled |
| configuration | Time density (LPT-RS) | 1, Disabled |
| PN model: Example -1 and Example -2 in 38.803 |  |  |

Table 2.1-2 Simulation results for normal PUSCH

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 | 70 % | 2 | pos1 | -3.1 |
|  |  | Normal | NTN-TDLC5-3000 | 70 % | 16 | pos1 | 9.6 |
|  |  | Normal | NTN-TDLC5-3000 | 70 % | 20 | pos1 | 13.7 |
|  |  | Normal | NTN-TDLC5-600 | 70 % | 20 | pos1 | 12.4 |
|  | 2 | Normal | NTN-TDLC5-3000 | 70 % | 2 | pos1 | -6.4 |
|  |  | Normal | NTN-TDLC5-3000 | 70 % | 16 | pos1 | 6.2 |
|  |  | Normal | NTN-TDLC5-3000 | 70 % | 20 | pos1 | 9.9 |

Table 2.1-3 Simulation results for PN impact for normal PUSCH

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | PT-RS | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No PN | PN model-1 | PN model-2 |
| 1 | 1 | Normal | NTN-TDLC5-3000 | 70 % | 16 | pos1 | On | 10.3 | 10.4 | 10.4 |
|  |  |  |  |  |  |  | Off | 9.6 | 9.6 | 9.7 |
|  |  |  |  |  | 20 | pos1 | On | 14.2 | 14.3 | 14.5 |
|  |  |  |  |  |  |  | Off | 13.7 | 13.8 | 14 |
|  | 2 | Normal | NTN-TDLC5-3000 | 70 % | 16 | pos1 | On | 7 | 7 | 7.1 |
|  |  |  |  |  |  |  | Off | 6.2 | 6.2 | 6.3 |
|  |  |  |  |  | 20 | pos1 | On | 10.6 | 10.7 | 10.8 |
|  |  |  |  |  |  |  | Off | 9.9 | 10 | 10.2 |

## 2.2 	FR2 PUSCH repetition type A

Table 2.2-1 Simulation parameters for FR2 PUSCH repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: FDD |
| HARQ | Maximum number of HARQ transmissions | 4 |
| RV sequence |  | 0, 3, 0, 3 |
| DM-RS | DM-RS configuration type | 1 |
| DM-RS duration |  | single-symbol DM-RS |
| Additional DM-RS symbols |  | Pos1 |
| Number of DM-RS CDM group(s) without data |  | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
| DM-RS port(s) |  | 0 |
| DM-RS sequence generation |  | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
| Allocation length |  | 10 |
| PUSCH aggregation factor (Note 2) |  | N2 |
| Frequency domain | RB assignment | Full applicable test bandwidth 120kHz SCS: 50MHz |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | Disabled |
| configuration | Time density (LPT-RS) | Disabled |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation NOTE 2: The intention of this configuration is to have two effective transmissions of the transport block. |  |  |

Table 2.2-2 Simulation results for FR2 PUSCH repetition type A

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Target BLER | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | NTN-TDLC5-3000 | 1% | G-FR2-A3A-7 | Pos1 | -13.1 |

## 2.3 	FR2 PRACH

Table 2.3-1 Simulation parameters for FR2 PRACH

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | NTN-TDLC5 |
| B4, C2 | 60 | 0.13 us | 0.13 us |
|  | 120 | 0.07 us | 0.07 us |

Table 2.3-2 Burst format for FR2 PRACH

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| B4, C2 | 60 | 69 | 0 | 0 |
|  | 120 | 69 | 0 | 0 |

Table 2.3-3 Simulation results for FR2 PRACH

| SCS (kHz) | Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (annex G) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format B4 | Burst format C2 |
| 60KHz | 1 | 1 | AWGN | 0 | -16.6 | -11.7 |
|  |  |  | NTN-TDLC5 Low | 3000 Hz |  | -12.3 |
|  | 1 | 2 | AWGN | 0 | -19.2 | -14.3 |
|  |  |  | NTN-TDLC5 Low | 3000 Hz |  | -14.3 |
| 120KHz | 1 | 1 | AWGN | 0 | -16.6 | -11.7 |
|  |  |  | NTN-TDLC5 Low | 3000 Hz | -16.9 | -13.2 |
|  | 1 | 2 | AWGN | 0 | -19.1 | -14.3 |
|  |  |  | NTN-TDLC5 Low | 3000 Hz | -18.8 | -15.2 |

## 2.4	FR1 PUSCH DM-RS bundling

Table 2.4-1 Simulation parameters for FR1 PUSCH DM-RS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example TDD UL-DL pattern (Note 1) |  | FDD |
| sHARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence (Note 2) | 0, 0, 0, 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos0, pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| pusch-TimeDomainWindowLength |  | 8 slots for FDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Timing offset and frequency offset model: maximum offset according to aTDW length |  | Timing offset = 0.21us for 15kHz SCS, 0.11us for 30kHz SCS Frequency offset = 4.3 Hz for 15kHz SCS, 2.2 Hz for 30kHz SCS |

Table 2.4-2 Simulation results for FR1 PUSCH DM-RS bundling, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ideal | Timing and frequency offset |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70 % | 4 | pos0 | -12.1 | -12.1 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | 4 | pos1 | -12.5 | -12.5 |

Table 2.4-3 Simulation results for FR1 PUSCH DM-RS bundling, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ideal | Timing and frequency offset |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70 % | 4 | pos0 | -12.2 | -12.2 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | 4 | pos1 | -12.4 | -12.4 |

# Conclusions

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

# References

[1]	R4- 2321187, Way Forward for NR_NTN_enh_SAN_UE_demod, Qualcomm