# 3GPP TSG-RAN WG4 Meeting # 110                               R4-2402033

# Athens, GR, 26 Feb – 01 Mar, 2024

# Title: 	Simulation results for multiple PRACH

# Source: 	ZTE Corporation

# Agenda item:	8.19.2

# Document for:	Discussion

# 1 Introduction

In RAN #102 meeting[1], the revised work item on Revised WID on further NR coverage enhancements was approved. In RAN4 #109 meeting[2] most issues were reached while some issues about the simulation parameters of multiple PRACH transmission need further discussion.

This contribution shares our simulation results for multiple PRACH transmission with different preamble format for FR2-1.

# 2	Discussion

Some simulation parameters about multiple PRACH are listed below[2].

| Parameters | Values |
| --- | --- |
| Frequency range | FR2-1 FR1 FFS |
| Sequence length | 139 |
| Preamble format | A2, B4, C2 |
| Repetition number | 2 |
| Antenna configuration | 1X2 |
| Channel model | AWGN, TDLA30-300 low for FR2-1 AWGN, TDLC300-100 low for FR1 studies |
| Frequency offset | Fading channel FR2-1 4000Hz AWGN 0 Fading channel FR1 400Hz |
| SCS | FR2-1 120kHz,FFS 60kHz FR1(if agreed) 15kHz and 30kHz |
| Test metric | Missing detection 1% False alarm 0.1% |

Table 2-1 provides the operating SNR for FR2-1 and the miss detection curves are appended in the Annex.

Table 2-1 Operating SNR for different preamble format with repetition number = 2, 120kHz SCS

| index | Preamble format | Channel mode | Operating SNR |
| --- | --- | --- | --- |
| 1 | A2 | AWGN w/o FO | -18.6 |
| 2 | A2 | TDLA30-300 low w/ 4000Hz FO | -11.5 |
| 3 | B4 | AWGN w/o FO | -21.7 |
| 4 | B4 | TDLA30-300 low w/ 4000Hz FO | -14.2 |
| 5 | C2 | AWGN w/o FO | -18.6 |
| 6 | C2 | TDLA30-300 low w/ 4000Hz FO | -11.6 |

# 3 Conclusion

In this contribution, we share our simulation results of 2 repetition PRACH for reference.

# 4	Reference

[1]  RP-233558 Revised WID on further NR coverage enhancements, China Telecom

[2]  R4-2321061 WF on NR_cov_enh2_demod, China Telecom, Nokia

# Annex

A-1 Miss detection curve for A2 preamble with AWGN, 120kHz SCS and 2 repetition

A-2 Miss detection curve for B4 preamble with AWGN, 120kHz SCS and 2 repetition

A-3 Miss detection curve for C2 preamble with AWGN, 120kHz SCS and 2 repetition

A-4 Miss detection curve for A2 preamble with TDLA30-300, 120kHz SCS and 2 repetition

A-5 Miss detection curve for B4 preamble with TDLA30-300, 120kHz SCS and 2 repetition

A-6 Miss detection curve for C2 preamble with TDLA30-300, 120kHz SCS and 2 repetition