3GPP TSG RAN4 Meeting #110							R4-2402391

Athens, Greece, Feb. 26 – March 1, 2024

### Agenda Item:		13.2

Source:	UIC (Union international Chemin de Fer)

Title:				Co-existence between RMR and MFCN in band n101

Document for:		Discussion

Related WIDs: 		NR_RAIL_EU_1900MHz_TDD, RAIL_HPUE_n100_n101

## Introduction

At RAN#102 (December 2023) document RP-233879 [1] has been submitted by a number of MFCN operators that

stated:

“The co-signing companies request TSG-RAN to request RAN WG 4 to investigate whether the requirement to “avoid causing interference on already established networks” has been achieved by the specifications for band n101 (including the extensions for HPUE) and, if not met, to work to correct the Release 17 and 18 specifications such that it is achieved”.

After discussion in RAN#102, RAN4 was tasked with RP-234008 [2] to:

“investigate what possibilities exist in the RAN4 specifications for n101 to avoid causing interference on already established networks, which corresponds to the objectives of WID NR_RAIL_EU_1900MHz_TDD, and report to RAN#103.

- Taking the applicable regulations of CEPT countries into account.”

In this contribution UIC provides an analysis of this subject and draws a number of conclusions.

## UIC analysis

The band n101 has been defined in RAN4 as a result of the WID RP-211542 [3]. Report TR 38.852 [4] has provided both the regulatory background and the technical conditions for this, and has concluded that:

“The study dealt with how the 1900-1910MHz spectrum block (RMR1900) can be used for rail communication using 5G NR applicable in Europe. The subject-areas 5G NR system, BS RF and UE RF were examined in detail.

General: The focus was on the use of RMR1900 under uncoordinated conditions, assuming that RMR BS operates independently of other operators e.g. MFCN. Co-location with other operators and resulting coordination is subject to national regulation and will be not further detailed by 3GPP RAN4.

System Parameter:	The RMR1900 spectrum block can be used by either a 10MHz CBW or a 5MHz CBW. The corresponding system parameters are to be transferred to 3GPP TS 38.101-1 [2] and 3GPP TS 38.104 [4].

BS RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver. The agreed  parameters are to be transferred to 3GPP TS 38.104 [4]. Furthermore, the coexistence with other 3GPP radio technologies has been included. These are to be captured in the corresponding normative 3GPP technical specifications.

UE RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly, and the necessary conclusions were derived for the use of Power Class 3 (23dBm). The agreed parameters in the present TR are to be transferred  to 3GPP TS 38.101-1 [2]”.

Observation 1: The technical conditions specified in ECC Decision (20)02 [5], i.e. EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver.

In TSG RAN#95-e document RP-220376 [6] has been approved which defined the following additional requirements based on the  ECC Decision (20)02 defined LRTC and BS receiver characteristics for RMR in band n101:

  - The base station maximum output power in band n101 is limited to 51 dBm/10MHz or 48 dBm/5MHz at the base station antenna connector. Note that this limit is derived from the 65 dBm EIRP limit in the ECC Decision (20)02 using an antenna gain of 18 dBi and 4 dB losses in accordance with the ECC Report 318 [7], in order to specify the limit at the base station antenna connector as explained in clause 7.1.1 in [4].

  - The maximum spurious emissions in 1920 to 1980 MHz are limited to -57 dBm/5MHz for base stations in band n101. This limit is derived from the -43 dBm/5MHz EIRP limit in the ECC Decision (20)02 using an antenna gain of 18 dBi and 4 dB losses to specify the limit at the base station antenna connector.

  - The out-of-band blocking requirements for base stations in band n101 which allow a maximum 3dB receiver desensitisation in the presence of a -20 dBm 5 MHz LTE interfering signal with centre frequencies between 1807.5 to 1877.5 MHz.

Observation 2: The technical conditions specified in ECC Decision (20)02 [5], i.e. the BS rated output power using corresponding assumptions to convert from the EIRP limits, the resulting emission values of the transmitter and the specifications of the receiver have been transferred to the 3GPP specifications.

For the Regulatory background the TR 38.852 [4] specifically refers to the Commission Implementing Decision (EU) 2021/1730 [9] and the ECC Decision(20)02 [5] that define the applicable regulatory conditions and the technical conditions for the use of the 1900 – 1910 MHz by Railway Mobile Radio. The Commission Implementing Decision (EU) 2021/1730 states in Whereas: 8)

“The harmonised technical conditions for RMR (FRMCS) base stations operating in the 1 900-1 910MHz band as defined in CEPT Report 76, assume that base stations providing electronic communications services, which use frequencies above 1920MHz for reception under Commission Implementing Decision (EU) 2020/667(4), have enhanced selectivity compared to the current Harmonised European Standards. Base stations providing electronic communications services, which are located in the vicinity of a RMR base station and do not meet the enhanced selectivity criterion, should, where necessary, be adapted, in order to mitigate harmful interference”.

As indicated in the Whereas 8), Commission Implementing Decision (EU) 2021/1730 has been based on the CEPT Report 76 [10].  CEPT Report 76 in its Executive summary states:

“The harmonised least restrictive technical conditions for wideband RMR (FRMCS) in 1900-1910 MHz are provided in section 4.1. The BEM for wideband RMR BS assumes that ECS BS have enhanced selectivity compared to the current Harmonised European Standards. This would facilitate coexistence with RMR BS transmitting up to 65 dBm e.i.r.p. ECS BS located near an RMR radio site which do not meet the enhanced selectivity criterion may need to be adapted so that they do not suffer interference. Operators of mobile networks in 1920-1980 MHz should have, sufficiently far in advance, information on the rollout of a new RMR BS in 1900-1910 MHz. Enhanced selectivity should be included (potentially as a specific receiver class) in the relevant ECS BS Harmonised European Standards in order to ensure the future availability of products placed on the market that fulfil this requirement”.

Furthermore it states in section 4.1.1 Technical conditions for RMR BS using wideband technologies:

“The BEM for wideband RMR BS assumes that ECS BS have enhanced selectivity compared to the current Harmonised European Standards. This facilitates coexistence with RMR BS transmitting up to 65 dBm e.i.r.p. ECS BS located near an RMR radio site which do not meet the enhanced selectivity criterion may need to be adapted so that they do not suffer interference. Operators of mobile networks in 1920-1980 MHz should have, sufficiently far in advance, information on the rollout of a new RMR BS in 1900-1910 MHz. Enhanced selectivity should be included (potentially as a specific receiver class) in the relevant ECS BS Harmonised European Standards in order to ensure the future availability of products placed on the market that fulfil this requirement”.

Also the ECC Decision (20)02, that has been based on CEPT report 76,  states in considering k) and l):

“k) that the least restrictive technical conditions (LRTC) for wideband RMR in 1900-1910 MHz assume that MFCN base stations (BS) receiving above 1920 MHz have an enhanced selectivity compared to the current Harmonised European Standards, which would facilitate coexistence with RMR BS transmitting up to 65 dBm e.i.r.p., and that current MFCN BS located near an RMR radio site may need to be adapted so that they do not suffer interference;”.

“l) that operators of commercial mobile networks in 1920-1980 MHz should have, sufficiently far in advance, information on the rollout of a new RMR BS in 1900-1910 MHz”.

The CEPT Report 76 has been based on the ECC Report 318 [7]. ECC Report 318 studied the compatibility of Railway Mobile Radio (RMR) with Mobile and Fixed Communication Networks (MFCN) as part of the answer to the mandate from the European Commission on FRMCS. For the compatibility of FRMCS in part of 1900-1920 MHz with MFCN the ECC Report 318 has studied:

- Impact of FRMCS BS on MFCN BS receiving above 1920 MHz;

- Impact of FRMCS high-power UE on MFCN BS receiving above 1920 MHz.

ECC Report 318 in the Executive summary 0.2 BEM Derivation at 1900 MHz states:

“This report looks at LRTC for operation of FRMCS in 1900-1910 MHz. The 100 metre MCL calculation approach and the statistical approach, assuming a MFCN BS selectivity as per CEPT Report 39 [4], result in LRTC requiring in-block e.i.r.p. limit for FRMCS BS of 40.7 dBm/10 MHz and 50.7 dBm/10 MHz respectively. However, the report also considers the operation of FRMCS in that band with macro coverage. This requires an in-block e.i.r.p. of 65 dBm/10 MHz for FRMCS BS and leads to a BEM as specified in Table 20. It may result in interference to some MFCN BS located near an FRMCS radio site.

With the current level of selectivity of MFCN base stations, those LRTC for FRMCS may result in interference to some MFCN base stations located near FRMCS BS sites. One way of addressing this interference is to coordinate FRMCS and MFCN deployments. However, this means that RMR operators may not be able to use 65 dBm at certain locations. If 65 dBm e.i.r.p. uncoordinated FRMCS base stations is desired, then these MFCN BS may need to be adapted when an FRMCS BS is rolled out in its proximity, so that it does not suffer interference from FRMCS.

Additional mitigation techniques need to be implemented on a case-by-case basis, such as adjustments of antenna directivity, azimuth, tilt, or improve the selectivity of the MFCN BS in the vicinity of the railway tracks2. Table 23 shows this enhanced selectivity.

The technical feasibility for receiver selectivity enhancement for MFCN AAS BS may not be achievable due to design constraints. These high selectivity requirements might hinder the migration of MFCN networks to 5G/NR with AAS BS, however studies need to be carried out.

In order to ensure that the MFCN operators have enough time to adapt the relevant radio sites, the RMR operator is required to perform an early notification procedure in advance of the rollout of a new FRMCS BS”.

Observation 3: In the Commission Implementing Decision (EU) 2021/1730, ECC Decision(20)02, CEPT Report 76 as well as ECC Report 318 [7] it is clearly stated that the regulation has assumed that the harmonised technical conditions for RMR (FRMCS) base stations operating in the 1900-1910MHz band assume that base stations providing electronic communications services, which use frequencies above 1920MHz for reception under Commission Implementing Decision (EU) 2020/667(4), have enhanced selectivity compared to the current Harmonised European Standards. Base stations providing electronic communications services, which are located in the vicinity of a RMR base station and do not meet the enhanced selectivity criterion, should, where necessary, be adapted, in order to mitigate harmful interference.

Observation 4: The ECC in Report 318 has considered that additional mitigation techniques need to be implemented on a case-by-case basis, such as adjustments of antenna directivity, azimuth, tilt, or improve the selectivity of the MFCN BS in the vicinity of the railway tracks. Such site engineering solutions have been considered by RAN 4 for TDD and FDD coexistence scenarios as described in TR 25.942 [11] section 8.4.

Observation 5: From CEPT report 76, ECC DEC(20)02 and ECC Report 318 it is clear that Operators of mobile networks in 1920-1980 MHz should have, sufficiently far in advance, information on the rollout of a new RMR BS in 1900-1910 MHz.

ETSI has created TS 103 807 V1.1.1 (2021-10) [12] Mobile Standards Group (MSG); IMT Cellular Networks to address additional requirements arising from EC/CEPT spectrum regulatory framework that apply to specific equipment in specific cases in certain countries and/or in certain geographical areas.

In its chapter 4 Enhanced Selectivity for protection of Band 1 BSs from FRMCS in 1900 MHz to 1910 MHz it is stated that the following requirement is only applicable to E-UTRA, UTRA, NR, NB-IoT or MSR BSs that are intended to be

deployed in close geographical proximity of FRMCS Base Stations unless mobile operators deem it is not necessary

(noting that there may be a risk of interference from FRMCS).

Observation 6: In response to the EC/CEPT regulation for the 1900-1910MHz band, ETSI has specified enhanced selectivity requirements for protection of Band 1 BSs.

In ECC Report 318 section 5 the Compatibility of FRMCS in 1900-1910 MHz with MFCN has been analysed based on the MCL. In order to select an MCL value, the following criterion has been selected: 7% of RMR sectors would have at least one MFCN neighbouring sector (from the operator the closest in frequency to the RMR lower band edge) with a coupling loss lower than that MCL value. The threshold of 7% has been chosen as a compromise value between efficient use of RMR and protecting the incumbent. This implies that around 7% of MFCN sectors that are near to railways may be impacted, which is significantly less than 7% of the total MFCN number of sectors . This number is further reduced as the coupling loss is expected to be higher than calculated, e.g. due to ground occupancy In urban areas.. Other factors reducing the probability of interference are the effects of terrain, vegetation, clutter, the temporal nature due to train speed and the uplink orientation of the FRMCS TDD frame.

Observation 7: The number of MFCN sectors that may be interfered by n101 BS will be less than around 7% of the MFCN sectors that are near railways due to the coupling loss expected to be higher than calculated, e.g. due to the effects of terrain, clutter, ground occupancy In urban areas, the temporal nature due to train speed and the uplink orientation of the FRMCS TDD frame. As the number of urban MFCN BS is over-represented in the above percentage the actual number of interfered MFCN BS will be lower.

For the high power UE the ECC Report 318 in the executive summary 0.4 High Power cab-radio states:

“For both 900 MHz and 1900 MHz bands, Monte Carlo studies based on SEAMCAT have been conducted and show that the interference from FRMCS cab-radio of 31 dBm output power to MFCN uplink is acceptable when uplink power-control is implemented and activated and with unwanted emissions as described in Annex 10.

Annex 9 provides a worst-case analysis based on an MCL calculation for the case without FRMCS cab-radio uplink power control. It concludes that this could result in harmful interferences unless unwanted emissions from cab-radio would be reduced to -53 dBm/MHz in the 880-915 MHz and 1920-1980 MHz frequency ranges.

FRMCS cab-radios shall therefore implement and activate uplink power control in the 900 MHz and 1900 MHz band. FRMCS high-power cab-radios are not permitted to operate without uplink power control.”.

The Commission Implementing Decision (EU) 2021/1730 in its part C technical conditions for wideband RMR in 1900-1920MHz (TDD) band and also the ECC Decision(20)02 in its Annex 3: least restrictive technical conditions for wideband RMR in 1900 – 1910 MHz (TDD) state that:

“Uplink power control is mandatory and shall be activated”.

Observation 8: From ECC Report 318 it is clear that ECC has performed Monte Carlo studies that show that the interference from FRMCS cab-radio of 31 dBm output power to MFCN uplink is acceptable when uplink power-control is implemented and activated.

Observation 9: from both the Commission Implementing Decision (EU) 2021/1730 and the ECC Decision(20)02 it is clear that Uplink power control is mandatory and shall be activated.

The ECC, CEPT and EU documents all have all been processed through their standard approval process steps:

- ECC Report 318 has been subject to the public consultation phase that was completed in ECC PT1#65 - June 2020 and subsequently approved in the 53rd ECC Plenary – June/July - 2020;

- ECC Decision (20)02 and CEPT Report 76 have been subject to the public consultation phases that were completed in WGFM#97 - October 2020 and subsequently approved in the 54th ECC Plenary - November 2021;

- Commission Implementing Decision (EU) 2021/1730 has been subject to the written procedure in the RSC#75 - July 2021.

Observation 10: The Commission Implementing Decision (EU) 2021/1730, the ECC Decision(20)02, The ECC Report 318 and the CEPT report 76 all have been subject to their regular consultation phases. These have not amended the assumption that base stations providing electronic communications services, which use frequencies above 1 920MHz for reception under Commission Implementing Decision (EU) 2020/667(4), have enhanced selectivity compared to the current Harmonised European Standards. Base stations providing electronic communications services, which are located in the vicinity of a RMR base station and do not meet the enhanced selectivity criterion, should, where necessary, be adapted, in order to mitigate harmful interference.

## Conclusion

This contribution provides an analysis of the co-existence between FRMCS BS in band n101 and existing MFCN networks, and the European regulation for the 1900-1910MHz band. It has drawn the following observations:

Observation 1: The technical conditions specified in ECC Decision (20)02 [5], i.e. EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver.

Observation 2: The technical conditions specified in ECC Decision (20)02 [5], i.e. the BS rated output power using corresponding assumptions to convert from the EIRP limits, the resulting emission values of the transmitter and the specifications of the receiver have been transferred to the 3GPP specifications.

Observation 3: In the Commission Implementing Decision (EU) 2021/1730, ECC Decision(20)02, CEPT Report 76 as well as ECC Report 318 [7] it is clearly stated that the regulation has assumed that the harmonised technical conditions for RMR (FRMCS) base stations operating in the 1900-1910MHz band assume that base stations providing electronic communications services, which use frequencies above 1920MHz for reception under Commission Implementing Decision (EU) 2020/667(4), have enhanced selectivity compared to the current Harmonised European Standards. Base stations providing electronic communications services, which are located in the vicinity of a RMR base station and do not meet the enhanced selectivity criterion, should, where necessary, be adapted, in order to mitigate harmful interference.

Observation 4: The ECC in Report 318 has considered that additional mitigation techniques need to be implemented on a case-by-case basis, such as adjustments of antenna directivity, azimuth, tilt, or improve the selectivity of the MFCN BS in the vicinity of the railway tracks. Such site engineering solutions have been considered by RAN 4 for TDD and FDD coexistence scenarios as described in TR 25.942 [11] section 8.4.

Observation 5: From CEPT report 76, ECC DEC(20)02 and ECC Report 318 it is clear that Operators of mobile networks in 1920-1980 MHz should have, sufficiently far in advance, information on the rollout of a new RMR BS in 1900-1910 MHz.

Observation 6: In response to the EC/CEPT regulation for the 1900-1910MHz band ETSI has specified enhanced selectivity requirements for protection of Band 1 BSs.

Observation 7: The number of MFCN sectors that may be interfered by n101 BS will be less than around 7% of the MFCN sectors that are near railways due to the coupling loss expected to be higher than calculated, e.g. due to the effects of terrain, clutter, ground occupancy In urban areas, the temporal nature due to train speed and the uplink orientation of the FRMCS TDD frame. As the number of urban MFCN BS is over-represented in the above percentage the actual number of interfered MFCN BS will be lower.

Observation 8: From ECC Report 318 it is clear that ECC has performed Monte Carlo studies that show that the interference from FRMCS cab-radio of 31 dBm output power to MFCN uplink is acceptable when uplink power-control is implemented and activated.

Observation 9: from both the Commission Implementing Decision (EU) 2021/1730 and the ECC Decision(20)02 it is clear that Uplink power control is mandatory and shall be activated.

Observation 10: The Commission Implementing Decision (EU) 2021/1730, the ECC Decision(20)02, The ECC Report 318 and the CEPT report 76 all have been subject to their regular consultation phases. These have not amended the assumption that base stations providing electronic communications services, which use frequencies above 1 920MHz for reception under Commission Implementing Decision (EU) 2020/667(4), have enhanced selectivity compared to the current Harmonised European Standards. Base stations providing electronic communications services, which are located in the vicinity of a RMR base station and do not meet the enhanced selectivity criterion, should, where necessary, be adapted, in order to mitigate harmful interference.

From the above observations the following conclusions are drawn:

- That the harmonised technical conditions of the European regulation for the 1900-1910MHz band have been transferred to the 3GPP specifications;

- That base stations providing electronic communications services, which are located in the vicinity of a RMR base station and do not meet the enhanced selectivity criterion, should, where necessary, be adapted, in order to mitigate harmful interference;

- That additional mitigation techniques need to be implemented on a case-by-case basis, and such site engineering solutions have been considered by RAN 4;

- That operators of mobile networks in 1920-1980 MHz should have, sufficiently far in advance, information on the rollout of a new RMR BS in 1900-1910 MHz;

- that the interference from FRMCS cab-radio of 31 dBm output power to MFCN uplink is acceptable when uplink power-control is implemented and activated;

- that the real number of potentially interfered MFCN BS near railways is expected to be less than around 7% ;

- that the regulation for FRMCS in 1900-1910 MHz has gone through its usual consultation steps and this has confirmed conclusions 2 and 5.

Based on the above, the UIC is of the opinion that no change to the current 3GPP specifications for RMR base stations nor for the high power UE in band n101 should be done. Any such change would be contrary to the European regulation.

### References:

[1]	RP-233879, “Harm to existing mobile networks caused by band n101”, Vodafone, Telecom Italia, Telefónica, Orange, Deutsche Telekom, Telia Company, KPN, Bouygues Telecom, Fraunhofer IIS.

[2]	RP-234008, “Way forward on the co-existence for existing mobile networks caused 	by band n101”, RAN4 chair (Huawei).

[3]	RP-211542, “Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR)”, UIC, Ericsson.

[4]	3GPP TR 38.852 v17.3.0, “Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR)”.

[5]	ECC Decision (20)02: "Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR)".

[6]	RP-220376, “RAN4 CRs for Open REL-17 NR or NR+LTE WIs - Batch 33”, RAN4.

[7]	ECC Report 318, “Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band”, 2020-07.

[9] 	Commission implementing decision (EU) 2021/1730 of 28 September 2021 on the harmonised use of the paired frequency bands 874,4-880,0 MHz and 919,4-925,0 MHz and of the unpaired frequency band 1 900-1 910MHz for Railway Mobile Radio (notified under document C(2021) 6862

[10] 	CEPT report 76. Report from CEPT to the European Commission in response to the Mandate on spectrum for the future railway mobile communications system (Task 5). Report approved on 20 November 2020 by the ECC

[11]	3GPP TR 25.942 v17.0.0, “Radio Frequency (RF) system scenarios”.

[12]	ETSI TS 103 807 V1.1.1 (2021-10) Mobile Standards Group (MSG). IMT Cellular Networks Base Stations (BS) Additional Regulatory Requirements.