3GPP TSG-WG4 Meeting #110	 R4-2402411

# Athens, Greece, February 26 – March 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2159 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_SL_enh2)CR to 38.101-1 on SL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For channel bandwidth of sidelink CA, only the configuration of 10MHz+10MHz, and 30MHz+40MHz were requested by companies. The rest configurations may not be necessary and should be removed. - For MPR of sidelink CA, the requirements for contiguous RB allocation was specified and is sufficient for Rel-18. - The MPR for sidelink PSFCH transmissions was not captured in the spec with the form in the endoresed draft CR R4-2321781 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the channel bandwidth configuration of 20MHz+.., and 30MHz +30MHz in Table 5.3E.1A-1. - Remove [with contiguous RB allocation] in the title of Table 6.2E.2.1A-1 - Correct the form of MPR for sidelink CA PSFCH transmissions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature of sidelink CA would be captured inaccurately in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

5.3E.1A	Channel bandwidth for Sidelink CA

For NR SL CA operation, the SL CA channel bandwidths for each band are specified in Table 5.3E.1A-1. The same (symmetrical) channel bandwidth is specified for both the transmission and reception path.

Table 5.3E.1A-1 NR SL intra-band contiguous CA operating bands for SL CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47B | SL_n47B | 10 | 10 |  |  | 70 | 0 |
|  |  | 30 | 40 |  |  |  |  |

<Next Change>

6.2E.2.1A	MPR for sidelink CA

For SL intra-band contiguous CA of PSCCH and PSSCH simultaneous transmission with contiguous RB allocation, the allowed MPR for the maximum output power are specified in Table 6.2E.2.1A-1.

Table 6.2E.2.1A-1: MPR for power class 3 SL CA

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 6.5 | ≤ 7.0 |

The contiguous allocation rule of inner and outer for SL intra-band contiguous CA refers to that for NR intra-band contiguous CA in 6.2A.2.1 in TS38.101-1.

For SL intra-band CA of PSFCH with single RB transmission on each carrier, the required MPR are specified as follow.

MA_PSFCH =	2.5		; 0.00< R ≤ 0.3

=    7.5	; 0.3< R ≤0.5

=    12  	; 0.5< R ≤1.0

Where,

R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier.

When single S-SSB is transmitted on intra-band contiguous carriers, required MPR for single cell V2X in Table 6.2E.2.2-2 is reused. For two S-SSB transmissions on intra-band contiguous carriers, the required MPR are specified as follow.

Table 6.2E.2.1A-2: MPR for two S-SSB transmissions on intra-band contiguous carriers for power class 3

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 3.5 | 9.0 | 13.0 |

<End of Change>