3GPP TSG RAN WG4 Meeting #110	R4-2402831

Athens, Greece, 26 Feb. 2024 – 01 Mar. 2024

Agenda Item:	8.16.1.1.4

Source: 	Ericsson

Title:	Discussion on UE feature list

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the UE feature list for L1/L2 based inter-cell mobility (LTM).

# Discussion

In this contribution we provide our view on the some of the aspects of UE feature list on top of moderator’s input. We highlight the differences from the moderator provided list in yellow.

| Index | Feature group | Components | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 39-1 | L1-RSRP measurements for multiple cells with RTD > CP | Capability of L1-RSRP measurements for more than 1 Cells with RTD > CP in one frequency layer | Per UE | Note: the max number of cells is up to 39-3-1 or 39-3a-1 | Optional with capability signalling |
| 39-2 | Inter-frequency L1-RSRP measurements without gap | Capability of Inter-frequency L1-RSRP measurements without gap for LTM | [Per band/BC] |  | Optional with capability signalling |
| 39-3-1 | Number of neighbour cells to be measured per intra-frequency layer | Capability of number of neighbour cells to be measured for L1-RSRP per intra-frequency layer | [per UE / Per band] | Candidate values: 2, 3, [… TBD] | Mandatory with capability signalling if UE supports 45-1 |
| 39-3-2 | [Number of SSB resources for L1-RSRP measurement within a slot] | The max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement FFS: This capability may be discussed in RAN1? | Per band | Note: component 4 is also counted in FG 2-24 | Mandatory with capability signalling if UE supports 45-1 or 39-3 |
| 39-3-3 | [Number of SSB resources for L1-RSRP measurement per intra-frequency layer] | The max number of SSB resources for L1-RSRP measurement that UE can measure per intra-frequency layer FFS: define a value in 38.133 as L3 measurement or introduce a UE capability | Per UE |  | Mandatory with capability signaling if UE supports 45-1 |
| [39-3-4] | [Number of frequency layers for L1-RSRP measurement] | - The max number of frequency layers configured to measure for intra- and inter-frequency without gap L1-RSRP measurement - The max number of frequency layers configured to measure for inter-frequency L1-RSRP measurement with measurement gap | Per UE | Define the values in 38.133 as L3 measurement or introduce a UE capability | Mandatory with capability signaling if UE supports 45-1, 45-1a, 39-2 |
| 39-3a-1 | Number of neighbour cells to be measured for L1-RSRP inter-frequency layer | Capability of number of neighbour cells to be measured for L1-RSRP inter-frequency per frequency layer | [per UE / Per band] | Candidate values: 2, 3, [… TBD] | Mandatory with capability signaling if UE supports 45-1a |
| 39-3a-2 | [Number of SSB resources for L1-RSRP measurement per inter-frequency layer] | The max number of SSB resources for L1-RSRP measurement that UE can measure per inter-frequency layer FFS: define a value in 38.133 as L3 measurement or introduce a UE capability | Per UE |  | Mandatory with capability signaling if UE supports 45-1a |
| 39-4 | Interruption on DL slot(s) due to PDCCH- ordered RACH transmission | Capability on whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission | [Per band pair] (between the target band for RACH transmission and band under UE’s current band combo) per band combination |  | Optional with capability signalling |
| 39-4a | Interruption due to RF retuning for PDCCH- ordered RACH | Indicates the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE’s configured UL BWP(s) of active serving cells | [Per BC] | Candidate values for interruption length Y = 0.25, 0.5, 1 and 2 | Optional with capability signalling |
| 39-4b | RF and BB processing delay | Indicates the time required to retune RF and BB before transmitting PRACH | [Per BC] | Candidate values for [3, 5]ms | Optional with capability signalling |
| 39-5 | Early ASN.1 decoding and validity check before cell switch | For UE supporting [early ASN.1 decoding and validity/compliance check], the capability is applicable for the cells on which PDCCH order is received or TCI states activation is performed. The number of cells on which UE supports [early ASN.1 decoding and validity/compliance check], is a UE capability indication | [Per UE] | Candidate values for [1, TBD] | Optional with capability signalling |
| 39-6 | Shorter UE processing time during cell switch | Capability of reduced TLTM_processing delay (refer to TS 38.133)]. Note: Additional conditions to apply the capability is FFS in RAN4. Additional components may be introduced to this FG later | [Per UE] | Candidate values: [10ms] | Optional with capability signalling |

Proposal 1: Consider above table for feature list discussion

# Summary and Conclusion

In this contribution we have analysed UE feature list and made following proposals.

Proposal 1: consider above table for feature list discussion.

# References

- R1-2304276