3GPP TSG-RAN WG4 Meeting # 110											R4-2403638

Athens, Greece, Feb. 26th – Mar. 1st , 2024

Title: 	WF on NR UE power classes

Agenda Item:	12.2.1

Source: 	Huawei, HiSilicon

Document for:	Approval

# Background

It has been observed that:

Observation 1: RAN2 adopts the principle of capability inheritance. A fallback band combination shall not be reported if it has the same or lower capabilities as a parent band combination.

Observation 2: The UE may report fallback BCs having additional functionality, either by adding entries in FeatureSetCombination of the same BandCombination IE, or by separate BandCombination entries.

Observation 3: RAN2 spec states that:

The Network configures serving cell(s) and BWP(s) configuration to comply with capabilities derived from the combination of FeatureSets at the same position in the FeatureSetsPerBand, regardless of activated/deactivated serving cell(s) and BWP(s).

# Way Forward

.

## <General Aspects>

- For any DL CA with single-carrier UL, shall the UE mandatorily support the power class indicated in ue-PowerClass for the UL band if it’s applicable in the spec for the CA configuration?

Option 1: Yes

Option 2: No. It is optional, subject to the power class capability reported by the UE.

Option 3: Yes, except for Power Class 1.5 for which the UE shall at least meet the minimum requirements for Power Class 2.

WF: Option 2 or 3

- If the high power class (PC2 or PC1.5) applicability note as in Clause 5 is not applied for a BC in the TS, shall we allow a UE to indicate the high power class for this BC? If yes, how to verify the MSD requirements for this high power class?

Option 1: Yes, if the MOP requirements for the UL are specified. The UE shall meet the MSD requirements for a lower power class while transmitting at this high power class.

- The relevant changes to the TS need to be further discussed and agreed in order to enable such new requirements. FFS from which release.

- This does not stop the HPUE basket WIs from specifying the MSD requirements for the high power class.

Option 2: No. This would bypass the HPUE basket WI procedure and complicate conformance tests.

- Whether the UE max Tx power is affected by cell activation/deactivation?

Option 1:

No. The network determines the max Tx power for a UE based on the capabilities of the configured band combination, regardless of cell activation/deactivation, according to the current specifications.

Option 2:

As baseline, the network determines the max Tx power for a UE based on the capabilities of the configured band combination. As enhancement, FFS whether the max Tx power can change based on cell activation/deactivation in an open release.

Option 3:

Yes. Make necessary changes to the TS since Rel-17.

- If a BC is not explicitly reported, how to determine the power class for the BC as well as the power class(es) for the UL component band(s)?

Option 1: Follow RAN2’s principle of capability inheritance, and derive the power class capabilities from a parent BC;

Option 2: Define RAN4’s own rules and inform RAN2 when necessary.

Agreement: Option 1

For brevity, the impact on max Tx power from p-max, duty cycle, P-MPR and etc are omitted in the following discussions. Those factors should be included in the formulas for PCMAX,f,c as in the current spec.

## <Max Tx power PCMAX,f,c for UL inter-band CA>

- Inter-band UL CA (2UL2CC):

The max Tx power PCMAX,f,c for a UL component band is determined by:

Option 1:

- If ue-PowerClassPerBandPerBC-r17 is present, min(ue-PowerClassPerBandPerBC-r17, power class of this BC);

- Otherwise, min(ue-PowerClass, power class of this BC).

Option 2:

Please propose.

WF: Further check option 1

- Inter-band+Intra-band UL CA (2UL3CC):

The max Tx power PCMAX,f,c for a UL component band is determined by:

Option 1:

- If ue-PowerClassPerBandPerBC-r17 is present, min(ue-PowerClassPerBandPerBC-r17, power class of this BC);

- Otherwise:

  - min(ue-PowerClass, power class of this BC) for the single-carrier UL

  - ue-PowerClass for the intra-band CA UL

Option 2:

- If ue-PowerClassPerBandPerBC-r17 is present, min(ue-PowerClassPerBandPerBC-r17, power class of this BC);

- Otherwise:

  - min(ue-PowerClass, power class of this BC) for both ULs

Option 3:

- If ue-PowerClassPerBandPerBC-r17 is present, min(ue-PowerClassPerBandPerBC-r17, power class of this BC);

- Otherwise:

  - min(ue-PowerClass, power class of this BC) for the single-carrier UL

  - default power class (i.e. PC3 or PC5) for the intra-band CA UL

WF: further check option 2

## <Max Tx power PCMAX,f,c for Single-carrier UL with inter-band CA DL>

This type of BC is typically viewed as a fallback of a parent BC (e.g. having UL CA). Based on RAN2’s reporting rules, a UE is allowed to report a fallback BC if its power class capability is higher than the parent BC.

- The fallback BC is reported:

Since the BC is reported, ue-PowerClassPerBandPerBC-r17 or powerClass may be present.

The max Tx power PCMAX,f,c for the UL component band is determined by:

Option 1:

- ue-PowerClassPerBandPerBC-r17 if present

- Otherwise, power class of this BC.

Option 2:

Power class of this BC;

Option 3:

ue-PowerClass for the UL band

Agreement: Option 1

- The fallback BC is NOT reported due to RAN2 fallback rule:

Provided that this BC is still supported by the UE based on the reported parent BC, the max Tx power PCMAX,f,c for the UL component band is determined by:

Option 1:

the power class derived from a parent BC

Option 2:

ue-PowerClass for the UL band

Option 3:

min{ue-PowerCLass, powerclass for the parent BC}

Agreement: Option 1

## <Max Tx power PCMAX,f,c for Intra-band CA UL with Inter+Intra-band CA DL>

Similarly, this type of BC may be viewed as a fallback of a parent BC (e.g. having UL CA). Based on RAN2’s reporting rules, a UE is allowed to report a fallback BC if its power class capability is higher than the parent BC.

- The fallback BC is reported:

Since the BC is reported, ue-PowerClassPerBandPerBC-r17 or powerClass may be present.

The max Tx power PCMAX,f,c for the UL component band is determined by:

Option 1:

- ue-PowerClassPerBandPerBC-r17 if present

- Otherwise, power class of this BC.

Option 2:

Power class of this BC;

Option 3:

ue-PowerClass for the UL band

Option 4:

- ue-PowerClassPerBandPerBC-r17 if present

- Otherwise, ue-PowerClass of this band for intra-band CA

Agreement: Option 1

- The fallback BC is NOT reported due to RAN2 fallback rule:

Provided that this BC is still supported by the UE based on the reported parent BC, the max Tx power PCMAX,f,c for the UL component band is determined by:

Option 1:

the power class derived from a parent BC

Option 2:

ue-PowerClass for the UL band

Option 3:

default power class (i.e. PC3 or PC5) for the UL band

Option 4:

min{ue-PowerCLass, powerclass for the parent BC}

Agreement: Option 1

## <Other open issues>

FFS the following open issues:

- Max Tx power PCMAX,f,c for single-carrier UL with Intra-band CA DL

- Which MPRc and A-MPRc applies per serving cell c of a configured band combination?

## <Online agreements>

Agreement:

- For UE that is configured in the single carrier mode (1 DL + 1 UL on this band), the power class is determined by ue-PowerClass for this NR band.

Agreement:

- The RAN4 common understanding is the ue-PowerClassPerBandPerBC-r17 capability can be used for 3Tx band combinations such as UL CA+TxD and UL CA+UL MIMO

# Reference

[1] R4-2401102 Topic summary for [110][143] NR_power_class	Moderator (Samsung)	RAN4#110