# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2404363

# Changsha, China, 15 April – 19 April, 2024

Agenda item:	6.16.7.1

Source:	Apple

Title:	On NR-NTN accuracy requirements in above 10 GHz bands

Document for:	Discussion

# Introduction

In RAN4#110 meeting, the NR-NTN RRM requirements in above 10 GHz bands have been discussed and WF was agreed in [1]. However, the measurement accuracy requirement has not been well discussed yet and it was not captured in the big CR [2]. In this contribution, we discuss the NR-NTN measurement accuracy requirements in above 10 GHz bands.

# UE UL timing requirement

The previous agreement and open issue for UE accuracy requirement over 10GHz is:

### Issue 2-12: Measurement accuracy

No agreement: (no discussion in this meeting) - Decide whether to remove an RF margin for different RX beams in the relative accuracy and tighten relative accuracy requirements.

Since the Rx beam sweeping is not specified for NTN over 10GHz, both intra-frequency and inter-frequency measurement happens on the same satellite. Regarding the intra-frequency/inter-frequency relative accuracy, since serving cell and neighbor cells are on the same satellite and single Rx beam is assumed in the delay requirement, that means same Rx beam is required for intra-frequency measurement and the RF margin for different Rx beams in this relative accuracy can be removed.

Proposal 1: remove an RF margin for different RX beams in the relative accuracy and tighten relative accuracy requirements.

Since the based band measurement for NTN above 10GHz and legacy FR2 are similar, we assume same absolute measurement accuracy requirement of TN FR2 can be applied for NTN UE above 10GHz.

Proposal 2: existing absolute measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz.

The TN measurement accuracy is defined as following example:

The parameter of SSB_RP has been specified in annex B of TS38.133, as following example:

The UE power class for legacy FR2 TN UE may not be applied for NTN UE above 10GHz. In UE RF spec session, one big CR has been agreed in last meeting in [3], and the UE class were defined quite different from legacy power class, as duplicated below,

Thus, instead of using UE power class, RAN4 shall define the minimum SSB_RP condition for accuracy requirement for 5 UE types specified in table 9.2.1.0-1 of TS38.101-5.

Proposal 3: instead of using UE power class, RAN4 shall define the minimum SSB_RP condition for accuracy requirement 5 five UE types specified in table 9.2.1.0-1 of TS38.101-5.

In the legacy TN FR2 accuracy requirement, the minimum SSB power is determined based on the following criteria:

In NTN above 10GHz, the same principle can be applied to derive the mini SSB_RP, which is based on the equation of Minimum SSB_RP = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n, where,

Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5 as following:

Y is the gain difference between fine and rough beams, which needs to be further discussed in RAN4; (the fine beam and rough beam gain for VSAT might be different from legacy TN FR2 UE)

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBP,n is 0 for NTN above 10GHz since CA/DC is not considered.

Proposal 4: RAN4 to define:

Minimum SSB_RP_NTN_FR2 = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n,

where,

Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5

Y is the gain difference between fine and rough beams, which needs FFS in RAN4

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBP,n is 0 for NTN above 10GHz since CA/DC is not considered.

# Conclusion

In this contribution, we discuss the NR-NTN measurement accuracy requirements in above 10 GHz bands.

Proposal 1: remove an RF margin for different RX beams in the relative accuracy and tighten relative accuracy requirements.

Proposal 2: existing absolute measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz.

Proposal 3: instead of using UE power class, RAN4 shall define the minimum SSB_RP condition for accuracy requirement 5 five UE types specified in table 9.2.1.0-1 of TS38.101-5.

Proposal 4: RAN4 to define:

Minimum SSB_RP_NTN_FR2 = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n,

where,

Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5

Y is the gain difference between fine and rough beams, which needs FFS in RAN4

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBP,n is 0 for NTN above 10GHz since CA/DC is not considered.

# References

[1] R4-2403492, WF on NTN RRM requirements, Qualcomm, RAN4 #110

[2] R4-2403572, Big CR to TS 38.133 on RRM requirements for NR NTN enhancement, Qualcomm, RAN4#110

[3] R4-2401116, Big CR on TS38.101-5 for UE RF Requirements, Samsung Electronics France SA, RAN4 #110