3GPP TSG-RAN WG4 Meeting #110bis	R4-2404377

Changsha, China, April 15-19, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance aspects on other NES features

Agenda item:	6.24.3.2

Document for:	Discussion

Introduction

According to the WID on network energy saving [1], the performance part started from RAN4#108bis meeting. In this paper, we will provide our views on RRM performance aspects on NES features other than SSB-less operation.

Discussion

Cell DTX/DRX

In last meeting, one open issue is if we shall define the test case for Cell DTX/DRX. In the core part discussion, it was agreed there is no impact on RRM requirements due to Cell DTX/DRX as most reference signals are not impacted by the non-active periods. However, it would be beneficial to verify the existing RRM requirements can still apply when Cell DTX/DRX is configured and activated. We would support defining at least one test case for Cell DTX/DRX. For instance, a test case can be defined for intra-frequency measurement with Cell DTX e.g. SA event triggered reporting test without gap under non-DRX and Cell DTX.

Issue 3-2-1: RRM performance impacts of Cell DTX/DRX

- Further discuss:

  - Option 2: No test case for Cell DTX/DRX.

    - Not support: E///, Nokia, ZTE

  - Option 3: Define one test case for Cell DTX: FFS the details

    - Option 3a: SA event triggered reporting tests without gap under Cell DTX.

    - QC, vivo, MTK, Apple, OPPO want to check whether option 3 is feasible.

Proposal 1: Define at least one test case for Cell DTX/DRX.

Proposal 2: Define a test case for intra-frequency measurement with Cell DTX e.g. SA event triggered reporting test without gap under non-DRX and Cell DTX.

NES-based CHO

RAN4 agreed on the handover delay requirement for NES-based conditional handover in RAN4#109 meeting. In the latest specification, the handover delay for NES-based conditional handover comprises measurement time delay which is defined based on if the DCI 2-X command comes before or after TEvent_DU + Tidentify_intra_with_index. Hence the test cases shall be defined for the two cases accordingly.

##### 6.1.4.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

For NES-based conditional intra-frequency handover:

-	If UE successfully decodes DCI 2-X command occurs earlier than TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index

-	If UE successfully decodes DCI 2-X command occurs later than TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-X command.

Proposal 3: Test cases shall be defined to verify the handover delay for NES-based conditional handover for the two cases where the DCI 2-X command comes before and after TEvent_DU + Tidentify_intra_with_index.

Conclusion

In this paper we have made the following proposals and observations related to the RRM performance aspects:

Proposal 1: Define at least one test case for Cell DTX/DRX.

Proposal 2: Define a test case for intra-frequency measurement with Cell DTX e.g. SA event triggered reporting test without gap under non-DRX and Cell DTX.

Proposal 3: Test cases shall be defined to verify the handover delay for NES-based conditional handover for the two cases where the DCI 2-X command comes before and after TEvent_DU + Tidentify_intra_with_index.

References

[1] RP-223540, New WID: Network energy savings for NR