# 3GPP TSG-RAN WG4 Meeting # 110bis														R4-2404990

# Changsha, China, 15-19 Apr, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2196 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-1 Modification for Lower MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Ambiguity on “highest power class for the given CA configuration”, there could be two interpretations, one is “highest supported power class for a UE for a given CA configruation”, the other is “highest power class specified in spec for a given CA configruation” |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change “Highest power class” to “Highest supported power class” to align with other part of the content 2. Remove the [] in IE name |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity exists |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change >

### 7.3A.7	Lower-MSD requirements for inter-band CA

A UE can report better MSD performance than the minimum requirements as specified in clause 7.3A.4, 7.3A.5 and 7.3A.6 by lowerMSD-r18 capability, except that the reporting for MSD caused by IMD with order higher than 5, IMD of UL intra-band CA or triple-beat is not supported in this release of the specification. The MSD performance after improvement is categorized into different lower-MSD capability classes, which are defined in Table 7.3A.7-1.

Table 7.3A.7-1: Lower-MSD capability classes

| Lower-MSD capability class | Maximum allowed actual MSD (i.e. Threshold) | Remark |
| --- | --- | --- |
| I | 0 dB | Actual MSD ≤ 0dB |
| II | 3 dB | Actual MSD ≤ 3dB |
| III | 6 dB | Actual MSD ≤ 6dB |
| IV | 9 dB | Actual MSD ≤ 9dB |
| V | 12 dB | Actual MSD ≤ 12dB |
| VI | 15 dB | Actual MSD ≤ 15dB |
| VII | 18 dB | Actual MSD ≤ 18dB |
| VIII | 22 dB | Actual MSD ≤ 22dB |

The reported lower-MSD capability classes are subject to the same uplink/downlink configurations as defined for the minimum MSD requirements in clause 7.3A.4, 7.3A.5 and 7.3A.6. If a UE can support more than one test points for a given REFSENS exception case, the reported lower-MSD capability class is applicable for the test point having the largest specified MSD value. Otherwise, it’s only applicable for the test point which can be supported by the UE. If one or multiple power classes are requested by the network, the UE can, if supported, report lowerMSD-r18 capability for the requested power classes; otherwise, the UE shall report lowerMSD-r18 capability for the highest supported power class for the given CA configuration.

The UE shall meet one of the following conditions in order to report lowerMSD-r18 capability for a given REFSENS exception case:

-	If the specified minimum requirement is tightly bounded by the range of a lower-MSD capability class (i.e, Thresholdi-1 < MSD ≤ Thresholdi, where i and (i-1) are two adjacent lower-MSD capability classes), the actual MSD shall be at least one-level lower (i.e., actual MSD ≤ Thresholdi-1); or

-	If the specified minimum requirement is larger than the maximum threshold (corresponding to lower-MSD capability class VIII), the actual MSD shall be no more than the maximum threshold.

Otherwise, the UE shall not report lowerMSD-r18 capability for this REFSENS exception case.

If the special MSD type “ALL” is indicated in the lowerMSD-r18 capability, the reporting conditions as specified above shall be met for each MSD type that has been specified in this release for the given CA configuration.

NOTE 1: 	The lowerMSD-r18 capability is verified by reusing the MSD test point parameters and only replacing the minimum MSD requirement value by the threshold of the reported lower-MSD capability class. UE indicating lower MSD capability shall indicate at least the power class in IE lowerMSD-r18 which is the same as the highest supported power class for the given CA configuration. And, similar to the specified MSD minimum requirements, only the highest supported power class or the power class required by the certification/regulation body per UL configuration is verified.

NOTE 2:	If the UE is equipped with four or eight Rx antenna ports for the victim band of the BC, the lowerMSD-r18 capability is verified with four or eight Rx antenna ports according to clause 7.2 under the condition mentioned above, but with the increased MSD values of ΔRIB,4R or ΔRIB,8R applied for the requirement based on the description in clause 7.3A.1.

<End of Change >