3GPP TSG-RAN WG4 Meeting # 110-bis					R4-2405055

China, Changsha, April 15 – April 19, 2024

Title:	TP to TR 38.898: Addition of PC2 for DC_40A_n78A and DC_40C_n78A

Source:	Nokia, nbn

Agenda item:	5.15.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.898 to add PC2 for DC_40A_n78A and DC_40C_n78A. DC_40A_n78A PC3 is completed as a fallback.

************************************* Start of TP*****************************************

## 5.x	DC_40_n78

### 5.x.1	Configurations

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

### 5.x.2	Maximum output power

Table 5.x.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_40A_n78A DC_40C_n78A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### 5.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Interference impact is identified as follows:

- The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

- The UL2/DL3 harmonic mixing falls into Rx frequencies of band 40.

- The UL3/DL2 harmonic mixing falls into RX frequencies of band n78.

- The 2nd, 3rd, 4th, and 5th order IMD do not fall into Rx frequencies of band 40 and n78 since both bands are TDD.

- Cross band isolation existing, the interference from n78 UL falls into band 40 DL.

New MSDs are defined in the following tables.

Table 5.x.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | 40 | 20 | 15 | 50 (RBstart=0) | 20 | 16.2 | NOTE 4 | UL2/DL3 |
| NOTE 4: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz |  |  |  |  |  |  |  |  |

Only DL BW 20 MHz is added in Table 5.x.3.2-1 as DL BW 5 MHz for UL band n78 and DL band 40 is already included in receiver harmonic mixing table for PC2 in TS38.101-3.

Table 5.x.3.2-2: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | 401 | 3350 | 100 | 30 | 270 (RBSTART=0) | 2397.5 | 5 | 6.5 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |

MSD due to cross band isolation for PC2 for UL band n78 and DL band 40 is already added to TS38.101-3. However, the value is too high (i.e., 11.6dB) which is apparently irrelevant given that band isolation for PC2 for: UL band n77 and DL band 41; UL band n78 and DL band 41; UL band n77 and DL band 40; UL band n77 and DL band 41 are all 6.5. Hence, a new MSD value, i.e., 6.5dB, is added to Table 5.x.3.2-2 for UL band n78 and DL band 40 accordingly.

### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

************************************* End of TP*****************************************