3GPP TSG-RAN WG4 Meeting #110bis	R4-2405341

# Changsha, China, April 15 – April 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | DraftCR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 38.863 to introduce some technical background for R18 NTN VSAT UE Tx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce some rationales and background for VSAT EIRP requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce some rationales and background for VSAT EIRP requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In current TR 38.863, the latest agreements for VSAT EIRP requirements are not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

7.4.2.3	Radiated transmitter characteristics for Ka band VSAT

7.4.2.3.1	VSAT maximum output power

There are many kinds of implementations for VSAT in Ka band. Based on the ITU regulation, VSAT could be fixed or mobile. The antenna beam steering types could be mechanical or electronic. Thus, the following VSAT types are considered.

Table 7.4.2.3.1-1: Assumptions of UE Types

| UE class | UE type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| Note 1: The UE types are assuming UE has only one antenna beam towards one satellite at a given time in this release. Note 2: The Mobile VSAT communicating with non-GSO is not considered in this release. |  |  |

As the 76.2dBm maximum peak EIRP is assumed for the adjacent coexistence studies, the 76.2dBm maximum peak EIRP was specified in Rel-18.

For type 3 in LEO600 scenario, at least 61dBm EIRP is assumed with the following UL budget.

Table 7.4.2.3.1-2: The calculation of UL budget for LEO600 scenario

|  | LEO600 |
| --- | --- |
| NTN VSAT EIRP (dBm) | 61 |
| Elevation angle (degree) | 30 |
| Slant range (km) | 1075 |
| center frequency (GHz) | 27 |
| freePathloss | 181.70 |
| SAN UL Received signal level at OTA (dBm) | -120.70 |

Referring to the endorsed draft CR R4-2318300, RAN4 has agreed to specify LEO SAN EIS with Range -120dBm to -129dBm. The 61dBm minimum peak EIRP can guarantee the uplink budget without failure.

For other types in GSO scenario, at least 70dBm EIRP is assumed with the following UL budget.

Table 7.4.2.3.1-3: The calculation of UL budget for GSO scenario

|  | LEO600 |
| --- | --- |
| NTN VSAT EIRP (dBm) | 70 |
| Elevation angle (degree) | 30 |
| Slant range (km) | 38611 |
| center frequency (GHz) | 27 |
| freePathloss | 212.80 |
| SAN UL Received signal level at OTA (dBm) | -142.80 |

RAN4 has agreed to specify GSO SAN EIS with Range -140dBm to -149dBm. The 70dBm minimum peak EIRP can guarantee most of scenarios without the uplink budget failure.

<<End of Change>>