# 3GPP TSG-RAN WG4 Meeting #110-bis                                                          R4-2405375

# Changsha, China, 15 April – 19 April, 2024

Agenda Item:	5.18.2

Source: 	CMCC, Huawei, HiSilicon, ZTE Corporation, Murata Manufacturing Co Ltd.

Title: 	(HPUE_FR1_FDD_NR_CADC_R18) TP for TR 38.850 to introduce PC2 CA_n8A-n41A with UL n8

Document for:	Approval

- Introduction

The HPUE request of CA_n8-n41 with PC2 UL n8 have been approved.

In this contribution, we will analyze the MSD studies for these band combinations.

- Reference

- . TR 38.850 Rel-18 High power UE (power class 2) for a single FR1 NR FDD band in UL of NR intra-band and inter-band CA/DC combinations with y bands downlink (y=1,2,3,4,5,6) and x bands uplink (x=1)

- TP to TR 38.850

## <<Start of Change for TR 38.850>>

  - 5.x	CA_ n8A-n41A

    - 5.x.1	UE maximum output power

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration |  | Uplink CA configuration or single uplink carrier |  | NR Band |  | Channel bandwidth (MHz) |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8A-n41A | n88 CA_n8A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |

    - 5.x.2	Reference sensitivity requirements

For PC3 CA_n8A-n41A, there are harmonic mixing MSD for this band combination:

3rd harmonic of band n8 UL fall into band n41 DL.

5.x.2.1	Reference sensitivity requirements with PC2 on n8 without TxD

For CA_n8-n41, this is the configuration and MSD for UL n8 with PC3 in TS 38.101-1.

Table 5.x.2.1-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n8 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 13 | NOTE 3 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 3.5 | NOTE 3 | UL3/DL1 direct-hit |

PC2 MSD for PC2 UL n8 without TxD is specified as below.

Table 5.x.2.1-2: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1 for UE not supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n8 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 15.9 | NOTE 2 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 6.2 | NOTE 2 | UL3/DL1 direct-hit |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |

5.x.2.2	Reference sensitivity requirements with PC2 on n8 with TxD

[TBD]

5.3.4	∆TIB and ∆RIB values

Void

## <<End Change for TR 38.850>>