# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2405385

Changsha, China, 15th – 19th April, 2024

Agenda Item:	6.20.1

Source: 	OPPO

Title: 	on Rel-18 maintenance work

Document for:	Approval

- Introduction

During the RAN4#110 meeting, a WF [1] for Rel-18 sidelink maintenance work has been agreed. Couple of the agreement are relevant to the Rel-19 sidelink package and with the agreed Rel-19 WID [2], we give further discussion on the maintenance work and try to finalize the Rel-18 work in time.

- Discussion

The agreement for the maintenance work has been captured as below for remaining NS values and the MPR for SL intra-band contiguous CA with non-contiguous RB allocations.

Issue 2-1-1: Remaining NS values for SL-U

<On-line Agreement>

- If SL is agreed as Rel-19 RAN4-led package and the remaining NS values are included, specify them in Rel-19. If not, specify them in Rel-18 maintenance.

Issue 3: MPR for SL intra-band contiguous CA with non-contiguous RB allocation

<Recommended WF>

- If SL CA is agreed as Rel-19 RAN4-led package, specify the corresponding MPR in Rel-19. If not, specify it in Rel-18 maintenance.

Further, the Rel-19 sidelink WID is captured below:

The core part of the work item includes:

- Specify the RF requirements for NR sidelink CA in n47:

  - Intra-band non-contiguous CA with power class 2 and power class 3, with Component Carrier (CC) combinations 10MHz + 10MHz and 10MHz + 20MHz

  - Intra-band contiguous CA with power class 2

- Introduce necessary changes for release independence specification, if identified

Note: Work for power class 3 starts first in RAN4. Whether there is any RRM core requirement to be specified will be confirmed in Q3 2024.

As can be seen above, the Rel-19 sidelink package has not included the NS values for SL-U as well as the MPR for intra-band contiguous CA with non-contiguous RB allocations.

Observation 1: The Rel-19 sidelink package has not included the NS values for SL-U as well as the MPR for intra-band contiguous CA with non-contiguous RB allocations.

In this case, it is suggested to keep the NS values for SL-U and the intra-band contiguous CA with non-contiguous RB allocation work within Rel-18 and should not influent the Rel-19 sidelink work..

Proposal 1: Keep the NS values for SL-U and the intra-band contiguous CA with non-contiguous RB allocation work within Rel-18 and should not influent the Rel-19 sidelink work.

Secondly, for the remaining NS values, as discussed also in the Rel-18 sidelink timeline, the NS values are too many as there are already 17 and 5 of them have been completed. From workload perspective, it is suggested to limit the work load for Rel-18 maintenance phase. If the total of 12 NS values are defined within 2 meetings, it will occupy too much time for sidelink interested companies and experts. From our understanding, the NS values defined up to Rel-17 can be finished in Rel-18 sidelink while the NS values defined in Rel-18 NR-U enhancement WID can be left for future. In this case, the Rel-17 finished NS values are NS 28,29.30.31; 53,54,58,59,60,61. This means we will further do NS 28,29,30 54,59 total 5 more considering the already finished NS values. This should be a reasonable workload.

Proposal 2: To finish the NS values defined up to Rel-17 of NR-U (NS 28,29,30 54,59) in Rel-18 maintenance timeline.

3	Conclusions

Observation 1: The Rel-19 sidelink package has not included the NS values for SL-U as well as the MPR for intra-band contiguous CA with non-contiguous RB allocations.

Proposal 1: Keep the NS values for SL-U and the intra-band contiguous CA with non-contiguous RB allocation work within Rel-18 and should not influent the Rel-19 sidelink work.

Proposal 2: To finish the NS values defined up to Rel-17 of NR-U (NS 28,29,30 54,59) in Rel-18 maintenance timeline.

4 References

[1] R4-2403868	 WF on NR sidlink UE RF requirements, LGE

[2] RP-240840 	New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band