# 3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2405435

# Changsha, China, 15th – 19th April, 2024

Agenda item:			6.16.7.1

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN RRM performance requirements in above 10 GHz bands

Document for:	Discussion

# Introduction

In this contribution, our views on impact of the beam type configuration for OTA test setting on above 10 GHz bands is provided.

# Performance requirements

### NTN bands above 10 GHz – Beam type impact

The test cases for above 10GHz have been discussed in WF [1]. However, one of specific test setting to be discussed in FR2-NTN is the RX beam type. Table 1 summarizes the FR2-NTN requirements and the corresponding FR2 TN SA test, as in A.7, TS 38.133. It can be observed the “Rough” beam type are used is the most cases in the current FR2 TN SA.

- Table 1: FR2-NTN requirements and the corresponding FR2 TN SA test in A.7, TS 38.133

| Test case | Comment on FR2-NTN requirement | Beam Assumption in current FR2 SA TN Test (A.7) |
| --- | --- | --- |
| RRC Idle and Inactive mobility in intra-satellite scenario | Follow FR1 TN core requirements but update test configurations | Rough |
| UL timing accuracy | New core requirements | Fine |
| L1-RSRP | Follow FR1 TN core requirements but update test configurations | Rough |
| RLM | Follow FR1 TN core requirements but update test configurations | Rough |
| L3 measurements in intra-satellite scenario | Follow FR1 TN core requirements but update test configurations | Rough |
| Intra-satellite Handover | Follow FR1 TN core requirements but update test configurations | [Rough] |
| Blind inter-satellite Handover | New core requirements | [Rough] |
| For relative accuracy for intra-frequency measurement FFS whether to define requirements for intra-sat only based on the assumption of same Rx beam. | Follow FR1 TN core requirements but update test configurations | Rough |

Observation 1: Rough beam type are used in the most of current TN FR2 tests.

However, there are two penitential issues for “rough” beam as the following.

- Issue #1: FR2 Minimum SSB_RP

- Issue #2: Noc calculation

Issue #1: FR2 Minimum SSB_RP

To use “rough” beam, the parameter Y and Z are required and depend on the UE power class.

Parameter Y is used for Minimum SSB_RP at Beam peak direction (i.e. typically serving cell):

Minimum SSB_RP = Reference sensitivity PC3, n260, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n

- Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction

| Value “Y” in dB, for each UE power class |  |  |  |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| FFS | 9.0 | 7.0 | FFS |

And Parameter Z is used Minimum SSB_RP at Spherical coverage directions (i.e., typically neighbor cell at Random direction)

- Minimum SSB_RP = EIS spherical coverage PC3, n260, 50MHz +Z -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBS,n,

- Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions

| Value “Z” in dB, for each UE power class |  |  |  |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| FFS | 9.0 | 7.0 | FFS |

However, for Ka band, the power class and the corresponding Y/Z needs to be discussed.

Issue #2: Noc calculation

The Noc setting depends on beam type (i.e. Fine beam or rough beam) and AOA directions (i.e. Beam peak or random direction). In other words, if the Y/Z are updated, the Noc calculation will need to be re-visited.

Observation 2: For rough beam type in Ka band, the power class and the corresponding Y/Z are missing for Ka band, and the Noc calculation needs to be re-visited.

Besides, the current OTA tests WIs are not covering Ka band.

Observation 3: The OTA test over Ka band has not been discussed and has not been covered in the current WI for OTA tests.

However, the beam direction could be projected based on Ephemeris in FR2-NTN, RAN4 need to further discuss whether to assume “rough” beam or the tests can be conducted with “fine” beams.

Proposal 1: RAN4 to further discuss the beam type for FR2-NTN OTA tests.

# Summary

The observations and proposals in this contribution are summarized:

Observation 1: Rough beam type are used in the most of current TN FR2 tests.

Observation 2: For rough beam type in Ka band, the power class and the corresponding Y/Z are missing for Ka band, and the Noc calculation needs to be re-visited.

Observation 3: The OTA test over Ka band has not been discussed and has not been covered in the current WI for OTA tests.

Proposal 1: RAN4 to further discuss the beam type for FR2-NTN OTA tests.

# Reference

- R4-2403492, WF on RRM requirements for NR_NTN_enh, RAN4#110, Feb. 2024.