3GPP TSG-RAN WG4 Meeting#110bis                              R4-2405631

Changsha, China, 15th – 19th April, 2024

Agenda item:	9.3.4

Source: 	ZTE Corporation

Title: 	Discussion on IMT parameters for 14800 to 15350 MHz

Document for:	Approval

Introduction

In the latest ITU-R meeting, WP5D has sent one LS to RAN/RAN4 to request the Parameters of terrestrial component of IMT in the following frequency ranges for sharing and compatibility studies. During the RAN#103 meeting, the corresponding SID for WP5D reply LS [3] is approved to facilitate the technical discussions in RAN4. In this contribution, we would like to share some views on IMT parameters for 14800-15350MHz.

Discussion

## 2.1. System parameters

For this frequency range 14800-15350MHz, similar as 6425-7125MHz and 10-10.5GHz, TDD operation should be the default assumption unless there are other operation mode (FDD or SDL or SUL) proposals from certain regions/countries or certain operators.

Proposal 1: assume TDD duplex mode as default assumption unless there are other operation mode proposals from certain regions/countries or certain operators.

Regarding the channel bandwidth and transmission bandwidth configuration, even though this frequency range might be targeted for the usage of both IMT-2020 and IMT-2030, this is still discussed under the 5G-A phase and it’s not clear on 6G system parameter assumption, therefore we propose to follow NR baseline assumptions for 14.8-15.35GHz, however the detailed channel bandwidth and its corresponding SCS need some further discussions.

First of all, based on the commercial network deployed so far in FR1 high bands (e..g higher than 2.5GHz), only 30kHz is used, therefore15KHz SCS is not relevant in this spectrum 14.8-15.35GHz from our understanding. For 15GHz, it might be mixture between FR1 and FR2-1 as following.

|  | Maximum channel bandwidth |  |  |
| --- | --- | --- | --- |
|  | 100MHz | 200MHz | 400MHz |
| 30KHz | [Yes] | N/A | N/A |
| 60kHz | [Yes] | [Yes] | N/A |
| 120KHz | [Yes] | [Yes] | [Yes] |

Regarding the applicable SCS and maximum channel bandwidth for this frequency range, we need to consider the following factors at least:

- The targeted deployment scenario

- The achievable BS antenna/power assumptions and UE antenna/power assumptions

- Others, e.g. the potential operators sharing these spectrum

- Regarding its targeted deployment scenario, if this is targeted to provide the wide area coverage instead of hotpot coverage, then 30kHz and 60kHz should be treated with high priorities since its longer CP could tolerate larger coverage. If the targeted scenario is still the hotpot coverage to boost the capacity in certain regions, then 60kHz and 120kHz could be treated as high priorities. More inputs/views from the operator are encouraged.

| Numerology | Subcarrier spacing | CP for long symbols | Distance | CP for other symbols | Distance |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 | 5.2us | 1560m | 4.69us | 1407m |
| 1 | 30 | 2.86us | 858m | 2.34us | 703m |
| 2 | 60 | 1.69us | 507m | 1.17us | 351m |
| 3 | 120 | 1.11us | 333m | 0.59us | 175m |

If the targeted deployment is wide area coverage, then to provide the comparable and competitive deployment with FR1 4GHz bands, at least 12dB higher EIRP value at BS side could be achieved to counteract the increasing pathloss at 15GHz as shown in the following figure.

Figure 1. FSPL comparison between 4GHz and 15GHz

In order to enable this larger BS EIRP performance at 15GHz, then larger BS antenna array size are expected to achieve better beamforming gain e.g. 4096 antenna elements (e.g. 33dBi beamforming gain) compared with192 BS antenna elements (e.g. 19.8dBi beamforming gain) at 4GHz. Another important factor should be gap of TRP or total conducted power between 4GHz and 15GHz. For 4GHz, its total transmission power per polarization could be up to 52dBm per 100MHz. In order to achieve similar conducted power as 4GHz, for 15GHz with 4096 antenna elements and also with FR2 BS hybrid beamforming architecture assumption, then its supposed transmission power per PA within beamforming chipset should be up to 18.9dBm which is much higher than the assumption used in Rel-15 (e.g. 7-10dBm per PA based CMOS assumption). Indeed its more practical conducted transmission power with 4096 antenna elements assumption should be 40-43dBm per polarization under the Rel-15 mature CMOS PA assumption. In other words, even though the beamforming gain at 15GHz with 4096 antenna elements could be achieve 12dBi beamforming gain to counteract the increasing pathloss, the maximum transmission power at 15GHz is still 10dB lower than that at 4GHz under the 100MHz assumption.

Observation 1: given the wide area coverage assumption for 15GHz as baseline, even with 4096 antenna elements implementation assuming FR2 hybrid architecture, its total transmission power might be still limiting factor to achieve similar coverage as 4GHz from BS perspective.

In order to mitigate the coverage gap between 15GHz and 4GHz, UE with phase antenna array (e.g. 2x2x2 antenna array) could help improve the coverage from both DL and UL coverage. 2x2 antenna assumption could basically offer around 10dBi array gain ideally (e.g. 5dBi antenna elements+6dBi beamforming gain).

Observation 2: UE with phase antenna array assumption could offer the additional help on mitigation the coverage gap between 4GHz and 15GHz.

In addition, if assuming 4096 antenna elements at BS side to offer the good coverage performance, due to its large antenna array size or aperture size, then the near field problem should be also taken into account. .e.g. 4096 antenna elements with 32x64x2 and its antenna element horizontal spacing is 0.5 lamda and its antenna element vertical spacing is 0.8 lamda, then its near/far filed boundary would be around 115.0m which is much larger than the minimum distance assumption in the existing WA BS class. In other words, for UEs within separation distance 35-115m, UE will be working under the spherical wave propagation instead of plane wave assumed in the past.

Observation 3: for BS with 4096 antenna elements assumption at 15GHz, both near filed problem should be taken into account according to the current WA BS class assumption.

For BS type 1-O and 2-O, BS classes are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

Another alternative solution is to follow the legacy IMT assumption for 6425-10500 MHz under the FR1 BS architecture, however due to the increasing operating frequency at 15GHz, then conducted power per element should be reduced accordingly e.g. 3dB further reduction on top of the assumption for 6425-10500 MHz, the exact value for 14.8-15.35GHz could be further discussed. In addition, considering the relatively high frequency range, then its antenna array size could be increased from 16x8 to 16x16 for WA/MR BS at least and 16x8 for Small cell BS.

|  |  |  |  | Rural | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |  |  |
| 1.2 |  | Element gain (dBi) (Note 1) |  | N/A | 6.4 | 5.5 | 5.5 | 5.5 |
| 1.3 |  | Horizontal/vertical 3 dB beamwidth of single element (degree) |  | N/A | 90º for H 65º for V | 90º for H 90º for V | 90º for H 90º for V | 90º for H 90º for V |
| 1.4 |  | Horizontal/vertical front-to-back ratio (dB) |  | N/A | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 |  | Antenna polarization |  | N/A | Linear ±45º | Linear ±45º | Linear ±45º | Linear ±45º |
| 1.6 |  | Antenna array configuration (Row × Column) |  | N/A | 16 × 16 elements | 16 ×16 elements | 16 × 8 elements | 4 × 4 elements |
| 1.7 |  | Horizontal/Vertical radiating element spacing |  | N/A | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.8 |  | Array Ohmic loss (dB) (Note 1) |  | N/A | 2 | 2 | ## 2 | 2 |
| 1.9 |  | Conducted power (before Ohmic loss) per antenna element (dBm) |  | N/A | [19] (Note 5) | [19] (Note 5) | [16] (Note 6) | [9] (Note 7) |
| 1.10 |  | Base station maximum coverage angle in the horizontal plane (degrees) |  | N/A | 120 | 120 | 120 | N/A (Note 8) |
| 1.11 |  | Base station vertical coverage range (degrees) (Note 3, 4) |  | N/A | 90-100 | 90-120 | 90-120 | N/A (Note 8) |
| 1.12 |  | Mechanical downtilt (degrees) (Note 4) |  | N/A | 6 | 10 | 10 | N/A (Note 8) |

From this alternative solution, even though the achievable EIRP from BS side at 15GHz could achieve similar performance as 6425-7125MHz, however from the propagation channel perspective, there are still 7dB gap observed at least. Similar logic as we considered before, UE with phase antenna array could be enabled to help improve the coverage on this frequency range.

Figure 2. FSPL comparison between 7GHz and 15GHz

Observation 4: UE with phase antenna array assumption could offer the additional help on mitigation the coverage gap between 7GHz and 15GHz.

In addition, if assuming 512 antenna elements at BS side to offer the good coverage performance, near/far filed problem might be not problematic g. 512 antenna elements with 16x16x2 and its antenna element horizontal spacing is 0.5 lamda and its antenna element vertical spacing is 0.8 lamda, then its near/far filed boundary would be around 9.0m which is within the minimum distance assumption (e.g. 35m) in the existing WA BS class.

## 2.2. Coexistence study

Similar as 6-10GHz study item in Rel-17, further coexistence studies for 15GHz are needed to investigate the necessary ACLR/ACS requirement etc. To follow the same principle,the following coexistence scenarios should be taken into account.

Table 2.2-1: Summary of considered scenario

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 15 GHz | Urban macro |  |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 15 GHz | Indoor hotspot |  |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 15 GHz | Dense urban | Down-prioritized |
| 4 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 15 GHz | Urban macro |  |
| 5 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 15 GHz | Indoor hotspot |  |
| 6 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 15 GHz | Dense urban | Down-prioritized |

Proposa1 2: for coexistence study for 14.8-15.35GHz, consider the coexistence cases as shown in Table 2.2-1 for further evaluation.

For the network layout, the existing assumption in TR 38.920 could be referred as baseline. Regarding cell radius for Wide area deployment and Medium range deployment, this might need more discussions since this assumption is highly dependent on the link budget (e.g.including both BS and UE assumption) or targeted deployment for this frequency range.

Proposa1 3: for the network layout,Propagation model,Transmission power control model,Received power model,ACLR and ACS modelling,Link level performance for 5G NR coexistence for 14.8-15.35GHz, consider the existing assumption in TR 38.921 as baseline and further discuss the cell radius for Urban Macro and Dense Urban scenario.

Regarding the noise figure at 15GHz, the following assumption in TR 38.921 or TR 38.820 could be used as starting point to trigger the initial coexistence study calibration.

Table 6.2.1-1: Typical noise figure for 7 – 24 GHz example frequencies [TR 38.921]

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 10 |
| 20 | 9 | 10 |

Proposa1 4: Regarding the noise figure at 15GHz, consider the NF value in Table 6.2.1-1 of TR 38.921 as starting point.

Regarding the BS transmission power and antenna assumption and also UE phase antenna array and its transmission power, this needs more discussions as we clarified in section 2.1 e.g. 512 antenna elements or 4096 antenna elements, whether UE is equipped with phase antenna array to increase the coverage to mitigate the gap between 15GHz and other lower bands.

Proposal 5: for antenna array and transmission power assumption of BS and UE, this need more discussions.

## 2.3. BS RF requirements

Regarding the BS RF requirements for 14800-15350MHz, it’s still premature to discuss all these requirements, however from our understanding, the following requirements should be discussed with high priorities:

- ACLR

- UEM, f_OBUE requirement

- NF

- ACS requirements;

- OOBB and f_OOBB requirement

- Proposal 6: prioritize the following BS RF requirements for the following discussions: ACLR, UEM, f_OBUE requirement, NF, ACS requirements, OOBB and f_OOBB requirements.

## 2.4. UE RF requirements

Regarding the UE RF requirement for 14800-15350MHz, similar as BS RF requirements, it’s still premature to discuss all these RF requirements, the following requirement should be discussed with high priorities:

- Maximum output power

- Power dynamic range

- Spectral mask

- ACLR

- Noise figure

- Sensitivity

- ACS

- Blocking

- Proposal 7: prioritize the following UE RF requirements for the following discussions: Maximum output power, Power dynamic range, spectral mask, ACLR, Noise figure, Sensitivity, ACS, Blocking.

Conclusions

In this contribution, we want to share some initial views on IMT parameters for 14800-15350MHz and proposals/observations are made as following:

Proposal 1: assume TDD duplex mode as default assumption unless there are other operation mode proposals from certain regions/countries or certain operators.

Observation 1: given the wide area coverage assumption for 15GHz as baseline, even with 4096 antenna elements implementation, its total transmission power might be still limiting factor to achieve similar coverage as 4GHz from BS perspective.

Observation 2: UE with phase antenna array assumption could offer the additional help on mitigation the coverage gap between 4GHz and 15GHz.

Observation 3: for BS with 4096 antenna elements assumption at 15GHz, both near filed problem should be taken into account according to the current WA BS class assumption.

Observation 4: UE with phase antenna array assumption could offer the additional help on mitigation the coverage gap between 7GHz and 15GHz.

Proposa1 2: for coexistence study for 14.8-15.35GHz, consider the coexistence cases as shown in Table 2.2-1 for further evaluation.

Proposa1 3: for the network layout,Propagation model,Transmission power control model,Received power model,ACLR and ACS modelling,Link level performance for 5G NR coexistence for 14.8-15.35GHz, consider the existing assumption in TR 38.921 as baseline and further discuss the cell radius for Urban Macro and Dense Urban scenario.

Proposa1 4: Regarding the noise figure at 15GHz, consider the NF value in Table 6.2.1-1 of TR 38.921 as starting point.

Proposal 5: for antenna array and transmission power assumption of BS and UE, this need more discussions.

- Proposal 6: prioritize the following BS RF requirements for the following discussions: ACLR, UEM, f_OBUE requirement, NF, ACS requirements, OOBB and f_OOBB requirements.

- Proposal 7: prioritize the following UE RF requirements for the following discussions: Maximum output power, Power dynamic range, spectral mask, ACLR, Noise figure, Sensitivity, ACS, Blocking.

References

- R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz.

- RP-210037, Reply LS to WP5D, Approved.

- RP-240787,New SI proposal: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz, Approved.

- TR 38.921

- https://www.itu.int/en/ITU-D/Regional-Presence/AsiaPacific/Documents/Events/2017/Aug-ISS2017/PAPER_Workshop_S3_Timur.pdf

- R4-2211224, Introduction of NR licensed band 6425 -7125 MHz, Approved.