3GPP TSG-RAN4 Meeting #110-bis	R4-2405723

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

Source: 	Qualcomm Incorporated

Title: 	Discussion on RRM performance requirement for fast CA/DC setup

Agenda item:	6.14.2.2

Document for:	Discussion

# Introduction

In this paper, we share our view on the remaining issues for fast SCell/SCG setup.

# Discussion

Since RAN4 define verification window for X, the test supposed to check whether either UE report the certain frequency carrier or UE shall not report certain carrier by changing RSRP. However, it is difficult to control UE measurement behavior especially when multiple frequency layers are configured for EMR. There is simple example with two FR2 frequencies configured for EMR CA/DC setup.

- UE A : measure band A first and then band B.

  - Band B is only valid from the validity check with X sec.

  - UE A will report band B.

- UE B : measure band B first and then band A.

  - Band A is only valid from the validity check with X sec.

  - UE B will report band A.

Depending on UE implementation, different UE can report different band for the same scenario. Therefore, it is very difficult to define the test case makes all UE have same results.

Even if the test case has single EMR frequency for test, it is difficult to define test as we discussed during the last meeting. RSRP variation-based test scenario is not definable because msg1 transmission occasion is unknown to TE.

Issue 2-3: Whether define test case for Improvement on SCell/SCG setup delay FFS the following options: - Option 1: do not define test case (QC) - Msg1 transmission occasion is unknown to TE. RSRP variation from X sec before T1 based test design is not definable. - If test is for validating UE not to report, TE does not differentiate wrong UE who is doing nothing and good UE who is performing validity check. Both good and wrong UE will pass by not reporting. - Option 2: define test case

Proposal: Due to difficulties of defining a test case and testability issue, RAN4 will not define the test case for R18 EMR feature.

# Conclusions

Proposal: Due to difficulties of defining a test case and testability issue, RAN4 will not define the test case for R18 EMR feature.

# References

[1] R4-2403548, WF on R18 Further NR mobility enhancement – part 2, Apple, RAN4 #110