3GPP TSG-RAN Meeting #110bis	R4-2405820

# Changsha, China, April 15th – 19th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_redcap_enh-Core) Draft CR to NR Measurements on Higher Priority Carriers in RRC_INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When configured eDRX_INACTIVE cycle is either a short or very long time period, UE shall search every layer of higher priority at least every Thigher_priority_search by using the LTE requirements. An extended search for higher priority carriers shall be defined when configured eDRX_INACTIVE has a medium time period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The equation for Thigher_priority_search for RRC_INACTIVE state is revised to align the definition in 36.133 when eDRX_INACTIVE ≤ 10.24 sec or >163.84 sec. Thigher_priority_search also defined with an extended time period when 10.24 sec < eDRX_INACTIVE cycle ≤ 163.84 sec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification may cause misinterpretation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1B.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

#### 5.1B.2.7	General requirements

The requirements in sub-clause 4.2B.2.7 shall apply when the UE is not configured with eDRX_INACTIVE.

When configured with eDRX_INACTIVE, the UE shall search every layer of higher priority at least every Thigher_priority_search = MAX(60 * Nlayers, one eDRX_INACTIVE cycle) seconds if eDRX_INACTIVE cycle ≤ 10.24 seconds or eDRX_INACTIVE cycle > 163.84 seconds; Thigher_priority_search = max(60, 1*eDRX_INACTIVE cycle length) * Nlayers seconds if 10.24 seconds < eDRX_INACTIVE cycle ≤ 163.84 seconds; where Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information.

--- End of change 1 ---