3GPP TSG-RAN WG4 Meeting #110bis												R4-2406072

Changsha, China, 15th – 19th April, 2024

Agenda item:			6.9.3

Source:	vivo

Title:	WF for [110bis][336] NR_FR1_TRP_TRS_enh

Document for:	Approval

Introduction

This is Way Forward for Rel-18 FR1 TRP TRS WI.

Topic #1: Test methodology related issues

Sub-topic 1-1 Single-layer UL-MIMO TRP test method

Issue 1-1-1: Performance metric for Coherent UE support multiple TPMI index 2~5

Proposed Agreements:

RAN4 consider both option 1 and option 2 as performance metric for Coherent UL-MIMO. In future requirement work, if started in Rel-19 based on available commercial UEs, further selecting option 1 and option 2 to specify Rel-19 requirements is FFS.

Topic #2: Rel-18 TRP TRS requirements related issues

Open issues summary

Sub-topic 2-1 Rel-18 AC lab alignment activity

Issue 2-1-1: Final Analysis and conclusion of RAN4 Rel-18 AC Lab alignment activity

Agreement:

Based on analysis in R4-2404644 and agreed pass fail limits, 3GPP Rel-18 FR1 TRP TRS AC lab alignment activity for BHH can be successfully concluded within the 8 labs with anechoic chamber system are well aligned.

Sub-topic 2-2 RC Harmonization and lab alignment

Issue 2-2-1: Updated RC Lab alignment outcome

Agreement:

  - based on the analysis in R4-2404644, 3GPP Rel-18 FR1 TRP TRS RC lab alignment activity can be successfully concluded (phase 3):

- 6 labs with Reverb chamber system are well aligned at n78

- 5 labs with Reverb chamber system are well aligned at n28

Issue 2-2-2: Updated RC Lab alignment schedule

Agreement:

Conclude final Rel-18 RC lab alignment activity in RAN4#111 meeting, with receiving the final results on RC from Samsung lab.

Issue 2-2-3: RC vs AC harmonization criteria

Agreements:

Average of AC value per-band per-device in R4-2404644 should be reference value. Pass fail limits as 0.75*Total harmonization MU. The MU value will be decided next meeting.

Sub-topic 2-3 additional CBW for band n28/n41/n77/n78 requirements

Issue 2-3-1: How to scale the defined large CBW to narrow CBW requirements?

Tentative Agreement:

Option 1: REFSENS RB scaled factor, or Option 2: 7dB, as the basic scaling factor for Band n41/77/78, additional allowance value on top of it is also considered, the exact value is FFS. Need input from companies.

Issue 2-3-2: How to reflect the requirements for additional CBW in spec?

Agreements:

The core specification shall only list the requirement based on the channel bandwidth for which performance campaign has been carried out. A specific NOTE indicating derivation/scaling of the requirement for additional channel bandwidths can be listed.

Issue 2-3-3: Feasibility of single point offset method for different CBW measurements of same band

Agreements:

Testing result preliminary confirmed the feasibility of single point offset method for different CBW measurements of same band.

Issue 2-3-4: detailed test parameters for additional CBW

Agreements:

RAN4 to confirm the Option 1 (Case B) (i.e., aligned with test frequencies defined in TS 38.508-1) as the test parameter for additional CBW.

Sub-topic 2-4 Rel-18 TRP TRS requirements work

Issue 2-4-1: TT for Rel-18 TRP TRS

Agreements:

  - Follow the same approach in Rel-17, reuse factor of 0.62. The recommended TT is as following:

    - TT value is recommended as a ratio of MU, where TT=0.62*MU; according to currently available analysis of MU of AC system in RAN4, these TT values should be:

      - For browsing mode: TRP TT is 1.1dB for above 3GHz bands and 1dB for below 3GHz bands, TRS TT is 1.3 dB for above 3GHz bands and 1.2 dB for below 3GHz bands

      - For talk mode: TRP TT is 1.2dB for above 3GHz bands and 1.1dB for below 3GHz bands, TRS TT is 1.5 dB for above 3GHz bands and 1.4 dB for below 3GHz bands

Issue 2-4-2: Reply LS to RAN5 on TT for Rel-18

Tentative agreements:

Agree the LS to RAN5 in rev of R4-2404648

Issue 2-4-3: Measurements plan shared by test labs for performance measurement campaign

Agreement:

Suggest the test labs to finalize the testing for both TRP and TRS case at same device, and also talk mode and browsing mode. Rapporteur to check with MCC this week about the latest UE information collection status and share with group.

Issue 2-4-6: How to define TRP PC3 requirements based on PC2

Proposed Agreements:

TRP(PC3) = TRP(PC2) – 2.5/2.7/3dB for TDD bands.