3GPP TSG-RAN WG4 Meeting #110bis	R4-2406350

Changsha, China, April 15-19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for FR1-FR1 to FR1-FR1 CHO+CPC test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test cases for Rel-18 CHO enhancements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases are added according to the work split for: - A.6.3.3.X1 Intra-frequency conditional handover including target MCG and candidate SCG for CPC – FR1-FR1 NR-DC to FR1-FR1 NR-DC handover Additionally, as RAN4 agreed to introduce one test case where CHO only configuration is provided, this draft CR also proposes a FR1-FR1 to FR1 intra-frequency conditional handover test case where the UE is expected to proceed with CHO without CPC: - A.6.3.3.X2 Intra-frequency conditional handover including target MCG and candidate SCG for CPC – FR1-FR1 NR-DC to FR1-FR1 NR-DC handover with complementary conditional handover configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 requirements are not tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.3.3.X1, A.6.3.3.X2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2404792 |  |  |  |  |  |  |  |  |

<< Change 1 >>

#### A.6.3.3.X1	Intra-frequency conditional handover including target MCG and candidate SCG for CPC – FR1-FR1 NR-DC to FR1-FR1 NR-DC handover

##### A.6.3.3.X1.1	Test Purpose and Environment

This test is to verify the requirement for conditional handover including target MCG and candidate SCG for CPC in FR1 NR-DC specified in clause 6.1.7.1. This test verifies the requirements for PCell conditional handover delay in section 6.1.7.1.1 and PSCell conditional change delay in section 6.1.7.1.2 for FR1-FR1 NR-DC to FR1-FR1 NR-DC CHO with CPC.

##### A.6.3.3.X1.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.X1.2-1. Conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.X1.2-2 and A.6.3.3.X1.2-3. Conditional PSCell change delay and interruption length are tested by using the parameters in A.6.3.3.X1.2-2 and A.6.3.3.X1.2-4.

The test consists of three successive time periods, with time durations of T1, T2, and T3, respectively.

At the start of time duration T1, the UE is connected to cell 1 (source PCell) and cell 2 (source PSCell). The UE may not have any timing information of cell 3 (target PCell) and cell 4 (target PSCell).

TE shall configure a condition implying conditional handover to cell 3 with a condition implying conditional PSCell change to cell 4 during T1, at a time earlier than TRRC before the beginning of T2.

At the start of T2, Cell 3 becomes detectable. At the start of T3, Cell 4 becomes detectable. UE meets the handover condition and meets the PSCell change condition during T3.

Table A.6.3.3.X1.2-1: Intra-frequency conditional handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cells (PCell and PSCell): NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cells (PCell and PSCell): NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.X1.2-2: General test parameters Intra-frequency conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cells |  | PCell: Cell 1 PSCell: Cell 2 |  |
|  | Neighbouring cells |  | Cell 3, Cell 4 |  |
| Final condition | Active cells |  | PCell: Cell 3 PSCell: Cell 4 |  |
| A3-Offset |  | dBm | 0 | A3-offset is applied for conditions for both CHO and CPC |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |
| T3 |  | s | 2 |  |

Table A.6.3.3.X1.2-3: Cell specific test parameters for FR1-FR1 CHO (Cell 1 and Cell 3)

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | 5 | 5 |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | 5 | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -94 | -Infinity | -93 | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -91 | -Infinity | -90 | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 | -70.05 | -63.85 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -58.49 | -63.94 | -57.75 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table A.6.3.3.X1.2-4: Cell specific test parameters for FR1-FR1 CPC (Cell 2 and Cell 4)

| Parameter |  |  | Unit | Cell 2 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 2 |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -94 | -Infinity | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -91 | -Infinity | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 | -70.05 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -58.49 | -63.94 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.6.3.3.X1.3 Test Requirements

TRRC + TEvent_DU occurs during T1 and T2, as the conditional handover condition for cell 3 becomes satisfied at the start of T2, and the conditional PSCell change condition for cell 4 becomes satisfied at the start of T3. The test shall verify that there are no interruptions during T1 and T2. The UE shall not send PRACH to cell 3 before the end of T2.

The UE shall start to transmit PRACH to Cell 3 less than max (Tmeasure_PCell, Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆_PCell + TPCell_DU + 2 ms = (800 + 10 + 25 + 20 + 20 + 2) ms = 877 ms from the start of T3 and the interruption during T3 shall not exceeed Tprocessing + T∆_PCell + TPCell_DU + 2 ms = (25 + 20 + 20 + 2) ms = 67 ms

The UE shall start to transmit PRACH to Cell 4 less than max (Tmeasure_PCell, Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆_PSCell + TPSCell_DU + 2 ms = (800 + 10 + 25 + 20 + 20 + 2) ms = 877 ms from the start of T3. The interruption during T3 shall not exceed Tprocessing + T∆_PSCell + TPSCell_DU + 2 ms = 25 ms + 20 + 20 ms + 2 ms = 67 ms.

<< Change 2 >>

#### A.6.3.3.X2	Intra-frequency conditional handover including target MCG and candidate SCG for CPC – FR1-FR1 NR-DC to FR1-FR1 NR-DC handover with complementary conditional handover configuration

##### A.6.3.3.X2.1	Test Purpose and Environment

This test verifies the requirement for conditional handover when the UE is configured with target MCG and candidate SCG for CPC in FR1 NR-DC and additionally configured with a complementary CHO configuration in FR1. The test verifies that the UE makes the correct decision to proceed with CHO without CPC in case CHO condition is met without CPC condition being met, meanwhile verifying the conditional handover delay requirement in section 6.1.4.2.

##### A.6.3.3.X2.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.X2.2-1. Both conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.X2.2-2, and A.6.3.3.X2.2-3.

The test consists of two successive time periods, with time durations of T1 and T2 respectively.

At the start of time duration T1, the UE is connected to cell 1 (source PCell) and cell 2 (source PSCell). The UE may not have any timing information of cell 3 (target PCell) and cell 4 (target PSCell).

TE shall configure a condition implying conditional handover to cell 3 with a condition implying conditional PSCell change to cell 4 during T1, at a time earlier than TRRC before the beginning of T2. Additionally, the TE shall configure a complementary condition only implying conditional handover to cell 3.

At the start of T2, cell 3 becomes detectable. UE meets the handover condition during T2.

Table A.6.3.3.X2.2-1: Intra-frequency conditional handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cells (PCell and PSCell): NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cells (PCell and PSCell): NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.X2.2-2: General test parameters Intra-frequency conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cells |  | PCell: Cell 1 PSCell: Cell 2 |  |
|  | Neighbouring cells |  | Cell 3, Cell 4 |  |
| Final condition | Active cells |  | PCell: Cell 3 PSCell: Cell 4 |  |
| A3-Offset in condition for PCell |  | dB | -10 |  |
| A4-Offset in condition for PSCell |  | dB | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.X2.2-3: Cell specific test parameters for FR1-FR1 CHO (Cell 1 and Cell 3)

| Parameter |  |  | Unit | Cell 1 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.6.3.3.X2.2-4: Cell specific test parameters for FR1-FR1 CPC (Cell 2 and Cell 4)

| Parameter |  |  | Unit | Cell 2 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 2 |  | 2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.3.3.X2.3 Test Requirements

TRRC + TEvent_DU occurs during T1 as the conditional handover condition for cell becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 3 less than Tmeasure + Tinterrupt + TCHO_execution = 800 +62 +10=872ms from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms.

The UE shall not send PRACH to Cell 4.

<< End of Changes >>