# 3GPP TSG-RAN WG4 Meeting #110bis	                                                       R4-2406356

# Changsha, China, 15 April – 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR | DraftCR | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TC set 1&2) DraftCR on test cases for L1 and intra-frequency measurement without gap for option B-1-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BWP_wor-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The feature for the support of bandwidth part operation without restriction is introduced in Rel-18 and the corresponding test cases should be clarified for option B-1-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the test cases for L1 and intra-frequency measurement without gap for option B-1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for L1 and intra-frequency measurement without gap for option B-1-1 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.1.X, A.4.6.1.X, A.4.6.4.X, A.5.5.1.X, A.5.6.1.X, A.5.6.3.X, A.6.5.1.X, A.6.6.1.X, A.6.6.4.X, A.7.5.1.X, A.7.6.1.X, A.7.6.3.X, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.4.5.1.X	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP

A.4.5.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when CD-SSB is outside active BWP. This test will partly verify the FR1 PSCell radio link monitoring requirements in clause 8.1.

The test environment is the same as in A.4.5.1.1 with following exceptions in Table A.4.5.1.1.1-2.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| DL dedicated BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.1.2 |
| UL dedicated BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.1.2 |

A.4.5.1.X.2	Test Requirements

The test requirements are the same as in A.4.5.1.1.2.

<End of Change 1>

<Start of Change 2>

A.4.6.1.X	EN-DC event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP

A.4.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of an event when CD-SSB is outside active BWP. This test will partly verify the intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

The test environment is the same as in A.4.6.1.1 with following exceptions in Table A.4.6.1.1.2-3.

| Parameter | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | T1 | T2 | T1 | T2 |
| Active DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.2 |  | DLBWP.1.2 |  |
| Active UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.2 |  | ULBWP.1.2 |  |

A.4.6.1.X.2	Test Requirements

The test requirements are the same as in A.4.6.1.1.3.

<End of Change 2>

<Start of Change 3>

A.4.6.4.X	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP

A.4.6.4.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of L1-RSRP measurement when CD-SSB is outside active BWP. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.4.6.4.1.1-1.

The test environment is the same as in A.4.6.4.1 with following exceptions in Table A.4.6.4.1.2-1.

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Dedicated BWP configuration | 1~6 |  | DLBWP.1.2 ULBWP.1.2 |

A.4.6.4.X.2	Test Requirements

The test requirements are the same as in A.4.6.4.1.3.

<End of Change 3>

<Start of Change 4>

A.5.5.1.X	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP

A.5.5.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when CD-SSB is outside active BWP. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

The test environment is the same as in A.5.5.1.1 with following exceptions in Table A.5.5.1.1.1-2.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| DL dedicated BWP configuration | Config 1, 2 |  | DLBWP.1.5 |
| UL dedicated BWP configuration | Config 1, 2 |  | ULBWP.1.5 |

A.5.5.1.X.2	Test Requirements

The test requirements are the same as in A.5.5.1.1.2.

<End of Change 4>

<Start of Change 5>

A.5.6.1.X	EN-DC event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP

A.5.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of an event when CD-SSB is outside active BWP. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.5.6.1.1.1-1.

The test environment is the same as in A.5.6.1.1 with following exceptions in Table A.5.6.1.1.1-3.

| Parameter | Unit | Config | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Active DL BWP configuration |  | 1~4 | DLBWP.1.1 |  | DLBWP.1.5 |  |
| Active UL BWP configuration |  | 1~4 | ULBWP.1.1 |  | ULBWP.1.5 |  |

A.5.6.1.X.2	Test Requirements

The test requirements are the same as in A.5.6.1.1.2.

<End of Change 5>

<Start of Change 6>

A.5.6.3.X	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP

A.5.6.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of L1-RSRP measurement when CD-SSB is outside active BWP. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.5.6.3.1.1-1.

The test environment is the same as in A.5.6.3.1 with following exceptions in Table A.5.6.3.1.2-1.

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.5 ULBWP.1.5 |

A.5.6.3.X.2	Test Requirements

The test requirements are the same as in A.5.6.3.1.3.

<End of Change 6>

<Start of Change 7>

A.6.5.1.X	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP

A.6.5.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when CD-SSB is outside active BWP. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

The test environment is the same as in A.6.5.1.1 with following exceptions in Table A.6.5.1.1.1-2.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.2 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.2 |

A.6.5.1.X.2	Test Requirements

The test requirements are the same as in A.6.5.1.1.2.

<End of Change 7>

<Start of Change 8>

A.6.6.1.X	SA event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP

A.6.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of an event when CD-SSB is outside active BWP. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

The test environment is the same as in A.6.6.1.1 with following exceptions in Table Table A.6.6.1.1.2-3.

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.2 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.2 |  |

A.6.6.1.X.2	Test Requirements

The test requirements are the same as in A.6.6.1.1.3.

<End of Change 8>

<Start of Change 9>

A.6.6.4.X	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP

A.6.6.4.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of L1-RSRP measurement when CD-SSB is outside active BWP. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.6.6.4.1.1-1.

The test environment is the same as in A.6.6.4.1 with following exceptions in Table A.6.6.4.1.2-1.

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.2 ULBWP.1.2 |

A.6.6.4.X.2	Test Requirements

The test requirements are the same as in A.6.6.4.1.3.

<End of Change 9>

<Start of Change 10>

A.7.5.1.X	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP

A.7.5.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when CD-SSB is outside active BWP. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

The test environment is the same as in A.7.5.1.1 with following exceptions in Table A.7.5.1.1.1-2.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.5 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.5 |

A.7.5.1.X.2	Test Requirements

The test requirements are the same as in A.7.5.1.1.2.

<End of Change 10>

<Start of Change 11>

A.7.6.1.X	SA event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP

A.7.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of an event when CD-SSB is outside active BWP. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.1.1.1-1.

The test environment is the same as in A.7.6.1.1 with following exceptions in Table A.7.6.1.1.1-3.

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.5 |  | DLBWP.1.5 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.5 |  | ULBWP.1.5 |  |

A.7.6.1.X.2	Test Requirements

The test requirements are the same as in A.7.6.1.1.2.

<End of Change 11>

<Start of Change 12>

A.7.6.3.X	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP

A.7.6.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of L1-RSRP measurement when CD-SSB is outside active BWP. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.7.6.3.1.1-1.

The test environment is the same as in A.7.6.3.1 with following exceptions in Table A.7.6.3.1.2-1.

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.5 ULBWP.1.5 |

A.7.6.3.X.2	Test Requirements

The test requirements are the same as in A.7.6.3.1.3.

<End of Change 12>