3GPP RAN WG4 Meeting #111		R4-2407064

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	7.10.3

Source:	Apple

Title:	On FR2 MIMO OTA measurement results

Release:	Rel-18

WI/SI:	NR_MIMO_OTA_enh-Perf

Document for:	Information

# 1	Introduction

The FR2 MIMO OTA Lab Alignment activity is concluded, RAN4 encourage aligned labs to provide FR2 MIMO OTA data points on band n261 to be included on performance requirement the pool of data [1].

This paper revisits the FR2 MIMOM OTA lab alignment results on [2] and provides anonymous FR2 MIMO OTA measurement result on band n261.

# 2	Discussion

After scrutinizing the measurement results from the lab alignment activity [3], Apple’s FR2 MIMO OTA system integrator confirmed that our results are 3 dB more optimistic due to the way the power is set at the center of the test zone. In the current setup, the power is set per polarization; however, 3GPP TS 38.151 [4] specifies the power to be set as the sum of two polarizations (section D.3.6 FR2 Power Validation). Hence, Apple’s FR2 MIMO OTA system integrator is proposing to add 3 dB to our MASC metric results, including RAN4 contributions, as a mitigation solution; in the meantime, a permanent solution will be provided.

Even though this new information will not change the outcome of the FR2 MIMO OTA lab alignment activity, it might be important to correct the inclusion of the proper PADs averaging results into the data pool as agreed on issue 2-3-2 [1].

Revisiting the FR2 MIMO OTA PAD alignment activity post-processing on [3], it was observed that the averaging method used to determine the baseline value per PAD is based on the linear logarithmic average (dB) instead of the linear power average (mW). Logarithmic averaging introduces an unnecessary error in the data analysis. Despite the minor difference in the quantities analysed and the fact that the outcome of this study is not altered, we would like to propose calculating the average values for performance requirements based on the technically correct approach, i.e., the linear power average in mW.

With the considerations above, the outcome of the FR2 MIMO OTA lab alignment can be viewed as follows:

# 3	Conclusion

Observation 1: 	Apple’s FR2 MIMO OTA lab alignment results need correction based on a technical issue found with the FR2 MIMO OTA integrated system.

Observation 2: 	The outcome of the FR2 MIMO OTA lab alignment activity remains the same.

Observation 3: 	The averaging method adopted during the lab alignment activity introduces an unnecessary error into the data analysis.

.

Proposal 1: 	Calculate the average values for performance requirements based on the technically correct approach, i.e., the linear power average in mW.

Proposal 2: 	3GPP RAN4 to accept the FR2 MIMO OTA data point on band n261 provided by Apple into the pool of data for performance requirement definition.

Proposal 3:	3GPP RAN4 to consider Apple’s additional n261 data points provided after the RAN4#111 TD submission deadline to be included in the FR2 MIMO OTA data pool.

# 4	References

- R4-2406083, Way Forward for [110bis][337] NR_MIMO_OTA_enh, CAICT, RAN4 #110-bis

- R4-2400194, On FR2 MIMO OTA Lab Alignment PADs measurement results, Apple, RAN4 #110

- R4-2405464, Analysis of 3GPP Rel-18 FR2 MIMO OTA lab alignment results, CAICT, RAN4 #110-bis

- TS 38.151 V17.7.0 (2024-03)