3GPP RAN WG4 Meeting #111		R4-2407094

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	12.2

Source:	Apple

Title:	Views on power class fallback issues

WI/SI:	TEI18

Release:	Rel-18

Document for:	Approval

# 1	Introduction

The concern of the power class fallback issues in current RAN4 specifications has been brought up in RAN4 #110 meeting [1] which could potentially cause misunderstanding on how the UE configured maximum output power can be applied, and that may result in double-counting the PCMAX power reduction as well as excessive allowance of MPR/A-MPR for HPUE. While the issue was generally acknowledged by most RAN4 companies in the past two RAN4 meetings, there was still no concrete way forward on how to mitigate this issue in current RAN4 specifications. In last RAN4 meeting, a way forward on “power class fallback issues” [2] was approved which provides the guidelines on how RAN4 specifications may be revised to mitigate the power class fallback issues. In this contribution, we share our views on the preferred option for each open issue as captured in the way forward and provide text proposals on how the current RAN4 specifications may be revised to mitigate the power class fallback issues.

# Discussion

## 2.1	Way forward on RAN4 specifications revisions to mitigate power class fallback issues

The following options for RAN4 specifications revisions to mitigate power class fallback issues have been considered in the way forward [2].

Option 1: Keep the texts for power class fallback behavior in MOP sections and change “apply all requirements for” a certain power class to a more restricted “apply maximum output power of” this power class (R4-2404660)

Option 2: Move the text descriptions on UL duty cycle and P-max conditions below the power class tables in “UE maximum output power” sub-clauses to ΔPPowerClass definitions in “Configured output power” sub-clauses (R4-2404186).

Option 3: Other options are not precluded.

Our preferred option is Option 2 as it is a cleaner solution which can effectively resolve the aforementioned power class fallback issue. In our view, the proper understanding on the UE behavior when UL duty cycle exceeds certain % number or when P-Max is 23 dBm or lower should be that UE configured maximum output power is capped below the UE power class capability via ΔPPowerClass or P-Max. Therefore, it makes more sense to keep this part of text descriptions in the same sub-clause of “configured transmitted power level” as the entire text descriptions are essentially used to define ΔPPowerClass but not PPowerClass.

If we chose Option 1 to keep the texts for power class fallback behavior in MOP sections, even changing the wording from “apply all requirements for” to a more restricted “apply maximum output power of”, it still implies PPowerClass would be changed to a lower power level than the indicated power class. As a result, PCMAX power reduction would still be double-counted and the power class fallback issue still remains.

Proposal 1: Choose Option 2 for RAN4 specifications revisions to mitigate the power class fallback issue.

## 2.2	Way forward for other power class fallback related issues

Issue 2.2-1: Should P-Max be included in the ΔPPowerClass definition?

Option 1: Yes

Option 2: No

In our view, P-Max should not be included in the conditions for defining ΔPPowerClass as ΔPPowerClass should only be related to UL duty cycle conditions. On the other hand, P-Max is already incorporated in the PCMAX formula as PEMAX, it does not seem to be very meaningful to include P-Max in the ΔPPowerClass definition.

Proposal 2: For Issue 2.2-1, choose Option 2 that P-Max should not be included in the ΔPPowerClass definition.

Issue 2.2-2: Is it agreeable to revise the PCMAX_L formula as below for single carrier and apply the similar change to other UL features?

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc, ΔPPowerClass)}

Option 1: Yes

Option 2: No

In our view, ΔPPowerClass should not be additive but parallel to other power reduction terms, such as MPR/A-MPR, otherwise, there would be excessive power reduction allowance. Therefore, we agree that the current PCMAX_L formula should be revised as proposed in the issue description.

Proposal 3: For Issue 2.2-2, choose Option 1 to revise the PCMAX_L formula as proposed in the issue description for single carrier and apply the similar change to other UL features.

Issue 2.2-3: How to describe power class fallback UE behavior during SRS transmission occasions for PC2 capable UE with txDiversity-r16 capability or PC1.5 capable UE?

Option 1: No change in current specifications, meaning that ΔPPowerClass = 3dB is sufficient.

Option 2: Change ΔPPowerClass = 3dB with the wordings that “for PC2 UE with txDiversity-r16 capability, all requirements for the default power class apply, and for PC1.5 UE, all requirements for PC2 apply.

Option 3: Other options are not precluded.

In our view, the power class fallback does occur during SRS antenna switching for PC2 capable UE with txDiversity-r16 capability or PC1.5 capable UE as during SRS antenna switching, only one of the two half-power transmitters can be physically connected to the antenna port to be switched to for transmitting SRS signal. ΔPPowerClass = 3dB in the current specifications may not fully represent the UE behavior as MPR/A-MPR should be based on the fallback power class instead of the reported power class. Therefore, we support Option 2 to change ΔPPowerClass = 3dB with the wordings that “for PC2 UE with txDiversity-r16 capability, all requirements for the default power class apply, and for PC1.5 UE, all requirements for PC2 apply.

Proposal 4: For Issue 2.2-3, choose Option 2 to change ΔPPowerClass = 3dB with the wordings that “for PC2 UE with txDiversity-r16 capability, all requirements for the default power class apply, and for PC1.5 UE, all requirements for PC2 apply.

Issue 2.2-4: If RAN4 agrees to make specifications revisions to mitigate the power class fallback issues, which release should be the starting release for the changes?

Option 1: Rel-17

Option 2: Rel-18

Since the proposed specifications revisions do not impact any core requirement but an improvement in current RAN4 specifications to avoid the power class fallback issues which may cause potential misinterpretation of PCMAX requirement, in our view, making the changes starting from current release (Rel-18) of specifications would be sufficient.

Proposal 5: For Issue 2.2-4, choose Option 2 to make specifications revisions to mitigate the power class fallback issues starting from Rel-18.

## 2.3	Proposed specifications revisions

In this section, we provide the text proposals on how the current RAN4 specifications may be revised based on the proposals in section 2 to mitigate the power class fallback issues. However, the changes related to SRS antenna switching could be rather complicated and may deviate the attention on the originally concerned power class fallback issues, they will not be considered in the proposed specifications revisions in this meeting.

Table 2.3-1 summarizes the proposed changes for the impacted sub-clauses in current RAN4 specifications including TS 38.101-1, TS 38.101-3, and TS 36.101, where “text descriptions” means texts describing the UL duty cycle and P-Max conditions for defining the ΔPPowerClass values.

| TS 38.101-1 |  |  |
| --- | --- | --- |
| Feature | Impacted sub-clauses | Proposed changes |
| Single carrier | 6.2.1 | Remove text descriptions |
|  | 6.2.4 | 1. Change PCMAX_L formula 2. Remove P-Max in ΔPPowerClass definition |
| Intra-band contiguous CA | 6.2A.1.1 | Move text descriptions to 6.2A.4.1.1 |
|  | 6.2A.4.1.1 | 1. Add text descriptions to ΔPPowerClass,CA 2. Change PCMAX_L formula 3. Remove P-Max in ΔPPowerClass,CA definition |
| Intra-band non-contiguous CA | 6.2A.1.2 | Move text descriptions to 6.2A.4.1.2 |
|  | 6.2A.4.1.2 | 1. Add text descriptions to ΔPPowerClass,CA 2. Change PCMAX_L formula 3. Remove P-Max in ΔPPowerClass,CA definition |
| Inter-band CA | 6.2A.1.3 | Move text descriptions to 6.2A.4.1.3 |
|  | 6.2A.4.1.3 | 1. Add text descriptions to ΔPPowerClass,CA 2. Remove P-Max in ΔPPowerClass,CA definition |
| SUL | 6.2C.1 | FFS |
| Inter-band UL CA with UL MIMO | 6.2H.3.1 | Move text descriptions to 6.2H.3.4 |
|  | 6.2H.3.4 | 1. Add text descriptions to ΔPPowerClass,CA 2. Remove P-Max in ΔPPowerClass,CA definition |
| Inter-band UL CA with Tx diversity | 6.2L.3.1 | Move text descriptions to 6.2L.3.4 |
|  | 6.2L.3.4 | 1. Add text descriptions to ΔPPowerClass,CA 2. Remove P-Max in ΔPPowerClass,CA definition |
| TS 38.101-3 |  |  |
| Feature | Impacted sub-clauses | Proposed changes |
| Intra-band contiguous EN-DC | 6.2B.1.1 | Remove text descriptions |
|  | 6.2B.4.1.1 | 1. Change PCMAX_L formula 2. Remove P-Max in ΔPPowerClass,CA definition |
| Intra-band contiguous NE-DC | 6.2B.4.1.1a | 1. Change PCMAX_L formula 2. ΔPPowerClass,NE-DC = 0 dB for PC3 only |
| Intra-band non-contiguous EN-DC | 6.2B.1.2 | Remove text descriptions |
| Inter-band EN-DC within FR1 | 6.2B.1.3 | Move text descriptions to 6.2B.4.1.3 |
|  | 6.2B.4.1.3 | 1. Add text descriptions to ΔPPowerClass,EN-DC 2. Change PCMAX_L formula 3. Remove P-Max in ΔPPowerClass,CA definition |
| Inter-band NE-DC within FR1 | 6.2B.4.1.3a | 1. Change PCMAX_L formula 2. ΔPPowerClass,NE-DC = 0 dB for PC3 only |
| Inter-band EN-DC with UL MIMO | 6.2H.1.3 | Remove text descriptions |
| Inter-band EN-DC with Tx Diversity | 6.2L.1.3 | Remove text descriptions |
| TS 36.101 |  |  |
| Feature | Impacted sub-clauses | Proposed changes |
| Single carrier | 6.2.2 | Remove text descriptions posed changes |
|  | 6.2.5 | 1. Change PCMAX_L formula 2. Remove P-Max in ΔPPowerClass,CA definition for B14 and B41 |
| Intra-band contiguous CA | 6.2.2A | Remove text descriptions for B41 |
|  | 6.2.5A | 1. Change PCMAX_L formula 2. Remove P-Max in ΔPPowerClass,CA definition for B41 |

Table 2.3-1 Proposed changes in RAN4 specifications on mitigating power class fallback issues

The text proposal below illustrates how the UL duty cycle conditions can be moved to ΔPPowerClass,CA for NR intra-band contiguous CA.

- ΔPPowerClass,CA = 3 dB for a power class 2 capable UE if the field of UE capability maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50%, or if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1; otherwise ΔPPowerClass,CA = 0 dB;

The same text proposal above can also be applied to intra-band non-contiguous CA.

For inter-band UL CA, the following text proposal illustrates how the UL duty cycle conditions can be moved to ΔPPowerClass,CA.

-	ΔPPowerClass,CA = 3 dB for a power class 2 capable UE if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.306 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2; otherwise ΔPPowerClass,CA = 0 dB;

NOTE: 	The average percentage of uplink symbols is defined as 50%  ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.306.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent,

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3,

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

For inter-band UL CA with UL MIMO and inter-band UL CA with Tx diversity, the following text proposal illustrates how the UL duty cycle conditions can be moved to ΔPPowerClass,CA.

-	ΔPPowerClass,CA:

–	For a power class 2 capable UE, it is 3dB if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.306 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2; otherwise ΔPPowerClass,CA = 0 dB;

NOTE: 	The average percentage of uplink symbols is defined in sub-clause 6.2A.4.1.

–	For a power class 1.5 capable UE, it is 6dB if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2; and it is 3dB if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; otherwise ΔPPowerClass,CA = 0 dB;

NOTE: 	The average percentage of uplink symbols is defined in sub-clause 6.2A.4.1.3 except if power class of one band within the band combination is power class 1.5 and the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 are both absent, the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%; or if only one of the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported, the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability; or the corresponding maxDutyNR,x or maxDutyNR,y is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 and 0.5*maxUplinkDutyCycle-PC2-FR1.

For intra-band contiguous NE-DC, the following changes to ΔPPowerClass,EN-DC is applied as currently only PC3 has been specified for this configuration.

-	ΔPPowerClass,NE-DC = 0 dB for a power class 3 UE;

For inter-band EN-DC, the following text proposal illustrates how the UL duty cycle conditions can be moved to ΔPPowerClass,EN-DC.

-	∆PPowerClass,EN-DC = 3 dB for a power class 2 capable EN-DC UE with an E-UTRA TDD and NR TDD Inter-band EN-DC band combination if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30%, or if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16; otherwise ∆PPowerClass,EN-DC = 0 dB;

-	∆PPowerClass,EN-DC = 0 dB for a power class 2 capable EN-DC UE with an E-UTRA FDD and NR TDD Inter-band EN-DC band combination if UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 where

–	if the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1, or

–	if the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2;

otherwise ∆PPowerClass,EN-DC = 3 dB;

For inter-band NE-DC, the following changes to ΔPPowerClass,EN-DC is applied as currently only PC3 has been specified for this configuration.

-	ΔPPowerClass,NE-DC = 0 dB for a power class 3 UE;

Based on the above text proposals, we also prepare the corresponding CRs in this meeting for consideration [3-5].

# 3	Conclusion

In this contribution, we share our views on the preferred option for each open issue as captured in the way forward and provide text proposals on how the current RAN4 specifications may be revised to mitigate the power class fallback issues.

Proposal 1: Choose Option 2 for RAN4 specifications revisions to mitigate the power class fallback issue.

(Move the text descriptions on UL duty cycle and P-max conditions below the power class tables in “UE maximum output power” sub-clauses to ΔPPowerClass definitions in “Configured output power” sub-clauses)

Proposal 2: For Issue 2.2-1, choose Option 2 that P-Max should not be included in the ΔPPowerClass definition.

Proposal 3: For Issue 2.2-2, choose Option 1 to revise the PCMAX_L formula as proposed below for single carrier and apply the similar change to other UL features.

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc, ΔPPowerClass)}

Proposal 4: For Issue 2.2-3, choose Option 2 to change ΔPPowerClass = 3dB with the wordings that “for PC2 UE with txDiversity-r16 capability, all requirements for the default power class apply, and for PC1.5 UE, all requirements for PC2 apply.

Proposal 5: For Issue 2.2-4, choose Option 2 to make specifications revisions to mitigate the power class fallback issues starting from Rel-18.

# 4	Reference

R4-2400180 “On misconception of power class fallback”, Apple, 3GPP RAN WG4 Meeting #110, Athens, Greece, February 26th – March 1st, 2024

R4-2406587 “WF on power class fallback issues”, Apple, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 14th – 19th, 2024

R4-2407095 “CR to 38.101-1 on mitigating power class fallback issues”, Apple, 3GPP RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

R4-2407096 “CR to 38.101-3 on mitigating power class fallback issues”, Apple, 3GPP RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

R4-2407097 “CR to 36.101 on mitigating power class fallback issues”, Apple, 3GPP RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024