3GPP TSG-RAN WG4 Meeting #111	R4-2407112

Fukuoka City, JP, May 20-24, 2024

Source:	China Telecom

Title:	Discussion on UE capability aspects for the advanced receiver for MU-MIMO

Agenda Item:	7.11.2.1

Document for:	Discussion

- Introduction

In RAN4 #110 meeting, the following UE capability definition for advanced receiver for MU-MIMO is agreed.

The following feature has been captured in the R18 UE feature list LS to RAN2: Index; Feature group; Components; Prerequisite feature groups; Mandatory/Optional. 36-1; MU-MIMO Interference Mitigation advanced receiver; R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when co-scheduled UE(s)’ modulation order is signaled; 3-4; …; Optional with capability signaling. 36-2a; MU-MIMO Interference Mitigation advanced receiver with modulation order detection; R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and 4RX] when co-scheduled UE(s)’ modulation order is not signaled; 36-1; …; Optional without capability signaling. 36-2b; MU-MIMO Interference Mitigation advanced receiver with modulation order detection; R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX] when co-scheduled UE(s)’ modulation order is not signaled; 36-1; …; Optional without capability signaling.

In addition, in the RAN4#110bis meeting, the following updates to the capability definition is agreed in [1].

- For 36-1, update the note in capability granularity column as below: - UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA, NE-DC, EN-DC and NR-DC operation - Update the ‘Components’ column as below: - For 36-1: - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when co-scheduled UE(s)’ modulation order is explicitly signalled by DCI index 1-5 in Table 7.3.1.2.2-12 of TS38.212. - For 36-2a: - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and 4RX] when co-scheduled UE(s)’ modulation order is not signalled DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled. - For 36-2b: - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX] when co-scheduled UE(s)’ modulation order is not signalled DCI index 6 in Table 7.3.1.2.2-12 of TS38.212 is signalled.

Same time, there are open issues in [1] that requires further discussion. In this paper, our view on the remaining issues for the capability definition for advanced receiver for MU-MIMO is given.

- Discussion

Capability definition for UE supporting modulation order blind detection

- Proposals on 36-2a and 36-2b: - Proposal 1: Combine 36-2a and 36-2b and remove number of layer descriptions if RAN4 agrees to not define 2+2 test under DCI 6 - Proposal 2: Remove FG 36-2b and keep 36-2a from UE feature list

It is now open in [1] whether rank 2+2 test requirements will be introduced for UE supporting modulation order blind detection. If both rank 1+1 and 2+2 requirements are introduced, it is straightforward what both 36-2a and 36-2b should be introduced.

Even if there will be no test requirements for rank 2+2 with modulation order blind detection, in our understanding, it is still reasonable to keep both 36-2a and 36-2b in the UE capability definition since there are UE vendors supporting rank 2+2 during the last meeting discussion.

Proposal 1: Keep both 36-2a and 36-2b with removing the [] in the UE capability definition regardless of whether rank 2+2 test will be introduced for UE supporting modulation order blind detection.

- Conclusion

Proposal 1: Keep both 36-2a and 36-2b with removing the [] in the UE capability definition regardless of whether rank 2+2 test will be introduced for UE supporting modulation order blind detection.

- References

- [1] R4-2406114, WF on the advanced receiver for MU-MIMO scenario, China Telecom