3GPP TSG-RAN WG4 Meeting #111				R4-2407114

Fukuoka City, JP, May 20-24, 2024

Source:	China Telecom

Title:	Discussion on test parameters for the advanced receiver for MU-MIMO: Simulation results

Agenda Item:	7.11.2.2

Document for:	Discussion

- Introduction

In RAN4 #111 meeting we discussed the Phase II test parameters for advanced receiver to cancel inter-user interference for MU-MIMO within the NR_demod_enh3-Perf WI. As an outcome the WF is approved in [1].

In this paper, phase II simulation results are given. Note that simulation results for both R-ML, E-IRC and baseline MMSE-IRC is given.

- Discussion

Table 1. Ideal simulation results for advanced receiver for MU-MIMO (with 1 co-UE)

| Case Number (New) | Case Number (110bis) | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | DCI Index | SNR @ 70% max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | R-ML | IRC | Gain over baseline |
| 1 | 2 | FDD 15KHz SCS | 10 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Medium | MCS 13 | QPSK | Index 1/2 | 16.0 |  |  |
| 2 | 4 |  |  |  | 2T4R |  |  |  |  |  |  | 13.9 |  |  |
| 3 | 7 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 15.6 |  |  |
| 4 | 11 | TDD 30KHz | 40 MHz | 1+1 | 2T2R |  | TDLC300-100 | ULA Medium | MCS 13 | QPSK |  | 18.4 |  |  |
| 5 | 13 |  |  |  | 2T4R |  |  |  |  |  |  | 15.4 |  |  |
| 6 | 16 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 15.6 |  |  |
| 7 | 20 | FDD 15KHz SCS | 10MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | Index 6 | 16.2 | 19.5 | 3.2 |
| 8 | 23 |  |  |  | 2T4R |  |  |  |  |  |  | 14.1 | 20.3 | 6.2 |
| 9 | 26 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM |  | 19.4 | 25.5 | 6.1 |
| 10 | 29 |  |  |  | 2T4R |  |  |  |  |  |  | 18.3 | 25.9 | 7.6 |
| 11 | 20a |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK |  | 10.2 | 12.4 | 2.2 |
| 12 | 23a |  |  |  | 2T4R |  |  |  |  |  |  | 5.2 | 6.4 | 1.2 |
| 13 | 31 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 10.6 | 13.5 | 2.9 |
| 14 | 32 |  |  |  |  |  |  | XP medium |  |  |  | 12.0 | 15.5 | 3.5 |
| 15 | 34 | TDD 30KHz SCS | 40 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK |  | 18.0 |  |  |
| 16 | 37 |  |  |  | 2T4R |  |  |  |  |  |  | 15.4 |  |  |
| 17 | 40 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM |  | 22.2 |  |  |
| 18 | 43 |  |  |  | 2T4R |  |  |  |  |  |  | 21.9 |  |  |
| 19 | 34a |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK |  | 10.9 |  |  |
| 20 | 37a |  |  |  | 2T4R |  |  |  |  |  |  | 5.7 |  |  |
| 21 | 45 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 10.6 |  |  |
| 22 | 46 |  |  |  |  |  |  | XP medium |  |  |  | 12.2 |  |  |

Table 2. Impairment simulation results for advanced receiver for MU-MIMO (with 1 co-UE)

| Case Number (New) | Case Number (110bis) | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | DCI Index | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | FDD 15KHz SCS | 10 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Medium | MCS 13 | QPSK | Index 1/2 | 18.0 |
| 2 | 4 |  |  |  | 2T4R |  |  |  |  |  |  | 15.9 |
| 3 | 7 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 17.6 |
| 4 | 11 | TDD 30KHz | 40 MHz | 1+1 | 2T2R |  | TDLC300-100 | ULA Medium | MCS 13 | QPSK |  | 20.4 |
| 5 | 13 |  |  |  | 2T4R |  |  |  |  |  |  | 17.4 |
| 6 | 16 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 17.6 |
| 7 | 20 | FDD 15 KHz SCS | 10MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | Index 6 | 18.2 |
| 8 | 23 |  |  |  | 2T4R |  |  |  |  |  |  | 16.1 |
| 9 | 26 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM |  | 21.4 |
| 10 | 29 |  |  |  | 2T4R |  |  |  |  |  |  | 20.3 |
| 11 | 20a |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK |  | 12.2 |
| 12 | 23a |  |  |  | 2T4R |  |  |  |  |  |  | 7.2 |
| 13 | 31 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 12.6 |
| 14 | 32 |  |  |  |  |  |  | XP medium |  |  |  | 14.0 |
| 15 | 34 | TDD 30KHz SCS | 40 MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK |  | 20.0 |
| 16 | 37 |  |  |  | 2T4R |  |  |  |  |  |  | 17.4 |
| 17 | 40 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM |  | 24.2 |
| 18 | 43 |  |  |  | 2T4R |  |  |  |  |  |  | 23.9 |
| 19 | 34a |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK |  | 12.9 |
| 20 | 37a |  |  |  | 2T4R |  |  |  |  |  |  | 7.7 |
| 21 | 45 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 12.6 |
| 22 | 46 |  |  |  |  |  |  | XP medium |  |  |  | 14.2 |

- Conclusion

This contribution provides phase II simulation results for advanced receiver for MU-MIMO.

- References

- [1] R4-2406114, WF on the advanced receiver for MU-MIMO scenario, China Telecom