3GPP TSG-RAN WG4 Meeting #111	R4-2407144

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	Discussion on NR NTN UE Demodulation

Agenda item:	7.16.8.2

Document for:	Discussion

Introduction

The introduction of NR NTN enhancements for Rel-18 has been outlined in the WID [1], with an aim to add functionality to increase features that were introduced in Rel 15, 16 and 17.

RAN4 Demodulation will specify the performance of these new enhancements.

The main objectives from the WID [1] are as follows:

- Specify RRM performance requirements and test cases [RAN4] - Specify UE demodulation and CSI reporting requirements [RAN4] - Specify satellite access node demodulation requirements [RAN4] - Specify satellite access node conformance tests [RAN4]

For the following enhancements

4.1.1 Coverage enhancement 4.1.2 NR-NTN deployment in above 10 GHz bands 4.1.3 Network verified UE location 4.1.4 NTN-TN and NTN-NTN mobility and service continuity enhancements

In this paper, we present Nokia’s view on the impact to UE demodulation requirements.

Discussion

Power Class

Whilst generating the draftCR for 38.101-5, it was noted that whilst in the corresponding TN specification, there was a specific need to highlight the power class of UEs for FR2 operations.

Unless otherwise stated, all minimum performance requirements defined in Clauses 8 are applicable to UE power class 3 only.

Nokia does not believe that this is necessary for inclusion in TS 38.101-5, however would like to clarify.

RAN4 shall not include power class explicitly in NTN-FR2 radiated requirements as this is implicitly implied

This is the approach in the current draftCR submitted under R4-2407145.

Clarification on TS 38.101-5 structure for radiated requirements

In developing the draftCR for 38.101-5 on Radiated Requirements, Nokia noticed a discrepancy in the structure between TS 38.101-4 and TS 38.101-5, although minor we would like to raise to RAN4 for clarification before submission of the bigCR during RAN4#111.

At the moment the worksplit identifies that clause 11.1 and 11.2 act as the General introduction to the radiated requirements, and this aligns with clause 8 of TS 38.101-5, but not with the radiated requirements of TS 38.101-4. Where in TS 38.101--4 the General section is all contained within clause 11.1.

The draftCR submitted before RAN4#111, R4-2407145 follows the structure of the conducted requirements of TS 38.101-4.

RAN4 to discuss the structure of the general sections of the radiated requirements of TS 38.101-5 before submission of the bigCR.

FRC Alignment with TN Specs

During the development of the draft CRs it was noted that the FRC numbering for different MCS is not aligned with that of TS 38.101-4.

The FRC annex numbering on NTN specifications does not align with those of the TN specifications.

We note this may cause confusion with TE vendor equipment and other testing functionality.

Furthermore, and the tables were introduced for only one specific case, Nokia would like to clarify whether within each sub-annex whether an attempt should be made to align FRCs with the relevant TN specification. This includes adding void clauses and tables potentially, as is seen in TS 38.101-4.

RAN4 to discuss whether TN and NTN FRCs should aim to be aligned.

Conclusion

This contribution provides our views on SAN demodulation requirements for Rel-18 NR NTN. Specifically, a series of observation and proposals are made as follows:

Proposal 1: RAN4 shall not include power class explicitly in NTN-FR2 radiated requirements as this is implicitly implied

Proposal 2: RAN4 to discuss the structure of the general sections of the radiated requirements of TS 38.101-5 before submission of the bigCR.

Observation 1: The FRC annex numbering on NTN specifications does not align with those of the TN specifications.

Proposal 3: RAN4 to discuss whether TN and NTN FRCs should aim to be aligned.

References

- RP-230809 “Revised WID on NR NTN (Non-Terrestrial Networks) enhancements”

- TS 38.101-5 v18.2.0. Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements. June 2023

- R4-2402865 “WF on NR_NTN_enh_SAN_UE_demod”, Moderator (Huawei, HiSilicon), RAN4#110, Athens, February, 2024