3GPP TSG-RAN WG4 Meeting # 111									          R4-2407159

# Fukuoka, Japan, 20 – 24 May 2024

# Title:	PC2 CA_n25-n41 MSD

# Agenda Item:	6.18.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# 1	Introduction

This contribution proposes power class 2 (PC2) UL4/DL3 Rx harmonic mixing MSD requirements for CA_n25A-n41A.

# Discussion

There is no PC3 MSD test point for CA_n25-n41. In this contribution, the 1Tx PC3 interference level is re-evaluated from the agreed Band n41 10MHz CBW PC3 MSD of 8.1dB for CA_n39-n41, where text proposal [1] assumed a 4.7dB lower total interference power affecting the diversity antenna than the primary antenna port.

For CA_n25-n41, we assume:

- 15dB lower interference levels due to better rejection of the Band n25 filter than the Band n39 filter at the Band n41 frequency range.

- A PC2 1Tx harmonic interference level 2.9dB greater than the PC3 interference level based on [2].

- 0.5dB reverse IMD is assumed for PC2 dual Tx MSD analysis.

- or consistency with [1], we evaluate the PC3, PC2 single Tx, and PC2 dual Tx MSD using an uncorrelated MRC combining and the results are captured in Figure 1.

Figure 1: Band n41 10MHz PC3, PC2 1Tx, PC2 Tx MSD analysis for CA_n25-n41.

Proposal: For Band n41 10MHz CBW UL3/DL4 Rx harmonic mixing MSD in CA_n25-n41, consider:

- Introducing 0.8dB PC3 MSD test point;

- Adopting the following PC2 test points.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD3 | MSD4 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 25 (RBstart=0) | 10 | 1.5 | 1.8 | NOTE 11 | UL4/DL3 |
| NOTE 3: Applicable to UE supporting PC2 with single Tx. NOTE 4: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |  |

# Conclusion

This contribution evaluates the PC2 MSD requirements for CA_n25A-n41A with the following proposal.

Proposal: For Band n41 10MHz CBW UL3/DL4 Rx harmonic mixing MSD in CA_n25-n41, consider:

- Introducing 0.8dB PC3 MSD test point;

- Adopting the following PC2 test points.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD3 | MSD4 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 25 (RBstart=0) | 10 | 1.5 | 1.8 | NOTE 11 | UL4/DL3 |
| NOTE 3: Applicable to UE supporting PC2 with single Tx. NOTE 4: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |  |

# References

[1] 	R4-2306550 TP for TR 38.894: Harmonic mixing MSD for simultaneous RxTx CA_n39-n41, 3GPP TSG-RAN WG4 Meeting #106bis-e, electronic meeting, ZTE Corporation, Huawei, Hisilicon, Skyworks Solutions, Inc.

[2] 	R4-2407156 Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting # 111, Fukuoka, Japan, Skyworks Solutions, Inc.