3GPP TSG-RAN WG4 Meeting # 111									          R4-2407163

# Fukuoka, Japan, 20 – 24 May 2024

# Title:	PC2 n28 NS_17

# Agenda Item:	6.19.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# 1	Introduction

This contribution is a follow up of [1] where we present the missing PC2 single TX power amplifier (PA) back-off required to meet the NS_17 requirements.

# Discussion

## 2.1 Power Amplifier Calibration

- Local Oscillator (LO) leakage: -28 dBc, IQ Image rejection: -28 dB, C-IM3 -60dBc, C-IM5 -70dBc.

- Power Class 3 (PC3) and PC2 operation, and 4dB post PA insertion losses.

- The PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB, at the lower channel edge, with 1 dB MPR:

  - PC3 to meet ACLR -30 dBc

  - PC2 to meet ACLR -31 dBc

- Waveforms: DFT-s-OFDM QPSK, 1RB edge and full RB allocation for 3, 5, and 10MHz channel bandwidth (CBW)

## 2.2 Measurement Results

The measured PA output back-off (OBO) is summarized in Table 2 for PC3, PC3+PC3 and 1xPC2. When no back-off is needed, we present (in brackets “[ ]”) the margin, relative to the NS_17 emission level requirements at MPR 0.

Table 2: Measured DFT-s-OFDM QPSK OBO for n28 NS_17.

| OBO/[Margin @MPR0] | PC3 | PC3+PC3 | PC2 |
| --- | --- | --- | --- |
| 3MHz_1RB Edge | [13.8] | [5.3] | n/a |
| 3MHz Full allocation | [3.4] | 1 | n/a |
| 5MHz 1RB Edge | [11.9] | [6.3] | [11.8] |
| 5M Full allocation | [3.9] | 1 | [2] |
| 10MHz 1RB Edge | ~3 | 4.5 | ~2.5 |
| 10M Full allocation | 3.5 | ~4.5 | 2.2 |

Observation: Measurements indicate the following PA back-off to meet the NS_17 emission requirements:

- For PC3: 3.5dB;

- For PC2 single Tx: 2.5dB;

- For PC2 dual-tx: 4.5dB.

Considering the long debates and off-line discussions regarding how to handle the PC3 and/or the PC2 A-MPR for UE supporting the full-band duplexers, we suggest postponing these studies to Release 19 as part of new spectrum effort for both Band 28 and Band n28.

Proposal: Complete the Band 28/n28 PC2 studies in Release 19 as part a new work item to enable the support for full-band duplexer architectures.

# Conclusion

This contribution presents PA back-off results to meet the NS_17 requirements assuming a full-band duplexer architecture.

Observation: Measurements indicate the following PA back-off to meet the NS_17 emission requirements:

- For PC3: 3.5dB;

- For PC2 single Tx: 2.5dB;

- For PC2 dual-tx: 4.5dB.

Proposal: Complete the Band 28/n28 PC2 studies in Release 19 as part a new work item to enable the support for full-band duplexer architectures.

# References

[1] 	R4-2405962 n28 NS_17 PC2, 3GPP TSG-RAN WG4 Meeting # 110-bis, Changsha, China, Skyworks Solutions, Inc.