3GPP TSG-RAN WG4 Meeting # 111									          R4-2407166

# Fukuoka, Japan, 20 – 24 May, 2024

# Title:	PC2 CA_n2-n66 CA_n25-n66 MSD

# Agenda Item:	6.18.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# 1	Introduction

This contribution proposes power class 3 (PC3) and power class 2 (PC2) cross-band MSD requirements for CA_n2A-n66A and CA_n25A-n66A.

# Discussion

Table 1 summarizes the source of cross-band interference affecting the downlink (DL) bands for the specified bandwidth combination sets (BCS) throughout TS 38.101-1 releases.

Table 1: CA_n2-n66, CA_n25-n66 source of cross-band interference vs releases.

| Band combination | UL band | DL band | max UL BW | UL RB Allocation | min DL BW | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | LCRB | (MHz) |  |
| CA_n2-n66 (all Releases) | n66 | n2 | 40 | 216 | 5 | ACLR ~ 4 |
|  | n2 | n66 | 20 | 50 | 5 | ACLR ~ 10 |
| CA_n25-n66 (Rel-18, Rel-17) | n66 | n25 | 45 | 240 | 5 | ACLR ~ 4 |
| CA_n25-n66 (Rel-16, PC3 only) | n66 | n25 | 40 | 216 | 5 | ACLR ~ 4 |
| CA_n25-n66 (all Releases) | n25 | n66 | 40 | 40 | 5 | ACLR ~ 5 |

No test points are specified for PC3. Table 1 indicates that some DL bands may be affected by ACLR ~4 to ~5. Therefore, we propose to evaluate the maximum sensitivity degreadation (MSD) for both PC3 and PC2 using power amplifier (PA) measurement data.

## 2.1 Measurement Results & MSD analysis

## 2.1.1 Power Amplifier Calibration & Front-end assumptions

For PA measurements, we assume:

- Local Oscillator (LO) leakage: -28 dBc, IQ Image rejection: -28 dB, C-IM3 -60 dBc, C-IM5 -70 dBc.

- PC3 operation and 4 dB post PA insertion losses.

- The PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR and:

  - ACLR -30 dBc for PC3 and transmit diversity (TxD) operation for each PA.

  - ACLR -31 dBc for PC2 operation.

- PA measurements are performed using DFT-s-QPSK, SCS 15kHz waveforms at MPR0.

- A band-pass filter is inserted at the PA input port.

For MSD analysis, we assume use of a quadplexer with similar isolation performance, as compared to that used for CA_n1-n3, i.e.:

- Tx-Rx quadplexer isolation: 50 dB;

- Tx-antenna duplex rejection in Rx range: 45 dB;

- Primary antenna to diversity antenna isolation: 10 dB;

- MRC combining and -1 dB Signal-to-Noise Ratio (SNR); and

- Flat-band noise power spectral density (PSD) at PA output due to transceiver input noise: -130 dBm/Hz.

## 2.1.2 Measured Noise Levels

Table 2 summarizes the measured interference levels affecting each DL band. Measurements show that Band n66 is affected by the flat-band noise PSD, even for the case of UL Band n25 and 40 MHz channel bandwidth (CBW). In Table 2, we report the sum of the measured PA noise and the flat-noise PSD contribution.

Table 2:  PC3, PC2 single-Tx and PC2 dual-Tx (PC3+PC3) measured noise levels.

| UL band: UL CBW / UL carrier (MHz) | DL Band: DL CBW / DL carrier (MHz) | UL RB allocation | PC3 [dBm/MBW] | PC2 single Tx [dBm/MBW] | PC2 dual Tx PA1 [dBm/MBW] | PC2 dual Tx PA2 [dBm/MBW] |
| --- | --- | --- | --- | --- | --- | --- |
| n2: 20 / 1900 | n66: 5 / 2112.5 | 50 (RBstart=56) | -60.7 | -58.8 | -60.3 | -60.2 |
| n25: 40 / 1895 | n66: 5 / 2112.5 | 40 (RBstart=176) | -60.7 | -58.8 | -60.3 | -60.2 |
| n66: 40 / 1760 | n2, n25: 5 / 1932.5 | 216 (RBstart=0) | -53.3 | -50.5 | -52.9 | -52.6 |
| n66: 45 / 1760 | n25: 5 / 1932.5 | 240 (RBstart=2) | -51.1 | -48.3 | -50.6 | -50.4 |

## 2.1.3 PC3, PC2 MSD

The detailed MSD analyses can be found in the Annex.

The PC3 MSD results are summarized in Table 3:

- Band n66 MSD is insignificant.

- Introduction of test points for Band n25 and Band n2 may be considered.

Table 3: PC3 MSD results.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.1 | >ACLR2 |
| n66 | n25 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 0.7 | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.0 | >ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | ≤0.2 | >ACLR2 |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | ≤0.2 | >ACLR2 |

The PC2 single Tx MSD results are summarized in Table 4.

- Band n66 MSD: questionable if a test point is needed. Our preference is to not introduce this test point.

- Introduction of test points for Band n25 and Band n2 may be considered.

Table 4: PC2 single Tx MSD results.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.9 | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.7 | >ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0.5 | >ACLR2 |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.5 | >ACLR2 |

The PC2 dual Tx MSD results are summarized in Table 5.

- Band n66 MSD may be considered at 0.7dB,

- Introduction of test points for Band n25 and Band n2 may be considered.

Table 5: PC2 dual Tx MSD results.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 2.8 | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 2.5 | >ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0.7 | >ACLR2 |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.7 | >ACLR2 |

# Conclusion

This contribution evaluates PC3 and PC2 MSD requirements for CA_n2A-n66A and CA_n25A-n66A, and present the following proposal.

Proposal: Consider introducing the PC3, PC2 single Tx and PC2 dual Tx test points of Table 6, Table 7, Table 8 respectively.

Table 6: PC3 MSD results.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.1 | >ACLR2 |
| n66 | n25 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 0.7 | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.0 | >ACLR2 |

Table 7: PC2 single Tx MSD results.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.9 | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.7 | >ACLR2 |

Table 8: PC2 dual Tx MSD results.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 2.8 | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 2.5 | >ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0.7 | >ACLR2 |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.7 | >ACLR2 |

# References

# Annex

CA_n2-n66: Band n2 5MHz CBW MSD analysis for n66 40 MHz UL CBW Lcrb=216 (RBstart=0).

CA_n25-n66: Band n25 5 MHz CBW MSD analysis for n66 40 MHz UL CBW Lcrb=216 (RBstart=0).

CA_n25-n66: Band n25 5MHz CBW MSD analysis for n66 45 MHz UL CBW Lcrb=240 (RBstart=2).

CA_n25-n66: Band n66 5 MHz CBW MSD analysis for n25 40 MHz UL CBW Lcrb=40 (RBstart=176) and CA_n2-n66: Band n66 5 MHz CBW MSD analysis for n2 20 MHz UL CBW Lcrb=50 (RBstart=56).